| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0611-v1 |
| --- | --- |

| Title: | Subjective assessment of CE11 proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vittorio Baroncini Andrey Norkin Anand Meher Kotra | Tel: Email: | baroncini@gmx.com anorkin@netflix.com anand.meher.kotra@huawei.com |
| Source: | JVET test chair and CE11 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a report of the subjective test for the proposals in Core Experiment 11 on deblocking filtering. The test was performed before the Macao meeting according with the CE11 description document JVET-K1031[2].

# Introduction

During the Ljubljana meeting an expert viewing to evaluate the submissions of the participants to the CE11 was done using a direct comparison video quality assessment strategy.

Three sequences (FoodMarket, CampFire and Tango) were coded using the “anchor” and the encoders of the Proponents participating to the CE11 project.

Each coded video was compared with the anchor; the order of presentation of anchor and coded videos was randomly changed.

A video (preceded by the latter A) was shown on the screen, followed a second video (preceded by the letter B); then the message “Vote N” was presented asking to the viewers to rate if “A was better than B”, “B was better than A” or “A was equal to B”.

The results of the scoring process were processed computing how many times a Proponent was better (or worst) than the anchor.

The graphs here below provide visual representation of the results of the AB comparison evaluation.  Results (grouped by test sequence) showed a rough idea of the performances of the Proposals.

The test was done in the viewing context more similar to a home environment (six people seated in a row in front of a TV set) other than a laboratory designed to perform a formal subjective assessment, and with the participation of video coding experts. Also the TV set was a standard LED TV of mid-low quality.

Pearson correlation of each viewer scores against the MOS values was also considered; this showed rather poor results for many cases. This led to consider the possibility of making a second subjective evaluation experiment using standard test method and a test laboratory.

This led to exclude around 50% of the viewers after the statistical post screening analysis based on Pearson correlation index.

# The formal subjective assessment

Considering the limited outcome of the expert viewing performed during the Ljubljana meeting, the Test Chair Vittorio Baroncini offered to perform a formal subjective assessment experiment at EVATech Labs, which was agreed.

The only condition (due to time and resources limitations) was to consider a limited number of test points (in the range from 80 to 120).

Twenty-six Proponents from the CE11 [3] participated by encoding three UHD test sequences (FoodMarket, CampFire, ParkRunning) and two HD test sequences (Kimono, Kristen&Sara).

| Anchor_ALF | CE11_2_1_S1_NoALF |
| --- | --- |
| Anchor_NoALF | CE11_2_1_S2_NoALF |
| CE11_1_1_NoALF | CE11_2_1_S3_NoALF |
| CE11_1_10_NoALF | CE11_2_1_S4_NoALF |
| CE11_1_11_NoALF | CE11_2_1_S5_NoALF |
| CE11_1_2_NoALF | CE11_2_1_S6_NoALF |
| CE11_1_3_NoALF | CE11_2_2_S1_NoALF |
| CE11_1_4_NoALF | CE11_2_2_S2_NoALF |
| CE11_1_5_NoALF | CE11_3_1_NoALF |
| CE11_1_6_NoALF | CE11_3_2_NoALF |
| CE11_1_7_S1_NoALF | CE11_3_3_S1_NoALF |
| CE11_1_7_S2_NoALF | CE11_3_3_S2_NoALF |
| CE11_1_8_NoALF | CE11_3_4_NoALF |
| CE11_1_9_NoALF | CE11_3_5_NoALF |

The test sequences were encoded also using 4 Anchors, producing a total of 150 test points.

The subjective assessments were done sitting fours viewers in front of a 55” TV sets (LG OLED B6) at a distance of 2H. A total of 15 viewers, all screened for viewing acuity colour blindness, participated to the test (8 females and 7 males), all aging around 19 years.

## Test material and coding conditions of the subjective test

The following test sequences and conditions have been used in the subjective test. The sequence length is 10 seconds. The following sequences and QP have been used.

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | FoodMarket | 3840x2160p | 60fps | 600 | RA | 39 |
| 02 | Park Running | 3840x2160p | 50fps | 500 | RA | 39 |
| 03 | Campfire | 3840x2160p | 30fps | 300 | RA | 39 |
| 04 | Kimono | 1920x1080p | 24fps | 240 | LDB | 34 |
| 05 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 34 |

The test conditions for the subjective test were CTC (VTM-2.0.1) except ALF switched off and the base QP used as in the table above.

Two anchors have been used: Anchor 1 with ALF switched on and Anchor-2 with ALF switched off.

## Subjective test design

The formal subjective assessment for CE11 was designed using the DSIS test method comparing the original non compressed video clips with the encoded ones.

The 150 test points were ordered randomly and divided in five test sessions; each test session included three “stabilization test cells” followed by 30 evaluation test cells.

The order of presentation of the video clips inside each test session was randomly designed taking care not to show the same sequence two consecutive times.

## Results of subjective test

The results are reported in the tables and the graphs here below.

| Campfire |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proponent | MOS | DS | CI |  | Proponent | MOS | DS | CI |
| CE11_3_2 | 3.93 | 0.88 | 0.45 |  | CE11_3_3_S2 | 4.53 | 0.74 | 0.33 |
| CE11_1_11 | 4.00 | 1.00 | 0.51 |  | CE11_2_1_S4 | 4.67 | 0.98 | 0.49 |
| CE11_2_1_S3 | 4.00 | 0.65 | 0.33 |  | CE11_3_1 | 4.67 | 0.82 | 0.41 |
| Anchor NoALF | 4.20 | 0.77 | 0.39 |  | CE11_2_1_S5 | 4.80 | 0.86 | 0.44 |
| CE11_1_4 | 4.27 | 1.03 | 0.45 |  | CE11_2_1_S6 | 4.87 | 1.06 | 0.54 |
| CE11_1_3 | 4.33 | 1.05 | 0.53 |  | CE11_3_3_S1 | 4.93 | 1.03 | 0.45 |
| CE11_1_7_S1 | 4.33 | 0.49 | 0.21 |  | CE11_1_1 | 5.00 | 1.00 | 0.51 |
| CE11_2_1_S1 | 4.40 | 0.74 | 0.37 |  | CE11_1_9 | 5.00 | 0.93 | 0.47 |
| CE11_1_7_S2 | 4.40 | 1.06 | 0.46 |  | CE11_1_8 | 5.07 | 0.96 | 0.49 |
| CE11_2_1_S2 | 4.47 | 0.64 | 0.32 |  | CE11_1_10 | 5.07 | 1.16 | 0.59 |
| CE11_1_5 | 4.47 | 0.52 | 0.26 |  | CE11_2_2_S2 | 5.13 | 0.52 | 0.23 |
| CE11_1_6 | 4.47 | 0.74 | 0.33 |  | CE11_1_2 | 5.60 | 0.83 | 0.42 |
| CE11_2_2_S1 | 4.47 | 0.74 | 0.33 |  | CE11_3_4 | 6.00 | 1.00 | 0.51 |
| Anchor ALF | 4.53 | 1.06 | 0.54 |  | CE11_3_5 | 6.00 | 0.53 | 0.23 |

| FoodMarket |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proponent | MOS | DS | CI |  | Proponent | MOS | DS | CI |
| CE11_3_3_S1 | 3.93 | 0.70 | 0.31 |  | CE11_2_1_S3 | 6.93 | 0.70 | 0.36 |
| CE11_1_7_S2 | 4.67 | 0.82 | 0.36 |  | CE11_1_8 | 7.13 | 1.13 | 0.57 |
| Anchor ALF | 5.93 | 0.80 | 0.40 |  | CE11_2_1_S1 | 7.13 | 1.13 | 0.57 |
| CE11_3_4 | 6.07 | 0.96 | 0.49 |  | CE11_3_2 | 7.13 | 1.06 | 0.54 |
| CE11_2_1_S4 | 6.20 | 0.94 | 0.48 |  | CE11_1_10 | 7.20 | 0.94 | 0.48 |
| CE11_1_11 | 6.27 | 0.80 | 0.40 |  | CE11_3_3_S2 | 7.33 | 0.72 | 0.32 |
| Anchor NoALF | 6.60 | 0.99 | 0.50 |  | CE11_1_9 | 7.47 | 0.74 | 0.38 |
| CE11_3_1 | 6.60 | 0.83 | 0.42 |  | CE11_1_5 | 7.60 | 0.63 | 0.32 |
| CE11_1_2 | 6.67 | 0.72 | 0.37 |  | CE11_1_6 | 7.73 | 0.46 | 0.20 |
| CE11_2_1_S2 | 6.73 | 0.96 | 0.49 |  | CE11_2_2_S1 | 7.93 | 0.26 | 0.11 |
| CE11_1_1 | 6.73 | 0.70 | 0.36 |  | CE11_2_2_S2 | 7.93 | 0.46 | 0.20 |
| CE11_2_1_S6 | 6.80 | 0.77 | 0.39 |  | CE11_1_4 | 8.00 | 0.53 | 0.23 |
| CE11_2_1_S5 | 6.87 | 0.92 | 0.46 |  | CE11_1_7_S1 | 8.07 | 0.70 | 0.31 |
| CE11_1_3 | 6.93 | 0.80 | 0.40 |  | CE11_3_5 | 8.27 | 0.59 | 0.26 |

| ParkRunning |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proponent | MOS | DS | CI |  | Proponent | MOS | DS | CI |
| CE11_3_3_S2 | 4.33 | 1.05 | 0.46 |  | Anchor NoALF | 7.07 | 0.96 | 0.49 |
| CE11_2_2_S1 | 4.53 | 0.83 | 0.37 |  | CE11_1_2 | 7.07 | 0.88 | 0.45 |
| CE11_2_1_S6 | 6.40 | 0.83 | 0.42 |  | CE11_1_1 | 7.13 | 0.99 | 0.50 |
| Anchor ALF | 6.60 | 1.06 | 0.53 |  | CE11_3_2 | 7.20 | 0.68 | 0.34 |
| CE11_1_3 | 6.73 | 0.59 | 0.30 |  | CE11_3_1 | 7.20 | 0.86 | 0.44 |
| CE11_2_1_S5 | 6.73 | 0.88 | 0.45 |  | CE11_3_4 | 7.27 | 0.88 | 0.45 |
| CE11_1_5 | 6.73 | 0.96 | 0.49 |  | CE11_2_1_S1 | 7.33 | 0.72 | 0.37 |
| CE11_1_11 | 6.80 | 1.01 | 0.51 |  | CE11_3_5 | 7.33 | 0.62 | 0.27 |
| CE11_1_9 | 6.93 | 1.16 | 0.59 |  | CE11_1_7_S1 | 7.47 | 0.83 | 0.37 |
| CE11_2_1_S2 | 6.93 | 0.80 | 0.40 |  | CE11_3_3_S1 | 7.53 | 1.06 | 0.46 |
| CE11_2_1_S3 | 7.00 | 1.25 | 0.63 |  | CE11_1_7_S2 | 7.60 | 0.63 | 0.28 |
| CE11_1_8 | 7.00 | 1.07 | 0.54 |  | CE11_1_10 | 7.67 | 0.90 | 0.46 |
| CE11_2_1_S4 | 7.00 | 1.00 | 0.51 |  | CE11_1_6 | 7.87 | 0.64 | 0.28 |
| CE11_1_4 | 7.00 | 0.85 | 0.37 |  | CE11_2_2_S2 | 8.27 | 0.59 | 0.26 |

| Kimono |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proponent | MOS | DS | CI |  | Proponent | MOS | DS | CI |
| CE11_2_1_S3 | 3.27 | 0.70 | 0.36 |  | CE11_2_1_S2 | 4.20 | 1.01 | 0.51 |
| CE11_1_2 | 3.53 | 0.99 | 0.50 |  | CE11_1_5 | 4.20 | 1.01 | 0.51 |
| CE11_3_2 | 3.73 | 0.88 | 0.45 |  | CE11_1_3 | 4.20 | 1.15 | 0.58 |
| Anchor ALF | 3.80 | 0.77 | 0.39 |  | CE11_2_2_S2 | 4.20 | 1.08 | 0.47 |
| CE11_3_4 | 3.80 | 0.94 | 0.48 |  | CE11_3_3_S2 | 4.27 | 0.88 | 0.39 |
| CE11_3_1 | 3.80 | 0.77 | 0.39 |  | CE11_2_1_S6 | 4.33 | 1.11 | 0.56 |
| CE11_1_8 | 3.80 | 0.94 | 0.48 |  | CE11_1_7_S2 | 4.33 | 0.82 | 0.36 |
| CE11_1_10 | 3.93 | 0.80 | 0.40 |  | CE11_1_6 | 4.47 | 0.74 | 0.33 |
| CE11_1_9 | 4.00 | 0.85 | 0.43 |  | CE11_2_2_S1 | 4.60 | 0.51 | 0.22 |
| CE11_1_11 | 4.07 | 0.59 | 0.30 |  | CE11_1_1 | 4.73 | 1.16 | 0.59 |
| Anchor NoALF | 4.13 | 0.83 | 0.42 |  | CE11_1_7_S1 | 4.80 | 1.08 | 0.47 |
| CE11_2_1_S5 | 4.13 | 1.13 | 0.57 |  | CE11_2_1_S4 | 4.87 | 1.13 | 0.57 |
| CE11_2_1_S1 | 4.13 | 0.74 | 0.38 |  | CE11_3_3_S1 | 5.20 | 0.77 | 0.34 |
| CE11_3_5 | 4.13 | 0.83 | 0.37 |  | CE11_1_4 | 5.47 | 0.64 | 0.28 |

| KristenAndSara |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Proponent | MOS | DS | CI |  | Proponent | MOS | DS | CI |
| CE11_2_1_S6 | 4.20 | 0.86 | 0.44 |  | CE11_1_2 | 4.87 | 0.92 | 0.46 |
| CE11_3_1 | 4.47 | 0.92 | 0.46 |  | CE11_3_4 | 5.07 | 0.59 | 0.30 |
| CE11_2_1_S2 | 4.47 | 1.06 | 0.54 |  | CE11_1_3 | 5.13 | 1.06 | 0.54 |
| CE11_1_7_S2 | 4.60 | 0.63 | 0.28 |  | CE11_1_4 | 5.13 | 0.52 | 0.23 |
| CE11_1_8 | 4.67 | 0.72 | 0.37 |  | CE11_1_9 | 5.27 | 1.10 | 0.56 |
| Anchor ALF | 4.67 | 0.82 | 0.41 |  | CE11_2_2_S1 | 5.27 | 1.03 | 0.45 |
| CE11_2_1_S5 | 4.73 | 1.03 | 0.52 |  | CE11_3_3_S1 | 5.40 | 1.06 | 0.46 |
| CE11_1_10 | 4.73 | 1.03 | 0.52 |  | CE11_1_6 | 5.47 | 1.06 | 0.46 |
| CE11_3_2 | 4.73 | 1.03 | 0.52 |  | CE11_2_1_S3 | 5.53 | 0.92 | 0.46 |
| CE11_2_1_S1 | 4.80 | 0.94 | 0.48 |  | Anchor NoALF | 5.87 | 0.52 | 0.26 |
| CE11_1_1 | 4.80 | 0.68 | 0.34 |  | CE11_3_3_S2 | 7.40 | 0.74 | 0.32 |
| CE11_2_1_S4 | 4.80 | 0.94 | 0.48 |  | CE11_2_2_S2 | 7.47 | 0.52 | 0.23 |
| CE11_1_5 | 4.87 | 0.99 | 0.50 |  | CE11_1_7_S1 | 7.73 | 0.59 | 0.26 |
| CE11_1_11 | 4.87 | 1.13 | 0.57 |  | CE11_3_5 | 8.00 | 0.65 | 0.29 |

# References

- “JVET common test conditions and software reference configurations”, JVET-K1010.

- A. Norkin and A. M. Kotra, "Description of Core Experiment 11 (CE11): Deblocking", JVET-K1031.

- A. Norkin and A. M. Kotra, "CE11: Summary report on deblocking", JVET-L0031.