| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, MO, 03–12 Oct. 2018 | Document: JVET-L0628-v2 |
| --- | --- |

| Title: | CE3: A combination of tests 3.1.2 and 3.1.4 for intra reference sample interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Jianle Chen Geert Van der Auwera Adarsh Krishnan Ramasubramonian Vadim Seregin Ted Hsieh Marta Karczewicz | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com geertv@qti.qualcomm.com aramasub@qti.qualcomm.com vseregin@qti.qualcomm.com thsieh@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Huawei Technologies Co. and Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contributions presents results of Core Experiment test CE3-3.1.4. Experimental results reportedly show -0.45% of luma BD-rate savings in all intra configuration and -0.17% of luma BD-rate savings over the BMS-2.0.1 software with the vtm configuration file (the suggested CE3 anchor).

# Introduction

This proposal reports test results for the experiment defined in [1]. The purpose of the test is to use the simplest design to achieve maximum coding performance and to prevent interference of WAIP, PDPC with the introduced 4-tap filters. The test comprises several modifications of the basis software that are described further. Intra prediction process by only directional intra modes is modified by this proposal. Non-directional intra prediction process is unmodified.

# Proposed techniques

This test proposes to modify the MDIS conditions in accordance with the aspect ratio of a block. In VTM2, the following MDIS table is included:

Table 1: MDIS thresholds (VTM2)

| sizeIndex | MDISthreshold[sizeIndex] |
| --- | --- |
| 0 | 20 |
| 1 | 20 |
| 2 | 20 |
| 3 | 14 |
| 4 | 2 |
| 5 | 0 |
| 6 | 20 → 0 (*) |
| 7 | 0 |

(*) In this proposal the MDISthreshold corresponding with sizeIndex=6 is modified to value 0.

This test suggests usage of simplified 6-bit 4-tap filters, the selection between these filters are performed according to the MDIS condition for directional intra prediction modes that provide non-fractional displacements, i.e. to all the directional modes excluding the following: 2, HOR_IDX, DIA_IDX, VER_IDX, 66.

Depending on the intra prediction mode, the following reference samples processing is performed:

- The directional intra-prediction mode is classified into one of the following groups:

  - vertical or horizontal modes (HOR_IDX, VER_IDX),

  - diagonal modes that represent angles which are multiple of 45 degree (2, DIA_IDX, VDIA_IDX),

  - remaining directional modes;

- If the directional intra-prediction mode is classified as belonging to group A, then then no filters are applied to reference samples to generate predicted samples;

- Otherwise, if a mode falls into group B, then a [1, 2, 1] reference sample filter may be applied (depending on the MDIS condition) to reference samples to further copy these filtered values into an intra predictor according to the selected direction, but no interpolation filters are applied;

- Otherwise, if a mode is classified as belonging to group C, then only an intra reference sample interpolation filter is applied to reference samples to generate a predicted sample that falls into a fractional or integer position between reference samples according to a selected direction (no reference sample filtering is performed).

Filter coefficients used by this test are given in Table 2. Instead of “Cubic” filter used in BMS2.0.1 the test proposes to reuse 4-tap DCT-IF filter that is used for subpixel motion compensation of chroma component (“MC chroma filter”). Coefficients of “MC chroma filter” are not modified as compared to BMS2.0.1.

## Table 2. 6-bit coefficients of 4-tap filters used in test CE3-3.1.4

| DCT-IF chroma filter | Gaussian filter 1 |
| --- | --- |
| { 0, 64, 0, 0 }, //0 { -1, 63, 2, 0 }, //1 { -2, 62, 4, 0 }, //2 { -2, 60, 7, -1 }, //3 { -2, 58, 10, -2 }, //4 { -3, 57, 12, -2 }, //5 { -4, 56, 14, -2 }, //6 { -4, 55, 15, -2 }, //7 { -4, 54, 16, -2 }, //8 { -5, 53, 18, -2 }, //9 { -6, 52, 20, -2 }, //10 { -6, 49, 24, -3 }, //11 { -6, 46, 28, -4 }, //12 { -5, 44, 29, -4 }, //13 { -4, 42, 30, -4 }, //14 { -4, 39, 33, -4 }, //15 { -4, 36, 36, -4 }, //16 { -4, 33, 39, -4 }, //17 { -4, 30, 42, -4 }, //18 { -4, 29, 44, -5 }, //19 { -4, 28, 46, -6 }, //20 { -3, 24, 49, -6 }, //21 { -2, 20, 52, -6 }, //22 { -2, 18, 53, -5 }, //23 { -2, 16, 54, -4 }, //24 { -2, 15, 55, -4 }, //25 { -2, 14, 56, -4 }, //26 { -2, 12, 57, -3 }, //27 { -2, 10, 58, -2 }, //28 { -1, 7, 60, -2 }, //29 { 0, 4, 62, -2 }, //30 { 0, 2, 63, -1 } //31 | { 16, 32, 16, 0 }, //int-pel { 15, 29, 17, 3 }, { 14, 29, 18, 3 }, { 14, 29, 18, 3 }, { 14, 28, 18, 4 }, { 13, 28, 19, 4 }, { 12, 28, 20, 4 }, { 12, 27, 20, 5 }, { 11, 27, 21, 5 }, { 11, 27, 21, 5 }, { 10, 26, 22, 6 }, { 10, 26, 22, 6 }, { 9, 26, 23, 6 }, { 9, 26, 23, 6 }, { 8, 25, 24, 7 }, { 8, 25, 24, 7 }, { 7, 25, 25, 7 }, // half-pel { 7 , 24, 25, 8}, { 7 , 24, 25, 8}, { 6 , 23, 26, 9}, { 6 , 23, 26, 9}, { 6 , 22, 26, 10}, { 6 , 22, 26, 10}, { 5 , 21, 27, 11}, { 5 , 21, 27, 11}, { 5 , 20, 27, 12}, { 4 , 20, 28, 12}, { 4 , 19, 28, 13}, { 4 , 18, 28, 14}, { 3 , 18, 29, 14}, { 3 , 18, 29, 14}, { 3 , 17, 29, 15} |

## Table 3. 6-bit coefficients of 4-tap filters used in test CE3-3.1.4 and CE3-3.1.2

| DCT-IF chroma filter (CE3-3.1.4) | Gaussian filter 2 (CE3-3.1.2) |
| --- | --- |
| { 0, 64, 0, 0 }, //0 { -1, 63, 2, 0 }, //1 { -2, 62, 4, 0 }, //2 { -2, 60, 7, -1 }, //3 { -2, 58, 10, -2 }, //4 { -3, 57, 12, -2 }, //5 { -4, 56, 14, -2 }, //6 { -4, 55, 15, -2 }, //7 { -4, 54, 16, -2 }, //8 { -5, 53, 18, -2 }, //9 { -6, 52, 20, -2 }, //10 { -6, 49, 24, -3 }, //11 { -6, 46, 28, -4 }, //12 { -5, 44, 29, -4 }, //13 { -4, 42, 30, -4 }, //14 { -4, 39, 33, -4 }, //15 { -4, 36, 36, -4 }, //16 { -4, 33, 39, -4 }, //17 { -4, 30, 42, -4 }, //18 { -4, 29, 44, -5 }, //19 { -4, 28, 46, -6 }, //20 { -3, 24, 49, -6 }, //21 { -2, 20, 52, -6 }, //22 { -2, 18, 53, -5 }, //23 { -2, 16, 54, -4 }, //24 { -2, 15, 55, -4 }, //25 { -2, 14, 56, -4 }, //26 { -2, 12, 57, -3 }, //27 { -2, 10, 58, -2 }, //28 { -1, 7, 60, -2 }, //29 { 0, 4, 62, -2 }, //30 { 0, 2, 63, -1 } //31 | { 16, 32, 16, 0 }, { 15, 29, 17, 3 }, { 15, 29, 17, 3 }, { 14, 29, 18, 3 }, { 13, 29, 18, 4 }, { 13, 28, 19, 4 }, { 13, 28, 19, 4 }, { 12, 28, 20, 4 }, { 11, 28, 20, 5 }, { 11, 27, 21, 5 }, { 10, 27, 22, 5 }, { 9, 27, 22, 6 }, { 9, 26, 23, 6 }, { 9, 26, 23, 6 }, { 8, 25, 24, 7 }, { 8, 25, 24, 7 }, { 8, 24, 24, 8 }, { 7, 24, 25, 8 }, { 7, 24, 25, 8 }, { 6, 23, 26, 9 }, { 6, 23, 26, 9 }, { 6, 22, 27, 9 }, { 5, 22, 27, 10 }, { 5, 21, 27, 11 }, { 5, 20, 28, 11 }, { 4, 20, 28, 12 }, { 4, 19, 28, 13 }, { 4, 19, 28, 13 }, { 4, 18, 29, 13 }, { 3, 18, 29, 14 }, { 3, 17, 29, 15 }, { 3, 17, 29, 15 } |

# Results

Experimental results were obtained using test conditions [2]. The proposed test is implemented on the basis of the CE3 anchor, i.e. BMS-2.0.1 software with vtm configuration file. Table 4 reportedly shows simulation results for AI and RA configuration over the CE3 anchor for the interpolation filters shown in Table 2 and the original MDIS conditions.

## Table 4. RD performance of 32-phase interpolation (Table 2) with the original MDIS conditions on top of the CE3 anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 (tag) with vtm config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.15% | -0.16% | 102% | 103% |
| Class A2 | -0.15% | -0.19% | -0.23% | 102% | 103% |
| Class B | -0.42% | -0.47% | -0.49% | 102% | 103% |
| Class C | -0.78% | -0.96% | -0.98% | 101% | 102% |
| Class E | -0.67% | -0.84% | -0.88% | 101% | 103% |
| Overall | -0.45% | -0.54% | -0.57% | 102% | 103% |
| Class D | -0.53% | -0.51% | -0.83% | 101% | 102% |
| Class F (optional) | -0.20% | -0.56% | -0.41% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 (tag) with vtm config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.10% | -0.08% | 101% | 103% |
| Class A2 | -0.09% | -0.07% | -0.11% | 102% | 104% |
| Class B | -0.19% | -0.06% | -0.04% | 101% | 102% |
| Class C | -0.29% | -0.12% | -0.10% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.09% | -0.08% | 101% | 103% |
| Class D | -0.24% | -0.16% | -0.01% | 101% | 103% |
| Class F (optional) | 0.14% | 0.02% | 0.13% | 101% | 101% |

Testing the interpolation filters presented in Table 3 with the original MDIS conditions gives the following results (Table 5).

## Table 5. RD performance of 32-phase interpolation (Table 3) with the unmodified MDIS conditions on top of CE3 anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.16% | -0.20% | 102% | 103% |
| Class A2 | -0.16% | -0.13% | -0.17% | 102% | 103% |
| Class B | -0.42% | -0.49% | -0.51% | 102% | 103% |
| Class C | -0.78% | -1.03% | -0.91% | 101% | 102% |
| Class E | -0.68% | -0.87% | -0.88% | 102% | 103% |
| Overall | -0.45% | -0.56% | -0.55% | 102% | 103% |
| Class D | -0.51% | -0.46% | -0.79% | 101% | 101% |
| Class F (optional) | -0.21% | -0.40% | -0.46% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.06% | -0.10% | 101% | 101% |
| Class A2 | -0.05% | 0.02% | 0.00% | 101% | 100% |
| Class B | -0.20% | -0.14% | -0.03% | 101% | 101% |
| Class C | -0.27% | -0.14% | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.17% | -0.09% | -0.06% | 101% | 101% |
| Class D | -0.23% | -0.18% | -0.02% | 100% | 98% |
| Class F (optional) | 0.12% | -0.05% | -0.05% | 101% | 101% |

# Conclusion

It is proposed to adopt the proposed filter into the VVC Working Draft 3 and the VTM3.0 software.

# References

- G. Van der Auwera, J. Heo, A. Filippov, JVET-K1023 “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010, “JVET common test conditions and software reference configurations for SDR video”, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd and Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).