| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0637 |
| --- | --- |

| Title: | Crosscheck of JVET-L0160 "AHG14: Intra Refresh Test conditions and Anchors generation Proposal" |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui | Tel: Email: | +81 44 754 2049 kazui.kimihiko@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck result of JVET-L0160 "AHG14: Intra Refresh Test conditions and Anchors generation Proposal".

In this crosscheck, the decoding result of a “progressive intra refresh” bitstream generated by the process described in JVET-L0160 is evaluated.

It is observed that a VTM decoder can decode the “progressive intra refresh” bitstream provided by the proponent of JVET-L0160 and output reconstructed pictures without visual artifacts after a recovery point.

Tested bitstream

Due to time constraint, the author obtained a bitstream subject to evaluation from the proponent of JVET-L0160, instead of a software package in which the non-normative method in JVET-L0160 is implemented.

The properties of the bitstream are the following;

| Property | Description |
| --- | --- |
| Test sequence | BQSquare (416x240/8-bit/60Hz/200 pictures) |
| QP | 37 |
| Major encoder option | Affine: 0 |
| Codebase | VTM2.0.1 |
| Interval of random access point | 26 pictures |
| Refresh direction | Left to right |

Testing method

The following modification is applied to VTM2.0.1 software to simulate random-access.  The scenario is to set the 27th picture as the random-access picture. The recovery point picture is the 52nd picture as the interval of random access point is 26.

<DecLib.cpp>

void DecLib::executeLoopFilters()

{

if( !m_pcPic )

{

return; // nothing to deblock

}

(snipped)

// The 27th picture is the random-access picture

if ( (m_pcPic->getPOC() >= 23) && (m_pcPic->getPOC() <= 26) )

{

cs.getRecoBuf().Y().fill(128); // clear DPB, for 8-bit sequence

cs.getRecoBuf().Cb().fill(128);

cs.getRecoBuf().Cr().fill(128);

cs.getMotionBuf().memset(0);    // clear MV storage

}

}

Result

It is observed any picture after the recovery point (i.e. 52th picture) has no visual artifacts.

The pictures output from VTM2.0.1 and VTM2.0.1 with the modification in Clause 2 are the following;

(RA: Random-access, RP: Recovery point)

| Picture | VTM2.0.1 | VTM2.0.1 with modification |
| --- | --- | --- |
| 0th IDR |  |  |
| 26th |  |  |
| 27th RA |  |  |
| 40th |  |  |
| 52th RP |  |  |

Remark

It is suggested to enable the decoded picture hash SEI message in the bitstream generation for checking exact match at decoder side easily.

References

[1] J.-M. Thiesse, D.Nicholson, D.Gommelet, “AHG14: Intra Refresh Test conditions and Anchors generation Proposal”, JVET-L0160, October 2018.

Patent rights declaration(s)

Fujitsu Limited does not have any current or pending patent rights relating to the technology described in this contribution.