| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0644 |
| --- | --- |

| Title: | Cross-check of JVET-L0256 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Poirier, F. Le Léannec | Email: | tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of the contribution JVET-L0256. It is reported that the results obtained during this cross-checking match the results described in JVET-L0256.

# Introduction

In JVET-L0256 “CE9-related: Complexity reduction and bit-width control for bi-directional optical flow (BIO)”, two methods are proposed to reduce BIO complexity: bit-depth reduction and using of bilinear interpolation filter instead of 8-tap filters.

# Code-level check

The proposal has been implemented on top of BMS2.1. Provided source code matches the description in [1].

First, a bit-depth reduction is implemented to use at most 15-bit multiplication and 32-bit internal bit-depth. This bit-width reduction allows SIMD implementation for some BIO derivation processes (gradient, correlation parameter, …). This SSE42 SIMD implementation is also proposed in the software.

Secondly, bilinear interpolation filter is used instead of 8-tap interpolation filter with a reduced area for the extended region for BIO derivation. BIO is further disabled if the CU as a height equal to 4 or CU has a size 4xN, and for CU using affine or ATMVP.

# Simulation results

The cross-check test is performed on top of BMS-2.1 using VTM configuration. The results are shown in the following tables and match the results shown in JVET-L0256.

## Performance of BIO bit-width control

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1(vtm.cfg) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.99% | -0.35% | -0.12% | 103% | 99% |
| Class A2 | -1.81% | -0.44% | -0.24% | 104% | 103% |
| Class B | -1.26% | -0.46% | -0.39% | 105% | 103% |
| Class C | -1.49% | -0.78% | -0.64% | 106% | 107% |
| Class E |  |  |  |  |  |
| Overall | -1.38% | -0.52% | -0.37% | 105% | 103% |
| Class D | -2.23% | -1.34% | -1.15% | 106% | 111% |

## Performance of BIO computational complexity reduction

## Performance of bit-width control method and BIO computational complexity reduction

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1(vtm.cfg) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.91% | -0.36% | -0.10% | 102% | 101% |
| Class A2 | -1.71% | -0.48% | -0.22% | 102% | 101% |
| Class B | -1.19% | -0.44% | -0.40% | 105% | 104% |
| Class C | -1.36% | -0.78% | -0.65% | 105% | 113% |
| Class E |  |  |  |  |  |
| Overall | -1.28% | -0.52% | -0.37% | 104% | 105% |
| Class D | -2.04% | -1.10% | -0.99% | 105% | 113% |

# References

- JVET, “CE9-related: Complexity reduction and bit-width control for bi-directional optical flow (BIO)”, JVET-L0256, 12th JVET Meeting, Macao, CN, 3-12 Oct., 2018

# Patent rights declaration(s)

Technicolor does not have any current or pending patent rights relating to the technology described in this contribution