| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0652-v2 |
| --- | --- |

| Title: | CE6-related: Combination test of CE 6-1.2-b and CE 6-2.1-a |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang, Jianle Chen Moonmo Koo Mehdi Salehifar Jaehyun Lim Seung Hwan Kim | Tel: Email: | +1-858-635-5226 yin.zhao@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com moonmo.koo@lge.com mehdi.salehifar@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | Huawei Technologies Co., Ltd LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports combination tests between following two tests:

- CE6-1.2-b: spatially varying transform (SVT)

- CE6-2.1-a: simplified MTS with reduced secondary transform

When inter MTS off, the observed BD-rate changes are -1.02% (AI), -1.19% (RA), and encoding time/decoding time is 94%/97% (AI), 114%/101% (RA). When inter MTS on, it is observed that BD-rate changes are -1.02% (AI), -1.07% (RA) and encoding time/decoding time is 94%/97% (AI), 111%/100% (RA).

Proposed Method

The features of CE 6-1.2-b [1] can be summarized as follows:

- A sub-block transform is used to transform and code a sub-part of the residual block of an inter-predicted CU.

- When the sub-block transform is used for a CU, the TU tiling method and the residual TU position are further coded.

- The sub-block transform is allowed when CU width and height are not larger than predefined maxSize

- When a CU uses the sub-block transform, the inter MTS is not allowed for the CU, and thus cu_mts_flag is inferred to be 0. Horizontal and vertical transform of the CU is adaptively determined based on the TU tiling method and residual TU position.

The configuration of CE 6-2.1-a [2] consists of the following features:

- The number of RST candidates to be tried is reduced from 3 to 1.

- The number of transform sets is reduced from 35 to 4.

- Worst case computational complexity is halved.

- Each own transform in a transform set is applied for both cases with MTS flag on and off.

- Using 1 MTS candidate for all modes, DST7 for both horizontal and vertical transform

Experimental results

The proposed method has been implemented based on BMS 2.0.1 reference software and experimentally evaluated under VTM configuration according to common test conditions defined in JVET-K1010 [3] and Core Experiment description in JVET-K1026 [4].

2.1) Inter MTS off

Anchor is BTM 2.0.1 with VTM configuration:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS2.0.1_vtmCfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.46% | -2.15% | -2.09% | 94% | 92% |
| Class A2 | -0.62% | -1.88% | -1.24% | 98% | 97% |
| Class B | -0.77% | -1.87% | -2.62% | 94% | 98% |
| Class C | -1.08% | -1.98% | -2.38% | 93% | 99% |
| Class E | -1.33% | -2.08% | -3.17% | 94% | 97% |
| Overall | -1.02% | -1.98% | -2.34% | 94% | 97% |
| Class D | -0.79% | -1.58% | -2.16% | 93% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS2.0.1_vtmCfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.39% | -1.22% | -1.76% | 112% | 101% |
| Class A2 | -1.22% | -1.34% | -1.34% | 117% | 102% |
| Class B | -1.17% | -1.40% | -2.17% | 115% | 101% |
| Class C | -1.04% | -1.09% | -1.36% | 113% | 101% |
| Class E |  |  |  |  |  |
| Overall | -1.19% | -1.27% | -1.71% | 114% | 101% |
| Class D | -0.95% | -0.97% | -1.35% | 117% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS2.0.1_vtmCfg |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.32% | -0.78% | -0.76% | 126% | 103% |
| Class C | -1.20% | -0.76% | -0.88% | 123% | 103% |
| Class E | -1.15% | -0.89% | -1.11% | 115% | 104% |
| Overall | -1.24% | -0.80% | -0.89% | 122% | 103% |
| Class D | -1.26% | 0.02% | -0.16% | 129% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

2.2) Inter MTS on

Anchor is BTM 2.0.1 with inter MTS on (--EMT = 3 --EMTFast = 3) under VTM configuration:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS2.0.1_vtmCfg_emt3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.46% | -2.15% | -2.09% | 94% | 92% |
| Class A2 | -0.62% | -1.88% | -1.24% | 98% | 97% |
| Class B | -0.77% | -1.87% | -2.62% | 94% | 98% |
| Class C | -1.08% | -1.98% | -2.38% | 93% | 99% |
| Class E | -1.33% | -2.08% | -3.17% | 94% | 96% |
| Overall | -1.02% | -1.98% | -2.34% | 94% | 97% |
| Class D | -0.79% | -1.58% | -2.16% | 93% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS2.0.1_vtmCfg_emt3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.32% | -1.44% | -1.91% | 109% | 99% |
| Class A2 | -1.06% | -1.42% | -1.42% | 112% | 99% |
| Class B | -1.05% | -1.56% | -2.24% | 112% | 100% |
| Class C | -0.90% | -1.12% | -1.38% | 110% | 103% |
| Class E |  |  |  |  |  |
| Overall | -1.07% | -1.39% | -1.78% | 111% | 100% |
| Class D | -0.85% | -0.90% | -1.34% | 112% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS2.0.1_vtmCfg_emt3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.98% | -0.96% | -0.64% | 118% | 98% |
| Class C | -0.96% | -0.95% | -1.05% | 116% | 99% |
| Class E | -0.81% | -0.96% | -1.40% | 110% | 99% |
| Overall | -0.93% | -0.96% | -0.97% | 115% | 99% |
| Class D | -1.02% | -0.22% | 0.34% | 120% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusion

In this contribution, it is reported that performance/encoding-time/decoding-time are -1.02%/94%/97% (AI), -1.19%/111%/100% (RA), and -1.24%/122%/103% (LD) for the combination of CE 6-1.2-a and CE 6-2.1-b. The luma coding gains of RST (CE 6-2.1-b) and sub-block transform (CE6-1.2-b) in RA are -0.58% and -0.61%, respectively, and the two tools together bring -1.19% gain in RA. Therefore, coding gains of the sub-block transform and RST are additive, and more than 1% BD-rate saving is achieved for AI/RA/LD configurations. The increase of decoder complexity and encoder run time are reasonable. Therefore, it is recommended that the two coding tools should be incorporated into the next version VTM.

Reference

- Yin Zhao et al., “CE6: Sub-block transform for inter blocks (CE6.1.2)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0358, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- M. Koo et al., “CE 6-2.1: Reduced Secondary Transform (RST)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0133, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- J. Boyce et al., “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- A. Said and X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signaling”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1026, 11th meeting, Ljubljana, SI, 10-18 July 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).