| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0662 |
| --- | --- |

| Title: | BoG on CE3.6: Intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | X. Zhao | Email: | xinzzhao@tencent.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG on CE3.6 and related contributions has mandates to review the following proposals: CE3 test 6.2.1 (JVET-L0165) and joint solution (JVET-L0222).

The BoG met on Saturday October 6th at 09:00 to 09:50.

# BoG Report

Both JVET-L0165 (Test 6.2.1) and JVET-L0222 (Combo Test) are based on 6 MPM.

It is agreed to use 6MPM for intra mode coding, and intra mode coding scheme without parsing dependency is recommended.

The non-MPM coding is same between these two candidates, truncated binary code. There is difference between these two candidates regarding the 5th MPM candidate.

It was asked whether there is fast algorithm at the encoder side regarding the intra mode coding of JVET-L0222, and it was clarified by proponents that there are no increased RD checks, and there are actually no encoder modifications regarding the proposed intra mode coding scheme.

It was mentioned that there is optimization regarding the MPM derivation process in JVET-L0165: using Planar as initialization of MPM list which may provide additional benefit for the encoder.

It was commented that the positions of the neighboring blocks are also modified in both two candidate solutions, and this individual aspect gives 0.07% gain (reported by CE related contribution JVET-L0154), while the total package gives 0.32% gain for the joint proposal (JVET-L0222) and 0.29% gain for JVET-L0165.

It was commented that JVET-L0222 may be not regarded as a CE test.

It is agreed that the joint proposal is a preferred solution for intra mode coding.

|  | Max number of neighbors to access | Line buffer required? | Max layers of if conditions | Max number of comparison operator | Max number of logical operators | Max number of assignment operators | Max number of increments | Max number of bit operation | Parsing dependency? | Number of Context modeling for MPM coding | number of full RDO checks | Has LUT? | LUT size | Number of condition check for remaining modes | Non-MPM coding |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.2.1 | 2 | N | 3 | 6 | 8 | 26 | 0 | 0 | N | 1 | 1 or 2 | N | - | 1 | TB |
| Combo | 2 | N | 3 | 8 | 5 | 27 | 4 | 0 | N | 1 | 1 or 2 | N | - | 1 | TB |

|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 6.2.1 | Extended number of MPM rather than 3 | -0.29% | -0.24% | -0.21% | 100% | 100% | -0.11% | -0.05% | 0.01% | 100% | 100% |

| Combined test of CE3.6 |  | JVET-L0222 (Huawei, MediaTek, LGE, Qualcomm) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 6.1, 6.2, 6.3, 6.4, 6.5 | Combined proposal of CE3.6 | -0.32% | -0.26% | -0.24% | 101% | 99% | -0.13% | -0.09% | -0.09% | 101% | 101% |

JVET-L0165 CE3-6.2.1: Extended MPM list [L. Li, J. Heo, J. Choi, J. Choi, S. Yoo, J. Lim (LGE)]

This contribution proposes an extended 6 MPM list with similar complexity compared to 3 MPM in VTM 2.0.1. In the proposed method, two neighboring intra modes (left and above) are deployed for MPM list generation. Following the structure of the 3 MPM generation in VTM2.0.1, MPM lists are generated. The following three tests are performed in this investigation.

- CE3-6.2.1a: CE3-6.2.1 with CTU boundary restriction

- CE3-6.2.1b: CE3-6.2.1 without CTU boundary restriction

The experiment results reportedly show -0.29%, -0.11% and -0.33% and -0.13% bit rate changes for VTM-AI, VTM-RA for CE3-6.2.1a, and VTM-AI and VTM-RA for CE3-6.2.1b, respectively.

JVET-L0222 CE3 6.6.1: A simple 6-MPM list construction with truncated binary coding for non-MPM signalling [A.M. Kotra, B. Wang, S. Esenlik, H. Gao, J. Chen (Huawei), M. G. Sarwer, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), L. Li, J. Heo, J. Choi, S. Yoo, J. Lim (LGE), A.K. Ramasubramonian, G. Van der Auwera, M. Karczewicz (Qualcomm)]

This contribution proposes a 6 most probable modes (MPM) based intra mode coding method. The MPM list is generated based on the left and top neighboring modes. The truncated binary coding is used to signal the remaining 61 non-MPM modes. It is reported that under All Intra configuration this proposal can achieve −0.32%/−0.26%/−0.24% Y/U/V BD-rate for VTM2.0.