| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0671-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-L0670 (Simplified DMVR for inclusion in VVC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh | Tel: Email: | +81-43-299-8526 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The cross checker reports that the source code provided by Huawei was carefully analysed and found to be consistent with the description in JVET-L0670. The cross checker could reproduce the results reported in JVET-L0670. The difference from measured encoder and decoder run time ratios to those reported in JVET-L0670 is acceptable.

Summary of Proposal JVET-L0670

JVET-L0670 [1] proposes a simplification of DMVR based on results of CE9 [2].

Simulation Results

Their software is based on BMS2.1rc1 and VTM configuration.

We used PC cluster for the simulation, however since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

Test results for RA configuration is shown in Table 1. All the BD-rate results are matched with JVET-L0670. For encoding/decoding complexity, similar tendency to the provided results by the proponent is observed.

Table 1  Summary of results for SDR

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.1rc1_vtm |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.05% | -1.74% | -2.10% | 101% | 104% |
| Class A2 | -2.47% | -2.57% | -2.65% | 100% | 102% |
| Class B | -1.37% | -1.39% | -1.44% | 101% | 105% |
| Class C | -1.18% | -1.34% | -1.40% | 101% | 109% |
| Class E |  |  |  |  |  |
| Overall | -1.67% | -1.68% | -1.80% | 101% | 105% |
| Class D | -1.06% | -1.23% | -1.26% | 101% | 108% |

References

- S. Esenlik, A. M. Kotra, B. Wang, H. Gao, J. Chen, S. Sethuraman, Simplified DMVR for inclusion in VVC, Joint Video Experts Team, doc. JVET-L0670, Macau, CN, Oct. 2018.

- S. Esenlik, Y.-W. Chen, F. Chen, Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation, Joint Video Experts Team, doc. JVET-K1029, Aug. 2018.

Patent rights declaration(s)

Sharp Corporation does not have any current or pending patent rights relating to the technology described in this contribution.