| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0674-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-L0129 (CE1-related: Chroma block coding and size restriction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Biao Wang | Tel: Email: | +4915226323685 biao.wang@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report of JVET-L0129 (CE1-related: Chroma block coding and size restriction). The simulations were carried out based on the common test condition (CTC) for AI and RA configurations. The test results are matched for all tested points.

# Introduction

This proposal of L0129 has two aspects:

- In chroma, restrictions in the chroma coding tree of intra slices prohibits splits that would result in any 2x2, 2x4, or 4x2 chroma blocks.

- In chroma, blocks of width or height of 2 use a 2x8 or 8x2 coefficient group size, respectively. As a consequence, coefficient groups always contain 16 coefficients (excepting the coefficient group containing the last significant coefficient).

# Simulation results

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.18% | -0.03% | 99% | 96% |
| Class A2 | 0.04% | 0.26% | 0.19% | 97% | 97% |
| Class B | 0.01% | 0.17% | 0.31% | 99% | 97% |
| Class C | 0.08% | 0.88% | 1.42% | 98% | 95% |
| Class E | -0.01% | 0.01% | 0.07% | 99% | 96% |
| Overall | 0.03% | 0.32% | 0.44% | 99% | 96% |
| Class D | 0.11% | 1.52% | 1.94% | 97% | 92% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

|  | Over VTM-2.0rc |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.09% | -0.02% | 99% | 99% |
| Class A2 | 0.02% | 0.18% | 0.04% | 99% | 99% |
| Class B | 0.03% | 0.18% | 0.37% | 99% | 98% |
| Class C | 0.04% | 0.77% | 0.92% | 98% | 92% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.32% | 0.37% | 99% | 97% |
| Class D | 0.06% | 0.80% | 1.47% | 99% | 87% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 1. Summary results of the proposed algorithm over VTM 2.0.1

# References

- C. Rosewarne, A. Dorrell, “CE1-related: Chroma block coding and size restriction”, JVET-L0129, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have any current or pending patent rights relating to the technology described in this contribution.