| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0676 |
| --- | --- |

| Title: | Cross-check of JVET-L0239 (CE3-related: Enabling different chroma sample location types in CCLM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Geert Van der Auwera 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com geertv@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The results of a partial cross-check of JVET-L0239-v1 is reported. It is reported that the cross-checker has conducted a partial cross-check of the bit-rates and PSNR reported for one of the 360° sequences, and the results obtained match the values reported by the proponent. The software implementation that was provided by the proponents to the cross-checkers was used for the cross-check. The cross-checker verified the aspects related to CCLM in the software implementation and reported that the implementation matches the description of the method provided in JVET-L0239-v1. It is reported for certain chroma sample location types (Type 2 and Type 3), the proposed method uses three lines of luma samples above the current block to derive the reference samples; CCLM in VTM-2.0/VVC Draft 2 only uses two lines of luma samples above the current block to derive the reference samples. It is also reported that the number of lines of luma samples that are left of the current block and that are used to derive the reference samples is not changed.

# Introduction

The CCLM tool is used to predict chroma samples (Cb and Cr) from reconstructed luma samples. A linear model is used for the prediction, and the parameters are derived from the neighbouring reconstructed reference samples; the luma samples are downsampled to be the same size as the chroma samples. For CCLM, currently two luma reference sample lines above, and three luma reference samples lines left of the current block are used. The model is derived at the encoder and decoder; the parameters are used to predict the values of Cb and Cr from the reconstructed luma samples.

# Cross-checked test

Three tests are proposed in JVET-L0239. The three tests differ in the the location of the chroma sample relative to the luma sample. Different filters were chosen for the different chroma sample locations relative to the luma sample positions. In addition to the default filter currently used by CCLM, which corresponds to chroma sample location type 0, one additional filter each is specified for the chroma sample location types 1, 2 and 3 – a total of three additional filters. Some different filtering operations are also performed on the boundaries for certain chroma sample location types.

# Software

No deviation was observed in the software implementation compared to the description provided in JVET-L0239-v1.

# Results

The PSNR numbers and bit rates numbers for QP 37 of the Trolley sequence for all the proposed methods were tested, and it is reported that they match the bitrates and PSNRs reported by the proponent.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).