| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0681 |
| --- | --- |

| Title: | BoG Report: Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Andrew Segall Anand Meher Kotra | Tel: Email: | anorkin@netflix.com asegall@sharplabs.com anand.meher.kotra@huawei.com |
| Source: | BoG |  |  |

_____________________________

# Summary

This is a report of the Breakout Group on Deblocking that met during the 12th meeting.  The Breakout Group was originally created as two Breakout Groups.  The first on Long Tap Deblocking Filters, and the second on Deblocking for 4 x N and N x 4 block boundaries. Each of these BoGs met separately on October 7th, and then were combined on October 8th.  The goal of the combined group was as follows:

- Long Tap Deblocking

  - Further analyze the results of subjective test in L0611

  - Identify if it is possible to conclude that visual improvement over VTM (ALF off) has been achieved

  - Identify if there is consistency that certain proposals perform better

- Deblocking for 4 x N and N x 4 block boundaries

  - Further analyze the proposals on 4x4 grid deblocking in terms of complexity (including decision mechanisms which block boundaries can be deblocked such that parallel processing is still possible)

  - Investigate possible interaction with CE11.2.2

- Perform additional expert viewing if necessary

- Review CE related documents on deblocking filters

- Recommend items to be investigated in the upcoming CE11

The BoG met on October 7th from 6:15PM to 9:30 PM, October 8th from 6:30PM to 9:55PM, and October 9th 3:00PM to 7:00PM.

# Review and Discuss Results of Subjective Testing

## Long Tap Deblocking

A general discussion of subjective testing for long tap deblocking was conducted on October 7th.  The notes from the discussion were:

It was commented that it may be beneficial to categorize the proposals that only modified the luma deblocking filters and those that modified both the luma and chroma deblocking filters.

It was commented that CE11.1.4 is operating on a 4x4 grid while other CE11.1.* proposals maybe operating on an 8x8 grid.  It was confirmed that CE11.1.4 is operating on a 4x4 grid by the cross checker.

It was commented that whether a long tap deblocking proposal operated on luma or luma and chroma is in the CE 11 summary report.

It was commented that some proposals only applied long deblocking when the width and height of a block was large, while other proposals applied long deblock filters when only the width (or height) was large.

It was commented that some proposals incorporated a decision to select long deblocking as an extension of the strong deblocking decision, while others always apply long deblocking for large blocks.

It was commented that some proposals used symmetrical filtering while other used asymmetrical filtering.

It was commented that some proposals did not extend the number of line buffers required to deblock the CTU boundary.  At least one participant expressed that this was a desirable aspect.

It was commented by a participant that long tap filters appear to provide a visual benefit compared to the anchor based on the subjective results provided by the Test Chair.

Following the discussion, two presentations were made reporting analysis of the subject test results.

In a first presentation, one participant reported their analysis of the subjective results and asserted that it appeared that there were 8 cases where long tap deblocking had outperformed the anchor.

In a second presentation, one participant reported a second analysis of the subject test results.  It was reported that the tests was conducted in two sessions, where each session consisted of a separate set of responses.

One participant observed that there may be some differences between the data in the two test sessions.  It was suggested to consult the Test Chair to better understand.

One participant commented that it was challenging to observe differences between technologies in the ParkRunning sequence in evaluation in their lab.

It was communicated by the JVET test chair that only Campfire sequence should be used for drawing conclusions since this was one sequence that behaved consistently in the test JVET-L0611.

It was found that at least one long-tap deblocking filter proposal had non-overlapping confidence interval with the Anchor NoALF on the Campfire sequence.  Another proposal had 1% overlap with the anchor, and one other proposal was rather close.

Some participants expressed opinion that we have evidence of the improvement over the VTM noALF anchor with long tap deblocking filters.

One participant expressed concerns that the conclusion was drawn based on one sequence.

It was recommended by the BoG to have a screening of a selected set of the CE11.1 proposals and the anchor to confirm that there is a subjective improvement from using long tap deblocking filters. The screening should be open to non-proponents and announced.

The group agreed that the selection below did not infer a recommendation to study the selected proposals in the CE or adopt any of them and should not be used to endorse proposals that were selected.

The visual demo of the proposals has been performed October 9, 12:00pm. The sequences were played in the order indicated below, and the comment from the participants were collected:

Campfire QP39:

AnchorNoALF

CE11.1.2

CE11.1.8

CE11.1.9

CE11.1.1

CE11.1.10

Comments: One participant says difference is small. Two participants see improvements in the fire and on the flag. Less fluctuation in the bottom of the picture for proposals compared to the anchor. One other proponent sees improvements in the subjective quality.

FoodMarket QP39

AnchorNoALF

CE11.1.5

CE11.1.8

CE11.1.9

CE11.1.1

CE11.1.10

Comments: Smoke in the beginning has less blocking artifacts with proposals. There was a comment that fade-in fade-out sequences may be used for blocking. Participant sees improvement in the smoke.

The discussion of the demo happened on Tuesday October 9, at 3pm.

It was asserted by one participant that some non-normative methods may help reducing large block artifacts and such methods can also be investigated in the next round of the CE.

Conclusion of the discussion:

- There is evidence that a problem with blocking artifacts in large blocks exists at higher QPs in VTM2.0.1.

- The BoG agrees that there is evidence that long-tap deblocking reduces large block artifacts on some content.

- Based on the available data, BoG could not determine which of the long-tap deblocking proposals performed more consistently than the other.

Recommendation: investigate the CE11.1 proposals in the next round of the CE.

## Deblocking for 4 x 4 grid

The main mandate here was to cross verify the subjective results of CE 11.3 presented in the proposal JVET-L0611. It was recommended by the Chair to perform an expert subjective test during the Macao meeting to cross verify the subjective results.

Due to the lack of resources, an expert subjective viewing could not be performed during the meeting. Instead an informal demo of all the CE 11.3 sequences which were submitted to the offline subjective test was conducted by the test chair.

The informal demo was conducted by showing the sequences in the following order:

- Anchor_NoALF

- Anchor_ALF

Then the sequences were displayed with ascending MOS scores for each sequence. The following proposals were shown in that order:

- CE 11_3_1_NoALF

- CE 11_3_3_S1_NoALF

- CE 11_3_3_S2_NoALF

- CE 11_3_4_NoALF

- CE 11_3_5_NoALF

The following comments expressed by the experts in the BoG who attended the informal subjective demo:

One expert commented that for RA sequences, there was no subjective evidence observed. It was suggested that large blocks may mask the need for deblocking on 4 x N and N x 4 block boundaries.

One participant observed subjective quality improvement around object boundaries for KristenAndSara sequence in LD configuration when compared to the NO_ALF Anchor.

One more participant observed no subjective evidence.

One participant commented that 4 x N and N x 4 blocks may not always happen but seem to appear sporadically and might be annoying if not deblocked.

One participant commented that the QP value used for 4 x N and N x4 deblocking should be determined, generally the higher the QP, the more chances that 4 x N and N x 4 blocks occur.

One participant commented we should first fix 4 x N and N x 4 blocks and then take care of longer tap filter.

One participant commented that the sub-block boundaries on 8 x 8 grid should also be deblocked.

A comment from participant was the group should determine a priority order of fixing types of deblocking artifacts and that it should be the following order:

- Longer tap

- Sub-PU

- 4 x 4 grid deblocking and parallel deblocking for 4 x N and N x 4 boundaries.

Revisit to conclude if there is a definite subjective evidence for deblocking on 4 x 4 grids. One more subjective test demo is planned to be conducted with some non-proponents as test viewers

The second viewing session of the 4x4 filtering grid proposals was been conducted on October 9, 2018, at 12pm. Sequences KristenAndSaraQP34 and Kimono were shown with the proposals listed above. The following comments were collected.

For KristenAndSara, QP34

One participant thought that some proposals were worse than anchor. One proponent saw improvements (smaller blocks). Two participants saw differences but were not sure what was better. A proponent saw improvements along object boundaries.

For Kimono, QP34

Kimono was more difficult to assess. Some participants saw difference in the face but were not sure which variant was better.

The discussion of the demo happened on Tuesday October 9, at 3pm.

BoG agree there is less difference was seen on deblocking for 4x4 than on long-tap deblocking. Some participants are convinced that non-deblocked boundaries may happen, and the CE may need to define separate test conditions for 4x4 deblocking during the editing period of the CE (so far same test conditions as for the long-tab deblocking have been used). As an example block boundaries of 4x64 for inter-slices and 32x4 for intra slice, where these boundaries may not be deblocked.

Recommendation: investigating deblocking on 4x4 boundaries in the next round of the CE.

### Possible interaction of deblocking on 4x4 grid with CE11.2.2.

The group agrees that there is no interaction between CE11.2.2 and proposals that suggest deblocking on the 4x4 grid.

# Review of CE Related Documents

## CE11 related – Deblocking (13)

Contributions in this category were discussed XXday XX Oct XXXX–XXXX (chaired by XXX).

JVET-L0226 CE11- related: Position dependent adaptive Tc clipping range for deblocking filter [A.M. Kotra, S. Esenlik, B. Wang, H. Gao, Z. Zhao, J. Chen (Huawei)]

The contribution proposes an adaptive Tc clipping range for the deblocking filtering. The current deblocking filter in VTM 2.0.1 uses a fixed clipping range, where all the samples after filtering are clipped to the following range of values [P – 2tc, P + 2tc]. It is asserted that this clipping range does not take into account the position of the sample from the edge and therefore in a few cases, the filtering may result in blurring or over smoothing. To address the asserted issue, the proposed technique defines an adaptive Tc clipping range as a “monotonically decreasing function” where-in the clipping range is defined based on the distance of the sample from the edge.  In other words, the closer the sample is to the edge, the larger the clipping range is allowed and therefore larger is the deviation allowed after filtering. Two different “monotonically decreasing functions” are proposed.

It was reported that no objective loss was observed.  Moreover, results were reported using the longer tap filter proposed in CE11.1.8.  In this configuration, a small gain was reported.

It was reported that subjective evaluation illustrated that the over-smoothing of a long tap filter was reduced.

Recommendation: Consider for CE study.

Proponent prefers variant f2 tested in the proposal.

JVET-L0621 Cross-check of JVET-L0226 (CE11-related: position dependent adaptive Tc clipping range for deblocking filter) [P. Onno, G. Laroche (Canon)] [late] [miss]

Presenter not presented.  It was reported by that the cross checker reported that the results matched.

JVET-L0393 CE11-related: Improvement of Extended Deblocking Filter [K. Unno, K. Kawamura, S. Naito (KDDI)]

Chaired by A. Norkin on 8/10/2018 in the Deblocking BoG

This contribution presents a proposal of extended deblocking filtering method. In the proposed method, long tap deblocking filters are applied to both luma and chroma samples. BD-rate gains for luma and chroma (Y, U, V) are (-0.01%, -0.53, -0.51), (-0.09%, -0.58%, -0.48), (0.00%, 0.30%, -0.03%), and (-0.03%, -0.02%, -0.23) for AI, RA, LDB, and LDP conditions, respectively, in case of comparing with VTM ALF On. Similarly, BD-rate gains for luma and chroma (Y, U, V) are (-0.02%, -0.70, -0.62), (-0.10%, -0.75%, -0.55), (0.00%, -0.28%, -0.46%), and (-0.10%, -0.80%, -0.95) under the AI, RA, LDB, and LDP conditions, respectively, in case of comparing with VTM anchor with ALF off.

The proposal is modification of CE11.1.2

Proposal 1:

Luma deblocking: Blocks size thresholds are switched by depending on Bs value of the adjacent block. For extended filtering to apply, blocks sizes should be >=16 for Bs=2 and  >=32 for Bs=2.

Block smoothness conditions are modified. The conditions are sum of absolute differences between sample values.

Filtering kernel is the same as in CE11.1.2

Proposal 2:

Chroma deblocking is added. Blocks size thresholds are switched by Bs as for luma (divided by 2). VTM luma strong filtering decisions and filter are used for chroma.

Luma complexity has not changed significantly relatively to CE11.2, while chroma was added.

The proponent suggests to further study this method in the CE11 (Proposal 1 + Proposal 2).. The proponent prefers this proposal to the one tested in CE11.1.2.

Recommendation: Consider for studying in the CE11 (Proposal 1 + Proposal 2).

JVET-L0566 Crosscheck for JVET-L0393 (CE11-related: Improvement of Extended Deblocking Filter) [W. Choi, K. Choi (Samsung)] [late] [miss]

The cross-checker confirmed that the software implementation is consistent with the described algorithm in JVET-L0393 and the test result is also matched to proponent’s one.

JVET-L0410 CE11-related: On deblocking tC table [A. Norkin (Netflix)]

Chaired by A. Segall on 8/10/2018 in the Deblocking BoG.

The document argues that the values chosen during the last meeting to extend the deblocking tC

table (as a consequence of extending the maximum QP value to 63) do not reflect the intent of

the tC table design. The document proposes new values to extend the tC table that keep linear

relation of tC with the quantization step size.

The proponent summarized that the tC table was updated in the previous meeting as a result of

extending the maximum QP range. It was asserted that the table was extended linearly.

However, that the design of the existing tC table was designed to have an exponential

relationship with the QP values.

The proponent reported that some visual evaluation was done, and that all results at impacted QP

values are visually objectionable.

One participant commented that at the beginning of the range, the tC table does not follow an

exponential relationship. It was commented that this may be related to the quantization of the

table.

One participant noted that this appears like a bug fix. Another participant agreed.

One participant noted that there are other proposals related to modifying the tC table (or process), and that this update should not indicate a disinterest in that work. The group agreed.

Recommendation: Adopt the updated tC table into the VVC.

JVET-L0460 CE11.1.11 related: Improvements to smoothness decision for long luma filters [K. Andersson, Z. Zhang, R. Sjöberg, W. Zhu (Ericsson), K. Misra, P. Cowan, A. Segall (Sharp Corporation)] [late]

This contribution was discussed on Sunday 7/10/2018 19:00 in the Deblocking BoG.

This contribution is related to CE 11.1.11, which is a joint proposal from Ericsson and Sharp.  The CE 11.1.11 deblocking design was asserted to achieve significant visual quality improvement when deblocking large blocks.  However, it is observed by the proponents that this improvement came with a loss in objective performance.  Though the loss is asserted to be small, the purpose of this document is to propose modifications to the CE11.1.11 design if the committee prefers an improvement in objective quality performance.  Two possible approaches are considered and described.  One approach modifies the long filter decision such that they are based on HEVC strong/weak decision and that smoothness decisions include all additional samples used by long filters. Another approach uses a modified threshold in the CE11.1.11 algorithm.  In both cases, all other aspects of the CE11.1.11 design are retained, including the proposed symmetric and asymmetric long filters, block size decisions, and chroma filtering.

It is asserted that both solutions improve objective quality while maintaining the subjective performance of CE11.1.11.  Specifically, the approach that modifies the long filter decision provides a luma BD rate impact of -0.01%/0.04%/0.15%/0.09% for AI/RA/LD/LDP compared to VTM-2.0.1 and -0.03%/-0.02%/0.15%/-0.06% for AI/RA/LD/LDP compared to VTM-2.0.1 when ALF is turned off.  Decoding time 101%/100%/101%/98% for AI/RA/LD/LDP compared to VTM-2.0.1 and 101%/99%/100%/101% for AI/RA/LD compared to VTM-2.0.1 when ALF is turned off.

Similarly, the approach that modifies the threshold provides a luma BD rate impact of -0.02%/0.00%/0.12%/0.06% for AI/RA/LD/LDP compared to VTM-2.0.1 and -0.04%/-0.05%/0.13%/-0.06% for AI/RA/LD/LDP compared to VTM-2.0.1 when ALF is turned off.  Decoding time (using heterogeneous cluster) 100%/102%/101%/101%for AI/RA/LD/LDP compared to VTM-2.0.1 and 103%/102%/103%/102% for AI/RA/LD compared to VTM-2.0.1 when ALF is turned off.

Recommendation: Consider for CE study

JVET-L0523 CE11-related: Very strong deblocking filtering with conditional activation signalling [C. Helmrich (HHI)] [late]

It was reported by the proponent that improved luma and chroma deblocking for the Versatile Video Coding specification is studied extensively in Core Experiment (CE) 11, described in JVET-L0031, and that while these designs generally succeed in increasing the video reconstruction quality for many SDR sequences, it was noticed that, for some SDR and, particularly, HDR material, even more aggressive deblocking filters (using even longer filter kernels) may be beneficial. It was asserted that using such very strong filtering, however, increases the risk of adverse artifacts in some image regions.

It was proposed that a simple very strong deblocking algorithm, whose activation can be controlled by the encoder may address the asserted issue.  To minimize the signaling overhead required for this “superstrong” deblocking mode, a conditional transmission of the activation flag was employed on a CTU-wise level based on the partitioning and residual coding information already present in the bit-stream.

It was reported that the activation flag is not signaled in every CTU, and that the Bjøntegaard delta (BD) rate increase caused by said flag (a 1-bit element) is less than 0.04 % on average.

It was reported that software could be made available.

It was proposed to study the proposed conditional activation together with (very) strong deblocking filters towards the next (13th) JVET meeting in 2019.

One participant commented if the flag applied to the entire CTU or just the CTU boundaries.

It was suggested that the flag signaled if strong deblocking is allowed (or not) in the CTU.

One participant commented that the ability to turn a very long filter off at a CTU level may be desirable.

One participant asked if the control was considered within the RDO loop.

Recommendation: Consider for CE study in combination with other long tap filters (but currently as a standalone proposal in the CE)

JVET-L0529 CE11.1.10-related: Smoothness threshold modification for long tap deblocking [W. Zhu, K. Misra, A. Segall (Sharp), M. Ikeda, T. Suzuki (Sony)] [late]

This contribution was discussed on Sunday 7/10/2018 19:10 in the Deblocking BoG.

This contribution is related to CE 11.1.10.  CE11.1.10 is a joint deblocking design proposed by Sony and Sharp that is asserted to achieve significant visual quality improvement.  While subjective quality is perhaps the most important aspect a deblocking design, the proponents also observe that the configuration of the CE11.1.10 proposal results in a small loss in objective performance compared to VTM 2.0.1.  The purpose of this contribution is to provide supplemental results of the CE11.1.10 design with improved objective performance.  This is achieved by modifying the smoothness threshold for luma stronger filter deblocking in the CE11.1.10 algorithm. Objective evaluations show luma BD rate impact of -0.03%/-0.07%/0.02%/-0.06% for AI/RA/LD/LDP compared to VTM-2.0.1 and -0.04%/-0.12%/0.01%/-0.16% for AI/RA/LD/LDP compared to VTM-2.0.1 when ALF is turned off.

Proponents claim that the subjective quality is same as in CE 11.1.10 but the objective gains are improved in JVET-L0529.

Recommendation: Consider CE study (possibly replacing CE11.1.10)

JVET-L0558 CE2/CE11-related: Deblocking TC offset for VTM [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)] [late]

This contribution was discussed on Sunday 8/10/2018 in the Deblocking BoG.

This contribution presents results of deblocking TC offset LoopFilterTcOffset_div2 set as -6 for AI, and TC offset -2 for RA and LDB configurations. The gains are -0.54% for AI, -0.07% for RA, and -0.22 for LD-B.

The proponent has not looked at the subjective quality yet. It has been commented that the results without ALF would be interesting (CE11 CTC include the ones with ALF on).

Recommendation: Further study (by the proponents) 1) Subjective quality. 2) Performance with ALF off (CE11 test conditions) if the subjective quality is good.

JVET-L0572 CE11-related: CTU line buffer reduction for long filter deblocking [A.M. Kotra, S. Esenlik, B. Wang, J. Chen (Huawei), W. Zhu, K. Misra, P. Cowan, A. Segall (Sharp)] [late]

The contribution is a joint proposal between Huawei and Sharp. Long filter deblocking is asserted to provide significant visual benefit over the current approach in VTM-2.0.1. However, such approaches also extend the number of CTU line buffers. The contribution proposes modifying the filtering and control processes at the top edge of the CTU boundary to access samples within 4 luma lines and 2 chroma lines. Aspects of long filter deblocking and CTU line buffer aware filtering are based on tests CE11.1.7b and CE11.1.8.

Four aspects are considered in this contribution:

- CTU line buffer reduction

  - modified three samples at the CTU boundary (top), and only using 4 for the filtering operations support.

- Samples beyond CTU line buffer reduction is not used in deblocking control process.

  - The gradient computation (for the strong filter) is skipped for HEVC.

- Subsampling of chroma gradient computation.

  - For chroma gradient computation, use only one line for the two-chroma-sample segment of the block boundary.

- Threshold change in luma strong deblocking filter.

  - Only for the luma strongest filtering test (to decrease the losses).

It is asserted that the visual impact of these changes is negligible. Objective evaluations show a luma BD rate impact of –0.02%/ 0.03%/0.15%,0.10%for AI/RA/LD/LDP compared to VTM-2.0.1 and -0.03%/0.02%,0.17%,0.03% for AI/RA/LD/LDP compared to VTM-2.0.1 when ALF is turned off.

Proponents were asked if they saw visual changes. Proponents answered that there is change but it is not noticeable.

One participant commented that it is desirable to keep the same number of modified samples irrespective of if it is across the block boundary since there may be possibility of artifacts.

One participant noted that first it can be decided whether to apply long deblocking filters and then to decide whether we should decrease the line buffers.

Another participant commented that the proposal consists of several small modifications.

Proposal mostly consists of 11.1.7b. The contribution of the CE11.1.8 are filters that are used across horizontal CTU boundary (according to the proponent, they can be obtained by modification of the filters from 11.1.7b).  The decisions (across CTU boundary) are based on 11.1.7b.

It was commented that 11.1.8 and this proposal are using common luma filters.

It was commented that line buffer is a problem, and a possible solution could consider joint use of line buffer buffers to decrease the line buffer requirements.

Recommendation: continue study the proposal in the CE. The group agrees that improvements to subjective quality should be prioritized and the line buffer reduction issue should be addressed second. It is likely that some of the aspects of this will be merged into CE11.1.7 during the next round of the CE.

JVET-L0661 Crosscheck of JVET-L0572 (CE11-related: CTU line buffer reduction for long filter deblocking) [M. Ikeda (Sony)] [late] [miss]

Cross-checker confirms that the objective results match the ones provided by the proponents.

JVET-L0614 CE11-related: Additional tests of CE 11.3.3 and 11.3.4 for 4 x N and N x 4 deblocking [A.M. Kotra, S. Esenlik, B. Wang, H. Gao, Z. Zhao, J. Chen (Huawei), Chia-Ming Tsai, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei (MediaTek)] [late] [miss]

Document reviewed on October 7th at 20:30. Chaired by Kenneth R. Andersson

In VTM2.0.1, deblocking is only considered on 8x8 grids. In this contribution, two approaches, Test1 and Test2, are proposed based on the subtest2 of JVET-L0225 and JVET-L0103. In Test1, for vertical edges, for the blocks whose widths are four samples, the proposed filter uses three samples in filtering decision and modifies one sample in filtering process. In Test2, for vertical edges, for the blocks whose widths are four samples, the proposed filter uses two samples in filtering decision and modifies one sample if weak filtering process is used and two samples if strong filtering process is used. Both in Test1 and Test2, for the blocks whose widths are larger than four samples, the numbers of modified samples in filtering process are the same as those in VTM2.0.1. Objective results for Test1 and Test2 are as follows:

Test1:
Over VTM2.0.1 with ALF off, luma BD-rates are 0.01% (AI), -0.26% (RA), -0.19% (LB), and -0.37% (LP) with negligible run-time changes.
Over VTM 2.0.1 with ALF on, luma BD-rates are 0.08% (AI), -0.12% (RA), -0.21% (LB), and -0.19% (LP) with negligible run-time changes.

Test2:
Over VTM2.0.1 with ALF off, luma BD-rates are 0.04% (AI), -0.22% (RA), -0.20% (LB), and -0.40% (LP) with negligible run-time changes.
Over VTM 2.0.1 with ALF on, luma BD-rates are 0.06% (AI), -0.12% (RA), -0.21% (LB), and -0.19% (LP) with negligible run-time changes.

Test 1: Related to CE11.3.3

Instead of 3 samples proponents use 2 samples in filter decision both for ”d” and HEVC strong/weak filter decision. The side with 4 samples is max modifies one sample, the other side can filter 3 samples if wider than 4 and HEVC strong/weak filter decision indicates strong filter.

Test 2: Related to CE11.3.4

Main difference is to allow asymmetric filtering.

Qualcomm is performing cross-check.

One proponent say that by doing deblocking of 4x4 grid as in this contribution and in CE11.3.1, 3.3, 3.4 proposals we get objective gain by doing more filtering and at the same time subjective benefit.

Recommendation: Revisit pending on outcome of CE11. This proposal may replace 11.3.1, 11.3.4 and 11.3.4

JVET-L0645 Cross-check of JVET-L0614 (CE11-related: Additional tests of CE 11.3.3 and 11.3.4 for 4 x N and N x 4 deblocking) [J. Dong (Qualcomm)] [late]

JVET-L0460 CE11.1.11 related: Improvements to smoothness decision for long luma filters [K. Andersson, Z. Zhang, R. Sjöberg, W. Zhu (Ericsson), K. Misra, P. Cowan, A. Segall (Sharp Corporation)] [late]

This contribution was discussed on Sunday 7/10/2018 19:00 in the Deblocking BoG.

This contribution is related to CE 11.1.11, which is a joint proposal from Ericsson and Sharp.  The CE 11.1.11 deblocking design was asserted to achieve significant visual quality improvement when deblocking large blocks.  However, it is observed by the proponents that this improvement came with a loss in objective performance.  Though the loss is asserted to be small, the purpose of this document is to propose modifications to the CE11.1.11 design if the committee prefers an improvement in objective quality performance.  Two possible approaches are considered and described.  One approach modifies the long filter decision such that they are based on HEVC strong/weak decision and that smoothness decisions include all additional samples used by long filters. Another approach uses a modified threshold in the CE11.1.11 algorithm.  In both cases, all other aspects of the CE11.1.11 design are retained, including the proposed symmetric and asymmetric long filters, block size decisions, and chroma filtering.

It is asserted that both solutions improve objective quality while maintaining the subjective performance of CE11.1.11.  Specifically, the approach that modifies the long filter decision provides a luma BD rate impact of -0.01%/0.04%/0.15%/0.09% for AI/RA/LD/LDP compared to VTM-2.0.1 and -0.03%/-0.02%/0.15%/-0.06% for AI/RA/LD/LDP compared to VTM-2.0.1 when ALF is turned off.  Decoding time 101%/100%/101%/98% for AI/RA/LD/LDP compared to VTM-2.0.1 and 101%/99%/100%/101% for AI/RA/LD compared to VTM-2.0.1 when ALF is turned off.

Similarly, the approach that modifies the threshold provides a luma BD rate impact of -0.02%/0.00%/0.12%/0.06% for AI/RA/LD/LDP compared to VTM-2.0.1 and -0.04%/-0.05%/0.13%/-0.06% for AI/RA/LD/LDP compared to VTM-2.0.1 when ALF is turned off.  Decoding time (using heterogeneous cluster) 100%/102%/101%/101%for AI/RA/LD/LDP compared to VTM-2.0.1 and 103%/102%/103%/102% for AI/RA/LD compared to VTM-2.0.1 when ALF is turned off.

Recommendation: study in the CE. (This may replace CE11.1.11)

### Luma deblocking complexity in CE11.3, deblocking on 4x4 grid

| Tests | Samples from block bound. modified | Samples from block bound. for deblocking decision | Max num. oper for filtering per line (add/mult/compar/shift) | Max number of oper. for decision for 8-sample boundary (add/mult/compar/shift) | Num. line buffers | Number of checks to determine block size |
| --- | --- | --- | --- | --- | --- | --- |
| CE11.3.1 | 2+1/1+2/1+1 | 3 + 3 | 1+1 weak filter of VTM (worst case 4 on both sides): 15 (6,2,6,1) 2+1 weak filter of VTM (>4 on one side and 4 on the other side): 22 (11,2,6,3) | (11/0/1/4) for “d” decision “d” decision Total 8 lines = 16 *2 = 32 | VTM | 3 |
| CE11.3.3 S1 | 3+1/1+3/1+1 | 3 + 3 | (19/1/0/11) 3+ 1 with VTM strong filter | (11/0/1/4) for “d” decision + (8/1/3/4) for strong filter Total 8 lines = 32*4 = 128 | VTM | 2 |
| CE11.3.3 S2 | 1+1 | 3+3 | (4/3/1/1) 1+1 weak filter of VTM | (11/0/1/4) for “d” decision + + (4/3/1/1) for one sample / no filtering decision Total = 16 *4 + (9*8)= 176 | VTM | 2 |
| CE11.3.4 | 2+2/1+1 | 3+3 (padding for outermost sample) | (16/1/8/7) 2+2 with VTM strong filter | (7/0/1/0) for d” decision + (8/1/3/4) for strong filter Total 8 lines = 24*4 = 96 | VTM | 2 |
| CE11.3.5 | 0 | 0 | 0 | 0 | VTM | 1 |
| JVET-L0614 Test1 | 2+1/1+2/1+1/3+1/1+3 | 3+3 (padding for outermost sample) | (19/1/0/11) 3+1 with VTM strong filter | (11/0/1/4) for “d” decision + (8/1/3/4) for strong filter Total 8 lines = 32*4 = 128 | VTM | 2 |
| JVET-L0614 Test2 | 2+1/1+2/1+1/2+2/3+2/2+3 | 3+3 (padding for outermost sample) | (21/2/10/9) 3+2 with VTM strong filter | (7/0/1/0) for d” decision + (8/1/3/4) for strong filter Total 8 lines = 24*4 = 96 | VTM | 2 |

In the worst case for filtering for two 4x4 adjacent blocks, if strong filter is applied, it can change upto 3 samples maximum on either side of the edge. This would have more operations when compared to CE 11.3.1.

11.3.5 does not perform any deblocking for 4x N and N x 4 blocks and therefore has less complexity when compared to applying strong filter for two 4 x4 adjacent blocks.

### Chroma deblocking complexity (CE11.3)

| Tests | Samples from block bound. modified | Samples from block bound. for deblocking decision | Max num. oper for filtering per line (add/mult/compar/shift) | Max number of oper. for decision for 8-sample boundary (add/mult/compar/shift) | Num. line buffers | Number of checks to determine block size |
| --- | --- | --- | --- | --- | --- | --- |
| CE11.3.1 | VTM | VTM | VTM | VTM | VTM |  |
| CE11.3.3 S1 | VTM | VTM | VTM | VTM | VTM |  |
| CE11.3.3 S2 | VTM | VTM | VTM | VTM | VTM |  |
| CE11.3.4 | VTM | VTM | VTM | VTM | VTM |  |
| CE11.3.5 | 0 | 0 | 0 | 0 | VTM | 1 |
| JVET-L0614 Test1 | VTM | VTM | VTM | VTM | VTM |  |
| JVET-L0614 Test2 | VTM | VTM | VTM | VTM | VTM |  |

### Parallel processing (CE11.3)

| Tests | Min unit size in luma samples that can be processed separately |
| --- | --- |
| CE11.3.1 | 4x4 grid |
| CE11.3.3 S1 | 4x4 grid |
| CE11.3.3 S2 | 4x4 grid |
| CE11.3.4 | 4x4 grid |
| CE11.3.5 | 4x4 grid |
| JVET-L0614 Test1 | 4x4 grid |
| JVET-L0614 Test2 | 4x4 grid |

# Recommendation of technologies to study in the CE11

It was agreed that we need to reduce the number of technologies tested in the CE since there is high cost for subjectively evaluating them and it makes the subjective testing difficult.

One proponent suggested to have a screening of the proposals during this meeting to determine which proposals seem more promising.

One proponent suggested to limit a number of variants of proposals from one company, and a number of combinations (e.g. one proposal addressing one particular aspect).

Promising technologies can be identified separately for each area (long filter, 4x4 deblocking).

If visual screening occurs, the proponents are encouraged to submit the most promising variant for visual screening.

One participant suggested that smaller number of line buffers between the horizontal CTU boundaries are encouraged (smaller line buffers). It was also suggested to use adaptive QP in subjective testing in the next round of the CE.

It was agreed to contact Vittorio regarding pre-screening of the CE11.1 (long tap deblocking) proposals for participation in the next round of the CE (and discussing how such pre-screening can be done).

Another proponent commented that side discussions among participants regarding voluntary reducing of number of proposals in sub-CEs (to provide a voluntary alternative to trimming number of proposals in the CE).

## Candidate technologies to study in CE11.1

| No | Tests | Plans | Samp.modif | Samp. read | Max oper. filt. line (add/mult/compar/shift) | Max oper. decision for 8-sample boundary (add/mult/compar/shift) | Num. line buff. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CE11.1.1 | May be dropped in favor of 11.1.9 | 7+7, 5+5 | 16 | 120 (46,24,28,22) | 30 (15,0,13,2) * 8 = + 120 op | 8 |
| 2 | CE11.1.2 | to be replaced by L0393 | 7+7 | 7+7 | 246 (138, 12, 28, 68) | 20(10,0,6,4) * 2 = + 20 op | 8 |
| 3 | CE11.1.3 |  | 7+7, 7+4, 4+7, 4+4 | 16 | 266 (168, 34, 28, 36) | 25 (12,0,11,2) per line | 8 |
| 4 | CE11.1.4 S1 | Proponent prefers this one to 11.1.4 S2. Possibly with combination with some other proposals | 4+4/4+3 | 6+6/6+4 | 123 (56/39/0/28) | 32(20/0/2/10) per line*4 = + 120op | 6 |
| 5 | CE11.1.4 S2 | possibly remove (proponent to confirm) | 4+4/4+3 | 6+6/6+4 | 123 (56/39/0/28) | 32(20/0/2/10) per line* 4 = + 128 op | 6 |
| 6 | CE11.1.5 |  | 7+7, 3+7 | 16 | 190(124/12/0/54) | 5(3/0/1/1) per line* 4 = + 20 op | VTM |
| 7 | CE11.1.6 | This proposal + possibly asymmetric decisions | 7+7 | 16 | 146 (104,0,28,14) | 48(25,0,14,9) per line* 4 =+ 192 op | 8 |
| 8 | CE11.1.7 S1 | (proponent may consider dropping) | 7+7, 7+3, 3+7 | 16, 12 | 97* (55, 0, 28, 14) | Worst case is same as HEVC. 20 (11, 0, 5, 4) * 4 = + 80 op | 8 |
| 9 | CE11.1.7 S2 | (preferred by proponent) + aspects of L0572 | 7+7, 7+3, 3+7 | 16, 12 | 97* (55, 0, 28 14) | Worst case is same as HEVC. 20 (11, 0, 5, 4) per line*4 = + 80 op | 8 |
| 10 | CE11.1.8 |  | 7+7, 3 + 7 for hor. edges between CTUs | 16 | 142(96/4/0/42) | 20(14, 2,2,2) per line * 4 lines = + 80 op | VTM |
| 11 | CE11.1.9 = CE11.1.9 + chroma | (somewhat preferred by proponent vs 11.1) | 7+7, 5+5 | 16 | 120 (46,24,28,22) | 30 (15,0,13,2) per line for 32x32 blocks* 8 = + 120 op | 8 |
| 12 | CE11.1.10 = CE11.1.7 luma + CE11.1.8 chroma | (Proponent may prefer this to 11.1.7S2 but not sure yet) | 7+7, 7+3, 3+7 | 16, 12 | 97* (55, 0, 28, 14) | Worst case is same as HEVC. 20 (11, 0, 5, 4) per line*4 = + 80 op | 8 |
| 13 | CE11.1.11 = CE11.1.7 luma + CE11.1.8 chroma | (Proponent may prefer this proposal 11.1.7S2 but not sure yet) | 7+7, 7+3, 3+7 | 16, 12 | 120 (46,24,28,22) | Worst case is same as HEVC. 20 (11, 0, 5, 4) per line*4 = + 80 op | 8 |
| 14 | JVET-L0226 based on CE11.1.8 |  |  |  |  |  |  |
| 15 | JVET-L0393 based on CE11.1.2 | Will replace CE11.1.2 |  |  |  |  |  |
| 16 | JVET-L0460 based on CE11.1.11 (11.1.1+11.1.7) |  |  |  |  |  |  |
| 17 | JVET-L0523 New proposal |  |  |  |  |  |  |
| 18 | JVET-L0529 based on 11.1.10 (11.1.6+11.1.7) | possibly replace CE11.1.10 |  |  |  |  |  |
| 19 | JVET-L0572 based on CE11.1.7b and CE11.1.8 | some of the aspects of this may be merged into CE11.1.7 |  |  |  |  |  |

* Numbers obtained with taking into account certain possible optimizations

## Candidate technologies to study in CE11.2

| No | Tests | Plans | Samples from block bound. modified | Num. line buffers |
| --- | --- | --- | --- | --- |
| 1 | CE11.2.1 S1 |  | VTM | VTM |
| 2 | CE11.2.1 S2 |  | VTM | VTM |
|  | CE11.2.1 S3 | (to be removed) | VTM | VTM |
|  | CE11.2.1 S4 | (to be removed) | VTM | VTM |
|  | CE11.2.1 S5 | (to be removed) | VTM | VTM |
|  | CE11.2.1 S6 | (to be removed) | VTM | VTM |
|  | CE11.2.2 S1 |  | VTM | VTM |
|  | CE11.2.2 S2 | Planned to be adopted | VTM | VTM |

## Candidate technologies to study in CE11.3

| No | Tests | Plans | Samples from block bound. modified | Samples from block bound. for deblocking decision | Max num. oper for filtering per line (add/mult/compar/shift) | Max number of oper. for decision for 8-sample boundary (add/mult/compar/shift) | Num. line buffers | Num checks block size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CE11.3.1 |  | 2+1/1+2/1+1 | 3 + 3 | 1+1 15 (6,2,6,1) 2+1 22 (11,2,6,3) | Total 8 lines = 16 *2 = 32 | VTM | 3 |
| 2 | CE11.3.2 | keep | VTM | VTM | VTM | VTM | VTM | VTM |
| 3 | CE11.3.3 S1 |  | 3+1/1+3/1+1 | 3 + 3 | (19/1/0/11) 3+ 1 with VTM strong filter | Total 8 lines = 32*4 = 128 | VTM | 2 |
| 4 | CE11.3.3 S2 | possibly combine 3.1, 3.3, 3.4 | 1+1 | 3+3 | (4/3/1/1) 1+1 weak filter of VTM | Total = 16 *4 + (9*8)= 176 | VTM | 2 |
| 5 | CE11.3.4 |  | 2+2/1+1 | 3+3 | (16/1/8/7) 2+2 with VTM strong filter | Total 8 lines = 24*4 = 96 | VTM | 2 |
| 6 | CE11.3.5 | keep | 0 | 0 | 0 | 0 | VTM | 1 |
| 7 | JVET-L0614 Test1 | this may replace 11.3.1, 11.3.4 and 11.3.4 | 2+1/1+2/1+1/3+1/1+3 | 3+3 | (19/1/0/11) 3+1 with VTM strong filter | Total 8 lines = 32*4 = 128 | VTM | 2 |
| 8 | JVET-L0614 Test2 | this may replace 11.3.1, 11.3.4 and 11.3.4 | 2+1/1+2/1+1/2+2/3+2/2+3 | 3+3 | (21/2/10/9) 3+2 with VTM strong filter | Total 8 lines = 24*4 = 96 | VTM | 2 |

# Summary of Recommendations

The following recommendations have been made by the BoG:

- Continue CE11

  - Continue CE11.1 long-tap deblocking filtering

  - Continue CE11.3 filtering on 4x4 grid

  - Continue remaining items in CE11.2 on general aspects

  - Consider CE related proposals in new round of the CE (listed in 4.1-4.3)

  - Reduce number of tests in CE11

  - Choose separate set of sequences and QP for subjective test in CE11.3

- Adopt JVET-L0410 (tC table fix) to VVC