| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 03–12 Oct. 2018 | Document: JVET-L0684-v1 |
| --- | --- |

| Title: | BoG report on CE14 and CE14 related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Li Zhang | Email: | lizhang.idm@bytedance.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

Mandates according to the meeting notes:

“The most interesting technologies are 14.1a and 14.3b. These are directly competing technologies. Both have roughly same compression gain, and similar increase in encoding/decoding time. What might be of more concern (and might also lead to a decision adopting none of them) is the complexity added at a critical position in the decoding, which could cause latency issues in particular for intra coding. A detailed analysis on this shall be performed, documenting worst case number of operations, cycles, also including the possibility that a different LUT may need to be used for the Hadamard filter for each next CU if the QP is switched to a different range. SIMD complexity aspects should also be addressed.

BoG (L. Zhang) to further investigate, and also look into CE-related contributions.”

# Complexity analysis of CE14.1.a and CE14.3.b

This section provides the data for different complexity aspects for the two selected CE14 test sets. Table 1 shows which blocks sizes that post-reconstruction filters could be enabled or disabled. Table 2 tabulates the complexity part for the two CE14 proposals based on CE14 descriptions while Table 3 shows the additional data for complexity comparisons.

Table 1. Enabling and Disabling Post-Reconstruction Filters

| CE14.1.a CE14.3.b | 4x4 | 4x8 | 8x4 | 8x8 | 4xN or Nx4 (N>8) | 8xN or Nx8 (N>8) | 16xN or Nx16 (N>8) | 32xN or Nx32 (N>8) | 64xN or Nx64 (N>8) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intra | - | X | X | X | X | X | X | X | X |
| inter | - | X | X | X | X | X | X | - | - |

Table 2. Complexity analysis according to CE14 descriptions

| Test | filter shape | Comp. complex. per sample* | Precis. of mult | Parallel friendly | Latency for filtering process (in clock cycles) | Latency for buffering | Memory required (bytes) | How to derive filter coeffs | Min. and max. filtered CU size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14.1.a | 5 pixel “plus”-shape; For inter, 5x5 area is used to calculate filter weights. | Current software implementation: Intra: 4 mult 9 adds 4 checks Inter: 4 mult 23 adds 10 checks Maximum hardware parallelism: Inter 6 mult 36 adds 20 checks | Intra: 9×8 and 12×9 Inter: 9×8 and 12×11 | yes | 3 | X | 63 | Intra: Inter: | Min: 4x8, 8x4 Max: Intra: 64x64 Inter: 16x64, 64x16 |
| 14.3.b | 3x3 | 0 mult 20 adds + 4 1-bit add for rounding 6 checks | n/a | yes | 2 | X | 70 (16 7-bit values per CU ) | Pre-calculated in LUT | same as above |

Table 3. Additional Information on Complexity Analysis

|  | possibility that a different LUT may need to be used for the Hadamard filter for each next CU if the QP is switched to a different range. | SIMD complexity | Wo/ SIMD complexity | Operations before filtering | Sequential operation | Number of LUTs/Weights calculations per sample |
| --- | --- | --- | --- | --- | --- | --- |
| CE14.3.b | 16*7bits per CU | 104% for AI, 101% for RA (from JVET-L326, pending cross-check) | 110%, 104% and 104% for AI, RA, LDB |  | 5 add; 1 look-up table (14 bytes); 2 check | 3 |
| CE14.1.a |  | 101.5% for AI, (from JVET-L0067, pending cross-check) | 105%, 102%, 103% for AI, RA and LDB | 1 parameter with 9 bit , 1 mult and 1 add per CU before filtering | For intra: 2 mult, 6 add and 2 checks; For inter: 2 mult, 12 add and 2 checks (or 2 mult, 8 add and 2 checks with maximum hardware parallelism) | 2 |

## Comments from hardware experts

- It is agreed that there should be no post-reconstruction filter for 4x4 intra and inter.

- One expert commented that no post-reconstruction filter for 4x8 and 8x4 intra and inter blocks is preferred.

- One expert commented that no post-reconstruction filter for intra blocks.

- Two hardware experts mentioned that CE14.3.b requires smaller size area compared to CE14.1.a

# CE14 related contributions

JVET-L0049 AHG16: An architecture study of bilateral filters [Y. Hu, M. Zhou (Broadcom)]

Suggestions from the proponent: if bilateral filter is applied, it is suggested bilateral filter is only applied to inter-coded blocks.

Q: have you studied the new CE14.1 results?

A: CE14.1.a could fit 3 cycles latency for the filtering process. Latency for sliding window and buffering size depend on block size.

Q: How about CE14.3.b?

A: Other hardware experts commented that for CE14.3.b, 2 cycles latency for the filtering process is safe. 1 cycle latency for the filtering process is impossible.

JVET-L0584 CE14.2-related: Extended applicability of bilateral filter (CE14.2.c) [D. Rusanovskyy, N. Shlyakhov, M. Karczewicz (Qualcomm)] [late] [miss]

It was covered in the CE14 summary report. No need to be presented.

JVET-L0615 CE14-related: Inter-only bilateral filtering [J. Ström, P. Wennersten, J. Enhorn, D. Liu, K. Andersson, R. Sjöberg] [late]

This contribution describes a variant of CE14.1a from JVET-L0172, where bilateral filtering has been turned off for all intra blocks. The BD-rate figures for an implementation in BMS-2.0.1 running in VTM configuration are reported to be -0.45% / -0.57% for RA/LD respectively.

It is commented that if CE14.3.b is only applied to inter coded blocks, the coding gain is -0.40% for RA. Confirmed by proponents of CE14.3.b.

Cross-checker reported that performance for low resolution is confirmed.

JVET-L0656 CE14.3-related: Hadamard transform domain filter with modified LUT [S. Ikonin, V. Stepin, D. Kuryshev, A. Karabutov, J. Chen (Huawei)] [late]

This document is addressing to topic described in document JVET-L0636 CE14: Crosscheck of CE14.3 (JVET-L326) [1]. Detailed problem analysis is provided and solution is proposed. Proposed solution does not increase LUT size and provides additional BD-rate gain improvements up to -0.71% for RA configuration.

Compared to the CE14.3.c, the threshold where  switches from using the LUT to using  has been moved from 123 to 239.

The cross-check report in JVET-L0636_r2 mentioned that

- The modification solves the quantization issue with respect to a ramp of slope 61.5, but that is unsurprising: Since the discontinuity has moved from 123 to 239, the new ramp with artifacts is going to be for a slope 239/2 = 119.5.

- The discontinuity due to subsampled LUT will cause quantization effects in the filtered output, especially for HDR sequences.

Simulation results show that there is no coding loss for JVET-L0656.

No one has reported whether there is subjective issue for SDR sequences and for HDR sequences, no one has checked yet.

JVET-L0677 CE14 related: Decoder run time analysis non-SIMD and SIMD

# Conclusions

It is agreed that:

- If any tool from CE14 is adopted, the proponents are requested to provide data for subjective viewing to check whether there are artifacts due to the adopted technology.

- If there is another round of CE, cross-checkers are requested to launch subjective viewing tests on SDR and HDR sequences.