| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0685 |
| --- | --- |

| Title: | BoG report on CE6.1: Primary transform and related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | X. Zhao | Email: | xinzzhao@tencent.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This BoG on CE6.1 is mandated to review the following three primary transform designs studied in CE6.1, and related contributions:

- COT with fast implementation similar to 6.1.4 (CE6-1.7g + CE6-1.4a)

- Aligning of MTS with DCT-2 basis (per 6.1.7a) with fast implementation (CE6-1.7a)

- Adjustment preprocessing of 6.1.6 (CE6-1.6)

Analysis of complexity, number of cycles to compute inverse transform, necessary bit depth of implementation stages is provided (confirmed by proponents). Comparisons versus complexity of fast implementations of DCT-2 are also provided.

The BoG met on Saturday October 7th at 18:20 to 22:20.

# BoG Report

Three primary transform designs proposed in CE6.1 that are considered as interesting candidates:

- C1: COT with fast implementation similar to 6.1.4 (CE6-1.7g + CE6-1.4a)

- C2: Aligning of MTS with DCT-2 basis (per 6.1.7a) with fast implementation (CE6-1.7a)

- C3: Adjustment preprocessing of 6.1.6 (CE6-1.6)

- C3a: CE6-1.6a

- C3b: CE6-1.6b

It was commented that CE6-1.7g + CE6-1.4a was not tested in CE.

Ideally, the transform should have the following properties:

- Sharing of as much as possible building blocks for different transform types and sizes

It was commented the term of “building blocks” need to be clarified.

- Implementation either as matrix multiply or fast algorithm, independent of specification

A fast algorithm refers to a method that has either benefit for HW or SW implementation. If it is noted that an algorithm has disadvantage for HW or SW implementation, it needs to be described.

It was commented that it should be clarified that whether a fast algorithm is applicable for encoder, decoder or both.

It was commented by the proponent of C2 that, “any fast implementation of DST-4/DCT-4 transforms lead fast implementation of the DCT-2 transform for double size, and any faster algorithm than the current partial butterfly for dct2 leads to faster DST-4/DCT-4 transforms, as they are only the odd basis. Thus, the current proposal aims at being fully compatible with the current dct2 design. Further normative/non-normative improvement of dct2 will improve the DST-4/DCT-4 transforms design.”

- C1: It is claimed to have a fast algorithm, which supports dual implementation, which means it can be implemented as either matrix multiply or fast algorithm, independent of specification. The fast algorithm is not DCT-2 style.

- C2: There is no fast algorithm available yet. However, it is commented by the proponent that maybe fast algorithm for C2 is available and can reduce each operation but need to confirm. The proponent mentioned that evidence will be shown next JVET meeting.

- C3a: This method does not support dual implementation. It is commented that the proposed method can be implemented as filtering operations stage added by matrix multiplication or DCT-2 style fast algorithm.

- C3: Same comments of C3a also applies.

- Implementation with 16-bit logic (at least for 10-bit video)

  - C1: Support 16-bit logic

- C2: Support 16-bit logic

- C3a: The CE test does not support 16-bit logic, but it is mentioned by the proponent that a modified implementation supports 16-bit logic between two stages (as proposed in JVET-L0682).

- C3b: Same comment of C3a applies.

- As low complexity as possible.

HW:

- It was questioned whether this non-DCT-2 style fast algorithm benefit for HW, but it does no harm vs matrix multiplication. It was commented that the fast algorithm may be more complex than DCT-2 butterfly. It was commented that the fast algorithm does not benefit for hardware. It was also commented that the fast algorithm may be not easy to implement for HW, however, it has a fall back solution, which is full matrix multiplication.

- C2: Same as matrix multiplication.

- C3a: It was commented by a HW expert that the fast method does not fit their decoder transform HW design because it has different input bit-depth of the normal forward DCT-2, and it does not have a fall back solution (single matrix multiplication). It was mentioned in JCTVC-G132 that forward+inverse DCT-2 has 17-21% more area than inverse DCT-2 only, it is unknown what is the HW cost to implement DST-7/DCT-8.

- C3b: Same comments of C3a apply to C3b.

SW:

- C1: The proposal is for 16-pt and 32-pt transforms used for MTS, but can be extended to 64-pt. Evidence on the benefit has been shown for SW (in terms of run-time). It was commented that HEVC 4x4 DST-7 has a similar fast algorithm implemented in the reference SW.

- C2: No fast algorithm.

- C3a: It was commented that the proposal may not benefit for SW optimization, and it could be even worse than full matrix multiplication, because the forward DCT-2 is used in decoder, which applies multiplications with over 16-bit input due to butterfly first structure of forward DCT-2. It was commented that it is the most important for SIMD optimization to have 16-bit multiplication. It was commented that forward DCT-2 should be avoided at the decoder for SW.

- C3b: Same comments of C3a apply to C3b.

Others:

- C1:

- C2: No fast algorithm

- C3a: It was commented by the proponent that the algorithm still benefit for encoder side. It was also mentioned that there was a JCTVC contribution JCTVC-G132, claiming that matrix multiplication is a useful feature for some scenarios, including DSP, ARM. However, it is questioned by the proponent whether this contribution has valid conclusion.

- C3b: Same comments of C3a apply to C3b.

- Re-usability of legacy building blocks might be desirable

  - C1: It was commented that existing implementations faster than matrix multiply would not be able to utilize the benefit of this proposal.

  - C2: It was commented that existing implementations faster than matrix multiply would not be able to utilize the benefit of this proposal.

  - C3a: It was commented that existing implementations faster than matrix multiply would not be able to utilize the benefit of this proposal. It was commented by the proponent that the proposed method re-use current DCT-2 with adjustment stages, and different opinions have been expressed by a non-proponent.

  - C3b: Same comment of C3a also applies.

- Specification as matrix multiply, or cascade of matrix multiplies, or other e.g. butterfly

  - C1: Single matrix multiply

  - C2: Single matrix multiply

  - C3a: Cascade of two matrix multiplies for 16-point and 32-point MTS transforms.

  - C3b: Cascade of two matrix multiplies for 16-point and 32-point MTS transforms.

It was commented that bullet #6 duplicates #7.

- Extraction of smaller transform sizes from largest size 64 (32 for MTS transforms)

For the following methods, the extraction of smaller transform sizes includes a sub-sampling of the largest transform core coefficients.

  - C1: For applying DCT-2, same as HEVC. For applying other transform types (DST-7, DCT-8), extraction (from a 64x64 matrix) is done according to transform type, where offsets are different for different transform cores, and sign changes are required.

  - C2: For applying DCT-2, same as HEVC. For applying other transform types (DST-4, DCT-4), extraction (from a 64x64 matrix) is done according to transform type, where offsets are different for different transform cores, and sign changes are required.

  - C3a: For applying DCT-2, same as HEVC. For applying other transform cores, no need to extract DST-7, DCT-8, but it is needed to extract DCT-2 and apply transpose, additional adjustment stage is required, but not for extraction.

  - C3b: Same comments of CE3a also applies.

Furthermore, each of the methods should be implementable with 16-bit logic (However, it should be investigated whether this might have impact on low QP cases)

It was requested in CE6 description document that:

- For experiments which have impact on the transform precision, i.e., modifications on shift operations in transform, modifications on bit-depth of intermediate data representation, results using low-QP configurations, i.e., QP 5, 10, 15 and 20 shall be provided as supplemental information.

- C1:

  - For the method proposed in CE6-1.4a: the low QP results will be provided in a late contribution.

- C2:

  - Low QP results not available.

- C3a:

  - For the method proposed in CE6-1.6a: the low QP results will be provided in a late contribution.

- C3b:

  - Low QP results not available.

Analysis of complexity, number of cycles to compute inverse transform, necessary bit depth of implementation stages, etc. should be provided (confirmed by proponents).

Complexity (operation counts):

Primary transform used for MTS0 (e.g., DCT-2, COT):

| Size | Mult |  |  | Add/Sub |  |  | Shift |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | C3 | C1 | C2 | C3 | C1 | C2 | C3 |
| 16-pt | 86 | 86 | 86 | 100 | 100 | 100 | 16 | 16 | 16 |
| 32-pt | 213 | 342 | 342 | 287 | 372 | 372 | 32 | 32 | 32 |
| 64-pt | 833 | 1366 | 1366 | 1069 | 1428 | 1428 | 64 | 64 | 64 |

It was commented by the proponent of C2 that, maybe fast algorithm for C2 is available and can reduce each operation, but it needs to be confirmed. The proponent will show the evidence by next meeting.

Primary transform used for MTS1 (e.g., DST-7/DCT-8, DST-4/DCT-4):

| Size | Mult |  |  |  | Add/Sub |  |  |  | Shift |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | C1 | C2 | C3a | C3b | C1 | C2 | C3a | C3b | C1 | C2 | C3a | C3b |
| 16-pt | 127 | 256 | 182 | 160 | 155 | 240 | 210 | 203 | 16 | 16 | 34 | 34 |
| 32-pt | 620 | 1024 | 534 | 470 | 718 | 992 | 594 | 587 | 32 | 32 | 66 | 66 |

Number of stages to compute inverse transform:

| Size | Mult |  |  |
| --- | --- | --- | --- |
|  | C1 | C2 | C3 |
| 16-pt | 1 | 1 | 2 |
| 32-pt | 1 | 1 | 2 |

Necessary bit depth of implementation stages:

C1:

Internal bit-depth may go up to 18-bit

C2:

Full matrix multiplication

C3:

16-pt DST-7/DCT-8: Internal bit-depth may go up to 19-bit

32-pt DST-7/DCT-8: Internal bit-depth may go up to 20-bit

Between Forward DCT-2 and adjustment: at least 20-bit (current CE code), it is mentioned that there is a modified version which has 16-bit internal bit-depth between the Forward DCT-2 and adjustment, the filter coefficients used for the adjustment stage is up to 128 (which may be regarded as 9-bit coefficient, but the range is within 8-bit).

Output is 16-bit

Memory (for storing primary transform cores based):

| VTM-2.0.1 | C1 | C2 | C3a | C3b |
| --- | --- | --- | --- | --- |
| 8.52 KB | 5.12 KB | 5.12 KB | 5.48KB | 5.33 KB |

Conclusions:

It was commented by several non-proponents representing HW companies that none of these three candidates would be a solution that they are confident to provide a complexity reduction compared to matrix multiply. They would encourage further work to find solution that would provide complexity reduction for HW implementations.

CE related contributions:

JVET-L0060 CE6-related: Unified matrix for transform [K. Choi, K. P. Choi (Samsung)]

This contribution presents a unified matrix for all kernels of transform. Currently, VTM2.0 uses three types of transform as DCT2, DCT8, and DST7 depending on supported block size. The proposed a matrix in this contribution can provide all three types of transform with all block sizes from 4x4 to 64x64 by using a 64x64 matrix designed with 8-bit representation. Testing results on the proposed method show 0.0%, 0.1%, 0.1% and 0.2% BD rate losses on average for All Intra (AI), Random Access (RA), Low Delayed B (LDB), and Low Delayed P (LDP) configurations, respectively, compared to VTM configure setting, and 0.0%, 0.1%, and 0.0% BD rate losses on average for AI, RA, and LDP respectively, and 0.1% gain for LDB configuration, compared to BMS configure setting.

A method of combining CE6-1.7g and 8-bit transform is proposed, however, the 8-bit transform is different from the one proposed in CE6-1.3. Further study is recommended if the current CE6-1.7g is to be continued.

JVET-L0304 CE6-Related: Multiplication Free Transform [M. Salehifar, M. Koo, S. Paluri, J. Lim, S. Kim (LGE)]

This contribution proposes multiplication free transform which consists of only additions (subtraction) and shifts, which could be applied for any transform including MTS candidates. The proposed technique is tested with different maximum number of the allowable terms for the shifts and additions. In particular it is tested with: 1) Allowing 5 terms of shifts, 2) Allowing 4 terms of shifts, and 3) Allowing 3 terms of shifts.

As first test results with allowing maximum 5 terms of shifts compare to VTM anchor, has identical performance.

As second test results with allowing maximum 4 terms of shifts compare to VTM anchor, BD-rate difference of 0.00% (AI), 0.01% (RA), and -0.01% (LDB) is observed.

As third test results with allowing maximum 3 terms of shifts compare to VTM anchor, BD-rate difference of 0.11% (AI), 0.05% (RA), and -0.01% (LDB) is observed.

It was commented that the proposed method may be too dedicated for particular transform type and may not really benefit in case different transform types need to be supported.

JVET-L0353 CE6-related: MTS using DST-4 and transposed DCT-2 [Y. Lin, J. Zheng, Q. Yu, N. Zhang (HiSilicon), C. Zhu (UESTC)]

This document presents a low-complexity MTS (multiple transform selection) approach for VVC. It is proposed to use DST-4 and transpose of DCT-2 as transform cores for the MTS, due to benefits of reusing circuit implementation of existing DCT-2 transform, reusing transform coefficients of existing DCT-2 transform and enabling partial butterfly implementation for the proposed transpose of DCT-2. Test results reportedly show coding performance of 0.06%/0.06%/-0.04%/0.03% on average for AI/RA/LDB/LDP configurations compared to BMS-2.0.1 with VTM configuration. Encoding and decoding time slightly decrease due to partial butterfly implementation of the proposed transpose of DCT-2.

It was noted that a forward DCT-2 is needed at the decoder side. The proposed method enables DCT-2 butterfly for one MTS transform type, and it is one stage calculation, for the other transform type, DST-4 is used, which does not have a fast method currently. The performance loss is relatively higher for Class A, average loss is small (below 0.1%). It was mentioned by the proponent that the coding performance of the proposed method is slightly better than CE6-1.7a under CTC. The proposed method looks interesting for complexity reduction of MTS transform types. Further study in CE is recommended.