| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L1022-v1 |
| --- | --- |

| Title: | Description of Core Experiment 2 (CE2): Sub-block based Motion Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Yuwen He, Ching-Yeh Chen, Chun-Chi Chen | Email: | yuwen.he@interdigital.com, chingyeh.chen@mediatek.com, chunchic@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of Core Experiment 2 (CE2) is to investigate sub-block based motion prediction techniques on top of VTM3.0. It comprises 5 categories,

- CE 2.1: Affine motion compensation

- CE 2.2: Affine merge mode

- CE 2.3: Sub-block based merge mode

- CE 2.4: Memory bandwidth reduction

- CE 2.5: ATMVP related

# CE2.1: Affine Motion Compensation

## Test specification

Please fill each test/sub-test in a single line here, e.g., 2.x.x or 2.x.x.1, and the source of the test, JVET-L0xxx. The test number will be assigned later by categorizing similar contributions together if needed.

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 2.1.1 | Adaptive precision for affine MVD coding | JVET-L0259 | D. Luo (InterDigital) |  |
| 2.1.2 |  | JVET-L0332 |  |  |

## Test condition and evaluation criteria

## Technical description

Please provide the description of each test/sub-test in the section of test specification

### Adaptive precision for affine MVD coding (JVET-L0259, InterDigital)

Test: 2.1.1

Description: Adaptive precision is proposed for affine MVD signaling. Three precisions are proposed for affine MVD coding at those control points. The precision set {prec_TL, prec_TR, prec_BL} is used to indicate the MVD precision for top-left, top-right and bottom-left control points. For 6-parameter affine model, three precision sets are used: {1-pel, 1/4-pel, 1/4-pel}, {1/4-pel, 1/4-pel, 1/4-pel}, and {1/8-pel, 1/8-pel, 1/8-pel}. For 4-parameter affine mode, the same three precisions are used, except that the precision for bottom-left control point is not needed.

### Xxx (JVET-Lxxx, company)

Test: 2.1.x

Description: TBD.

### Xxx (JVET-Lxxx, company)

Test: 2.1.x

Description: TBD.

# CE2.2: Affine Merge Mode

## Test specification

Please fill each test/sub-test in a single line here, e.g., 2.x.x or 2.x.x.1, and the source of the test, JVET-L0xxx. The test number will be assigned later by categorizing similar contributions together if needed.

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 2.2.1 |  | JVET-L0193 |  |  |
| 2.2.2 |  | JVET-L0194 |  |  |
| 2.2.3.a | Generation of a table to store motion information of coded affine CUs | JVET-L0305 | J. Zhao (LGE) G. Li (Tencent) |  |
| 2.2.3.b | The derivation of affine HMVP candidate for merge list from the stored affine motion information. |  |  |  |
| 2.2.3.c | Simplified pruning for affine HMVP candidates |  |  |  |
| 2.2.3.d | Applying HMVP candidates for affine motion vector prediction |  |  |  |
| 2.2.3.e | Combined test of x.x.4 and affine HMVP for merge |  |  |  |
| 2.2.4.a | Affine merge with offsets and CP level signaling | JVET-L0320 | G. Li (Tencent) |  |
| 2.2.4.b | Affine merge with offsets and block level signaled |  |  |  |
| 2.2.4.c | Affine merge with offset and extension of value range |  |  |  |
| 2.2.4.d | POC distance based offset mirroring for Bi-prediction |  |  |  |
| 2.2.4.e | POC distance based offset scaling for Bi-prediction |  |  |  |
| 2.2.5 | Affine MV offset merge candidates | JVET-L0389 | Y.-C. Yang (Foxconn) |  |
| 2.2.6.a | Simplification for 6-param constructed candidates | JVET-L0390 | Y.-J. Chang |  |
| 2.2.6.b | Simplification for 4-param constructed candidates |  | (Foxconn) |  |
| 2.2.6.c | 2.2.6.a + 2.2.6.b |  |  |  |
| 2.2.7a | a) Low complexity affine merge with temporal candidates | JVET-L0522 | F. Galpin (Technicolor) |  |
| 2.2.7b | b) Medium complexity affine merge with temporal candidates |  |  |  |
| 2.2.7c | c) Improved affine flag coding |  |  |  |
| 2.2.7d | 2.2.7a + 2.2.7c |  |  |  |
| 2.2.7e | 2.2.7b + 2.2.7c |  |  |  |

## Test condition and evaluation criteria

## Technical description

Please provide the description of each test/sub-test in the section of test specification

### Xxx (JVET-Lxxx, company)

Test: 2.2.x

Description: TBD.

### Xxx (JVET-Lxxx, company)

Test: 2.2.x

Description: TBD.

### History Based Affine Merge Candidate (JVET-L0305 LGE & Tencent)

Test: 2.2.3

Description:

In these tests, history based affine merge candidate (Affine HMVP) methods for affine merge list generation are investigated by considering the following aspects.

- Generation of a table to store motion information of coded affine CUs

- The derivation of affine HMVP candidate for merge list from the stored affine motion information.

### Affine merge with prediction offset (JVET-L0320, Tencent)

Test: 2.2.4

Description:

These tests affine merge with prediction offset were proposed in JVET-L0320.

The proposed method selects the first available affine merge candidate as a base predictor. Then it applies a motion vector offset to each control point’s motion vector value from the base predictor. If there’s no affine merge candidate available, the proposed methods will not be used.

Two signaling methods were proposed:

- Per control point (CP) signaling

The selected base predictor’s inter prediction direction, and the reference index of each direction is used without change.

For each control point, a zero_MVD flag is used to indicate whether the control point needs offset signaling. If zero_MVD flag is true, there’s no other signaling needed for the control point. Otherwise, a distance index and an offset direction index is signaled for the control point.

Distance index is signaled to indicate which distance offset to use from the offset table.

| Distance IDX | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Distance-offset | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel |

The direction index can represent four directions as shown below, where only x or y direction may have an MV difference, but not in both directions.

| Offset Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-dir-factor | +1 | –1 | 0 | 0 |
| y-dir-factor | 0 | 0 | +1 | –1 |

If the inter prediction is Uni-prediction, the signaled distance offset is applied on the offset direction for each control point predictor. Results will be the MV value of each control point.

If the inter prediction is Bi-prediction, the signaled distance offset is applied on the signaled offset direction for control point predictor’s L0 motion vector; the offset to be applied on L1 MV is applied on a mirrored basis.

- Per block signaling

In this method, the distance offset index and the offset direction index are signaled per block. The same offset will be applied to all available control points. The number of control points is determined by the base predictor’s affine type. The distance offset table and the offset direction tables are the same as in 1).

Since the signaling is done for all the control points of the block at once, the zero_MVD flag is not used in this method.

### Affine MV offset merge candidates (JVET-L0389, Foxconn)

Test: 2.2.5

Description:

New Affine merge candidates are generated based on the MVs offsets of the first Affine merge candidate. In Uni-prediction, the MV offsets are applied to the MVs of the first candidate. In Bi-prediction with List 0 and List 1 on the same direction, the MV offsets are applied to the first candidate as follows:

MVnew(L0), i = MVold(L0) + MVoffset(i)

MVnew(L1), i = MVold(L1) + MVoffset(i)

In Bi-prediction with List 0 and List 1 on the opposite direction, the MV offsets are applied to the first candidate as follows:

MVnew(L0), i = MVold(L0) + MVoffset(i)

MVnew(L1), i = MVold(L1) - MVoffset(i)

On top of new adoptions in Affine merge aspect, this sub-test will test various offset directions combined with various offset magnitudes for the proposed Affine MV offset merge candidates.

### Simplification of the constructed Affine merge candidates (JVET-L0390, Foxconn)

Test: 2.2.6

Description:

The first aspect in JVET-L0390 is related to reduction of the maximum number of available merge candidates. This test focuses on the simplification of the constructed Affine merge candidates. The modifications are as follows:

- At most M constructed candidates are allowed to be selected out of 6-parameter Affine model (6-param) constructed candidates

- At most N constructed candidates are allowed to be selected out of 4-parameter Affine model (4-param) constructed candidates

The modifications can reduce the number of if-else checks.

### Constructed temporal candidates (JVET-L0522, Technicolor)

Test: 2.2.7

Description:

The tests driven in this section will be performed on top of the VTM 3 for affine merge mode by using the concepts described in JVET-L0522 (constructed temporal candidates and improved affine flag coding).

# CE2.3: Sub-block based merge mode

## Test specification

Please fill each test/sub-test in a single line here, e.g., 2.x.x or 2.x.x.1, and the source of the test, JVET-L0xxx. The test number will be assigned later by categorizing similar contributions together if needed.

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 2.3.1.a | Planar Motion Vector Prediction (PMVP) as a single mode | JVET-L0070 | Z. Zhang (Huawei) |  |
| 2.3.1.b | PMVP with sub-block MV clip as a single mode |  |  |  |
| 2.3.1.c | PMVP as a candidate in sub-block (affine) merge list |  |  |  |
| 2.3.1.d | PMVP combination test with other sub-block method |  |  |  |
| 2.3.2.a | the threshold is set as 1 pixel for the sub-blocks inside the 4x16, 16x4 and 8x8 blocks | JVET-L0319 | M. Gao (Tencent) |  |
| 2.3.2.b | the threshold is set to 0 pixel for 4x16 and 16x4 blocks along vertical and horizontal directions, respectively; and the threshold is set as 1 pixel for the sub-blocks inside 8x8 blocks |  |  |  |
| 2.3.3.a | Regression-based Motion Vector Field (RMVF) as a separate merge mode | JVET-L0171 | R. Ghaznavi-Youvalari (Nokia) |  |
| 2.3.3.b | RMVF combined with the affine merge modes |  |  |  |
| 2.3.3.c | RMVF combined with the other merge modes |  |  |  |

## Test condition and evaluation criteria

## Technical description

Please provide the description of each test/sub-test in the section of test specification

### Planar Motion Vector Prediction (JVET-L0070, Huawei)

Test: 2.3.1

Description:

To generate a smooth fine granularity motion field, Figure 1 gives a brief description of the planar motion vector prediction (PMVP) process.

Planar motion vector prediction is achieved by averaging a horizontal and a vertical linear interpolation on sub-block basis as follows.

# Figure 1. Planar motion vector prediction process

- Separately signaled for each coding unit.

- Once there are two different MVs in the L-shaped area, the mode is activated.

- Ref_idx in L0 and L1 is set to 0, all the MVs in the L-shaped area are scaled if needed.

- In the case a neighboring block in the L-shaped area is intra coded, it is padded in the same manner as intra reference pixel padding, separately for L0 and L1.

- BR temporal motion is used for planar interpolation. If BR temporal motion is not available, exactly the same planar prediction for intra mode is applied.

- It is a merge mode.

Bi-directional prediction is assumed for the worst case. uSize denotes the sub-block size. W and H denote the width and height of a block.

### CE4-related: Sub-block MV clipping in planar motion vector prediction (JVET-L0319, Tencent)

Test: 2.3.2

Description: The MVs for all 4x4 sub-blocks inside the 4x16, 16x4 or 8x8 blocks are constrained such that the max difference between integer parts of these sub-blocks is no more than a given threshold.

### Merge Mode with Regression-based Motion Vector Field (JVET-L0171, Nokia)

Test: 2.3.3

Description:

JVET-L0171 proposed a new sub-block merge mode with Regression-based Motion Vector Field (RMVF). RMVF uses a 6-parameter motion model for calculating the motion vectors of sub-blocks.

MVX_subPUMVY_subPU= axxaxyayxayyXsubPUYsubPU+ bxby

Motion parameters are calculated based on one line and row of spatially neighboring 4x4 sub-blocks using their motion vectors and center locations as an input to a linear regression method.

In this test RMVF is implemented as a new merge mode in VTM-3.

As part of the experiment the complexity aspects of the tool will be analyzed.

# CE2.4: Memory Bandwidth Reduction

## Test specification

Please fill each test/sub-test in a single line here, e.g., 2.x.x or 2.x.x.1, and the source of the test, JVET-L0xxx. The test number will be assigned later by categorizing similar contributions together if needed.

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 2.4.1 |  | JVET-L0046/JVET-L0048 |  |  |
| 2.4.2 |  | JVET-L0104 |  |  |
| 2.4.3 | Restricted motion vector field for affine mode | JVET-L0122 | J. Li (Panasonic) |  |
| 2.4.4 | Affine restrictions for the worst-case bandwidth reduction - The worst-case bandwidth is bi-prediction of an INTER_8x4/INTER_4x8 block. - The worst-case bandwidth is uni-prediction of an INTER_4x4 block. - The worst-case bandwidth is bi-prediction of an INTER_8x8 block. | JVET-L0396 | L. Pham Van (Qualcomm) |  |
| 2.4.5 | Modified affine inheritance from above CTU | JVET-L0273 | H. Huang (Qualcomm) |  |
| 2.4.6 | Size constrain for inherited affine motion prediction | JVET-L0281 | H. Huang (Qualcomm) |  |
| 2.4.7 |  | JVET-L0330 |  |  |
| 2.4.8 | Affine sub-block size restrictions - 8x4 for BI, 4x4 for UNI - 8x4 for BI and UNI - 4x8 for BI, 4x4 for UNI - 4x8 for BI and UNI - Adaptive 8x4/4x8 for BI, 4x4 for UNI - Adaptive 8x4/4x8 for BI and UNI - 8x8 for BI, 4x4 for UNI - 8x8 for BI and UNI | JVET-L0371 | H. Chen (Huawei) |  |
| 2.4.9 | Clipping sub-block MVs within 8x8 block in affine prediction | JVET-L0317 | M. Gao (Tencent) |  |

## Test condition and evaluation criteria

## Technical description

Please provide the description of each test/sub-test in the section of test specification

### Xxx (JVET-L0xxx, company)

Test: 2.4.1

Description: TBD

### Xxx (JVET-L0xxx, company)

Test: 2.4.2

Description: TBD

### Restricted motion vector field for affine mode (JVET-L0122, Panasonic)

Test: 2.4.3

Description:

To reduce the worst-case memory bandwidth of an affine CU, the sub-block motion vectors of the affine CU are constrained to be within a pre-defined motion vector field which is calculated according to the motion vector of first sub-block, the size of the affine CU and the prediction direction of the affine CU (i.e. uni-prediction or bi-prediction). Assume that the memory is retrieved per CU instead of per sub-block and the motion vectors of first (top left) and the second sub-blocks are (v0x,v0y) and (vix, viy), values of vix and viy exhibit the following constraints:

vix ∊ [v0x-H, v0x+H],

viy ∊ [v0y-V, v0y+V].

The values H and V are chosen to guarantee that the worst case memory bandwidth of the affine CU does not exceed that of normal inter MC of a 8×8 bi-prediction block. If the motion vector of any sub-block exceeds the pre-defined motion vector field, the motion vector is clipped. An illustration of the idea of constrained sub-block motion vector is given in the below figure.

### Affine restrictions for the worst-case bandwidth reduction (JVET-L0396, Qualcomm)

Test: 2.4.4

Description:

In this test, the performance of affine restrictions for worst-case bandwidth reduction is evaluated.

1) The worst-case bandwidth is bi-prediction of an INTER_8x4/INTER_4x8 block.

2) The worst-case bandwidth is uni-prediction of an INTER_4x4 block.

3) The worst-case bandwidth is bi-prediction of an INTER_8x8 block.

### Modified affine inheritance from above CTU (JVET-L0273, Qualcomm)

Test: 2.4.5

Description:

In the proposed method, deriving a 6-parameter affine inherited candidate from a neighboring block in the above CTU is illustrated in Fig. 1. The top-left CPMV v0 and top-right CPMVs v1 are derived by the the bottom-left MV vLB and bottom-right MV vRB of the neighboring candidate block:

v0=vRB-vLBneiW*posCurX-posNeiX+vLB

v1=vRB-vLBneiW*posCurX+curW-posNeiX+vLB

Wherein neiW is the width of the neighboring block, curW is the width of current block, posNeiX is the x coordinate of the top-left pixel of neighboring block, and posCurX is the x coordinate of the top-left pixel of current block.

The bottom-left CPMV v2 is derived as follows:

If the MV from left neighbor vleft is available and has the same reference picture as  vLB

v2 =vleft;

Else if the MV from bellow-left neighbor vbellowleft is available and has the same reference picture as  vLB

v2 =vbellowleft;

Else

deltaHorX,deltaHorY=vRB-vLBneiW

∇vVer=(-deltaHorY,deltaHorX)

v2 =vRB-vLBneiW*posCurX-posNeiX+∇vVer*curH+vLB

Wherein curH is the height of current block.

### Size constrain for inherited affine motion prediction (JVET-L0281, Qualcomm)

Test: 2.4.6

Description:

It’s proposed to disable inherited affine motion prediction from a neighboring affine coded block if the width or height of the neighboring block is less than threshold.

### Xxx (JVET-L0xxx, company)

Test: 2.4.7

Description: TBD

### Affine sub-block size restrictions (JVET-L0371, Huawei)

Test: 2.4.8

Description: In affine mode, different sub-block shapes are used, which depend on the prediction direction and the CU size, as shown in Table 1.

  - For uni-prediction affine coded CU, the sub-block size is set equal to 4x4.

  - For bi-prediction affine coded CU, and if the width of the CU is larger than or equal to its height, the sub-block size is set equal to 8x4;

  - For bi-prediction affine coded CU, and if the height of the CU is smaller than its height, the sub-block size is set equal to 4x8;

Table 1 Sub-block shapes

| Prediction direction and CU size | Sub-block size |
| --- | --- |
| uni-prediction | 4x4 |
| bi-prediction, CU width >= CU height | 8x4 |
| bi-prediction, CU width < CU height | 4x8 |

Affine sub-block restrictions:

- 8x4 for BI, 4x4 for UNI

- 8x4 for BI and UNI

- 4x8 for BI, 4x4 for UNI

- 4x8 for BI and UNI

- Adaptive 8x4/4x8 for BI, 4x4 for UNI

- Adaptive 8x4/4x8 for BI and UNI

- 8x8 for BI, 4x4 for UNI

- 8x8 for BI and UNI

### Sub-block MV clipping in affine prediction (JVET-L0317, Tencent)

Test: 2.4.9

Description: For affine prediction, it is proposed to constrain the MVs of four 4x4 sub-blocks within one 8x8 block such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel.

# CE2.5: ATMVP related

## Test specification

Please fill each test/sub-test in a single line here, e.g., 2.x.x or 2.x.x.1, and the source of the test, JVET-L0xxx. The test number will be assigned later by categorizing similar contributions together if needed.

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 2.5.1 |  | JVET-L0092 |  |  |
| 2.5.2 |  | JVET-L0105 |  |  |
| 2.5.3 | non-sub-block ATMVP | JVET-L0119 | K. Abe (Panasonic) |  |
|  |  | JVET-L0207 |  |  |
| 2.5.4 | Double ATMVP with uniformity check | JVET-K0245 | Yuling Hsiao (MediaTek) |  |

## Test condition and evaluation criteria

## Technical description

Please provide the description of each test/sub-test in the section of test specification

### Xxx (JVET-L0xxx, company)

Test: 2.5.1

Description: TBD

### Xxx (JVET-L0xxx, company)

Test: 2.5.2

Description: TBD

### Non-sub-block ATMVP (JVET-L0119, Panasonic)

Test: 2.5.3

Description:

The proposed non-sub-block ATMVP is based on the existing ATMVP. The corresponding area is derived by the same method as current ATMVP. But it selects a set of MV in the centre position of corresponding area for entire target CU as shown in the figure below. The derived MV is CU unit, therefore, all processes after MV derivation are just same as other merge candidates. And it can also solve the subjective image quality problem of strong sub-block boundary noise.

### Double ATMVP with uniformity check (JVET-K0245 and JVET-J0018, MediaTek)

Test: 2.5.4

Description:

### In this test, two MVs from merge candidate list instead of one MV are used as temporal MVs to generate two ATMVP candidates. The first and second temporal MVs are the first and second available MVs in the merge candidate list, respectively. There are two types of uniformity checks; one is to check whether the motion information at the four corners of ATMVP sub-blocks is the same as the motion information at the center, and treat it as a normal temporal candidate (non-sub-block ATMVP) if they are the same; the other is to check whether the motion information at each of the four corners is the same for the two corresponding ATMVP sub-blocks, and the second ATMVP candidate is not inserted if they are the same at all the four corners. The no pruning of the first ATMVP candidate and second ATMVP candidate will also be evaluated.

# Test condition and evaluation criteria

Each testing item is required to be compared against VTM, evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in JVET-L1010. Simulation with RA and LDB configurations are mandatory.

Sub-CE specific test condition and evaluation criteria may be defined for a sub-CE if considered informational for tool evaluation. Please refer to the section of test condition and evaluation criteria in each sub-CE for information.

# SW integration guideline

All tests are required to be implemented on top of VTM3.0.

# Timeline

| T1 | xxx. xx, 2018 | Finalize CE description |
| --- | --- | --- |
| T2 | xxx. xx, 2018 | Deliver software packages and begin cross-checking |
| T3 | xxx. xx, 2018 | Software frozen, testers provide full results, and final crosschecking begins |

# Participant list

| Company/ Organization | Participant Name | Email Address |
| --- | --- | --- |
| Technicolor | F. Galpin T. Poirier A. Robert P. Bordes E. Francois F. Le Léannec | antoine.robert@technicolor.com franck.galpin@technicolor.com tangi.poirier@technicolor.com philippe.bordes@technicolor.com edouard.francois@technicolor.com fabrice.leleannec@technicolor.com |
| HHI | B. Bross I. Zupancic | benjamin.bross@hhi.fraunhofer.de ivan.zupancic@hhi.fraunhofer.de |
| Huawei | H. Chen H. Yang J. Chen A. Filippov V. Rufitskiy | chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huaweic.om alexey.filippov@huawei.com vasily.rufitskiy@huawei.com |
| LGE |  |  |
| Tencent | G. Li X. Xu X. Li M. Gao | guichunli@tencent.com xiaozhongxu@tencent.com xlxiangli@tencent.com maxwellgao@tencent.com |
| USTC |  |  |
| HiSilicon | N. Zhang J. Zheng X. Chen | zhangna25@hisilicon.com zhengjianhua@hisilicon.com anderson.chen@hisilicon.com |
| BBC |  |  |
| ETRI |  |  |
| Panasonic | K. Abe J. Li R.-L. Liao | abe.kiyo@jp.panasonic.com jingya.li@sg.panasonic.com ruling.liao@sg,panasonic.com |
| Fujitsu | J. Zhu K. Kazui | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Sony | K. Kondo | kenji.kondo@sony.com |
| Qualcomm |  |  |
| Alibaba |  |  |
| Hikvision |  |  |
| Sharp | A. Segall K. Misra | asegall@sharplabs.com misrak@sharplabs.com |
| Ericsson |  |  |
| Samsung Electronics |  |  |
| Nokia | R. Ghaznavi-Youvalari A. Aminlou J. Lainema | ramin.ghaznavi-youvalari@nokia.com alireza.aminlou@nokia.com jani.lainema@nokia.com |
| InterDigital | D. Luo Y. He X. Xiu | daniel.luo@interdigital.com yuwen.he@interdigital.com xiaoyu.xiu@interdigital.com |
| Arris |  |  |
| MediaTek | P. Chuang C.-Y. Chen C.-W. Hsu Y.-W. Huang Z.-Y. Lin Y. Hsiao C.- Chia Chen | peter.chuang@mediatek.com chingyeh.chen@mediatek.com cw.hsu@mediatek.com yuwen.huang@mediatek.com zhi-yi.lin@mediatek.com yuling.hsiao@mediatek.com chunchia.chen@mediatek.com |
| NHK |  |  |
| Netflix |  |  |
| Orange |  |  |
| Broadcom | M. Zhou W. Wan P. Chen | minhua.zhou@broadcom.com Wade.wan@broadcom.com Peisong.chen@broadcom.com |
| Canon |  |  |
| PKU |  |  |
| Chips&Media |  |  |
| Aachen University |  |  |
| HIT |  |  |
| WILUS |  |  |
| ITRI |  |  |
| Bytedance | H. Liu K. Zhang L. Zhang | liuhongbin.01@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Kwai | X. Wang Y.-W. Chen | xianglinwang@kwai.com yiwenchen@kwai.com |
| HNU |  |  |
| Dolby | P. Yin | pyin@dolby.com |
| Foxconn | Y.-J. Chang Y.-C. Yang | morriss.yj.chang@foxconn.com johnny.yc.yang@foxconn.com |