| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L1023-v1 |
| --- | --- |

| Title: | Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Jin Heo Alexey Filippov | Email: | geertv@qti.qualcomm.com jin78.heo@lge.com Alexey.Filippov@huawei.com |
| Source: | CE3 coordinators |  |  |

_____________________________

# Abstract

This document describes Core Experiment 3 (CE3) on intra prediction and mode coding. The experiments are conducted after the 12th JVET meeting and the results, including cross-checks, are reported as input contributions to the 13th JVET meeting.

# Introduction

In CE3, a total of 28 tests are defined, which are organized in the following sub-CE categories:

- CE3.1: Intra prediction modes (5 tests)

- CE3.2: Cross-component prediction (8 tests)

- CE3.3: Intra mode coding (15 tests)

# Test conditions and guidelines

CTC conditions as agreed at the 12th JVET meeting (Macau SAR, CN) for SDR will be used for the evaluation of objective performance (JVET-L1010). The anchor is the ‘VVC Test Model’ or VTM. Both the ‘All Intra’ and ‘Random Access’ test configurations shall be used and providing the results for each of these configurations is mandatory. The VTM version 3.0 software shall be used with ‘vtm’ configuration files to produce the VTM anchor. Tools shall be implemented into the VTM version 3.0 software.

Proposals will primarily be compared with respect to objective compression efficiency while complexity will also be assessed.

BD-rates, encoding and decoding time comparisons, shall be reported by all proponents for at least the following case: #2 vs. #1. Additional test results may be provided, for example: #2 vs. #3.

Numbering is defined as follows for the case where ‘ToolA’ is replacing an existing ‘ToolB’ in VTM:

1.           VTM,						VTM anchor

2.           VTM + ToolA,			VTM + ToolA (as a “tool on” test)

3.           VTM − ToolB            	VTM with ToolB disabled

Note: If ‘ToolB’ does not exist, then #3 = #2.

Source codes corresponding with the test descriptions contained in this document shall be uploaded to the CE3 GitLab project according to the agreed timeline below:

TBD

# Timeline

| T1 | November 2, 2018 | CE test description finalized |
| --- | --- | --- |
| T2 | Test model SW release + 2 weeks | Initial software of CE tests released; initial study by cross-checkers begins |
| T3 | December 19, 2018 | Software of CE tests is finalized, and full test results are provided; final cross-check begins |
| T4 | December 31, 2018 | Document upload: test results, cross-check reports, CE3 summary report |

## CE3.1 Intra modes

### Contribution documents

JVET-L0076 CE3: Line-based intra coding mode (Tests 2.1.1 and 2.1.2) [S. De Luxán Hernández, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-L0199 CE3: Non-linear weighted intra prediction (tests 2.2.1 and 2.2.2) [P. Helle, J. Pfaff, M. Schäfer, R. Rischke, T. Hinz, P. Merkle, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-L0131 CE3-4.1: Harmonization of linear interpolation intra prediction (LIP) with simplified position dependent intra prediction combination (PDPC) and wide-angle intra prediction (WAIP) [J. Heo, J. Choi, J. Choi, S. Yoo, L. Li, J. Lim (LGE)]

### Defined tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.1.1 | Intra Sub-Partitions coding mode with a better trade-off between gain and encoding run-time | Santiago de Luxán Hernández (HHI) |  |
| 1.1.2 | Test 1.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples | Santiago de Luxán Hernández (HHI) |  |
| 1.2.1 | Affine linear weighted intra prediction modes with encoder speedup | P. Merkle (HHI) |  |
| 1.2.2 | Affine linear weighted intra prediction modes with a fixed number of weights needed per NxN block | P. Merkle (HHI) |  |
| 1.3 | Harmonization simplified linear interpolation intra prediction (LIP) with PDPC | Jin Heo (LGE) |  |

### Test descriptions

#### Intra Sub-Partitions coding mode (JVET-L0076 HHI)

The Intra Sub-Partitions (ISP) coding mode is an updated version of the Line-Based Intra (LIP) coding that corrects the hardware-related issues of the previous design of the algorithm, as explained in JVET-L0076. The ISP tool divides luma intra-predicted blocks vertically or horizontally into 2 or 4 sub-partitions depending on the block size dimensions, as shown in Table 1. Figure 1 and Figure 2 show examples of the two possibilities. All sub-partitions fulfill the condition of having at least 16 samples.

Table 1: Number of sub-partitions depending on the block size

| Block Size | Number of Sub-Partitions |
| --- | --- |
| 4×4 | Not divided |
| 4×8 and 8×4 | 2 |
| All other cases | 4 |

Figure 1: Example of division of 4×8 and 8×4 blocks

Figure 2: Example of division of all blocks except 4×8, 8×4 and 4×4

For each of these sub-partitions, a residual signal is generated by entropy decoding the coefficients sent by the encoder and then inverse quantizing and inverse transforming them. Then, the sub-partition is intra predicted and finally the corresponding reconstructed samples are obtained by adding the residual signal to the prediction signal. Therefore, the reconstructed values of each sub-partition will be available to generate the prediction of the next one, which will repeat the process and so on. All sub-partitions share the same intra mode.

Based on the intra mode and the split utilized, two different classes of processing orders are used, which are referred to as normal and reversed order. In the normal order, the first sub-partition to be processed is the one containing the top-left sample of the CU and then continuing downwards (horizontal split) or rightwards (vertical split). As a result, reference samples used to generate the sub-partitions prediction signals are only located at the left and above sides of the lines.  On the other hand, the reverse processing order either starts with the sub-partition containing the bottom-left sample of the CU and continues upwards or starts with sub-partition containing the top-right sample of the CU and continues leftwards.

A more detailed description can be found in JVET-L0076.

Based to the description of ISP coding mode in JVET-L0076, it is proposed to study how to find a better trade-off between gain and encoding run-time for the two experiments carried out in CE3: Summary Report on Intra Prediction and Mode Coding (JVET-L0023).

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.1.1 | Intra Sub-Partitions coding mode with a better trade-off between gain and encoding run-time | Santiago de Luxán Hernández (HHI) |  |
| 1.1.2 | Test 1.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples | Santiago de Luxán Hernández (HHI) |  |

#### Affine linear weighted intra predictors (JVET-L0199 HHI)

Affine Linear Weighted intra predictors (ALWIP) take as their input reference samples from one or more reference lines above or left of the block, apply a matrix multiplication to this input and add an offset. The matrices and the offsets used are fixed in advance. For large blocks, the reference samples may be downsampled first. Also, for large blocks, the predictors may predict into the domain of an energy compactifying transform, e.g., the DCT. If this is the case, most of the frequency components (typically high frequency components) of the prediction signal are always set to zero. Which frequency components are always set to zero is fixed in advance and does not depend on the reference samples. A more detailed description of the ALWIP modes can be found in JVET-L0199.

As required in the discussion during the Macao-meeting, in the tests under considerations, two aspects will be studied with respect to a complexity-performance-tradeoff. First, an encoder speedup will be studied. Second, a variant with a fixed maximal number of weights per NxN-block will be investigated.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.2.1 | Affine linear weighted intra prediction modes with encoder speedup | P. Merkle (HHI) |  |
| 1.2.2 | Affine linear weighted intra prediction modes with a fixed number of weights needed per NxN block | P. Merkle (HHI) |  |

#### Harmonization of linear interpolation intra prediction (LIP) with PDPC and wide-angle intra prediction (WAIP) (JVET-L0131 LGE)

LIP is applied on top of intra prediction. The LIP flag for an intra coded CU is signaled at the CU level to indicate whether LIP intra prediction mode is used or not. LIP prediction is only applied to angular intra prediction modes except for Planar and DC modes.

The LIP intra prediction method first computes the intensity value of the bottom right (BR) neighboring sample. Then, it generates the intensity values of the bottom row and the right column of the neighboring samples. These samples serve as references on the opposite side of the CU and are derived using the intensity values of the BR neighboring sample and the intensity values of other corner neighboring samples, i.e., the top right neighboring sample, the bottom left neighboring sample.

The first stage of the intra angular mode prediction involves extrapolating samples from a projected reference sample location based on a given direction. The projected reference sample location is computed with bilinear interpolation which is used to obtain the value of the projected reference sample using two closest reference samples located at integer position.

The second stage in the prediction process requires an identical approach to generate another predicted sample value at the same sample location. The second predicted sample value is obtained by projecting the location of the sample onto the opposite side of the reference sample array by applying the selected prediction direction and interpolating the value to 1/32 sample position accuracy. The bilinear interpolation is performed linearly using the two closest opposite side reference samples in the direction of prediction.

The LIP takes advantage of the difference in accuracy between the input intensities in the final interpolation process. Specifically, larger weights are applied to the positions that are closer to more reliable neighboring positions. Final predicted sample value is calculated.

Figure 3: Linear interpolation intra prediction (LIP)

In this test, harmonization of linear interpolation intra prediction (LIP) and position dependent intra prediction combination (PDPC) will be investigated. Simplified method on generation of LIP boundary samples will also be searched.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1.3 | Harmonization simplified LIP with PDPC | Jin Heo (LGE) |  |

## CE3.2 Cross-component prediction tools

### Contribution documents

JVET-L0251 CE3: Extended LM modes (Tests 5.2.1, 5.2.2, 5.2.3, and 5.2.4) [A. K. Ramasubramonian, G. Van der Auwera, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-L0388 CE3: Multiple neighbor-based linear model (Test 5.3.1) [H.-Y. Jiang, H.-J. Jhu, Y.-J. Chang (Foxconn)]

JVET-L0419 CE3: Adaptive multiple cross-component linear model (Test 5.9.1) [S.-P. Wang, P.-H. Lin, C.-H. Yau, C.-L. Lin, C.-C. Lin (ITRI)]

JVET-L0239 CE3-related: Enabling different chroma sample location types in CCLM [P. Hanhart, Y. He, Y. Ye (InterDigital)]

JVET-L0342 CE3-related: Classification-based mean value for CCLM coefficients derivation [X. Ma, F. Mu, H. Yang, J. Chen (Huawei)]

JVET-L0329 CE3-related: CCLM prediction with single-line neighbouring luma samples [K. Zhang, L. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

### Defined tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.1 | CCLM + MDLM + MMLM | Adarsh K. Ramasubramonian (Qualcomm) |  |
| 2.2 | CCLM + MDLM + MMLM + Above-MMLM + Left-MMLM | Hong-Jheng Jhu (Foxconn) |  |
| 2.3 | CCLM + MDLM + Adaptive multiple cross-component linear model | Po-Han Lin (ITRI) |  |
| 2.4 | Modified CCLM downsampling filter for “type-2” content | Philippe Hanhart (Interdigital) |  |
| 2.5.1 | CCLM + MDLM, using MaxMin method, and β derived by using Cmean and Lmean | Xiang Ma (Huawei) |  |
| 2.5.2 | CCLM + MDLM, using classification-based mean value method | Xiang Ma (Huawei) |  |
| 2.5.3 | CCLM + MDLM, using MaxMin method; using the first top row and the second left column | Xiang Ma (Huawei) |  |
| 2.5.4 | CCLM + MDLM, using classification-based mean value method; using the first top row and the second left column | Xiang Ma (Huawei) |  |
| 2.6 | CCLM accessing less neighbouring luma samples, interacted with other adopted coding tools | Kai Zhang (Bytedance) |  |

### Test descriptions

#### Multiple model LM (JVET-L0251 Qualcomm)

The CCLM mode in VTM-3.0 employs one linear model for predicting the chroma samples from the luma samples for the whole CU. In MMLM, two models are defined for the prediction of chroma samples from luma samples. Neighbouring luma samples and neighbouring chroma samples of the current block (aligned with CCLM and MDLM) are classified into two groups. Each group is used as a training set to derive a linear model (i.e., a particular α and β are derived for a particular group). This test re-uses the min-max method for parameter estimation (as used by CCLM in VTM-3.0) and the downsampling filters used for the derivation of the linear model parameters in CCLM. Furthermore, the samples of the current luma block are also classified based on the same rule for the classification of neighbouring luma samples. Figure 4 shows an example of classifying the neighbouring samples into two groups based on the value Threshold. Threshold is calculated as the average value of the neighbouring reconstructed luma samples. A neighbouring sample with Rec′L[x,y] <= Threshold is classified into group 1; while a neighbouring sample with Rec′L[x,y] > Threshold is classified into group 2. Decoder complexity involved in the MMLM derivation is also studied.

Figure 4: An example of classifying the neighbouring samples into two groups

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.1 | CCLM + MDLM + MMLM | Adarsh K. Ramasubramonian (Qualcomm) |  |

#### Multiple neighbor LM (JVET-L0388 Foxconn)

A cross component linear model predicts chroma samples based on the reconstructed luma samples of the same CU. The parameters of the linear models are derived by minimizing the regression error using the reconstructed luma samples and chroma samples in the neighbouring data set. Multiple neighbouring data sets for LM and MMLM are proposed in JVET-J0026, JVET-K0073 and JVET-L0388 to use multiple neighbouring data sets to cover various linear relationships between luma samples and chroma samples in CU.

The test provided would also like to address the complexity concerns of MMLM and MNLM.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.2 | Multiple neighbour LM (MNLM) (CCLM + MDLM + MMLM + Above-MMLM + Left-MMLM) | Hong-Jheng Jhu (Foxconn) |  |

#### Adaptive multiple cross-component linear model (JVET-L0419 ITRI)

This test utilizes a grouping process to generate single or multiple groups for linear model prediction. Firstly, samples from the top and left neighbouring reconstructed blocks are classified into groups according to both luma and chroma values. The classification method scans each neighbouring sample and determines if it can join an existing group according to the criteria (1) and (2). If the neighbouring sample cannot satisfy the criteria of any existing group, a new group will be created. As a result, all the neighbouring samples will be classified into a number of groups as illustrated in Figure 5.

Ygroup_min – Ymargin < Yp < Ygroup_max + Ymargin  	(1)

Cgroup_min – Cmargin < Cp < Cgroup_max + Cmargin   	(2)

Note that Yp and Cp denotes the luma and chroma value of a neighboring sample. Ygroup_max and Cgroup_max denotes the maximum luma and chroma value in a group. Ygroup_min and Cgroup_min denotes the minimum luma and chroma value in a group. Ymargin and Cmargin denotes the margin of the group for luma and chroma respectively.

Figure 5: Group classification of neighboring pixels

After that, LM parameters αg and βg for each group will be derived by the simplified straight line equation which is adopted in this meeting. When predicting a chroma sample, the nearest group will be determined according to an averaged luma value. LM parameters αg and βg of the nearest group is then used to derive the chroma value with the following equation:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.3 | CCLM + MDLM + Adaptive multiple cross-component linear model | Po-Han Lin (ITRI) |  |

#### Modified CCLM downsampling filter for “type-2” content (JVET-L0239 Interdigital)

To perform the cross-component prediction, for the 4:2:0 chroma format, the reconstructed luma samples need to be downsampled to match the sampling of the chroma signal. The default downsampling filter used in CCLM mode is as follows:

recL'i,j= recL2i-1,2j+2∙recL2i,2j+recL2i+1,2j+recL2i-1,2j+1+2∙recL2i,2j+1+recL2i+1,2j+1+4≫3

This downsampling assumes the “type-0” phase relationship for the positions of the chroma samples relative to the positions of the luma samples, i.e., collocated sampling horizontally and interstitial sampling vertically.

For “type-2” phase relationship, i.e., collocated sampling horizontally and vertically, the downsampling filter should be modified to avoid a misalignment between the chroma samples and the downsampled luma samples. For “type-2” content, the following 3-tap, 5-tap, and 9-tap downsampling filters may be used for the linear model determination and the prediction to avoid a phase shift:

recL'i,j= recL2i-1,2j+2∙recL2i,2j+recL2i+1,2j+2≫2

recL'i,j= recL2i,2j-1+recL2i-1,2j+4∙recL2i,2j+recL2i+1,2j+recL2i,2j+1+4≫3

recL'i,j= recL2i-1,2j-1+2∙recL2i,2j-1+recL2i+1,2j-1+2∙recL2i-1,2j+4∙recL2i,2j+2∙recL2i+1,2j+recL2i-1,2j+1+2∙recL2i,2j+1+recL2i+1,2j+1+8≫4

To avoid increasing the number of line buffer, these modifications will not be applied at the top CTU boundary.

The tests will be conducted on HDR content, which uses “type-2”. CTC conditions as agreed at the 12th JVET meeting (Macao, CN) for HDR will be used for the evaluation of objective performance (JVET-L1011).

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.4.x | Modified CCLM downsampling filter for “type-2” content - LM determination: 3-tap filter; prediction: 3-tap filter - LM determination: 3-tap filter; prediction: 5-tap filter - LM determination: 3-tap filter; prediction: 9-tap filter - LM determination: 5-tap filter; prediction: 5-tap filter - LM determination: 9-tap filter; prediction: 9-tap filter | Philippe Hanhart (InterDigital) |  |

#### Classification-based mean value for CCLM coefficients derivation  (JVET-L0342 Huawei)

In JVET-L meeting, MaxMin method (JVET-L0191) for CCLM coefficients derivation is adopted, in which, both the linear model coefficients α and β are derived by a linear equation directly. In the proposed method, a new coefficients derivation method based on classification-based mean value is proposed. In the proposed method, the luma template samples are classified into two luma classes by the mean value of them. Correspondingly, two chroma classes are obtained. Then two luma mean values of the two luma classes, and two chroma mean values of the two chroma classes are obtained, respectively. The coefficients are derived based on the luma mean values and the chroma mean values.

Figure 6: Classification and mean value for CCLM mode

As shown in Figure 6, Lmean is the mean value of the luma template samples. Cmean is the mean value of the chroma template samples. The luma template samples are classified into two luma classes by Lmean. Then, mean values of the two luma classes LLmean and LRmean are obtained. Correspondingly, CLmean and CRmean are obtained. The CCLM coefficients are derived as:

α=CLmean-CRmeanLLmean-LRmean

β=Cmean-α*Lmean

In the existing method, to get the luma template samples, 2 rows and 3 columns of neighboring luma samples of current luma block are needed for down-sampling, except the CU at CTU top boundary. In the proposed method, only 1 row and 1 column of neighboring luma samples of current luma block are needed for all the CU. Which means, only the first top row and the second left column will be used for down-sampling to get the luma template samples. As shown in Figure 7.

Figure 7: Luma template samples (1 row, 1 column, with down-sampling).

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.5.1 | CCLM + MDLM, using MaxMin method, and β derived by using Cmean and Lmean. | Xiang Ma (Huawei) |  |
| 2.5.2 | CCLM + MDLM, using classification-based mean value method. | Xiang Ma (Huawei) |  |
| 2.5.3 | CCLM + MDLM, using MaxMin method; using the first top row and the second left column. | Xiang Ma (Huawei) |  |
| 2.5.4 | CCLM + MDLM, using classification-based mean value method; using the first top row and the second left column. | Xiang Ma (Huawei) |  |

#### CCLM accessing less neighbouring luma samples (JVET-L0329 Bytedance)

In JVET-L0329, down-sampling filters with less taps are applied on the neighbouring luma samples. The interaction of this complexity reduction and other adopted coding tools will be further studied in the CE.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 2.6 | CCLM accessing less neighbouring luma samples | Kai Zhang (Bytedance) |  |

## CE3.3 Intra mode coding

### Contribution documents

JVET-L0139 CE3-related: Simplified MDMS [J. Choi, J. Heo, S. Yoo, L. Li, J. Choi, J. Lim (LGE)]

JVET-L0164 CE3-related: Decoder-side Intra Mode Derivation [E. Mora, A. Nasrallah, M. Raulet (ATEME)]

JVET-L0280 CE3-related: Intra mode coding [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-L0630 CE3-related: Simplification of MDMS derivation [C.-H. Yao, P.-H. Lin, C.-C. Lin, B.-J. Fuh, C.-L. Lin (ITRI)]

### Defined tests

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3.1.1 | Simplified MDMS with 5 chroma intra modes | Jangwon Choi (LGE) |  |
| 3.1.2 | Simplified MDMS with the reduced chroma intra modes | Jangwon Choi (LGE) |  |
| 3.2.1 | Decoder-side intra mode derivation with reduced reconstruction latency | E. Mora (ATEME) |  |
| 3.2.2 | Application of the decoder-side intra mode derivation for the chroma component | E. Mora (ATEME) |  |
| 3.2.3 | Template-pixel candidate pruning | E. Mora (ATEME) |  |
| 3.2.4 | Differential coding of the intra mode with decoder-side derived mode as predictor | E. Mora (ATEME) |  |
| 3.2.5 | Alternative selection of the gradient filter | E. Mora (ATEME) |  |
| 3.2.6 | Alternative parsing dependency removal for DIMD in MPM list construction | E. Mora (ATEME) |  |
| 3.2.7 | Restriction of DIMD to certain block sizes | E. Mora (ATEME) |  |
| 3.3 | 6 MPMs and 32 remaining modes | Liang Zhao (Tencent) |  |
| 3.4.1 | One DM + one chroma neighbor mode + default modes | Po-Han Lin (ITRI) |  |
| 3.4.2 | One DM + two chroma neighbor modes + default modes | Po-Han Lin (ITRI) |  |
| 3.4.3 | One DM + one chroma neighbor mode + N/-N derivation + default modes | Po-Han Lin (ITRI) |  |
| 3.4.4 | One DM + two chroma neighbor modes + N/-N derivation + default modes | Po-Han Lin (ITRI) |  |
| 3.4.5 | Reduce candidate | Po-Han Lin (ITRI) |  |

### Test descriptions

#### Simplified MDMS (JVET-L0139 LGE)

In this test, simplified MDMS (Multiple Direct Mode Signaling) methods are further investigated to reduce required complexity. Specifically, the following points will be investigated.

- The derivation of chroma intra candidate modes is engaged by the reduced number of DM and spatial point searching compared to the original MDMS.

- The reduction of the number of context models used for chroma intra mode coding.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3.1.1 | Simplified MDMS with 5 chroma intra modes | Jangwon Choi (LGE) |  |
| 3.1.2 | Simplified MDMS with the reduced chroma intra modes | Jangwon Choi (LGE) |  |

#### Decoder-side Intra Mode Derivation (JVET-L0164 Ateme)

Decoder-side Intra Mode Derivation (DIMD) is an intra coding mode wherein the luma intra prediction mode is not sent in the bitstream but derived using previously encoded / decoded pixels, in an identical fashion at the encoder and at the decoder. The selection of DIMD is signalled in the bitstream for intra coded blocks using a simple flag. DIMD competes with other coding modes at the encoder, including the classical intra coding mode (where the intra prediction mode is coded). Figure 8 shows the intra block decoding process. A DIMD flag is first parsed. If that flag equals 0, the intra prediction mode is parsed from the bitstream as usually done. If not (flag equals 1), the parsing stops and the intra prediction mode is derived during the prediction process.

Figure 8: Proposed intra block decoding process.

To derive the intra prediction mode for a block, a gradient analysis is performed on the pixels of a neighboring 2-pixels wide L-shaped template surrounding the block. This allows to determine a main angular direction for the template, which has a high chance to be equal to the one of the current block. A 3x3 Sobel gradient filter is used, defined by the following matrices:

Mx=-101-202-101    and    My=-1-2-1000121

For each pixel of the template, whenever possible, we compute a horizontal and a vertical gradient, Gx and Gy, respectively. Computation of gradients is not possible in the following cases:

- For template-pixels that are closest to the current block due to a lack of neighbors

- For template-pixels that are outside of the current picture

- For template-pixels that are located on picture boundaries due to a lack of neighbors

- For template-pixels that are located one line above the current CTU. This is imposed to avoid additional line buffers

For each template-pixel for which a gradient computation was possible, an edge intensity G is calculated as such: G=Gx+Gy, and an intra angular direction corresponding to the edge orientation is derived from Gx and Gy. The intra angular direction is used to index a histogram of gradients or HoG, first initialized to zero. The histogram value at that intra angular direction is increased by G. Once all the pixels in the template have been processed, the histogram will contain cumulative values of gradient intensities, for each intra angular mode. The mode that shows the first highest peak in the histogram is selected as intra prediction mode for the current block. If the maximum value in the histogram is 0, then the DC mode is selected as intra prediction mode for the current block.

To avoid a parsing dependency on reconstructed pixels, the intra prediction mode in DIMD is derived in the reconstruction process, meaning it is unavailable during the parsing process. This has two consequences:

- For the chroma component of a DIMD block, the initial chroma mode candidate list is not modified (usually if the luma mode equals one of the candidates, this candidate would be replaced by the vertical diagonal mode)

- DIMD-coded blocks are treated just like inter blocks during MPM list construction, meaning they are considered unavailable.

Based on the above description, it is first proposed to reduce the reconstruction latency that DIMD implies (because of the additional intra mode derivation steps needed to be done during prediction). It is furthermore believed that additional coding efficiency can be achieved by several means. Applying DIMD for chroma, pruning template-pixels if the edge orientation at those pixels does not cross the current block, coding a delta mode with the derived intra prediction mode as predictor, modifying the way DIMD blocks impact MPM list construction, and selecting alternative gradient filters are proposed as tests to achieve this mean. Reduction of encoding complexity can in addition be achieved through the restriction of DIMD to only certain block sizes.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3.2.1 | Decoder-side intra mode derivation with reduced reconstruction latency | E. Mora (ATEME) |  |
| 3.2.2 | Application of the decoder-side intra mode derivation for the chroma component | E. Mora (ATEME) |  |
| 3.2.3 | Template-pixel candidate pruning | E. Mora (ATEME) |  |
| 3.2.4 | Differential coding of the intra mode with decoder-side derived mode as predictor | E. Mora (ATEME) |  |
| 3.2.5 | Alternative selection of the gradient filter | E. Mora (ATEME) |  |
| 3.2.6 | Alternative parsing dependency removal for DIMD in MPM list construction | E. Mora (ATEME) |  |
| 3.2.7 | Restriction of DIMD to certain block sizes | E. Mora (ATEME) |  |

#### 6MPMs and 32 remaining modes (JVET-L0280 Tencent)

In this test, it is proposed to signal 6 MPMs and 32 remaining modes for each coding unit. The remaining modes are derived using angular modes in the MPM list. To code an intra prediction mode, an MPM flag is firstly signaled. If the MPM flag is not true, fix length codeword with bypass coding is used to signal the remaining modes.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3.3 | 6 MPMs and 32 remaining modes | Liang Zhao (Tencent) |  |

#### Chroma intra prediction simplification (JVET-L0630 ITRI)

MDMS (Multiple Direct Mode Signaling) is a chroma signalling method for the non-LM modes. The concept of MDMS is similar to MPM in the luma component. MDMS now has 5 candidates for signalling. The derivation order of MDMS is shown in the Figure 9. First, DM mode which is the intra prediction mode of the corresponding luma block is added. With the separate tree method, the chroma block could correspond to multiple luma blocks. Therefore, it checks the corresponding luma blocks with 5 positions in the chroma block to get multiple DM modes. Then, the neighbouring blocks of the current chroma block are checked and added to the list if available. After that, if the candidates in the MDMS list are still smaller than 5, planar mode, DC mode, +1/-1 derivation, vertical mode, horizontal mode, angle mode 2 and diagonal mode is applied sequentially.

Figure 9: The derivation order of MDMS.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3.4.1 | One DM + one chroma neighbor mode + default modes | Po-Han Lin (ITRI) |  |
| 3.4.2 | One DM + two chroma neighbor modes + default modes | Po-Han Lin (ITRI) |  |
| 3.4.3 | One DM + one chroma neighbor mode + N/-N derivation + default modes | Po-Han Lin (ITRI) |  |
| 3.4.4 | One DM + two chroma neighbor modes + N/-N derivation + default modes | Po-Han Lin (ITRI) |  |
| 3.4.5 | Reduce candidate | Po-Han Lin (ITRI) |  |

# Participants

The following table enumerates participants with tools in CE3 as well as participating cross-checkers and interested persons.

| Name | Company | Email | CE3.x Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Geert Van der Auwera Vadim Seregin Nan Hu Ted Hsieh Adarsh Krishnan Ramasubramonian Luong Pham Van Chun-Chi Chen Yan Zhang Chao-Hsiung Hung | Qualcomm | geertv@qti.qualcomm.com vseregin@qti.qualcomm.com nanh@qti.qualcomm.com thsieh@qti.qualcomm.com aramasub@qti.qualcomm.com lphamvan@qti.qualcomm.com chunchic@qti.qualcomm.com yzh@qti.qualcomm.com chaohsiu@qti.qualcomm.com | 2 |  |
| Xin Zhao Liang Zhao | Tencent | xinzzhao@tencent.com leolzhao@tencent.com | 3 |  |
| Fabien Racapé Gagan Rath Fabrice Urban Franck Galpin Francois Edouard | Technicolor | Fabien.Racape@technicolor.com Gagan.rath@technicolor.com Fabrice.urban@technicolor.com Franck.galpin@technicolor.com Edouard.Francois@technicolor.com |  |  |
| Jani Lainema | Nokia | jani.lainema@nokia.com |  |  |
| Virginie Drugeon | Panasonic | Virginie.Drugeon@eu.panasonic.com |  |  |
| Jaehyun Lim SeungHawn Kim Jin Heo Jangwon Choi Sunmi Yoo Ling Li Hyeongmun Jang | LGE | jaehyun.lim@lge.com seunghwan3.kim@lge.com jin78.heo@lge.com jangwon84.choi@lge.com sunmi.yoo@lge.com l.li@lge.com hm.jang@lge.com | 1, 3 |  |
| Jungwon Kang Jinho Lee | ETRI | jungwon@etri.re.kr jinosoul@etri.re.kr |  |  |
| Jonathan Pfaff Heiko Schwarz Benjamin Bross Philipp Merkle Santiago de Luxán Hernández Philipp Helle | HHI | jonathan.pfaff@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de benjamin.bross@hhi.fraunhofer.de philipp.merkle@hhi.fraunhofer.de santiago.de.luxan@hhi.fraunhofer.de philipp.helle@hhi.fraunhofer.de | 1 |  |
| Yue Yu Limin Wang | ARRIS | Yue.Yu@arris.com Limin.Wang@arris.com |  |  |
| Yao-Jen Chang Hong-Jheng Jhu Tsung-Hua Li | Foxconn | morriss.yj.chang@foxconn.com ted0958.am96@g2.nctu.edu.tw tim.th.li@fii-foxconn.com | 2 |  |
| Alexey Filippov Sergey Ikonin Jianle Chen Anand Meher Kotra Xiang Ma Vasily Rufitskiy Sychev Maxim Cheung Auyeung Biao Wang | Huawei | Alexey.Filippov@huawei.com Sergey.Ikonin@huawei.com jianle.chen@huawei.com anand.meher.kotra@huawei.com maxiang6@huawei.com Vasily.Rufitskiy@huawei.com Sychev.Maxim@huawei.com cheung.auyeung@huawei.com biao.wang@huawei.com | 2 |  |
| Kei Kawamura Yoshitaka Kidani Sei Naito | KDDI | ki-kawamura@kddi.com yo-kidani@kddi.com se-naitou@kddi.com |  |  |
| Peter Chuang, ChingYeh Chen, Yu-Wen Huang, Chih-Wei Hsu, Chia-Ming Tsai, Zhi-Yi Lin, Mohammed Sarwer | MediaTek | peter.chuang@MediaTek.com chingyeh.chen@MediaTek.com yuwen.huang@MediaTek.com cw.hsu@MediaTek.com Chia-ming.Tsai@MediaTek.com Zhi-Yi.Lin@MediaTek.com Mohammed.Sarwer@MediaTek.com |  |  |
| Yinji Piao Narae Choi Kiho Choi Jie Chen | Samsung | yj1003.piao@samsung.com narae1.choi@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com |  |  |
| Ching-Chieh Lin Po-Han Lin | ITRI | jacklin@itri.com pohan@itri.com | 2 |  |
| Steve Keating | Sony | Steve.Keating@sony.com |  |  |
| Shimpei Nemoto | NHK | nemoto.s-fy@nhk.or.jp |  |  |
| Andre Seixas Dias Saverio Blasi Gosala Kulupana | BBC | Andre.SeixasDias@bbc.co.uk saverio.blasi@bbc.co.uk gosala.kulupana@bbc.co.uk |  |  |
| Andrew Segall Kiran Misra Yukinobu Yasugi Takeshi Chujoh Tomohiro Ikai Eiishi Sasaki Tomonori Hashimoto | Sharp | asegall@sharplabs.com misrak@sharplabs.com yasugi.yukinobu@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp eiichi.sasaki@sharp.co.jp tomonori.hashimoto@sharp.co.jp |  |  |
| Zhi Zhang Rickard Sjoberg | Ericsson | zhi.a.zhang@ericsson.com rickard.sjoberg@ericsson.com |  |  |
| Yongbing Lin Quanhe Yu Jianhua Zheng | HiSilicon | linyongbing@hisilicon.com yuquanhe@hisilicon.com zhengjianhua@hisilicon.com |  |  |
| Dongcheol Kim | WILUS | dongcheol.kim@wilusgroup.com |  |  |
| Xingchun Qi Xuguang Zuo Shurui Ye Liying Xu | Hikvision | qixingchun@hikvision.com zuoxuguang@hikvision.com yeshurui@hikvision.com xuliying5@hikvision.com |  |  |
| D. Y. Kim | Chips& Media | brad.kim@chipsnmedia.com |  |  |
| Elie Mora Anthony Nasrallah | ATEME | e.mora@ateme.com a.nasrallah@ateme.com | 3 |  |
| Lin Tao | Tongji Univ. | lintao@tongji.edu.cn |  |  |
| Xianglin Wang Yi-Wen Chen | Kwai | xianglinwang@kwai.com yiwenchen@kwai.com |  |  |
| Sriram Sethuraman | Ittiam | sriram.sethuraman@ittiam.com |  |  |
| Chris Rosewarne Andrew Dorrell Patrice Onno Guillaume Laroche Jonathan Taquet | Canon | Chris.Rosewarne@cisra.canon.com.au Andrew.Dorrell@cisra.canon.com.au Patrice.Onno@crf.canon.fr guillaume.laroche@crf.canon.fr jonathan.taquet@crf.anon.fr |  |  |
| Byeungwoo Jeon Jeeyoon Park | SKKU | bjeon@skku.edu jiyoonpark@skku.edu |  |  |
| Liu Hongbin Kai Zhang | Bytedance | liuhongbin.01@bytedance.com zhangkai.video@bytedance.com |  |  |
| Hari Kalva | Florida Atlantic Univ. | hari.kalva@fau.edu |  |  |
| Minhua Zhou Peisong Chen Wade Wan | Broadcom | minhua.zhou@broadcom.com peisong.chen@broadcom.com wade.wan@broadcom.com |  |  |
| Rahul Vanam Xiao Yu Xiu Yuwen He Saurav Bandyopadhyay | Interdigital | rahul.vanam@interdigital.com xiaoyu.xiu@interdigital.com yuwen.he@interdigital.com Saurav.Bandyopadhyay@InterDigital.com | 2 |  |
| Pierrick Philippe | Orange | pierrick.philippe@orange.com |  |  |
| Weimin Zeng Eric Chai | Ubilinx | weimin.zeng@ublnx.com eric.chai@ublnx.com |  |  |
| Dan Grois | Comcast | groisz@yahoo.com |  |  |
| Jongseok Lee | Kwangwoon University | suk2080@kw.ac.kr |  |  |
| Fannie Yu | Oppo | yuyuanfang@oppo.com |  |  |
| Peng Yin | Dolby | pyin@dolby.com |  |  |
| Haechul Choi | Hanbat University | choihc@hanbat.ac.kr |  |  |
| Daehyeok Gwon Heeji Han Jaeryun Choe | Naver Corporation | skyeye0530@naver.com sarhakor@naver.com tjswo9395@naver.com |  |  |
| Grace Yu | Intel | grace.yu@intel.com |  |  |
| Yiming Li Zizheng Liu | Wuhan University | userlym@whu.edu.cn lzz8246@whu.edu.cn |  |  |
| Jianqing Zhu Kimihiko Kazui | Fujitsu | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |  |  |