| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L1030-v1 |
| --- | --- |

| Title: | Description of Core Experiment 10: Combined and multi-hypothesis prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu Martin Winken | Email: | cw.hsu@mediatek.com martin.winken@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

_____________________________

- Abstract

The main goal of this Core Experiment (CE) is to improve the inter prediction of the Test Model by the use of combined predictions. Five sub CEs are created to test different methods of combined predictions, including:

CE10.1: multi-hypothesis prediction,

CE10.2: overlapped block motion compensation,

CE10.3: multiple shape prediction partitions,

CE10.4: diffusion filtering of inter- and intra-prediction signals,

CE10.5: local illumination compensation.

Participants

| Nr. | Name | Company | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Man-Shu Chiang ChunChia Chen Zhi-Yi Lin Yu-Cheng Lin | MediaTek | Man-Shu.Chiang@mediatek.com chunchia.chen@mediatek.com Zhi-Yi.Lin@mediatek.com Yucheng.Lin@mediatek.com | X | X |
| 2 | Martin Winken Jennifer Rasch | Fraunhofer HHI | martin.winken@hhi.fraunhofer.de Jennifer.Rasch@hhi.fraunhofer.de | X | X |
| 3 | Weiwei Xu Yin Zhao | Huawei | xuweiwei3@huawei.com yin.zhao@huawei.com | X |  |
| 4 | Yongjo Ahn | Digital Insights | yjahn@digitalinsights.co.kr | X |  |
| 5 | Poirier Tangi Le Léannec Fabrice | Technicolor | tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com | X |  |
| 6 | Anish Mahendra Tamse | Samsung | tamse.anish@samsung.com | X |  |
| 7 | Kiyofumi Abe | Panasonic | abe.kiyo@jp.panasonic.com | X |  |

CE10.1: Multi-hypothesis prediction

JVET-L0100 (MediaTek)

In JVET-L0100, multi-hypothesis prediction mode is applied to improve uni-prediction of advanced motion vector prediction (AMVP) mode, skip/merge mode, and intra mode. The goal is to combine an existing prediction mode with an extra merge indexed prediction. When applied to skip or merge mode, the merge index for the extra merge indexed prediction is implicitly derived.

Within this CE, the performance of the method described in JVET-L0100 is to be studied for different requirements in terms of the motion compensation bandwidth, run time reduction, and coding gain improvement. Settings affecting these requirements are e.g., the motion vector limitation and merge candidate generation.

JVET-L0148 (HHI)

In JVET-L0148, a multi-hypothesis inter prediction mode is described.  In this multi-hypothesis inter prediction mode, one or more additional prediction signals are signaled, in addition to the conventional uni/bi prediction signal.  The resulting overall prediction signal is obtained by sample-wise weighted superposition.

Within this CE, the performance of the method described in JVET- L0148is to be studied for different settings resulting in different requirements in terms of memory access bandwidth and computational complexity.  Settings affecting these requirements are e.g., the number of additional hypotheses and the length of the sub-pel interpolation filter.

JVET-L0375 (Huawei)

This proposal simplifies the Inter-Intra mode adopted in Macao meeting from three aspects. The first aspect is to reduce the 4 intra modes used in the Inter-Intra mode to only Planar mode. The second aspect is to simplify the Planar prediction process used in the Inter-Intra mode by skipping reference sample smoothing and PDPC. The third aspect is to use unified weights (4:4) for the whole CU in Inter-Intra prediction sample averaging.

Intra Planar prediction in VTM includes four steps: 1) reference sample availability check and substitution, 2) reference sample filtering, 3) horizontal and vertical planar predictor generation and averaging, and 4) PDPC. The second aspect uses the least steps (i.e., steps 1 and 3) to generate the intra Planar prediction samples used in the Inter-Intra mode.

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE10.1.1.a | Multi-hypothesis prediction for uni-prediction as described in 2.1 - Explicit signaling with merge index - Additional 1 hypothesis for uni-prediction - Total 2 hypothesis - Disabled for small block sizes | Man-Shu Chiang (MediaTek) | Martin Winken (HHI) |
| CE10.1.1.b | Multi-hypothesis prediction for skip/merge mode as described in 2.1 - Implicit derivation of hypothesis - Additional 1 or 2 hypothesis - Total at most 4 hypothesis - Reduced interpolation filter for multi-hypotheses | Man-Shu Chiang (MediaTek) | Martin Winken (HHI) |
| CE10.1.2.a | Multi-hypothesis prediction as described in 2.2 - Additional 2 hypothesis - Total at most 4 hypothesis - Disabled for small block sizes - full-pel only for multi-hypotheses | Martin Winken (HHI) | Man-Shu Chiang (MediaTek) |
| CE10.1.3.a | Simplification on Inter-Intra mode in 2.3 - Aspect 1: Intra mode is only Planar (reduced from 4 intra modes) | Weiwei Xu (Huawei) |  |
| CE10.1.3.b | Simplification on Inter-Intra mode in 2.3 - Aspect 2: intra Planar prediction used in the Inter-Intra mode is simplified by skipping reference sample smoothing and PDPC | Weiwei Xu (Huawei) |  |
| CE10.1.3.c | Simplification on Inter-Intra mode in 2.3 - Aspect 3: same weights (4:4) for a whole CU | Weiwei Xu (Huawei) |  |
| CE10.1.3.d | CE10.1.3.a + CE10.1.3.b | Weiwei Xu (Huawei) |  |
| CE10.1.3.e | CE10.1.3.a + CE10.1.3.b + CE10.1.3.c | Weiwei Xu (Huawei) |  |

CE10.2: Overlapped block motion compensation

JVET- L0255 (MediaTek, InterDigital)

In JVET- L0255, the overlapping block motion compensation (OBMC) method with low complexity, parallel blending and restricted bandwidth is proposed. The target of the proposed test is to make sure the MC bandwidth and operations will not be worse than a bi-directional MC. This test includes the CU-boundary OBMC and the sub-block OBMC. In the CU-boundary OBMC, the OBMC operations may be enabled or disabled (in sub-block wise or whole CU base) according to the condition of uni-prediction or bi-prediction of the current CU and neighboring CU. In the sub-block OBMC, when generating an sub-block motion compensation (MC) (e.g., 4-by-4), it generates a larger size of MC block (e.g., 6-by-6 and 8-by-8). The extended MC region will be blended with neighboring sub-blocks to generate the weighted prediction current CU. To restrict the bandwidth to be less than bi-directional MC, it will enable or disable the extended MC region according to the condition of uni-prediction or bi-prediction of the current sub-block and neighboring sub-block. The overlapped blending is based on a parallel blending method.

Additionally, some OBMC from one spatial neighboring block is skipped based on the similarity between the MVs of the current block and the neighboring block. Specifically, two different MV difference threshold values i.e., 1.5-pel and 1-pel, are applied when the neighboring block is uni-predicted or bi-predicted.

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE10.2.1 | overlapping block motion compensation (OBMC) - apply uni-prediction constraint to blending lines - OBMC early termination based on MV difference | ChunChia Chen (MediaTek) Xiaoyu Xiu (InterDigital) |  |

CE10.3: Multiple shape prediction partitions

JVET-L0125 (Digital Insights)

In JVET-L0125, triangle partitions (diagonal and inverse diagonal direction) are applied to AMVP mode with uni-prediction constraint. Within this CE, the performance of triangle partitions for AMVP mode in JVET- L0125 is to be studied.

JVET-L0208 (Technicolor)

In JVET-L0208, multiple prediction unit shapes are tested. A CU can be split using 2 prediction units, in either diagonal or inverse diagonal direction, or horizontal or vertical direction with multiple positions. Each prediction unit in the CU has its own uni-prediction motion vector and reference frame index which are derived from a uni-prediction candidate list.

With regards to the shape and the merge index coding, one or more flags signal the shape of the prediction units, then an index indicates the motion predictor. In addition to triangular prediction units, more or alternative shapes may be used. Some exemplary PU shapes are shown on the table below.

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 10.3.1 | Two triangle prediction units (PUs) for AMVP mode | Y. Ahn (Digital Insights) |  |
| 10.3.2.a | Multiple prediction shapes using two rectangle prediction units in place of triangle PUs | T. Poirier (Technicolor) |  |
| 10.3.2.b | Multiple prediction shapes using two rectangle PUs added to the triangle PUs | T. Poirier (Technicolor) |  |
| 10.3.2.c | Multiple prediction shapes using rectangle PUs including symmetric and asymmetric PU partitioning | T. Poirier (Technicolor) |  |
| 10.3.2.d | Multiple prediction shapes using triangle PUs including both symmetric and asymmetric triangle partitioning around the two diagonals of the block. | T. Poirier (Technicolor) |  |
| 10.3.2.e | Multiple prediction shapes using rectangle and triangle PUs with both symmetric and asymmetric partitioning. | T. Poirier (Technicolor) |  |

CE10.4: Diffusion filtering of inter- and intra-prediction signals

JVET-L0157 (HHI)

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE10.4.1 |  |  |  |

CE10.5: Local Illumination Compensation

JVET-L0056 (Samsung)

At the end of processing of each CU, linear models (Ax + B = y) are computed by comparing the prediction pixels (x) and the reconstructed pixels (y). The models are computed using linear regression. The lower edge and right edge pixels are used for model computation both the models are stored. Each model contains parameters for each colour component and for each of the two bi-prediction directions (if available). These parameters (sets of A and B) are then stored in the PU for later access.

Furthermore the parameters are only computed for CUs whose width and height are longer than or equal to 8. This reduces the hardware implementation cost considerably as compared to when it is computed for all the CUs.

The conditions under which this tool is applicable depends on whether the neighboring blocks are skip coded and/or IC coded and the size of the current block. If applicable, a flag is signaled to indicate whether IPR is to be applied to a given inter coded CU or not. If the tool is to be applied, the motion compensation pixels (x) are refined using the application of the linear model (Ax + B) to obtain the new set of prediction pixels. The model used is derived from the current CU’s neighbors. Top, top left, left and left bottom neighbours are considered for borrowing the parameter. Since multiple neighbors may be available, pruning is performed to obtain a single model. Intra coded neighbours are discarded in the pruning process. Next, the neighbours which have trivial model [x = y] are discarded in the pruning process. Models not having the same reference indices are discarded next. If no model is remaining, a trivial model is used. If model(s) still remain, a predefined priority order is used for checking the neighbors.

Additionally, when performing RDO check for determining whether IPR will be used or not, mean-removed SAD (Sum of Absolute Difference) is used instead of the regular SAD for the ME (Motion Estimation) process.

The linear regression method used is same as that in JEM. Also, using two sides for parameter derivation improves the gain.

Key points

- LIC model pre-computation and storage after processing a CU

- LIC model pruning to get the only one model applied to a CU

JVET-L0120 (Panasonic)

The proposed method is based on the LIC in JEM with the following modifications:

- Remove all encoder and decoder LIC processes other than the reconstruction stage.

- Apply LIC process after DMVR refined MC.

- Set bS equal to 1 in DBF when at least one of the blocks uses LIC.

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 10.5.1 | - Use above neighbor parameter for LIC - Use above and left neighbor parameters for LIC | T. Anish (Samsung) |  |
| 10.5.2 | LIC in JEM with the following modifications (JVET-L0120) - Remove all LIC processes other than the reconstruction stage - Apply LIC process after DMVR refined MC - Set bS equal to 1 in DBF when at least one of the blocks uses LIC | K. Abe (Panasonic) |  |

Test conditions and evaluation criteria

The VVC Test Model (VTM) [1] will be used as anchor, where VTM cfg is used. Proposals will be evaluated using BD rates, encoder run time, decoder time with Common Test Conditions (CTC) [2] as agreed at the Macao meeting. Each proponent is suggested to provide additional information related to memory bandwidth and computational complexity.

Expected interdependency with other CE

This CE is related to CE3: Intra prediction and mode coding, and CE4: Inter prediction and MV coding. It is recommended to do combined tests with selected outcomes of CE3 and CE4 to provide information for the group.

Time-line and Responsibilities

General timeline for core experiments (as agreed by JVET).

| T1 | Nov 2, 2018 | - Final CE description uploaded |
| --- | --- | --- |
| T2 | 2 weeks after VTM-3 SW release | - Proponents completed initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | Dec 7, 2018 | - Proponents froze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |

Test results and cross check results are reported to CE coordinators (for inclusion into CE report).

Reference

[1] Description of Test Model

[2] Common test conditions and software reference configurations

Appedix: Full participant list

| Company/ Organization | Participant Name | Email Address |
| --- | --- | --- |
| HHI | Martin Winken Jennifer Rasch Jonathan Pfaff Anastasia Henkel Ivan Zupancic | martin.winken@hhi.fraunhofer.de Jennifer.Rasch@hhi.fraunhofer.de jonathan.pfaff@hhi.fraunhofer.de anastasia.henkel@hhi-extern.fraunhofer.de ivan.zupancic@hhi.fraunhofer.de |
| MediaTek | Chih-Wei Hsu ChunChia Chen Man-Shu Chiang Yu-Wen Huang ChingYeh Chen Peter Chuang Shih-Ta Hsiang Zhi-Yi Lin | cw.hsu@mediatek.com chunchia.chen@mediatek.com Man-Shu.Chiang@mediatek.com yuwen.huang@mediatek.com chingyeh.chen@mediatek.com Peter.Chuang@mediatek.com Shih-ta.Hsiang@mediatek.com Zhi-Yi.Lin@mediatek.com |
| RWTH Aachen University | Max Blaeser Mathias Wien Johannes Sauer | blaeser@ient.rwth-aachen.de wien@ient.rwth-aachen.de sauer@ient.rwth-aachen.de |
| Tencent | Jing Ye Xiaozhong Xu Guichun Li Meng Xu Xiang Li | jingye@tencent.com xiaozhongxu@tencent.com guichunli@tencent.com mengxxu@tencent.com xlxiangli@tencent.com |
| ETRI | Sung-Chang Lim Jung Won Kang | sclim@etri.re.kr jungwon@etri.re.kr |
| SHARP | Zhu Weijia Choi, Byeongdoo Misra, Kiran Tomonori Hashimoto Zhou Tianyang Chujoh Takeshi Ikai Tomohiro Yukinobu Yasugi | zhuw@sharplabs.com choib@sharplabs.com misrak@sharplabs.com tomonori.hashimoto@sharp.co.jp zhou.tianyang@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp yasugi.yukinobu@sharp.co.jp |
| Panasonic | Ru-Ling Liao Chong Soon Lim Li Jingya | ruling.liao@sg.panasonic.com chongsoon.lim@sg.panasonic.com jingya.li@sg.panasonic.com |
| HiSilicon | Zhengjianhua Yongbing Lin | zhengjianhua@hisilicon.com linyongbing@hisilicon.com |
| Broadcom | Minhua Zhou Wade Wan Peisong Chen | minhua.zhou@broadcom.com wade.wan@broadcom.com peisong.chen@broadcom.com |
| Qualcomm | Yu Han Chun-Chi Chen Nan Hu Wei-Jung Chien | yuhan@qti.qualcomm.com chunchic@qti.qualcomm.com nanh@qti.qualcomm.com wchien@qti.qualcomm.com |
| Samsung | Yinji Piao Kiho Choi Jie Chen Seungsoo Jeong Anish Tamse | yj1003.piao@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com ss00.jeong@samsung.com tamse.anish@samsung.com |
| LGE | Seung Hwan Kim Ling Li Sunmi Yoo | seunghwan3.kim@lge.com l.li@lge.com sunmi.yoo@lge.com |
| Huawei | Xu Wei Wei Hai Tao Yang Jianle Chen Zhang Na Alexey Filippov Vasily Rufitskiy | xuweiwei3@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com zhangna25@huawei.com alexey.filippov@huawei.com vasily.rufitskiy@huawei.com |
| Canon | Chris Rosewarne Christophe Gisquet | Chris.Rosewarne@cisra.canon.com.au christophe.gisquet@gmail.com |
| Yonsei University | Sang-hyo Park | spark@barunict.kr |
| Netflix | Andrey Norkin | anorkin@netflix.com |
| Peking University | Zhao Wang, | zhaowang@pku.edu.cn |
| BBC | Saverio Blasi | Saverio.Blasi@bbc.co.uk Andre.SeixasDias@bbc.co.uk |
| Alibaba | YuChen Sun | yuchen.s@alibaba-inc.com |
| Apple | Alexis Tourapis | atourapis@apple.com |
| PIXTREE | Jaeseob Shin | jsshin@pixtree.com |
| Sony | Masaru Ikeda | Masaru.Ikeda@sony.com |
| Technicolor | Leleannec Fabrice Antoine Robert Tangi Poirier Edouard Francois Angi Poirier | Fabrice.Leleannec@technicolor.com antoine.robert@technicolor.com tangi.poirier@technicolor.com Edouard.francois@technicolor.com angi.poirier@technicolor.com |
| Nvidia | Ankur | ankur81saxena@gmail.com |
| Arris | Panusopone, Krit Hong, Seungwook Wang, Limin | Krit.Panusopone@arris.com Seungwook.Hong@arris.com Limin.Wang@arris.com |
| ITRI | Po-Han Lin Ching-Chieh Lin | pohan@itri.com JackLin@itri.com |
| Hikvision | Xuguang Zuo | zuoxuguang@hikvision.com |
| Florida Atlantic University | Hari Kalva | hari.kalva@fau.edu |
| Ericsson | Jacob Ström Ruoyang Yu | jacob.strom@ericsson.com ruoyang.yu@ericsson.com |
| Digital Insights | Yongjo Ahn | yjahn@digitalinsights.co.kr |
| Chips&Media | Wook Jeong | nick.jeong@chipsnmedia.com |
|  |  | yaojen.chang.foxconn@gmail.com |
| Bytedance | Kai Zhang Hongbin Liu Li Zhang | zhangkai.video@bytedance.com liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Dolby Laboratories | Peng Yin | pyin@dolby.com |
| Ittiam Systems Pvt. Ltd | S. Sethuraman | sriram.sethuraman@ittiam.com |
| Kwai | Yi-Wen Chen Xianglin Wang | yiwenchen@kwai.com xianglinwang@kwai.com |
|  |  | i.jaehoon.kim@icloud.com |