| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L1030-v3 |
| --- | --- |

| Title: | Description of Core Experiment 10: Combined and multi-hypothesis prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu Martin Winken | Email: | cw.hsu@mediatek.com martin.winken@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

_____________________________

- Abstract

The main goal of this Core Experiment (CE) is to improve the inter prediction of the Test Model by the use of combined predictions. Five sub CEs are created to test different methods of combined predictions, including:

CE10.1: multi-hypothesis prediction,

CE10.2: overlapped block motion compensation,

CE10.3: multiple shape prediction partitions,

CE10.4: diffusion filtering of inter- and intra-prediction signals,

CE10.5: local illumination compensation.

Participants

| Nr. | Name | Company | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Man-Shu Chiang ChunChia Chen Zhi-Yi Lin Yu-Cheng Lin | MediaTek | Man-Shu.Chiang@mediatek.com chunchia.chen@mediatek.com Zhi-Yi.Lin@mediatek.com Yucheng.Lin@mediatek.com | X | X |
| 2 | Martin Winken Jennifer Rasch | Fraunhofer HHI | martin.winken@hhi.fraunhofer.de Jennifer.Rasch@hhi.fraunhofer.de | X | X |
| 3 | Weiwei Xu Yin Zhao | Huawei | xuweiwei3@huawei.com yin.zhao@huawei.com | X | X |
| 4 | Yongjo Ahn | Digital Insights | yjahn@digitalinsights.co.kr | X | X |
| 5 | Poirier Tangi Le Léannec Fabrice Bordes Philippe | Technicolor | tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com Philippe.bordes@technicolor.com | X |  |
| 6 | Anish Mahendra Tamse | Samsung | tamse.anish@samsung.com | X | X |
| 7 | Kiyofumi Abe | Panasonic | abe.kiyo@jp.panasonic.com | X | X |

CE10.1: Multi-hypothesis prediction

JVET-L0100 (MediaTek)

In JVET-L0100, multi-hypothesis prediction mode is applied to improve uni-prediction of advanced motion vector prediction (AMVP) mode, skip/merge mode, and intra mode. The goal is to combine an existing prediction mode with an extra merge indexed prediction. When applied to skip or merge mode, the merge index for the extra merge indexed prediction is implicitly derived.

Within this CE, the performance of the method described in JVET-L0100 is to be studied for different requirements in terms of the motion compensation bandwidth, run time reduction, and coding gain improvement. Settings affecting these requirements are e.g., the length of the sub-pel interpolation filter. In addition, cache miss analysis in JVET-K0451 can be provided to study the memory bandwidth issue for multi-hypothesis prediction mode.

Since multi-hypothesis prediction for intra mode, e.g., Inter-Intra mode, with size >= 64 luma samples was adopted, different size constraint for larger CUs will be tested as suggested in the meeting.

JVET-L0148/-L0679 (HHI)

In JVET-L0148, a multi-hypothesis inter prediction mode is described.  In this multi-hypothesis inter prediction mode, one or two additional prediction signals can be signaled, in addition to the conventional bi prediction signal.  The resulting overall prediction signal is obtained by sample-wise weighted superposition.  For each additional hypothesis, the following data are transmitted:

- AMVP index (0 or 1)

- Motion vector difference

- Reference index

- Weight index

In JVET-L0679, a simplification is described where the maximum number of AMVP list construction processes that have to be invoked is limited to two (instead of four as in JVET-L0148).

For the additional hypotheses, no sub-pel interpolation filtering is used.  The motion vectors of the additional hypotheses are signaled at luma full-pel accuracy.  For chroma, the motion vectors are quantized to the closest (upper/left) chroma sample.

The additional hypothesis motion parameters are used for the reconstruction of the current block and, possibly, as part of a spatial merging candidate.  Merging of additional hypothesis is only allowed from the left CTU and from within the current CTU, but not from CTUs above.  The additional hypothesis motion parameters are not used for temporal motion vector prediction or deblocking filtering.

Multi-hypothesis inter prediction is only applied to blocks for which both of the following conditions are true:

- block size > 8x8 luma samples

- min(width, height) >= 8

Within this CE, the performance of the method described in JVET- L0679 is to be studied for different settings resulting in different requirements in terms of memory access bandwidth and computational complexity.  For investigating the impact of multi-hypothesis inter prediction on cache-related aspects, the cache model that can be compiled into the reference decoder software by setting #define JVET_J0090_MEMORY_BANDWITH_MEASURE to 1 will be used, in conjunction with the cache config file provided in JVET-K0451.

JVET-L0375 (Huawei)

This proposal simplifies the Inter-Intra mode adopted in Macao meeting from three aspects. The first aspect is to reduce the 4 intra modes used in the Inter-Intra mode to only Planar mode. The second aspect is to simplify the Planar prediction process used in the Inter-Intra mode by skipping reference sample smoothing and PDPC. The third aspect is to use unified weights (4:4) for the whole CU in Inter-Intra prediction sample averaging.

Intra Planar prediction in VTM includes four steps: 1) reference sample availability check and substitution, 2) reference sample filtering, 3) horizontal and vertical planar predictor generation and averaging, and 4) PDPC. The second aspect uses the least steps (i.e., steps 1 and 3) to generate the intra Planar prediction samples used in the Inter-Intra mode.

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE10.1.1.a | Multi-hypothesis prediction for uni-prediction as described in 2.1 - Explicit signaling with merge index - Additional 1 hypothesis for uni-prediction - Total 2 hypothesis - Disabled for small block sizes | Man-Shu Chiang (MediaTek) | Martin Winken (HHI) |
| CE10.1.1.b | Multi-hypothesis prediction for skip/merge mode as described in 2.1 - Implicit derivation of hypothesis - Additional 1 or 2 hypothesis - Total at most 4 hypothesis - Full-pel only for multi-hypotheses | Man-Shu Chiang (MediaTek) | Martin Winken (HHI) |
| CE10.1.2.a | Multi-hypothesis prediction as described in 2.2 - Additional 1 or 2 hypothesis - Total at most 4 hypothesis - Up to 4 different reference pictures - Disabled for small block sizes - Total at most 2 AMVP list construction processes or 1 merge list construction process and 1 AMVP list construction process - No sub-pel interpolation for multi-hypotheses (neither luma nor chroma) - Merging of additional hypothesis possible, but only from left CTU or within current CTU | Martin Winken (HHI) | Man-Shu Chiang (MediaTek) |
| CE10.1.2.b | Like CE10.1.2.a, but - No merging of additional hypotheses | Martin Winken (HHI) | Man-Shu Chiang (MediaTek) |
| CE10.1.2.c | Like CE10.1.2.a, but - Up to 2 different reference pictures | Martin Winken (HHI) | Man-Shu Chiang (MediaTek) |
| CE10.1.2.d | Like CE10.1.2.a, but - Additional hypotheses are only applied for luma | Martin Winken (HHI) | Man-Shu Chiang (MediaTek) |
| CE10.1.3.a | Simplification on Inter-Intra mode in 2.3 - Aspect 1: Intra mode is only Planar (reduced from 4 intra modes) | Weiwei Xu (Huawei) | Man-Shu Chiang (MediaTek) |
| CE10.1.3.b | Simplification on Inter-Intra mode in 2.3 - Aspect 2: intra Planar prediction used in the Inter-Intra mode is simplified by skipping reference sample smoothing and PDPC | Weiwei Xu (Huawei) | Ling Li (LGE) |
| CE10.1.3.c | Simplification on Inter-Intra mode in 2.3 - Aspect 3: same weights (4:4) for a whole CU | Weiwei Xu (Huawei) | Ling Li (LGE) |
| CE10.1.3.d | CE10.1.3.a + CE10.1.3.b | Weiwei Xu (Huawei) | Andre Seixas Dias (BBC) |
| CE10.1.4 | Multi-hypothesis prediction for intra mode as described in 2.1 (Inter-Intra mode) - Disabled for the blocks with sizes larger than 1024 luma samples | Man-Shu Chiang (MediaTek) | Weiwei Xu (Huawei) |
| CE10.1.5.a | CE10.1.3.a + CE10.1.4 | Weiwei Xu (Huawei) Man-Shu Chiang (MediaTek) | Martin Winken (HHI) |
| CE10.1.5.b | CE10.1.3.a + CE10.1.4 - No PDPC in Inter-Intra mode | Weiwei Xu (Huawei) Man-Shu Chiang (MediaTek) | Martin Winken (HHI) |

CE10.2: Overlapped block motion compensation

JVET- L0255 (MediaTek, InterDigital)

In JVET- L0255, the overlapping block motion compensation (OBMC) method with low complexity, parallel blending and restricted bandwidth is proposed. The target of the proposed test is to make sure the MC bandwidth and operations will not be worse than a bi-directional MC. This test includes the CU-boundary OBMC and the sub-block OBMC. In the CU-boundary OBMC, to reduce the bandwidth and interpolation complexity, two methods will be tested, which including a) the OBMC operations may be enabled or disabled (in sub-block wise or whole CU base) according to the condition of uni-prediction or bi-prediction of the current CU and neighboring CU; b) the MV of the neighboring blocks are converted to integer MV.  In the sub-block OBMC, when generating an sub-block motion compensation (MC) (e.g., 4-by-4), it generates a larger size of MC block (e.g., 6-by-6 and 8-by-8). The extended MC region will be blended with neighboring sub-blocks to generate the weighted prediction current CU. To restrict the bandwidth to be less than bi-directional MC, it will enable or disable the extended MC region according to the condition of uni-prediction or bi-prediction of the current sub-block and neighboring sub-block. The overlapped blending is based on a parallel blending method.

Additionally, some OBMC from one spatial neighboring block is skipped based on the similarity between the MVs of the current block and the neighboring block. Specifically, two different MV difference threshold values i.e., 1.5-pel and 1-pel, are applied when the neighboring block is uni-predicted or bi-predicted.

According to the suggested comments during the meeting, the following aspects will be also tested: No apply OBMC across CTU-row boundary. OBMC enabling flag is added when current block size is smaller than 16x16.

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE10.2.1 | Apply uni-prediction constraint on overlapping block motion compensation (OBMC) - current block should be uni-prediction - OBMC blocks from top or left neighboring blocks are generated by uni-prediction - OBMC early termination based on MV difference | ChunChia Chen (MediaTek) Xiaoyu Xiu (InterDigital) | Sung-Chang Lim (ETRI) |
| CE10.2.2 | Apply integer MV constraint on OBMC - OBMC blocks from top or left neighboring blocks are generated by converting neighboring MVs to integer MVs - OBMC early termination based on MV difference | Zhi-Yi Lin (MediaTek) Xiaoyu Xiu (InterDigital) | Yoshitaka KIDANI(KDDI) |
| CE10.2.3 | - CE10.2.1 - sub-block OBMC is applied when current block is uni-prediction and coded in affine mode | ChunChia Chen (MediaTek) Xiaoyu Xiu (InterDigital) | Sung-Chang Lim (ETRI) |
| CE10.2.4 | - CE10.2.2 - sub-block OBMC is applied when current block is uni-prediction and coded in affine mode | Zhi-Yi Lin (MediaTek) Xiaoyu Xiu (InterDigital) | Yoshitaka KIDANI(KDDI) |

CE10.3: Multiple shape prediction partitions

JVET-L0125 (Digital Insights)

In JVET-L0125, triangle partitions (diagonal and inverse diagonal direction) are applied to AMVP mode with uni-prediction constraint. Within this CE, the performance of triangle partitions for AMVP mode in JVET- L0125 is to be studied.

JVET-L0208 (Technicolor)

In JVET-L0208, multiple prediction unit shapes are tested. A CU can be split using 2 prediction units, in either diagonal or inverse diagonal direction, or horizontal or vertical direction with multiple positions. Each prediction unit in the CU has its own uni-prediction motion vector and reference frame index which are derived from a uni-prediction candidate list.

With regards to the shape and the merge index coding, one or more flags signal the shape of the prediction units, then an index indicates the motion predictor. In addition to triangular prediction units, more or alternative shapes may be used. Some exemplary PU shapes are shown on the table below.

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE10.3.1 | Two triangle prediction units (PUs) for AMVP mode | Y. Ahn (Digital Insights) | - Jaeil Kim (SK telecom) |
| CE10.3.2.a | Multiple prediction shapes using two rectangle prediction units in place of triangle PUs | T. Poirier (Technicolor) |  |
| CE 10.3.2.b | Multiple prediction shapes using two rectangle PUs added to the triangle PUs | T. Poirier (Technicolor) |  |
| CE 10.3.2.c | Multiple prediction shapes using rectangle PUs including symmetric and asymmetric PU partitioning | T. Poirier (Technicolor) |  |
| CE 10.3.2.d | Multiple prediction shapes using triangle PUs including both symmetric and asymmetric triangle partitioning around the two diagonals of the block. | T. Poirier (Technicolor) |  |
| CE 10.3.2.e | Multiple prediction shapes using rectangle and triangle PUs with both symmetric and asymmetric partitioning. | T. Poirier (Technicolor) |  |

CE10.4: Diffusion filtering of inter- and intra-prediction signals

JVET-L0157 (HHI)

In the first test, CE10.4.1, an encoder speedup for the Uniform Diffusion filters, described in JVET-L0157, is investigated. The filter masks are simplified and no iterations are used. There are three types of filters, two directional filters and one weighted interpolation of the directional filters.

In the second test, CE10.4.2, a restriction of the Diffusion filters to specific types of predictions is investigated.

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE10.4.1 | Uniform Diffusion filters with encoder speedup and simplified filter masks | Jennifer Rasch (HHI) | Jacob Ström (Ericsson) |
| CE10.4.2 | Uniform Diffusion filters restricted to specific types of predictions | Jennifer Rasch (HHI) | Jacob Ström (Ericsson) |

CE10.5: Local Illumination Compensation

JVET-L0056 (Samsung)

At the end of processing of each CU, linear models (Ax + B = y) are computed by comparing the prediction pixels (x) and the reconstructed pixels (y). The models are computed using linear regression. The lower edge and right edge pixels are used for model computation both the models are stored. Each model contains parameters for each colour component and for each of the two bi-prediction directions (if available). These parameters (sets of A and B) are then stored in the PU for later access.

Furthermore the parameters are only computed for CUs whose width and height are longer than or equal to 8. This reduces the hardware implementation cost considerably as compared to when it is computed for all the CUs.

The conditions under which this tool is applicable depends on whether the neighboring blocks are skip coded and/or IC coded and the size of the current block. If applicable, a flag is signaled to indicate whether IPR is to be applied to a given inter coded CU or not. If the tool is to be applied, the motion compensation pixels (x) are refined using the application of the linear model (Ax + B) to obtain the new set of prediction pixels. The model used is derived from the current CU’s neighbors. Top, top left, left and left bottom neighbours are considered for borrowing the parameter. Since multiple neighbors may be available, pruning is performed to obtain a single model. Intra coded neighbours are discarded in the pruning process. Next, the neighbours which have trivial model [x = y] are discarded in the pruning process. Models not having the same reference indices are discarded next. If no model is remaining, a trivial model is used. If model(s) still remain, a predefined priority order is used for checking the neighbors.

Additionally, when performing RDO check for determining whether IPR will be used or not, mean-removed SAD (Sum of Absolute Difference) is used instead of the regular SAD for the ME (Motion Estimation) process.

The linear regression method used is same as that in JEM. Also, using two sides for parameter derivation improves the gain.

Key points

- LIC model pre-computation and storage after processing a CU

- LIC model pruning to get the only one model applied to a CU

JVET-L0120 (Panasonic)

The proposed method is based on the LIC in JEM with the following modifications:

- Remove all encoder and decoder LIC processes other than the reconstruction stage.

- Apply LIC process after DMVR refined MC.

- Set bS equal to 1 in DBF when at least one of the blocks uses LIC.

JVET-L0203 (Technicolor)

The proposed method proposes an implementation of Local Illumination Compensation (LIC) with reduced memory bandwidth.

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| - CE10.5.1.a | - Use above neighbor parameter for LIC | - T. Anish (Samsung) |  |
| - CE10.5.1.b | - Use above and left neighbor parameters for LIC | T. Anish (Samsung) |  |
| CE10.5.2 | LIC in JEM with the following modifications (JVET-L0120) - Remove all LIC processes other than the reconstruction stage - Apply LIC process after DMVR refined MC - Set bS equal to 1 in DBF when at least one of the blocks uses LIC | K. Abe (Panasonic) | T. Anish (Samsung) |
| CE10.5.3 | - (LIC) with reduced memory bandwidth. | P. Bordes (Technicolor) | K. Abe (Panasonic) |

Test conditions and evaluation criteria

The VVC Test Model (VTM) [1] will be used as anchor, where VTM cfg is used. Proposals will be evaluated using BD rates, encoder run time, decoder time with Common Test Conditions (CTC) [2] as agreed at the Macao meeting. Each proponent is suggested to provide additional information related to memory bandwidth and computational complexity.  For studying cache-related aspects, the cache model that can be compiled into the reference decoder software by setting #define JVET_J0090_MEMORY_BANDWITH_MEASURE to 1 will be used, in conjunction with the cache config file provided in JVET-K0451.

Expected interdependency with other CE

This CE is related to CE2: Sub-block based motion prediction, CE3: Intra prediction and mode coding, and CE4: Inter prediction and MV coding. It is recommended to do combined tests with selected outcomes of CE2, CE3 and CE4 to provide information for the group.

Time-line and Responsibilities

General timeline for core experiments (as agreed by JVET).

| T1 | Nov 2, 2018 | - Final CE description uploaded |
| --- | --- | --- |
| T2 | Dec 6, 2018 | - Finished implementation of the technologies into the code base - Initial study by cross-checkers can begin - No software changes after T2 |
| T3 | Dec 19, 2018 | - Proponents report results to CE coordinators for inclusion in CE report |

Reference

[1] Description of Test Model

[2] Common test conditions and software reference configurations

Appedix: Full participant list

| Company/ Organization | Participant Name | Email Address |
| --- | --- | --- |
| HHI | Martin Winken Jennifer Rasch Jonathan Pfaff Anastasia Henkel Ivan Zupancic | martin.winken@hhi.fraunhofer.de Jennifer.Rasch@hhi.fraunhofer.de jonathan.pfaff@hhi.fraunhofer.de anastasia.henkel@hhi-extern.fraunhofer.de ivan.zupancic@hhi.fraunhofer.de |
| MediaTek | Chih-Wei Hsu ChunChia Chen Man-Shu Chiang Yu-Wen Huang ChingYeh Chen Peter Chuang Shih-Ta Hsiang Zhi-Yi Lin | cw.hsu@mediatek.com chunchia.chen@mediatek.com Man-Shu.Chiang@mediatek.com yuwen.huang@mediatek.com chingyeh.chen@mediatek.com Peter.Chuang@mediatek.com Shih-ta.Hsiang@mediatek.com Zhi-Yi.Lin@mediatek.com |
| RWTH Aachen University | Max Blaeser Mathias Wien Johannes Sauer | blaeser@ient.rwth-aachen.de wien@ient.rwth-aachen.de sauer@ient.rwth-aachen.de |
| Tencent | Jing Ye Xiaozhong Xu Guichun Li Meng Xu Xiang Li | jingye@tencent.com xiaozhongxu@tencent.com guichunli@tencent.com mengxxu@tencent.com xlxiangli@tencent.com |
| ETRI | Sung-Chang Lim Jung Won Kang | sclim@etri.re.kr jungwon@etri.re.kr |
| SHARP | Zhu Weijia Choi, Byeongdoo Misra, Kiran Tomonori Hashimoto Zhou Tianyang Chujoh Takeshi Ikai Tomohiro Yukinobu Yasugi | zhuw@sharplabs.com choib@sharplabs.com misrak@sharplabs.com tomonori.hashimoto@sharp.co.jp zhou.tianyang@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp yasugi.yukinobu@sharp.co.jp |
| Panasonic | Ru-Ling Liao Chong Soon Lim Li Jingya | ruling.liao@sg.panasonic.com chongsoon.lim@sg.panasonic.com jingya.li@sg.panasonic.com |
| HiSilicon | Zhengjianhua Yongbing Lin | zhengjianhua@hisilicon.com linyongbing@hisilicon.com |
| Broadcom | Minhua Zhou Wade Wan Peisong Chen | minhua.zhou@broadcom.com wade.wan@broadcom.com peisong.chen@broadcom.com |
| Qualcomm | Yu Han Chun-Chi Chen Nan Hu Wei-Jung Chien | yuhan@qti.qualcomm.com chunchic@qti.qualcomm.com nanh@qti.qualcomm.com wchien@qti.qualcomm.com |
| Samsung | Yinji Piao Kiho Choi Jie Chen Seungsoo Jeong Anish Tamse | yj1003.piao@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com ss00.jeong@samsung.com tamse.anish@samsung.com |
| LGE | Seung Hwan Kim Ling Li Sunmi Yoo | seunghwan3.kim@lge.com l.li@lge.com sunmi.yoo@lge.com |
| Huawei | Xu Wei Wei Hai Tao Yang Jianle Chen Zhang Na Alexey Filippov Vasily Rufitskiy | xuweiwei3@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com zhangna25@huawei.com alexey.filippov@huawei.com vasily.rufitskiy@huawei.com |
| Canon | Chris Rosewarne Christophe Gisquet | Chris.Rosewarne@cisra.canon.com.au christophe.gisquet@gmail.com |
| Yonsei University | Sang-hyo Park | spark@barunict.kr |
| Netflix | Andrey Norkin | anorkin@netflix.com |
| Peking University | Zhao Wang, | zhaowang@pku.edu.cn |
| BBC | Saverio Blasi | Saverio.Blasi@bbc.co.uk Andre.SeixasDias@bbc.co.uk |
| Alibaba | YuChen Sun | yuchen.s@alibaba-inc.com |
| Apple | Alexis Tourapis | atourapis@apple.com |
| PIXTREE | Jaeseob Shin | jsshin@pixtree.com |
| Sony | Masaru Ikeda | Masaru.Ikeda@sony.com |
| Technicolor | Leleannec Fabrice Antoine Robert Tangi Poirier Edouard Francois Philippe Bordes | Fabrice.Leleannec@technicolor.com antoine.robert@technicolor.com tangi.poirier@technicolor.com Edouard.francois@technicolor.com philippe.bordes@technicolor.com |
| Nvidia | Ankur | ankur81saxena@gmail.com |
| Arris | Panusopone, Krit Hong, Seungwook Wang, Limin | Krit.Panusopone@arris.com Seungwook.Hong@arris.com Limin.Wang@arris.com |
| ITRI | Po-Han Lin Ching-Chieh Lin | pohan@itri.com JackLin@itri.com |
| Hikvision | Xuguang Zuo | zuoxuguang@hikvision.com |
| Florida Atlantic University | Hari Kalva | hari.kalva@fau.edu |
| Ericsson | Jacob Ström Ruoyang Yu | jacob.strom@ericsson.com ruoyang.yu@ericsson.com |
| Digital Insights | Yongjo Ahn | yjahn@digitalinsights.co.kr |
| Chips&Media | Wook Jeong | nick.jeong@chipsnmedia.com |
|  |  | yaojen.chang.foxconn@gmail.com |
| Bytedance | Kai Zhang Hongbin Liu Li Zhang | zhangkai.video@bytedance.com liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Dolby Laboratories | Peng Yin | pyin@dolby.com |
| Ittiam Systems Pvt. Ltd | S. Sethuraman | sriram.sethuraman@ittiam.com |
| Kwai | Yi-Wen Chen Xianglin Wang | yiwenchen@kwai.com xianglinwang@kwai.com |
|  |  | i.jaehoon.kim@icloud.com |
| SK telecom | Jaeil Kim Taeyoung Na | jaeilkim@sk.com taeyoung.na@sk.com |