| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0032-v1 |
| --- | --- |

| Title: | Description of Core Experiment 12 (CE12): mapping functions |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | E. François, P. Yin | Tel: Email: | edouard.francois@technicolor.com pyin@dolby.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

The purpose of this CE is to investigate the performance of mapping for content coding with focus on SDR. The considered technologies are in-loop reshaping and related variants. The considered technologies will be compared using objective metrics to be reported by the proponents and the cross-checkers.

# Tools description

This CE will study the following tools:

- [JVET-L0246]: SDR In-loop Reshaping

- [JVET-L0247]: Universal low complexity reshaper for SDR and HDR video

## [JVET-L0246, JVET-L0247, Dolby]: SDR in-loop reshaping

The proposed universal reshaper has two main components: 1) in-loop reshaping applied to the luma component; 2) complementary luma dependent chroma residue scaling applied to chroma components.

### Universal low complexity in-loop luma reshaping architecture

The in-loop luma reshaper is implemented as a pair of look-up tables (LUTs); but only one of the two LUTs need to be signaled as the other one can be computed from the signaled LUT.

Each LUT is a one-dimensional, 10-bit, 1024-entry mapping table (1D-LUT). One LUT is a forward LUT, FwdLUT, that maps input luma code values Yi to altered values Yr: Yr=FwdLUT[Yi]. The other LUT is an inverse LUT, InvLUT, that maps altered code values Yr to Yi : Yi=InvLUT[Yr]. (Yi represents the reconstruction values of Yi.)

For intra slices, only the InvLUT is applied.  For inter slices, both FwdLUT and InvLUT are applied.

LUTs are applied before loop filtering for both intra and inter slices. Processing operations and data flow are identical for SDR and HDR.

The processing operations for intra slices are illustrated in Figure 1.  InvLUT maps intra reconstructed values in the reshaped domain to intra reconstructed values in the original domain. (Yi=InvLUT[Yr])

The processing operations for inter slices are illustrated in Figure 2.  FwdLUT maps motion-compensation values in the original domain to the reshaped domain. (FwdLUT[Ypred])  InvLUT then maps inter reconstructed values in the reshaped domain to inter reconstructed values in the original domain. (Yi=InvLUT[Yres+FwdLUT[Ypred]]).

Figure 1. Intra slice reconstruction with in-loop luma reshaper

Figure 2. Inter slice reconstruction with in-loop luma reshaper

### Chroma residue scaling

Chroma residue scaling is a simple multiplicative process implemented with fixed-point integer operation. Chroma residue scaling compensates for luma signal interaction with the chroma signal. Chroma residue scaling is applied at the TU level.

Encoder side: cResScale = cRes * cScale

Decoder side: cRes = cResScale/cScale

cRes is the original chroma residual signal and cResScale is the scaled chroma residual signal. cScale is a luma-dependent scaling factor calculated using FwdLUT and using the average luma prediction signal of the TU for inter mode, and the average luma reconstructed signal of the TU for intra mode.

# Tests to be performed

In the following tests, variants of in-loop reshaping for SDR, based on JVET-L0247, will be evaluated.

## CE12-1: In-loop Reshaping for SDR on coding efficiency

| Test | Proponent | Description | Xchecker |
| --- | --- | --- | --- |
| CE12-1 | Dolby (JVET-L0247) | Test in-loop reshaping for SDR focused on coding efficiency improvement, such as making reshaping function rate adaptive, disable towards higher rates. | E. François (Technicolor) |

## CE12-2: In-loop Reshaping for SDR on implementation investigation

| Test | Proponent | Description | Xchecker |
| --- | --- | --- | --- |
| CE12-2 | Dolby (JVET-L0247) | Test in-loop reshaping for SDR focused on implementation investigation on pipelining of the block-wise prediction loop: such as piece-wise linear interpolation replacing LUT-based mapping, disable intra-block in inter slices. | J. Xu (ByteDance), E. François (Technicolor) |

# Experimental Test Conditions

## Software

VTM3 software shall be used for these experiments.

## Test Sequences, Bit Rates, Coding Conditions

Tests sequences, bit rates, and coding conditions defined in the common tests conditions described in document JVET-K1010 for SDR will be used.

## Evaluation of Results

The following data shall be reported for each test:

- objective metrics, encoding and decoding runtime ratios, as specified in the SDR CTCs (JVET-K1010).

  - Decoding time shall be computed with YUV output activated.

- complexity analysis

- analysis of the bitrate distribution

# Time-line

| T1 | End of meeting#12 + 2 weeks | Final CE description |
| --- | --- | --- |
| T2 |  | VTM3 delivery |
| T3 | T2 + 3 weeks | Completion of tool integration, test configuration announcement, made software available |
| T4 | T3 + 1 week | Changes to software branches is frozen and proponent starts reporting results to CE coordinators for inclusion in CE report. Cross-checks start. |
| T5 | Beginning of Meeting#13 – 1 week | Completion of cross-check. Reporting of complete results to CE coordinators. |

# Participants

| Name | Company | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- |
| Taoran Lu Fangjun Pu Peng Yin Sean McCarthy | Dolby | tlu@dolby.com fpu@dolby.com pyin@dolby.com smcca@dolby.com | X |  |
| Jizheng Xu Li Zhang | Bytedance | xujizheng@bytedance.com lizhang.idm@btyedance.com |  | X |
| Edouard François Christophe Chevance Franck Hiron | Technicolor | edouard.francois@technicolor.com christophe.chevance@technicolor.com franck.hiron@technicolor.com |  | X |
| Dmytro Rusanovskyy Adarsh K. Ramasubramonian | Qualcomm | dmytror@qti.qualcomm.com |  |  |
| Andrew Segall Kiran Misra Takeshi Chujoh Tomohiro Ikai | Sharp | asegall@sharplabs.com misrak@sharplabs.com chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |  |  |
| Alexei Filippov Vasily Rufitskiy Sergei Ikonin | Huawei | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com Sergey.Ikonin@huawei.com |  |  |
| Sriram Sethuraman | Ittiam | Sriram.sethuraman@ittiam.com |  |  |
| Yi-Wen Chen Xianglin Wang | Kwai | yiwenchen@kwai.com xianglinwang@kwai.com |  |  |
| Peisong Chen Minhua Zhou Wade Wan | Broadcom | peisong.chen@broadcom.com minhua.zhou@broadcom.com wade.wan@broadcom.com |  |  |
| Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |  |  |
| Pankaj Topiwala | FastVDO | pankajtva@gmail.com |  |  |
| Alexis Tourapis | Apple | atourapis@apple.com |  |  |
| Chirag Pujara Amith DSouza | Samsung | chirag.p@samsung.com amith.ds@samsung.com |  |  |
| Dan Grois | Comcast | grois@ieee.org |  |  |
| Koohyar Minoo |  | koohyar.minoo@gmail.com |  |  |
| Limin Wang |  | cuiwang@gmail.com |  |  |
| Jangwon Choi Hyeongmun Jang | LGE | jangwon84.choi@lge.com hm.jang@lge.com |  |  |
| Ching-Chieh Lin Po-Han Lin | ITRI | jacklin@itri.com pohan@itri.com |  |  |
| Jacob Ström | Ericsson | jacob.strom@ericsson.com |  |  |