| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0022-v1 |
| --- | --- |

| Title: | CE2: Summary report on sub-block based motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yuwen He, Ching-Yeh Chen, Chun-Chi Chen | Email: | yuwen.he@interdigital.com chingyeh.chen@mediatek.com chunchic@qti.qualcomm.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of Core Experiment 2 (CE2) is to investigate sub-block based motion prediction techniques on top of VTM-3.0. It comprises 5 categories,

- CE 2.1: Affine motion compensation

- CE 2.2: Affine merge mode

- CE 2.3: Sub-block based merge mode

- CE 2.4: Complexity reduction

- CE 2.5: ATMVP and related topics

For each test, a comparative study along with related tests is conducted, results and complexity analysis are provided. Crosschecking reports of all tests are integrated in this document as well.

# CE2.1: Affine motion compensation

## Categorization & comparative study

### Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 2.1.1 | Adaptive precision for affine MVD coding: {1-pel,1/4-pel,1/4-pel}, {1/4-pel,1/4-pel,1/4-pel}, {1/8-pel,1/8-pel,1/8-pel} | JVET-M0420 |
| 2.1.2 | Adaptive MVD precision for affine inter mode coding: {1-pel,1-pel,1-pel}, {1/4-pel,1/4-pel, 1/4-pel}, {1/16-pel,1/16-pel,1/16-pel} | JVET-M0246 |

### Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.1.1 | -0.19% | -0.08% | -0.12% | 105% | 101% | -0.13% | -0.16% | 0.04% | 106% | 100% |
| 2.1.2 | -0.23% | -0.21% | -0.27% | 104% | 99% | -0.20% | -0.08% | -0.30% | 112% | 100% |

### Complexity analysis

### Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 2.1.1 | J. Lainema (Nokia) | 2 | F | Y | M |
|  | C.-H. Hung (Qualcomm) |  |  |  |  |
| 2.1.2 | T.-H. Li (Foxconn) | 1 | F | Y (*VTM3 encoder bug) | M |
|  | C.-H. Hung (Qualcomm) |  |  |  |  |

## Technical description

### CE2.1.1 in JVET-M0420 (InterDigital)

In VTM-3.0, affine MVD precision for the signaling is fixed ({1/4-pel, 1/4-pel, 1/4-pel}). Adaptive precision is proposed for affine MVD signaling. Three precisions are proposed for affine MVD coding at those control points. The precision set {prec_TL, prec_TR, prec_BL} is used to indicate the MVD precision for top-left, top-right and bottom-left control points. For 6-parameter affine model, three precision sets are used: {1-pel, 1/4-pel, 1/4-pel}, {1/4-pel, 1/4-pel, 1/4-pel}, and {1/8-pel, 1/8-pel, 1/8-pel}. For 4-parameter affine mode, the same three precisions are used, except that the precision for bottom-left control point is not needed.

### CE2.1.2 in JVET-M0246 (Bytedance)

Extend Adaptive Motion Vector Resolution (AMVR) to affine inter mode coding, wherein MVD in affine inter mode can be coded with adaptive precision.

# CE2.2: Affine merge mode

## Categorization & comparative study

### Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 2.2.1.a | Bypass Affine flag for Skip CU | JVET-M0380 |
| 2.2.1.b | CABAC context change for Affine Flag | JVET-M0380 |
| 2.2.2.a | 2 contexts: one for Affine and one for regular Merge index | JVET-M0381 |
| 2.2.2.b | 1 context: one for both Affine Merge index and regular Merge index | JVET-M0381 |
| 2.2.3.a | Applying affine HMVP candidates to affine merge | JVET-M0125 |
| 2.2.3.b | Applying affine HMVP candidates for affine motion vector prediction | JVET-M0125 |
| 2.2.3.c | Combined test of 2.2.3a and 2.2.3b | JVET-M0125 |
| 2.2.3.d | Replace existing inherited affine candidates with affine HMVP candidates | JVET-M0125 |
| 2.2.4.a | Affine merge with offsets and block level signaled | JVET-M0431 |
| 2.2.4.b | Affine merge with offset and extension of value range | JVET-M0431 |
| 2.2.4.c | POC distance based offset mirroring for Bi-prediction | JVET-M0431 |
| 2.2.4.d | POC distance based offset scaling for Bi-prediction | JVET-M0431 |
| 2.2.5 | Affine MV offset merge candidates | JVET-M0476 |
| 2.2.6.a | Simplification for 6-param and 4-param constructed candidates by setting P = 3 in 7 availability cases | JVET-M0477 |
| 2.2.6.b | Simplification for 6-param and 4-param constructed candidates by setting P = 3 in 2 availability cases | JVET-M0477 |
| 2.2.7.a | Low complexity affine merge with temporal candidates | JVET-M0256 |
| 2.2.7.b | Medium complexity affine merge with temporal candidates | JVET-M0256 |
| 2.2.7.c | Withdraw |  |
| 2.2.7.d | Withdraw |  |
| 2.2.7.e | Withdraw |  |
| 2.2.8.a | In affine AMVP, apply some pruning on candidates. Rounding, clipping applied before pruning | JVET-M0282 |
| 2.2.8.b | In affine merge, apply some pruning on inherited candidates | JVET-M0282 |
| 2.2.8.c | 2.2.8.a + 2.2.8.b | JVET-M0282 |
| 2.2.8.d | 2.2.8.c + 4.1.5.c | JVET-M0282 |

### Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.2.1.a | 0.09% | 0.21% | 0.20% | 102% | 98% | 0.03% | 0.04% | 0.06% | 103% | 92% |
| 2.2.1.b | -0.01% | 0.02% | -0.01% | 100% | 100% | 0.00% | 0.02% | 0.06% | 99% | 98% |
| 2.2.2.a | 0.00% | 0.00% | -0.01% | 100% | 98% | 0.03% | -0.05% | 0.10% | 99% | 94% |
| 2.2.2.b | 0.07% | 0.04% | 0.06% | 100% | 104% | 0.05% | 0.03% | 0.15% | 100% | 96% |
| 2.2.3.a | -0.06% | -0.04% | -0.02% | 100% | 100% | -0.06% | 0.04% | 0.09% | 101% | 100% |
| 2.2.3.b | -0.01% | 0.01% | 0.02% | 100% | 100% | 0.00% | 0.00% | 0.09% | 102% | 101% |
| 2.2.3.c | -0.07% | -0.04% | -0.04% | 100% | 100% | -0.03% | -0.05% | -0.18% | 101% | 102% |
| 2.2.3.d | 0.07% | 0.10% | 0.06% | 100% | 100% | 0.09% | 0.09% | 0.02% | 101% | 101% |
| 2.2.4.a | -0.16% | -0.13% | -0.14% | 104% | 100% | 0.01% | -0.05% | -0.07% | 107% | 100% |
| 2.2.4.b | -0.17% | -0.12% | -0.13% | 104% | 100% | -0.02% | 0.09% | -0.04% | 106% | 100% |
| 2.2.4.c | -0.20% | -0.08% | -0.09% | 104% | 100% | 0.04% | -0.01% | 0.19% | 105% | 100% |
| 2.2.4.d | -0.20% | -0.09% | -0.09% | 104% | 99% | 0.02% | 0.07% | 0.07% | 106% | 99% |
| 2.2.5 | -0.06% | -0.02% | -0.05% | 104% | 101% | -0.03% | 0.06% | -0.03% | 105% | 99% |
| 2.2.6.a | 0.00% | 0.01% | -0.01% | 100% | 100% | 0.07% | -0.01% | 0.18% | 99% | 99% |
| 2.2.6.b | 0.02% | 0.04% | 0.00% | 100% | 100% | 0.08% | 0.14% | 0.04% | 99% | 98% |
| 2.2.7.a | -0.08% | -0.02% | -0.13% | 105% | 102% | -0.03% | -0.12% | -0.09% | 116% | 115% |
| 2.2.7.b | -0.10% | -0.10% | -0.14% | 106% | 102% | -0.07% | -0.21% | -0.08% | 117% | 116% |
| 2.2.8.a | 0.00% | 0.01% | 0.00% | 100% | 100% | -0.01% | 0.04% | -0.07% | 100% | 100% |
| 2.2.8.b | -0.01% | -0.01% | 0.00% | 101% | 100% | -0.03% | 0.00% | -0.05% | 100% | 100% |
| 2.2.8.c | -0.02% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.05% | 0.10% | 100% | 100% |
| 2.2.8.d | -0.04% | -0.01% | -0.01% | 100% | 100% | -0.03% | -0.12% | -0.17% | 100% | 100% |

### Complexity analysis

|  | AMVP/Merge list size | Max number of potential candidates | Max number of MV comp. | Max number of ref idx comp. | Max number of MV scaling | Max number of conditional cases to enable constructed candidates | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.2.1.a | same | Same | Same | same | same | same |  |
| 2.2.1.b | same | Same | Same | same | same | same |  |
| 2.2.2.a | same | Same | Same | same | same | same |  |
| 2.2.2.b | same | Same | Same | same | same | same |  |
| 2.2.3.a | same | Merge +5 | Same | same | same |  | *Affine HMVP 170 bytes |
| 2.2.3.b | same | AMVP + 2 | Same | +2 | same |  | *Do ref index selection for affine HMVP based on the selected ref. in AMVP *Affine HMVP 68 bytes |
| 2.2.3.c | same | Merge +5 / AMVP +2 | Same | +2 | same |  | *Affine HMVP 170 bytes |
| 2.2.3.d | same | same | Same | -1 | same |  | Reduce CPMV buffer from 1296 bytes to 170 bytes. Note: 1296 bytes requires optimized affine CPMV buffer update. Otherwise, storage of a full CTU's CPMV needs 6192 bytes. Position comp: merge + 5/AMVP +4 |
| 2.2.4.a | same | Same | Same | same | same | same |  |
| 2.2.4.b | same | Same | Same | same | same | same |  |
| 2.2.4.c | same | Same | same | same | same | same |  |
| 2.2.4.d | same | same | same | same | same | same |  |
| 2.2.5 | Merge+20 | +20 | same | same | same | same |  |
| 2.2.6.a | same | -3 | same | same | same | same |  |
| 2.2.6.b | same | -3 | same | same | same | 2(-5) |  |
| 2.2.7.a | same | same | + 38 | -12 | +2 |  |  |
| 2.2.7.b | same | +3 | +98 | +26 | +3 |  |  |
| 2.2.8.a | same | same | +3 | same | same |  |  |
| 2.2.8.b | same | same | +6 | +2 | same |  |  |
| 2.2.8.c | same | same | +3 for AMVP +6 for Merge | +0 for AMVP +2 for Merge | same |  |  |
| 2.2.8.d |  |  |  |  |  |  |  |

### Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 2.2.1.a | F. Galpin (Technicolor) | 2 | F | Y | M |
| 2.2.1.b | F. Galpin (Technicolor) | 2 | F | Y | M |
| 2.2.2.a | F. Galpin (Technicolor) | 2 | F | Y | M |
| 2.2.2.b | F. Galpin (Technicolor) | 2 | F | Y | M |
| 2.2.3.a | H. Chen (Huawei) | 2 | F | Y | M |
| 2.2.3.b | H. Chen (Huawei) | 2 | F | Y | M |
| 2.2.3.c | H. Chen (Huawei) | 2 | F | Y | M |
| 2.2.3.d | H. Chen (Huawei) | 2 | F | Y | M |
| 2.2.4.a | L. Li (LGE) |  |  |  |  |
| 2.2.4.b | L. Li (LGE) |  |  |  |  |
| 2.2.4.c | L. Li (LGE) |  |  |  |  |
| 2.2.4.d | L. Li (LGE) |  |  |  |  |
| 2.2.5 | H. Liu (Bytedance) | 1 | F | Y | M |
| 2.2.6.a | H. Liu (Bytedance) | 1 | F | Y | M |
| 2.2.6.b | H. Liu (Bytedance) | 1 | F | Y | M |
| 2.2.7.a | H. Huang (Qualcomm) | 1 | F | Y | G |
| 2.2.7.b | H. Huang (Qualcomm) | 1 | F | Y | G |
| 2.2.8.a | G. Laroche (Canon) | 1 | F | Y | M |
| 2.2.8.b | G. Laroche (Canon) | 1 | F | Y | M |
| 2.2.8.c | G. Laroche (Canon) | 1 | F | Y | M |
| 2.2.8.d | G. Laroche (Canon) | 1 | F | Y | M |

## Technical description

### On Affine mode restriction in JVET-M0380 (Canon)

Test: CE2.2.1.a (Bypass Affine flag for Skip CU)

The affine motion compensation prediction is particularly efficient for complex motion like rotation, zoom in, zoom out and perspective motion. The Skip mode is efficient for static content or constant motion. It seems that these modes compensate different kinds of motion and are not compatible. This test proposes to bypass the affine mode flag coding when the current CU is “skip” coded.

Test: CE2.2.1.b (Affine flag context simplification)

This test aligns the block positions used to derive the affine flag context with the Merge candidate positions. The new proposed block positions for the Affine flag context are A1 and B1 instead of A2 and B3 in VTM3. With this modification, only 5 block positions (instead of 7) needs to be checked before decoding or parsing the Affine flag.

### On Merge Index coding in JVET-M0381 (Canon)

Test: CE2.2.2

The proposed modification will consist in reducing the number of CABAC coded bins for the Affine Merge index. Similarly to L0194, which was related to the classical Merge mode, only one context for the first bit of the Affine Merge index will be used while the 4 other bits will be CABAC bypassed.

### History Based Affine MV Candidate in JVET-M0125 (LGE & Tencent)

Test: CE2.2.3

In these tests, history based affine MV candidate (Affine HMVP) methods will be applied to affine merge list or affine AMVP list generation or both. The following aspects will be investigated:

- Generation of a table to store motion information of coded affine CUs

- The derivation of affine HMVP candidate from the stored affine motion information.

- Simplified pruning for affine HMVP candidates.

- Impact of various history table sizes, e.g. 4, 6, 8

- Coexist or replace existing affine inherited candidates

### Affine merge with prediction offset in JVET-M0431 (Tencent)

Test: CE2.2.4

These tests affine merge with prediction offset were proposed in JVET-L0320. The proposed method selects the first available affine merge candidate as a base predictor. Then it applies a motion vector offset to each control point’s motion vector value from the base predictor. If there’s no affine merge candidate available, the proposed methods will not be used.

Two signaling methods were proposed:

- Per control point (CP) signaling

The selected base predictor’s inter prediction direction, and the reference index of each direction is used without change.

For each control point, a zero_MVD flag is used to indicate whether the control point needs offset signaling. If zero_MVD flag is true, there’s no other signaling needed for the control point. Otherwise, a distance index and an offset direction index is signaled for the control point.

Distance index is signaled to indicate which distance offset to use from the offset table.

| Distance IDX | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Distance-offset | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel |

The direction index can represent four directions as shown below, where only x or y direction may have an MV difference, but not in both directions.

| Offset Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-dir-factor | +1 | –1 | 0 | 0 |
| y-dir-factor | 0 | 0 | +1 | –1 |

If the inter prediction is Uni-prediction, the signaled distance offset is applied on the offset direction for each control point predictor. Results will be the MV value of each control point.

If the inter prediction is Bi-prediction, the signaled distance offset is applied on the signaled offset direction for control point predictor’s L0 motion vector; the offset to be applied on L1 MV is applied on a mirrored basis.

- Per block signaling

In this method, the distance offset index and the offset direction index are signaled per block. The same offset will be applied to all available control points. The number of control points is determined by the base predictor’s affine type. The distance offset table and the offset direction tables are the same as in 1).

Since the signaling is done for all the control points of the block at once, the zero_MVD flag is not used in this method.

### Affine MV offset merge candidates in JVET-M0476 (Foxconn)

Test: CE2.2.5

New Affine merge candidates are generated based on the MVs offsets of the first Affine merge candidate or the first sub-block MV merge candidate. In Uni-prediction, the MV offsets are applied to the MVs of the first candidate. In Bi-prediction with List 0 and List 1 on the same direction, the MV offsets are applied to the MVs of the first candidate as follows:

MVnew(L0), i = MVold(L0) + MVoffset(i)

MVnew(L1), i = MVold(L1) + MVoffset(i)

In Bi-prediction with List 0 and List 1 on the opposite direction, the MV offsets are applied to the MVs of the first candidate as follows:

MVnew(L0), i = MVold(L0) + MVoffset(i)

MVnew(L1), i = MVold(L1) - MVoffset(i)

On top of new adoptions in Affine merge aspect, this sub-test will test various offset directions combined with various offset magnitudes for the proposed Affine MV offset merge candidates. The various numbers of the Affine MV offset merge candidates will be tested.

### Simplification of the constructed Affine merge candidates in JVET-M0477 (Foxconn)

Test: CE2.2.6

The first aspect in JVET-L0390 is related to reduction of the maximum number of available merge candidates. This test focuses on the simplification of the constructed Affine merge candidates. Define 6-param constructed candidates and 4-param constructed candidates as follows:

0: Null

1: [CP0, CP1, CP2]

2: [CP0, CP1, CP3]

3: [CP0, CP2, CP3]

4: [CP1, CP2, CP3]

5: [CP0, CP1]

6: [CP0, CP2].

The constructed candidate indexes enabled in 7 availability cases are shown as the following table:

| V: Availability X:Unavailability |  |  |  | Constructed candidate index |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP0 | CP1 | CP2 | CP3 |  |  |  |  |  |  |
| V | V | V | V | 1 | 2 | 3 | 4 | 5 | 6 |
| V | V | V | X | 1 | 5 | 6 |  |  |  |
| V | V | X | V | 2 | 5 |  |  |  |  |
| V | X | V | V | 3 | 6 |  |  |  |  |
| X | V | V | V | 4 |  |  |  |  |  |
| V | V | X | X | 5 |  |  |  |  |  |
| V | X | V | X | 6 |  |  |  |  |  |
| others |  |  |  | 0 |  |  |  |  |  |

The modifications are as one or combination of the following descriptions:

- At most M constructed candidates are allowed to be selected out of 6-parameter Affine model (6-param) constructed candidates, where M < 4

Example 1: Set M = 1 to select at most 1 constructed candidates out of 6-parameter Affine models in 7 availability cases. In other cases, the constructed candidates are disabled.

| V: Availability X:Unavailability |  |  |  | Constructed candidate index |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP0 | CP1 | CP2 | CP3 |  |  |  |  |  |  |
| V | V | V | V | 1 | 5 | 6 |  |  |  |
| V | V | V | X | 1 | 5 | 6 |  |  |  |
| V | V | X | V | 2 | 5 |  |  |  |  |
| V | X | V | V | 3 | 6 |  |  |  |  |
| X | V | V | V | 4 |  |  |  |  |  |
| V | V | X | X | 5 |  |  |  |  |  |
| V | X | V | X | 6 |  |  |  |  |  |
| Others |  |  |  | 0 |  |  |  |  |  |

- At most N constructed candidates are allowed to be selected out of 4-parameter Affine model (4-param) constructed candidates, where N < 2

Example 2: Set N = 1 to select at most 1 constructed candidates out of 4-parameter Affine models in 7 availability cases. In other cases, the constructed candidates are disabled.

| V: Availability X:Unavailability |  |  |  | Constructed candidate index |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP0 | CP1 | CP2 | CP3 |  |  |  |  |  |  |
| V | V | V | V | 1 | 2 | 3 | 4 | 5 |  |
| V | V | V | X | 1 | 5 |  |  |  |  |
| V | V | X | V | 2 | 5 |  |  |  |  |
| V | X | V | V | 3 | 6 |  |  |  |  |
| X | V | V | V | 4 |  |  |  |  |  |
| V | V | X | X | 5 |  |  |  |  |  |
| V | X | V | X | 6 |  |  |  |  |  |
| Others |  |  |  | 0 |  |  |  |  |  |

- At most P constructed candidates are allowed to be selected out of constructed candidates, where P < 6

Example 3: Set P = 3 to select at most 3 constructed candidates out of 6-parameter and 4-parameter Affine models in 7 availability cases. In other cases, the constructed candidates are disabled. This is Test (a).

| V: Availability X:Unavailability |  |  |  | Constructed candidate index |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP0 | CP1 | CP2 | CP3 |  |  |  |  |  |  |
| V | V | V | V | 1 | 2 | 3 |  |  |  |
| V | V | V | X | 1 | 5 | 6 |  |  |  |
| V | V | X | V | 2 | 5 |  |  |  |  |
| V | X | V | V | 3 | 6 |  |  |  |  |
| X | V | V | V | 4 |  |  |  |  |  |
| V | V | X | X | 5 |  |  |  |  |  |
| V | X | V | X | 6 |  |  |  |  |  |
| Others |  |  |  | 0 |  |  |  |  |  |

Example 4: Set P = 3 to select at most 3 constructed candidates out of 6-parameter and 4-parameter Affine models in 2 availability cases, where CP0, CP1, and CP2 are all available. In the other cases, the constructed candidates are disabled. This is Test (b).

| V: Availability X:Unavailability |  |  |  | Constructed candidate index |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP0 | CP1 | CP2 | CP3 |  |  |  |  |  |  |
| V | V | V | V | 1 | 2 | 3 |  |  |  |
| V | V | V | X | 1 | 5 | 6 |  |  |  |
| Others |  |  |  | 0 |  |  |  |  |  |

The modifications can reduce the number of if-else checks.

### Constructed temporal candidates in JVET-M0256 (Technicolor)

Test: CE2.2.7

The tests driven in this section will be performed on top of the VTM 3 for affine merge mode by using the concepts described in JVET-L0522 (constructed temporal candidates and improved affine flag coding).

### Motion Vector Pruning in JVET-M0282 (Technicolor)

Test: CE2.2.8

This proposal includes two aspects. First, before any pruning operation takes place the involved motion vectors are computed in their final representation. This means the MVs are set to the appropriate precision level, rounded to the right motion vector resolution or clipped, before being compared to candidates already present in the concerned list. Second aspect consists in adding some pruning operations between candidates to improve the trade-off between coding efficiency and complexity.

# CE2.3: Sub-block based merge mode

## Categorization & comparative study

### Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 2.3.1.a.1 | Planar motion vector prediction (PMVP) as a single mode with the sub-block size setting: 8x4 for BI, 4x4 for UNI | JVET-M0104 |
| 2.3.1.a.2 | PMVP as a single mode with the sub-block size setting: 4x8 for BI, 4x4 for UNI |  |
| 2.3.1.a.3 | PMVP as a single mode with the sub-block size setting: Adaptive 8x4/4x8 for BI, 4x4 for UNI |  |
| 2.3.1.a.4 | PMVP as a single mode with the sub-block size setting: 8x8 for BI, 4x4 for UNI |  |
| 2.3.1.a.5 | PMVP as a single mode with the sub-block size setting: Adaptive 8x4/4x8 for BI and UNI |  |
| 2.3.1.a.6 | PMVP as a single mode with the sub-block size setting: 8x8 for BI and UNI |  |
| 2.3.1.b | Withdraw |  |
| 2.3.1.c | PMVP as a candidate in sub-block (affine) merge list |  |
| 2.3.1.d | Withdraw |  |
| 2.3.2.a | The threshold is set as 1 pixel for the sub-blocks inside the 4x16, 16x4 and 8x8 blocks | JVET-M0485 |
| 2.3.2.b | The threshold is set to 0 pixel for 4x16 and 16x4 blocks along vertical and horizontal directions, respectively; and the threshold is set as 1 pixel for the sub-blocks inside 8x8 blocks |  |
| 2.3.3.a | Regression-based Motion Vector Field (RMVF) as a separate merge mode as described in JVET-L0171 | JVET-M0302 |
| 2.3.3.b | 2.3.3.a + the RMVF-coded blocks are used for affine merge candidate inheritance |  |
| 2.3.3.c | 2.3.3.a + complexity reduction (i.e., lower number of neighboring motion information for RMVF parameter derivation) |  |
| 2.3.3.d | 2.3.3.b + 2.3.3.c |  |

### Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.3.1.a.1 | -0.11% | -0.10% | -0.17% | 103% | 103% | -0.08% | 0.01% | 0.09% | 104% | 103% |
| 2.3.1.a.2 | -0.10% | -0.18% | -0.18% | 103% | 103% | -0.05% | 0.03% | 0.01% | 104% | 103% |
| 2.3.1.a.3 | -0.11% | -0.13% | -0.22% | 103% | 103% | -0.04% | -0.11% | -0.07% | 104% | 104% |
| 2.3.1.a.4 | -0.11% | -0.07% | -0.15% | 103% | 101% | -0.05% | -0.07% | -0.19% | 104% | 102% |
| 2.3.1.a.5 | -0.10% | -0.15% | -0.18% | 103% | 102% | -0.06% | 0.01% | 0.02% | 104% | 103% |
| 2.3.1.a.6 | -0.11% | -0.10% | -0.17% | 103% | 103% | -0.08% | 0.01% | 0.09% | 104% | 103% |
| 2.3.1.c |  |  |  |  |  |  |  |  |  |  |
| 2.3.2.a (over VTM-3.0) | -0.12% | -0.18% | -0.21% | 104% | 107% | -0.08% | -0.05% | 0.05% | 106% | 112% |
| Over 2.3.1 |  |  |  |  |  |  |  |  |  |  |
| 2.3.2.b | -0.12% | -0.15% | -0.23% | 104% | 107% | -0.08% | 0.22% | 0.08% | 106% | 112% |
| Over 2.3.1 |  |  |  |  |  |  |  |  |  |  |
| 2.3.3.a | -0.18% | -0.17% | -0.26% | 105% | 102% | -0.02% | -0.10% | 0.09% | 106% | 102% |
| 2.3.3.b | -0.20% | -0.25% | -0.24% | 104% | 101% | -0.02% | 0.12% | 0.21% | 103% | 102% |
| 2.3.3.c | -0.19% | -0.13% | -0.19% | 105% | 102% | 0.01% | 0.05% | -0.03% | 106% | 101% |
| 2.3.3.d | -0.19% | -0.22% | -0.23% | 105% | 100% | 0.00% | 0.07% | -0.16% | 106% | 101% |

### Complexity analysis

### Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 2.3.1.a.1 | X. Zheng (DJI) |  |  |  |  |
| 2.3.1.a.2 | X. Zheng (DJI) |  |  |  |  |
| 2.3.1.a.3 | Ikai (Sharp) |  |  |  |  |
| 2.3.1.a.4 | Ikai (Sharp) |  |  |  |  |
| 2.3.1.a.5 | M. Gao (Tencent) |  |  |  |  |
| 2.3.1.a.6 | M. Gao (Tencent) |  |  |  |  |
| 2.3.1.c | X. Zheng (DJI) |  |  |  |  |
| 2.3.2.a | X. Chen (HiSilicon) |  |  |  |  |
| 2.3.2.b | X. Chen (HiSilicon) |  |  |  |  |
| 2.3.3.a | T. Solovyev (Huawei) |  |  |  |  |
| 2.3.3.b | T. Solovyev (Huawei) |  |  |  |  |
| 2.3.3.c | T. Solovyev (Huawei) |  |  |  |  |
| 2.3.3.d | T. Solovyev (Huawei) |  |  |  |  |

## Technical description

### CE2.3.1 in JVET-M0104 (HiSilicon)

To generate a smooth fine granularity motion field, the figure below gives a brief description of the planar motion vector prediction (PMVP) process.

Planar motion vector prediction is achieved by averaging a horizontal and a vertical linear interpolation on sub-block basis as follows.

- Separately signaling for each coding unit.

- Once there are two different MVs in the L-shaped area, the mode is activated.

- Ref_idx in L0 and L1 are set to 0, all the MVs in the L-shaped area are scaled if needed.

- In the case a neighboring block in the L-shaped area is intra coded, it is padded in the same manner as intra reference pixel padding, separately for L0 and L1.

- BR temporal motion is used for planar interpolation. If BR temporal motion is not available, exactly the same planar prediction for intra mode is applied.

- It is a merge mode.

### CE2.3.2 in JVET-M0485 (Tencent)

The MVs for all 4x4 sub-blocks inside the 4x16, 16x4 or 8x8 blocks are constrained such that the max difference between integer parts of these sub-blocks is no more than a given threshold.

### CE2.3.3 in JVET-M0302 (Nokia)

JVET-L0171 proposed a new sub-block merge mode with Regression-based Motion Vector Field (RMVF). RMVF uses a 6-parameter motion model for calculating the motion vectors of sub-blocks.

MVX_subPUMVY_subPU= axxaxyayxayyXsubPUYsubPU+ bxby

Motion parameters are calculated based on one line and row of spatially neighboring 4x4 sub-blocks using their motion vectors and center locations as an input to a linear regression method.

In this test RMVF is implemented as a new merge mode in VTM-3.0. As part of the experiment the complexity aspects of the tool will be analyzed.

# CE2.4: Complexity reduction

## Categorization & comparative study

### Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 2.4.1.a | 4x4 block is used as the sub-block size for a uni-directional affine coded CU while 8x4/4x8 block is used as the sub-block size for a bi-directional affine coded CU according to the width/height of the CU. | JVET-M0226 |
| 2.4.1.b | 4x4 block is used as the sub-block size for a uni-directional affine coded CU while 8x4 block is used as the sub-block size for a bi-directional affine coded CU. |  |
| 2.4.1.c | 4x4 block is used as the sub-block size for a uni-directional affine coded CU while 4x8 block is used as the sub-block size for a bi-directional affine coded CU. |  |
| 2.4.2 | Restricted motion vector field for affine mode. | JVET-M0309 |
| 2.4.3.a | Affine restrictions for the worst-case bandwidth reduction: The worst-case bandwidth is bi-prediction of an INTER_8x4/INTER_4x8 block. (*changes only luma) | JVET-M0150 |
| 2.4.3.b | The worst-case bandwidth is uni-prediction of an INTER_4x4 block. (*changes only luma) |  |
| 2.4.3.c | The worst-case bandwidth is bi-prediction of an INTER_8x8 block. (*changes only luma) |  |
| 2.4.4.a | Affine sub-block size restrictions: 8x4 for BI, 4x4 for UNI. (* changes applied to both luma and chroma) | JVET-M0472 |
| 2.4.4.b | 4x8 for BI, 4x4 for UNI. (* changes applied to both luma and chroma) |  |
| 2.4.4.c | Adaptive 8x4/4x8 for BI, 4x4 for UNI. (* changes applied to both luma and chroma) |  |
| 2.4.4.d | 8x8 for BI, 4x4 for UNI. (* changes applied to both luma and chroma) |  |
| 2.4.5.a | clip sub-block MVs in 8x8 block for UNI and BI prediction. | JVET-M0488 |
| 2.4.5.b | Withdraw. |  |
| 2.4.6 | Modified affine inheritance from above CTU. | JVET-M0054 |
| 2.4.7 | Size constrain for inherited affine motion prediction. | JVET-M0053 |
| 2.4.8 | Affine model inheritance with reduced memory requirement. | JVET-M0262 |
| 2.4.9 | Withdraw. |  |

### Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.4.1.a | 0.11% | 0.10% | 0.09% | 100% | 99% | 0.10% | 0.12% | 0.11% | 101% | 97% |
| 2.4.1.b | 0.10% | 0.09% | 0.08% | 101% | 99% | 0.11% | 0.00% | 0.20% | 99% | 95% |
| 2.4.1.c | 0.12% | 0.07% | 0.09% | 99% | 96% | 0.13% | 0.16% | 0.16% | 99% | 95% |
| 2.4.2 | 0.01% | 0.00% | 0.05% | 101% | 100% | 0.01% | -0.05% | 0.07% | 101% | 100% |
| 2.4.3.a | -0.01% | 0.02% | 0.01% | 99% | 100% | 0.02% | -0.04% | -0.05% | 101% | 101% |
| 2.4.3.b | 0.27% | 0.14% | 0.18% | 97% | 98% | 0.27% | 0.22% | 0.08% | 99% | 100% |
| 2.4.3.c | 0.03% | 0.06% | 0.01% | 99% | 99% | -0.01% | -0.08% | -0.07% | 101% | 100% |
| 2.4.4.a | 0.10% | 0.09% | 0.08% | 99% | 99% | 0.11% | 0.00% | 0.20% | 98% | 98% |
| 2.4.4.b | 0.12% | 0.07% | 0.09% | 99% | 99% | 0.13% | 0.16% | 0.16% | 99% | 99% |
| 2.4.4.c | 0.11% | 0.10% | 0.09% | 99% | 99% | 0.10% | 0.12% | 0.11% | 99% | 99% |
| 2.4.4.d | 0.22% | 0.18% | 0.16% | 98% | 98% | 0.20% | 0.24% | 0.31% | 97% | 99% |
| 2.4.5.a | 0.01% | 0.00% | -0.03% | 100% | 100% | -0.02% | -0.10% | 0.06% | 102% | 103% |
| 2.4.6 | -0.01% | 0.01% | -0.02% | 100% | 100% | 0.02% | -0.03% | 0.02% | 100% | 99% |
| 2.4.7 | 0.02% | 0.02% | 0.03% | 100% | 100% | 0.05% | -0.11% | 0.05% | 100% | 99% |
| 2.4.8 | 0.00% | -0.02% | 0.02% | 100% | 99% | 0.03% | 0.15% | -0.04% | 100% | 103% |

### Complexity analysis

| Test# | Bandwidth for uni/bi-prediction | Affine MVs storage | Others (additional operations for MV clipping or derivation, and so on.) |
| --- | --- | --- | --- |
| Translational model | 6.25 / 8.56 |  |  |
| Affine model | 8.33 / 16.66 |  |  |
| 2.4.1.a | 8.33 / 11.84 | Same | 2 comparisons per CU |
| 2.4.1.b | (1 for width/height + 1 for uni/bi) |  |  |
| 2.4.1.c | 8.33 / 11.84 | Same | 1 comparison per CU |
| 2.4.2 | (1 for uni/bi) |  |  |
| 2.4.3.a | 8.33 / 11.84 | Same | 1 comparison per CU |
| 2.4.3.b | (1 for uni/bi) |  |  |
| 2.4.3.c | 8.33 / 8.56 | Same | MV clipping for subblk MVs |
| 2.4.4.a | 8.33 / 16.66 | Same | 3 comparisons per CU |
| 2.4.4.b | (1 for width/height + 1 for uni/bi + 1 for restriction) |  |  |
| 2.4.4.c | 8.33 / | Same |  |
| 2.4.4.d | 8.33 / 16.66 | Same | 13 comparisons per CU |
| 2.4.5.a | (1 for uni/bi + 12 for restriction) |  |  |
| 2.4.6 | Same | Same |  |
| 2.4.7 | Same | 25% |  |
| 2.4.8 | Same |  |  |

### Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 2.4.1.a | T. Zhou (Sharp) | 1 | F | Y | M |
| 2.4.1.b | T. Zhou (Sharp) | 1 | F | Y | M |
| 2.4.1.c | T. Zhou (Sharp) | 1 | F | Y | M |
| 2.4.2 | Y.-L. Hsiao (MediaTek) | 2 | F | Y | M |
| 2.4.3.a | S. Jeong (Samsung) | 2 | F | Y | M |
| 2.4.3.b | S. Jeong (Samsung) | 2 | F | Y | M |
| 2.4.3.c | S. Jeong (Samsung) | 2 | F | Y | M |
| 2.4.4.a | Y.-W. Chen (Kwai) | 2 | F | Y | M |
| 2.4.4.b | Y.-W. Chen (Kwai) | 2 | F | Y | M |
| 2.4.4.c | Y.-W. Chen (Kwai) | 2 | F | Y | M |
| 2.4.4.d | Y.-W. Chen (Kwai) | 2 | F | Y | M |
| 2.4.5.a | C.-C. Chen (Mediatek) | 2 | F | Y | M |
| 2.4.6 | H. Chen (Huawei) | 2 | F | Y | M |
| 2.4.7 | H. Chen (Huawei) | 2 | F | Y | M |
| 2.4.8 | T.-H, Li (Foxconn) |  |  |  |  |

## Technical description

### CE2.4.1 in JVET-M0226 (Kwai)

In JVET-L0104, it is proposed to restrict a 4x4 block from using bi-directional motion compensation. The details of the proposal are as follows:

- Disallow bi-directional inter mode (AMVP mode and Merge mode) for each 4x4 inter CU.

- Enlarge the minimum sub-block size of ATMVP merge mode from 4x4 to 8x8; ATMVP is not allowed when either width or height of a CU is equal to 4.

- 4x4 block is used as the sub-block size for a uni-directional affine coded CU while 8x4/4x8 block is used as the sub-block size for a bi-directional affine coded CU

The items 1) and 2) have been adopted into VVC and item 3) is further studied in this CE. The tests related to the item 3) are illustrated as shown in the table in section 4.1 test 2.4.1.

### CE2.4.2 in JVET-M0309 (Panasonic)

To reduce the worst-case memory bandwidth of an affine CU, the sub-block motion vectors of the affine CU are constrained to be within a pre-defined motion vector field which is calculated according to the motion vector of first sub-block, the size of the affine CU and the prediction direction of the affine CU (i.e. uni-prediction or bi-prediction). Assume that the memory is retrieved per CU instead of per sub-block and the motion vectors of first (top left) and the second sub-blocks are (v0x,v0y) and (vix, viy), values of vix and viy exhibit the following constraints:

vix ∊ [v0x-H, v0x+H],

viy ∊ [v0y-V, v0y+V].

The values H and V are chosen to guarantee that the worst case memory bandwidth of the affine CU does not exceed that of normal inter MC of a 8×8 bi-prediction block. If the motion vector of any sub-block exceeds the pre-defined motion vector field, the motion vector is clipped. An illustration of the idea of constrained sub-block motion vector is given in the below figure.

### CE2.4.3 in JVET-M0150 (Qualcomm)

In this test, the performance of affine restrictions for worst-case bandwidth reduction is evaluated.

a) The worst-case bandwidth is bi-prediction of an INTER_8x4/INTER_4x8 block.

b) The worst-case bandwidth is uni-prediction of an INTER_4x4 block.

c) The worst-case bandwidth is bi-prediction of an INTER_8x8 block.

### CE2.4.4 in JVET-M0472 (Huawei)

In affine mode, different sub-block shapes are used, which depends on the prediction direction and the CU size, as shown in the table below.

  - For uni-prediction affine coded CU, the sub-block size is set equal to 4x4.

  - For bi-prediction affine coded CU, and if the width of the CU is larger than or equal to its height, the sub-block size is set equal to 8x4;

  - For bi-prediction affine coded CU, and if the width of the CU is smaller than its height, the sub-block size is set equal to 4x8;

| Prediction direction and CU size | Sub-block size |
| --- | --- |
| uni-prediction | 4x4 |
| bi-prediction, CU width >= CU height | 8x4 |
| bi-prediction, CU width < CU height | 4x8 |

Affine sub-block restrictions:

- 8x4 for BI, 4x4 for UNI

- 4x8 for BI, 4x4 for UNI

- Adaptive 8x4/4x8 for BI, 4x4 for UNI

- 8x8 for BI, 4x4 for UNI

### CE2.4.5 in JVET-M0488 (Tencent)

For affine prediction, it is proposed to constrain the MVs of four 4x4 sub-blocks within one 8x8 block such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel.

### CE2.4.6 in JVET-M0054 (Qualcomm)

In the proposed method, deriving a 6-parameter affine inherited candidate from a neighboring block in the above CTU is illustrated in Fig. 1. The top-left CPMV v0 and top-right CPMVs v1 are derived by the the bottom-left MV vLB and bottom-right MV vRB of the neighboring candidate block:

v0=vRB-vLBneiW*posCurX-posNeiX+vLB

v1=vRB-vLBneiW*posCurX+curW-posNeiX+vLB

Wherein neiW is the width of the neighboring block, curW is the width of current block, posNeiX is the x coordinate of the top-left pixel of neighboring block, and posCurX is the x coordinate of the top-left pixel of current block.

The bottom-left CPMV v2 is derived as follows:

If the MV from left neighbor vleft is available and has the same reference picture as  vLB

v2 =vleft;

Else if the MV from bellow-left neighbor vbellowleft is available and has the same reference 
      picture as  vLB

v2 =vbellowleft;

Else

deltaHorX,deltaHorY=vRB-vLBneiW

∇vVer=(-deltaHorY,deltaHorX)

v2 =vRB-vLBneiW*posCurX-posNeiX+∇vVer*curH+vLB

Wherein curH is the height of current block.

### CE2.4.7 in JVET-M0053 (Qualcomm)

It’s proposed to disable inherited affine motion prediction from a neighboring affine coded block if the width or height of the neighboring block is less than threshold.

### CE2.4.8 in JVET-M0262 (Bytedance)

In affine model inheritance, less MVs stored in neighbouring blocks are required.

### [withdraw]

# CE2.5: ATMVP and related topics

## Categorization & comparative study

### Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 2.5.1 | Simplification of ATMVP, add additional CU size restriction (>64) | JVET-M0165 |
| 2.5.2.a | Withdraw | JVET-M0227 |
| 2.5.2.b | Withdraw |  |
| 2.5.2.c | Insert the new ATMVP into the affine merge candidate list in additional to the original ATMVP for non-low-delay picture with pruning |  |
| 2.5.2.d | Withdraw |  |
| 2.5.2.e | Withdraw |  |
| 2.5.2.f | Withdraw |  |
| 2.5.2.g | Withdraw |  |
| 2.5.2.h | For non-low-delay picture, insert the new ATMVP; for low-delay picture, insert VTM-3.0 ATMVP |  |
| 2.5.3.a | Withdraw |  |
| 2.5.3.b | Withdraw |  |
| 2.5.4 | Non-sub-block ATMVP | JVET-M0086 |

### Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.5.1 | 0.00% | 0.00% | 0.05% | 100% | 100% | 0.00% | -0.04% | 0.08% | 100% | 101% |
| 2.5.2.c | -0.04% | -0.07% | -0.06% | 100% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| 2.5.2.h | 0.26% | 0.14% | 0.15% | 101% | 101% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| 2.5.4 | 0.19% | 0.28% | 0.26% | 100% | 99% | 0.52% | 0.52% | 0.66% | 101% | 97% |

### Complexity analysis

### Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 2.5.1 | M. Gao (Tencent) | 1 | F | Y | M |
| 2.5.2.c | Y.-L. Hsiao (MediaTek) | 2 | F | Y | M |
| 2.5.2.h | G. Li (Tencent) | 2 | F | Y | M |
| 2.5.4 | Y.-L. Hsiao (MediaTek) | 2 | F | Y | M |

## Technical description

### CE2.5.1 in JVET-M0165 (MediaTek)

In JVET-L0092, a simplification method for ATMVP is described to simply disable the ATMVP for small size of CU. This method can reduce the hardware processing cycles for small CUs. Currently, the sub-block merge mode is disabled when width or height is smaller than 8. This CE test wants to disable ATMVP when the number of samples in one CU is smaller than 128, 256, or other value. Within this CE, the performance of the method described in JVET-L0092 is to be studied for different requirements, such as disabling ATMVP for small area, small width, small height, and so on. Due to combination of ATMVP and affine merge into a unified subblock-based merge list, the coherence of the disabling rule related to affine merge is also be studied.

### CE2.5.2 in JVET-M0227 (Kwai)

In JVET-L0105, a new ATMVP-like merge candidate is generated and inserted into the merge candidate list in addition to the original ATMVP. The derivation process of the new ATMVP is similar to the original ATMVP, with slightly different rules used in selecting motion vectors (MV) from corresponding reference sub-blocks. More specifically, the MV selection rules for TMVP from a reference block are used in deriving MVs for each subblock under the new ATMVP mode.

### [withdraw]

### CE2.5.4 in JVET-M0086 (Panasonic)

The proposed non-sub-block ATMVP is based on the existing ATMVP. The corresponding area is derived by the same method as current ATMVP. But it selects a set of MV in the centre position of corresponding area for entire target CU as shown in the figure below. The derived MV is CU unit, therefore, all processes after MV derivation are just same as other merge candidates. And it can also solve the subjective image quality problem of strong sub-block boundary noise.

# Recommendations

Review CE2 document if needed. Review all CE2-related documents.