| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0023-v1 |
| --- | --- |

| Title: | CE3: Summary Report on Intra Prediction and Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Jin Heo Alexey Filippov | Email: | geertv@qti.qualcomm.com jin78.heo@lge.com Alexey.Filippov@huawei.com |
| Source: | CE3 coordinators |  |  |

_____________________________

# Abstract

This is the summary report of the third Core Experiment (CE3) [1]. The goal of CE3 is to study intra prediction tools, including mode coding, for potential inclusion into the VVC standard.

The following is the list of defined sub-tests in CE3:

- CE3.1: Intra prediction modes (5 tests)

- CE3.2: Cross-component prediction (11 tests)

- CE3.3: Intra mode coding (10 tests)

This document summarizes the objective results (BD-rates, runtimes), cross-check reports, and related input contributions.

# Test conditions and guidelines

CTC conditions as agreed at the 12th JVET meeting (Macau, CN) for SDR (JVET-L1010) are followed for the evaluation of objective performance. The anchor is the ‘VVC Test Model’ or VTM. Results are provided for the ‘All Intra’ and ‘Random Access’ test configurations. The VTM version 3.0 software is used with ‘vtm’ configuration files to produce the VTM anchor. Tools are implemented into the VTM version 3.0 software. Proposals are compared with respect to objective compression efficiency while complexity is also assessed.

The source codes of the tests and full test results are uploaded by proponents into the following CE3 GitLab repository:

https://vcgit.hhi.fraunhofer.de/JVET-L-CE3/VVCSoftware_VTM.git

# Cross-check reporting

Each test is covered by at least one cross-checker and the cross-check reports are included within the respective sub-test sections. Within CE3 it was agreed that cross-checkers answer the following questions:

- Did the cross-checker study the software? If so, did the cross-checker find any significant inconsistencies between the description and the provided software?

- Do the cross-checked results closely match the proponent results for AI and RA conditions?

- Were additional results cross-checked?

- Additional comments that may be helpful to the group?

This report summarizes the answers in tables reporting whether the objective test results match in one table column and all other comments in a second table column.

# Overview of CE3 test description document changes

The following changes were made to the CE3 test description document [1] after the T2 deadline (December 6, 2018) had passed. Besides changes to contact persons and assignment of cross-checkers, the following changes were requested on the JVET reflector and discussed for clarification, if needed:

- CE3.2:

  - Added test CE3.2.2.1 (MMLM; combination of tests CE3.2.1 and CE3.2.2)

  - Added test CE3.2.6.2 (CCLM; testing 3 columns of neighbouring samples on left side of block, 1 row above block)

- CE3.3:

  - Reduction and clarification of test CE3.3.1 from 7 subtests to 4 (decoder-side intra mode derivation)

# CE3.1 on ‘Intra prediction modes’

## Defined tests [1]

| Test # | Description | Doc. # |
| --- | --- | --- |
| 1.1.1 | Intra Sub-Partitions coding mode with a better trade-off between gain and encoding run-time | JVET-M0102 |
| 1.1.2 | Test 1.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples |  |
| 1.2.1 | Affine linear weighted intra prediction modes with encoder speedup | JVET-M0043 |
| 1.2.2 | Affine linear weighted intra prediction modes with a fixed number of weights needed per NxN block |  |
| 1.3 | Harmonization simplified linear interpolation intra prediction (LIP) with PDPC | JVET-M0252 |

## CE3.1: Test results

### CE3.1: VTM-3.0 anchor

|  |  | All Intra Main10 - Over VTM-3.0 |  |  |  |  | Random Access Main10 - Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1.1 | Intra Sub-Partitions coding mode with a better trade-off between gain and encoding run-time | -0.59% | -0.44% | -0.47% | 112% | 103% | -0.29% | -0.31% | -0.15% | 102% | 103% |
| 1.1.2 | Test 1.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples | -0.46% | -0.34% | -0.34% | 112% | 104% | -0.24% | -0.28% | -0.18% | 102% | 102% |
| 1.2.1 | Affine linear weighted intra prediction modes with encoder speedup | -1.36% | -1.02% | -1.01% | 153% | 105% | -0.85% | -0.92% | -0.98% | 112% | 99% |
| 1.2.2 | Affine linear weighted intra prediction modes with a fixed number of weights needed per NxN block | -0.95% | -0.42% | -0.46% | 153% | 101% | -0.57% | -0.73% | -0.83% | 110% | 98% |
| 1.3 | Harmonization simplified linear interpolation intra prediction (LIP) with PDPC | -0.08% | -0.14% | -0.11% | 106% | 100% | -0.04% | 0.00% | -0.01% | 101% | 100% |

### CE3.1: Additional test results

|  |  | All Intra Main10 - Over VTM-3.0 |  |  |  |  | Random Access Main10 - Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1.2.1 | Test 1.1.2 with 1-D transform and entropy coding for blocks that cannot be vertically divided (JVET-M0102) | -0.52% | -0.40% | -0.42% | 112% | 104% | -0.25% | -0.25% | -0.16% | 102% | 102% |

### CE3.1: Cross-check reports

| Test# | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 1.1.1 | Ching-Chieh Lin (ITRI) |  |  |
| 1.1.2 | Ching-Chieh Lin (ITRI) |  |  |
| 1.2.1 | Alexey Filippov (Huawei) |  |  |
|  | Fabien Racape (Technicolor) | y | The software matches the CE description. Bd-rate results are exact; the execution times are within the measurement error range. |
| 1.2.2 | Alexey Filippov (Huawei) |  |  |
|  | Fabien Racape (Technicolor) | y | The software matches the CE description. Bd-rate results are exact; the execution times are within the measurement error range. |
| 1.3 | Vasily Rufitskiy (Huawei) |  |  |

## CE3.1: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-M0426 (HHI) | 1.1.2 | CE3-related: Improvement on the Intra Sub-Partitions Coding Mode |

# CE3.2 on ‘Cross-component prediction’

## Defined tests [1]

| Test # | Description | Doc. # |
| --- | --- | --- |
| 2.1 | CCLM + MDLM + MMLM | JVET-M0097 |
| 2.2 | CCLM + MDLM + MMLM + Above-MMLM + Left-MMLM | JVET-M0475 |
| 2.2.1 | Combination of 2.1 and 2.2 | JVET-M0098 |
| 2.3 | CCLM + MDLM + Adaptive multiple cross-component linear model | JVET-M0504 |
| 2.4 | Modified CCLM downsampling filter for “type-2” content | JVET-M0142 |
| 2.5.1 | CCLM + MDLM, using MaxMin method, and β derived by using Cmean and Lmean | JVET-M0401 |
| 2.5.2 | CCLM + MDLM, using classification-based mean value method |  |
| 2.5.3 | CCLM + MDLM, using MaxMin method; using the first top row and the second left column |  |
| 2.5.4 | CCLM + MDLM, using classification-based mean value method; using the first top row and the second left column |  |
| 2.6.1 | CCLM accessing less neighbouring luma samples, interaction with other adopted coding tools: one row above and one column left luma samples are accessed | JVET-M0263 |
| 2.6.2 | 2.6.1 with three columns of left luma samples as in the anchor |  |

## CE3.2: Test results

### CE3.2: VTM-3.0 anchor

|  |  | All Intra Main10 - Over VTM-3.0 |  |  |  |  | Random Access Main10 - Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.1 | CCLM + MDLM + MMLM | -0.25% | -1.71% | -2.19% | 100% | 100% | -0.12% | -1.36% | -1.70% | 99% | 99% |
| 2.2 | CCLM + MDLM + MMLM + Above-MMLM + Left-MMLM | -0.29% | -1.94% | -2.51% | 100% | 100% | -0.15% | -1.70% | -1.99% | 101% | 100% |
| 2.2.1 | Combination of 2.1 and 2.2 | -0.27% | -1.71% | -2.26% | 99% | 99% | -0.13% | -1.30% | -1.73% | 99% | 98% |
| 2.3 | CCLM + MDLM + Adaptive multiple cross-component linear model | 0.01% | -0.12% | -0.07% | 101% | 100% | 0.00% | 0.03% | 0.01% | 101% | 100% |
| 2.4 | Modified CCLM downsampling filter for “type-2” content | See below (*) |  |  |  |  |  |  |  |  |  |
| 2.5.1 | CCLM + MDLM, using MaxMin method, and β derived by using Cmean and Lmean | -0.09% | -0.44% | -0.47% | 100% | 99% | -0.08% | -0.31% | -0.39% | 100% | 99% |
| 2.5.2 | CCLM + MDLM, using classification-based mean value method | -0.18% | -0.88% | -0.90% | 100% | 99% | -0.11% | -0.74% | -0.84% | 100% | 99% |
| 2.5.3 | CCLM + MDLM, using MaxMin method; using the first top row and the second left column | 0.05% | 0.45% | 0.48% | 100% | 100% | 0.02% | 0.51% | 0.51% | 100% | 99% |
| 2.5.4 | CCLM + MDLM, using classification-based mean value method; using the first top row and the second left column | -0.14% | -0.51% | -0.51% | 100% | 101% | -0.09% | -0.41% | -0.46% | 100% | 99% |
| 2.6.1 | CCLM accessing less neighbouring luma samples, interaction with other adopted coding tools: one row above and one column left luma samples are accessed | 0.09% | 0.71% | 0.79% | 99% | 97% | 0.06% | 0.88% | 0.95% | 100% | 99% |
| 2.6.2 | 2.6.1 with three columns of left luma samples as in the anchor | 0.03% | 0.16% | 0.21% | 100% | 97% | 0.00% | 0.35% | 0.35% | 100% | 99% |

(*) Summary of test 2.4:

|  |  | All Intra – Over VTM3.0 WCG_EXT=1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Description | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | PsnrY | PsnrU | PsnrV | EncT | DecT |
| 2.4.a | LM determination: 3-tap filter; prediction: 3-tap filter | -2.5% | -0.2% | -0.2% | -4.2% | -8.6% | -0.2% | -3.4% | -6.7% | 101% | 101% |
| 2.4.b | LM determination: 3-tap filter; prediction: 5-tap filter | -2.2% | -0.2% | -0.2% | -4.7% | -8.6% | -0.2% | -4.0% | -6.8% | 101% | 102% |
| 2.4.c | LM determination: 5-tap filter; prediction: 5-tap filter | -2.6% | -0.3% | -0.2% | -5.5% | -9.6% | -0.2% | -4.6% | -7.6% | 99% | 101% |

|  |  | Random Access – Over VTM3.0 WCG_EXT=1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Description | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | PsnrY | PsnrU | PsnrV | EncT | DecT |
| 2.4.a | LM determination: 3-tap filter; prediction: 3-tap filter | -1.9% | -0.2% | -0.1% | -4.2% | -5.8% | -0.1% | -3.3% | -4.5% | 101% | 99% |
| 2.4.b | LM determination: 3-tap filter; prediction: 5-tap filter | -1.7% | -0.1% | -0.1% | -4.7% | -6.0% | -0.1% | -3.7% | -4.6% | 101% | 100% |
| 2.4.c | LM determination: 5-tap filter; prediction: 5-tap filter | -2.1% | -0.2% | -0.2% | -5.1% | -6.6% | -0.2% | -4.1% | -5.1% | 102% | 101% |

### CE3.2: Additional test results

|  |  | All Intra Main10 - Over VTM-3.0 |  |  |  |  | Random Access Main10 - Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |

### CE3.2: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 2.1 | Hong-Jheng Jhu (Foxconn) | y | I study the software. It is consistent between the description and the provided software. The results are close. It is found that the following sequence has minor difference in RA config: CatRobot1, Cactus, BasketballDrill, SlideShow. As we know, there is a bug in VTM3.0 that leads to different outcomes under different platform. Our results are based on Windows Server 2012 and Qualcomm's results are based on Linux Server. |
| 2.2 | Adarsh K. Ramasubramonian (Qualcomm) | y | No discrepancies were found in the software and the description of the test. For many rate points in the RA configurations, there is a minor mismatch in the BD-rate/PSNR values; however, none are significant enough to affect the BD-rate numbers by more than 0.01%. It may be attributed to the different cluster platforms used. |
| 2.2.1 | Narae Choi (Samsung) | y | I checked the code. There are no significant inconsistencies between the description and the provided software. Two additional min/max values and an additional flag with a context model are used for MMLM mode. |
| 2.3 | Santiago de Luxán Hernández (HHI) |  |  |
| 2.4 | Christophe Chevance (Technicolor) | y | The code was checked. It conforms the description. The results for tests a, b and c are closely matched by the cross-checker. A substantial gain in chroma, and slight gain in luma, is obtained. No noticeable impact on encoding and decoding was observed. |
| 2.5.1 | Yoshitaka Kidani (KDDI) |  |  |
| 2.5.2 | Liang Zhao (Tencent) | y | There is no significant inconsistencies between the description and the provided software. |
|  | Yoshitaka Kidani (KDDI) |  |  |
| 2.5.3 | Liang Zhao (Tencent) | y | There is no significant inconsistencies between the description and the provided software. |
|  | Ching-Chieh Lin (ITRI) | y | There are no inconsistencies between the CE description and the uploaded CE software. |
| 2.5.4 | Ching-Chieh Lin (ITRI) | y | There are no inconsistencies between the CE description and the uploaded CE software. |
| 2.6.1 | Jangwon Choi (LGE) | y | There are no inconsistencies between the CE description and the uploaded CE software. The decoding times are slightly higher ( 2~3% ) than the proponent results. |
| 2.6.2 | Jangwon Choi (LGE) | y | There are no inconsistencies between the CE description and the uploaded CE software. The decoding times are slightly higher ( 2~3% ) than the proponent results. |

## CE3.2: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |

# CE3.3 on ‘Intra mode coding’

## Defined tests [1]

| Test # | Description | Doc. # |
| --- | --- | --- |
| 3.1.1 | Decoder-side intra mode derivation | JVET-M0094 |
| 3.1.2 | Decoder-side intra mode selection with predictor coding |  |
| 3.1.3 | Decoder-side intra mode derivation with extended gradient filtering for directly neighboring pixels |  |
| 3.1.4 | Decoder-side intra mode derivation restricted for blocks with a number of samples <= 128 |  |
| 3.2 | 6 MPMs and 32 remaining modes | JVET-M0495 |
| 3.3.1 | Simplified MDMS with 5 chroma intra modes | JVET-M0218 |
| 3.3.2 | Simplified MDMS with the reduced chroma intra modes |  |
| 3.4.1 | Simplified MDMS by reducing the number of operations | JVET-M0503 |
| 3.4.2 | Simplified MDMS by reducing the number of candidates |  |
| 3.5 | Simplified MDMS with one DM | JVET-M0203 |

## CE3.3: Test results

### CE3.3: VTM-3.0 anchor

| Test # | Description | All Intra Main10 - Over VTM-3.0 |  |  |  |  | Random Access Main10 - Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.1.1 | Decoder-side intra mode derivation | -0.11% | -0.09% | -0.08% | 122% | 103% | -0.07% | -0.06% | -0.07% | 104% | 101% |
| 3.1.2 | Decoder-side intra mode selection with predictor coding | -0.01% | -0.05% | -0.02% | 122% | 102% | 0.01% | 0.06% | 0.04% | 103% | 100% |
| 3.1.3 | Decoder-side intra mode derivation with extended gradient filtering for directly neighboring pixels | -0.12% | -0.04% | -0.05% | 124% | 104% | -0.05% | -0.01% | 0.03% | 103% | 101% |
| 3.1.4 | Decoder-side intra mode derivation restricted for blocks with a number of samples <= 128 | -0.09% | -0.09% | -0.06% | 113% | 103% | -0.05% | 0.10% | 0.01% | 100% | 100% |
| 3.2 | 6 MPMs and 32 remaining modes | -0.03% | -0.04% | -0.05% | 97% | 100% | 0.02% | 0.14% | 0.13% | 99% | 99% |
| 3.3.1 | Simplified MDMS with 5 chroma intra modes | -0.03% | -0.88% | -0.86% | 100% | 100% | -0.01% | -0.46% | -0.50% | 100% | 100% |
| 3.3.2 | Simplified MDMS with the reduced chroma intra modes | -0.02% | -0.29% | -0.20% | 97% | 100% | 0.00% | -0.12% | -0.05% | 99% | 100% |
| 3.4.1 | Simplified MDMS by reducing the number of operations | -0.02% | -0.96% | -1.03% | 101% | 99% | 0.04% | -0.98% | -1.04% | 100% | 100% |
| 3.4.2 | Simplified MDMS by reducing the number of candidates | 0.03% | -0.34% | -0.32% | 97% | 100% | 0.10% | -0.86% | -0.92% | 99% | 100% |
| 3.5 | Simplified MDMS with one DM | -0.01% | -0.61% | -0.64% | 100% | 100% | 0.06% | -0.68% | -0.76% | 100% | 101% |

### CE3.3: Additional test results

| Test # | Description | All Intra Main10 - Over VTM-3.0 |  |  |  |  | Random Access Main10 - Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.5.1 | Simplified MDMS with one DM by using aligned chroma coding of VVC-3.0 | -0.02% | -0.58% | -0.61% | 100% | 100% | -0.02% | -0.37% | -0.39% | 100% | 100% |

### CE3.3: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 3.1.1 | Virginie Drugeon (Panasonic) | y | No significant inconsistencies were found between the description and the provided software. The derived DIMD mode is added to the list of checked modes in the rate distortion search. How much of the gain is due to an improved rate distortion search only? The code to derive the DIMD angular mode currently contains divisions. |
| 3.1.2 | Adarsh K. Ramasubramonian (Qualcomm) |  |  |
| 3.1.3 | Alexey Filippov (Huawei) |  |  |
| 3.1.4 | Vasily Rufitskiy (Huawei) |  |  |
| 3.2 | Adarsh K. Ramasubramonian (Qualcomm) | y | No discrepancies were found in the software and the description of the test. For some rate points in the RA configurations, there is a minor mismatch in the BD-rate/PSNR values; however, none are significant enough to affect the BD-rate numbers. The test software derives the MPM and non-MPM lists only once for all the modes during the encoder intra search. That provides minor encoding time savings; however that could be applied to the anchor too. |
| 3.3.1 | Ching-Chieh Lin (ITRI) | y | The software is checked. There are no inconsistencies between the CE description and the uploaded CE software. |
| 3.3.2 | Ching-Chieh Lin (ITRI) | y | The software is checked. There are no inconsistencies between the CE description and the uploaded CE software. |
| 3.4.1 | Jangwon Choi (LGE) | y | There are no inconsistencies between the CE description and the uploaded CE software. |
| 3.4.2 | Jangwon Choi (LGE) | y | There are no inconsistencies between the CE description and the uploaded CE software. |
| 3.5 | Adarsh K. Ramasubramonian (Qualcomm) |  |  |

## CE3.3: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 12th JVET Meeting, Macau, CN, October 2018, JVET-L1023-v3