| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0024-v1 |
| --- | --- |

| Title: | CE4: Summary report on inter prediction and motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haitao Yang Shan Liu Kai Zhang | Email: | haitao.yang@huawei.com shanl@tencent.com zhangkai.video@bytedance.com |
| Source: | CE4 coordinators |  |  |

_____________________________

- Abstract

This contribution provides a summary report of Core Experiment 4 on inter prediction and motion vector coding. CE4 comprises 5 categories,

- Merge mode simplification

- Merge mode enhancement

- Parallel processing for merge mode

- Motion vector coding

- Motion compensation constraints for complexity reduction

All techniques are implemented on top of and test against VTM 3.0. Simulation results and crosschecking reports of each test specified in this document are provided.

CE4.1: Merge mode simplification

Test specification

| Test# | Source | Description |
| --- | --- | --- |
| 4.1.1.a | JVET-M0124 | Limit the number of history candidates to check with threshold 5 |
| 4.1.1.b | JVET-M0124 | Limit the number of history candidates to check with threshold 6 |
| 4.1.1.c | JVET-M0124 | 4.1.1.a with condition on the number of existing candidates |
| 4.1.1.d | JVET-M0124 | 4.1.1.b with condition on the number of existing candidates |
| 4.1.2.a | JVET-M0126 | HMVP with simplified pruning algorithm (number of HMVP to prune with threshold 3) |
| 4.1.2.b | JVET-M0126 | HMVP with simplified pruning algorithm (number of HMVP to prune with threshold 2) |
| 4.1.5.a | JVET-M0281 | Motion vector pruning: rounding, clipping before any MV pruning |
| 4.1.5.b | JVET-M0281 | In merge apply some pruning on combined HEVC candidates, and/or pairwise averaged candidates |
| 4.1.5.c | JVET-M0281 | 4.1.5.a + 4.1.5.b |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.1.a | 0.00% | 0.07% | 0.02% | 100% | 100% | 0.07% | 0.14% | 0.22% | 101% | 99% |
| 4.1.1.b | 0.00% | 0.02% | 0.06% | 100% | 101% | 0.02% | 0.01% | 0.00% | 101% | 100% |
| 4.1.1.c | -0.04% | 0.00% | -0.01% | 100% | 100% | 0.05% | 0.06% | 0.00% | 101% | 99% |
| 4.1.1.d | -0.02% | 0.00% | 0.01% | 100% | 99% | 0.03% | -0.04% | -0.02% | 100% | 100% |
| 4.1.2.a | 0.03% | 0.06% | 0.03% | 100% | 100% | 0.08% | 0.07% | 0.13% | 100% | 99% |
| 4.1.2.b | 0.03% | 0.02% | 0.04% | 100% | 100% | 0.08% | -0.03% | 0.08% | 99% | 98% |
| 4.1.5.a | 0.00% | -0.01% | 0.00% | 100% | 100% | -0.01% | -0.04% | -0.01% | 100% | 100% |
| 4.1.5.b | -0.02% | 0.00% | 0.00% | 100% | 100% | 0.02% | 0.09% | -0.09% | 99% | 100% |
| 4.1.5.c | -0.02% | -0.03% | -0.01% | 100% | 100% | -0.02% | 0.06% | 0.07% | 99% | 100% |

Complexity analysis of list construction

| Test# | Difference in the number of candidate comparison (+/- xx) | Max number of pruning stages | Others |
| --- | --- | --- | --- |
| 4.1.1.a | -4 | unchanged |  |
| 4.1.1.b | -2 | unchanged |  |
| 4.1.1.c | -3 | unchanged |  |
| 4.1.1.d | -2 | unchanged |  |
| 4.1.2.a | -8 | -2 | HMVP table = 10 |
| 4.1.2.b | -10 | -3 | HMVP table=10, runtime saving of HMVP functions is 59%(RA), 62%(LB) |
| 4.1.5.a | +0 | +0 |  |
| 4.1.5.b | +6 | +1 | Discard pairwise calculation |
| 4.1.5.c | +6 | +1 | Discard pairwise calculation |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.1.1.a | L. Zhang (ByteDance) | 2 | F | Y | M |
| 4.1.1.b | L. Zhang (ByteDance) | 2 | F | Y | M |
| 4.1.1.c | L. Zhang (ByteDance) | 2 | F | Y | M |
| 4.1.1.d | L. Zhang (ByteDance) | 2 | F | Y | M |
| 4.1.2.a | C.-C. Chen (MediaTek) | 2 | F | Y | M |
| 4.1.2.b | C.-C. Chen (MediaTek) | 2 | F | Y | M |
| 4.1.5.a | T. Solovyev (Huawei) |  |  |  |  |
| 4.1.5.b | T. Solovyev (Huawei) |  |  |  |  |
| 4.1.5.c | T. Solovyev (Huawei) |  |  |  |  |

Detailed description of tests

CE4.1.1 in JVET-M0124 (LGE)

A method was proposed to limit the number of HMVP candidates to check based on how many spatial and temporal candidates already exist in the merge list. The idea is that if there are not many spots left in the merge list for the HMVP candidates, it may not need to check all the available HMVP candidates in the table. The benefit is to reduce the number of pruning checks when inserting HMVP to the merge list.

Four subtests with different thresholds are tests as listed below.

- NHMVPChecked = 5 - NST  .    Threshold 5 is picked because it equals to maxNumMergeCand -1 , which is the max number of HMVP candidates allowed to be added to the merge list, so 5- NST is how many spots left in the merge list for HMVP candidates, i.e. NmrgToBeAdded

- NHMVPChecked = 6 - NST  .       6 is picked because it equals the maxNumMergeCand

- NHMVPChecked = (NST >=3) ? 5 - NST  :  NHMVP   .  This is applying a) when NST >=3

- NHMVPChecked = (NST >=3) ? 6 - NST  :  NHMVP  .  This is applying b) when NST >=3

CE4.1.2 in JVET-M0126 (Qualcomm)

Update HMVP table without pruning

In this conurbation, redundancy check is removed from HMVP table update.

Subsampling HMVP table when adding HMVP candidates in merger list

In this contribution, the size of HMVP table is increased to 10, and one out of every 3 is picked to add in the merge list. At most 4 HMVP candidates can be used.

In order to reduce the number of pruning, partial pruning is used when adding a new HMVP candidate in merge list. In the sub test CE4.1.2.a, only the first 3 HMVP candidates are pruned by the left and above spatial candidates. In the sub test CE4.1.2.b, only the first 2 HMVP candidates are pruned by the left and above spatial candidates.

HMVP in AMVP mode

The size of HMVP table is increased to 10, and one out of every 3 HMVP candidates is picked to add in the AMVP list.

Remove dependency between HMVP and pairwise candidates

In VTM3.0, pairwise candidates are derived from the candidates in the merge list which brings dependency between pairwise predictors and HMVP predictors. In this contribution, pairwise candidates are generated not using HMVP predictors.

CE4.1.5 in JVET-M0281 (Technicolor)

In AMVP, a motion predictor is a motion vector dedicated to a particular reference picture, and a list must contain 2 motion predictors. The process is repeated for each reference frame of each reference frame list.

It has been adopted that when performing the AMVR rounding, the ¼-pel rounding should be performed at the same time. This rounding step takes place just before the existing pruning. But for motion predictors that do not use AMVR, the needed ¼-pel rounding is performed at the end of the list construction.

This contribution then proposes to perform all the rounding operations before pruning even if AMVR is off, i.e. either ¼-pel or AMVR and ¼-pel rounding is done before any pruning.

In Merge mode, a motion predictor is one or two motion vector(s) with its associated reference picture, and a list must contain 6 motion predictors.

The pruning process of the spatial candidates has been simplified and the one of the temporal candidate has been removed. But, for pair-wise candidates, no pruning process exists.

This contribution then proposes to perform an early and simple pruning of the pair-wise candidates. For each used pair (A, B), the pair-wise candidate is not calculated:

- If both A and B are bi-directional and both motion vectors are equal,

- If both A and B are uni-directional or only A is bi-directional and motion vectors of the common list are equal,

- If only B is bi-directional and motion vectors and reference frame of the common list are equal.

CE4.2: Merge mode enhancement

Test specification

| Test# | Source | Description |
| --- | --- | --- |
| 4.2.1.a | JVET-M0059 | Non-scaling STMVP (one candidate) |
| 4.2.2.a | JVET-M0106 | STMVP using a single adjacent candidate, a single adjacent left candidate and a TMVP candidate |
| 4.2.2.b | JVET-M0106 | 4.2.2.a with merge list size = 8 |
| 4.2.2.c | JVET-M0106 | Multiple STMVP candidates |
| 4.2.2.d | JVET-M0106 | Multiple STMVP candidates with merge list size = 8 |
| 4.2.3.a | JVET-M0221 | STMVP simplification (no scaling, remove the non-adjacent candidates part of JVET-L0399) |
| 4.2.4.a | JVET-M0404 | History based STMVP (one candidate) without scaling |
| 4.2.5.a | JVET-M0291 | Merge offset extension based on 3 candidates |
| 4.2.5.b | JVET-M0291 | 8 x N (Position fixed offset candidates) |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.2.1.a | -0.09% | -0.04% | -0.06% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| 4.2.2.a | -0.11% | -0.03% | 0.00% | 101% | 99% | 0.00% | -0.12% | 0.02% | 107% | 110% |
| 4.2.2.b | -0.17% | -0.10% | -0.09% | 101% | 100% | -0.06% | -0.14% | -0.25% | 107% | 110% |
| 4.2.2.c | -0.15% | -0.10% | -0.13% | 100% | 100% | 0.00% | -0.18% | 0.01% | 107% | 110% |
| 4.2.2.d | -0.23% | -0.15% | -0.13% | 101% | 100% | -0.06% | -0.16% | -0.11% | 107% | 111% |
| 4.2.3.a | 0.00% | 0.00% | -0.01% | 101% | 101% | 0.11% | 0.18% | 0.29% | 100% | 101% |
| 4.2.4.a |  |  |  |  |  |  |  |  |  |  |
| 4.2.5.a | 0.07% | 0.17% | 0.14% | 100% | 95% | 0.03% | 0.13% | 0.24% | 100% | 97% |
| 4.2.5.b | 0.02% | 0.17% | 0.14% | 100% | 97% | -0.07% | 0.10% | 0.10% | 100% | 94% |

Complexity analysis of list construction

| Test# | Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of pruning stages | Max number of MV scaling | Others |
| --- | --- | --- | --- | --- | --- | --- |
| 4.2.1.a | 6 |  |  |  |  |  |
| 4.2.2.a | 6 |  |  |  |  |  |
| 4.2.2.b | 8 |  |  |  |  |  |
| 4.2.2.c | 6 |  |  |  |  |  |
| 4.2.2.d | 8 |  |  |  |  |  |
| 4.2.3.a | 6 |  |  |  |  |  |
| 4.2.4.a |  |  |  |  |  |  |
| 4.2.5.a | 6 |  |  |  |  |  |
| 4.2.5.b | 6 |  |  |  |  |  |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.2.1.a | Y.-W. Chen (Kwai Inc.) |  |  |  |  |
| 4.2.2.a | Y.Han (Qualcomm) | 2 | P | Y | M |
| 4.2.2.b |  |  |  |  |  |
| 4.2.2.c | Y.Han (Qualcomm) | 2 | P | Y | M |
| 4.2.2.d |  |  |  |  |  |
| 4.2.3.a | F. Le Léannec (Technicolor) | 2 | F | Y | M |
| 4.2.4.a | T. Zhou (Sharp) |  |  |  |  |
| 4.2.5.a | L. Xu (Hikvision) | 2 | F | Y | RA:102%/100% LB:101%/97% |
| 4.2.5.b | L. Xu (Hikvision) | 2 | F | Y | RA:101%/100% LB:101%/97% |

Detailed description of tests

CE4.2.1 in JVET-M0059 (Sharp)

The proposed method derives an averaging candidate as STMVP candidate using two spatial merge candidate and one collocated merge candidate.

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(8-118)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagSbCol )
	mergeCandList[ i++ ] = STMVP
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col

For the spatial candidates, the first and second candidates in the current merge candidate list is used.

For the temporal candidate, the same position as VTM / HEVC collocated position is used.

If three candidates, of which the reference are equal to zero, are available, the following apply.

mvLX[0] = (mvLX_A[0] * 3 + mvLX_L[0] * 3 + mvLX_C[0] * 2 ) / 8

mvLX[1] = (mvLX_A[1] * 3 + mvLX_L[1] * 3 + mvLX_C[1] * 2 ) / 8

If two motion information, of which the reference are equal to zero, is available, the following apply

mvLX[0] = (mvLX_A[0] + mvLX_C[0] ) / 2

mvLX[1] = (mvLX_A[1] + mvLX_C[1] ) / 2

or

mvLX[0] = (mvLX_B[0] + mvLX_C[0] ) / 2

mvLX[1] = (mvLX_B[1] + mvLX_C[1] ) / 2

Note: If the temporal candidate is unavailable, the STMVP mode is off.

CE4.2.2 in JVET-M0106 (Technicolor)

Test 4.2.2.a

The proposed STMVP technique is based on JVT-L0207. It consists in constructing the STMVP merge candidate as follows.

- One top-neighboring motion vector (MV) of current CU is selected among spatial position (w/2,-1), (2*w,-1) and (w-1,-1), as shown on Figure 1. The first found position where all available MVs use the reference picture index 0 is selected.

- One left-neighboring motion vector (MV) of current CU is selected among spatial position (-1,h/2), (-1,2*h) and (-1,h-1), as shown on Figure 1. The first found position where all available MVs use the reference picture index 0 is selected.

- The same temporal MV predictor as in JVET-L0207 is used. The temporal candidate is set as always available, given that the Right-Bottom position of the current block is inside the picture. The y-position of the temporal candidate is clipped to ensure it lies inside the current CTU row (see Figure 2).

- The STMVP candidate is considered as available if at least 2 spatial-temporal MV predictors are found. If only one neighboring MV is retrieved, no STVMP candidate is included into the merge list.

- No MV scaling is applied.

- The STMVP candidate is computed as the average between the 2 or 3 retrieved spatial and temporal MV predictors.

Test 4.2.2.b

Test 4.2.2.b involves 2 additional aspects over test 4.2.2.a.

- A second STMVP candidate is added to the merge candidate list, if 3 motion vectors (thus including the temporal predictor) had been used to generate the first STMVP candidate. The additional STMVP candidate consists in the average of two spatial MV predictors used to construct the first STMVP candidate.

- Pairwise average candidates are disallowed.

CE4.2.3 in JVET-M0221 (Qualcomm)

In this proposal, four modifications are proposed to simplify the complexity in the non-sub-PU STMVP in JVET_L0399, as described as follows:

- No MV scaling for the two spatial neighbors.

- STMVP candidates with non-local spatial neighbors are removed. As a result, only one STMVP is added into merge candidate list.

- For STMVP with local spatial neighbors, instead of searching up to two positions for above (left) spatial neighbors, only one position is checked, as shown in Figure 1. The positions are the same as the 1st and 2nd checked candidates in the non-sub-block merge candidate list.

Current PU

S0

S1

- If three candidates are available, i.e., the temporal candidate (MVT) on co-located block is available and the two spatial candidates are available with reference index equals to zero, the STMVP candidate is calculated as

MVSTMVP=MVS0+MVS14+MVT2

Note that the denominators in divisions are always of power of two, and therefore can be implemented using shift operations.

CE4.2.4 in JVET-M0404 (Tencent)

The proposed method derives a new candidate using last two history MVs and one collocated merge candidate (referred as HMVP-TMVP). The new candidate is inserted after A1, B1, B0 and A0 candidates, as follows:

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagSbCol )
	mergeCandList[ i++ ] = HMVP+TMVP
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col

For the two MVs from HMVP, the last coded and second to last coded MVs in the buffer are used (referred as A and B). For the temporal candidate, the same position as VTM-3.0 collocated position (referred as C) is used.

If three candidates, of which the reference indices are equal to zero, are available, the following apply.

mvLX[0] = (mvLX_A[0] * 3 + mvLX_B[0] * 3 + mvLX_C[0] * 2 ) / 8

mvLX[1] = (mvLX_A[1] * 3 + mvLX_B[1] * 3 + mvLX_C[1] * 2 ) / 8

If two motion information, of which the reference indices are equal to zero, is available, the following apply

mvLX[0] = (mvLX_A[0] + mvLX_C[0] ) / 2

mvLX[1] = (mvLX_A[1] + mvLX_C[1] ) / 2

or

mvLX[0] = (mvLX_B[0] + mvLX_C[0] ) / 2

mvLX[1] = (mvLX_B[1] + mvLX_C[1] ) / 2

Note: If the temporal candidate is unavailable, the HMVP-TMVP mode is off.

CE4.2.5 in JVET-M0291 (Hisilicon)

Solution 1

For the merge candidates, three candidate can be selected, and choice one to be the starting point.

Distance index indicates the pre-defined distance from the starting point information. Pre-defined distance is as follows:

| Distance IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel |

Solution 2

For solution 2, 4 new diagonal directionMV offsets for each distance is induced as figure 1 shows.Distance of diagonal offsets are half of vertical and horizontal direction distance of exited offsets.

Distance index indicates the pre-defined distance from the starting point information. Pre-defined distance is as follows:

| Distance IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel |

For solution 2, Bug of MMVD flag signalling (When MMVD mode is selected, no need to transfer triangle flag or Intra-Inter combined mode flag) is fixed.

CE4.3: Parallel processing for merge mode

Test specification

| Test# | Source | Description |
| --- | --- | --- |
| 4.3.1.a | JVET-M0170 | Share merge list for translational merge and sub-CU merge with fixed threshold = 32 |
| 4.3.1.b | JVET-M0170 | Share merge list for translational merge and sub-CU merge with fixed threshold = 64 |
| 4.3.1.c | JVET-M0170 | Share merge list for translational merge and sub-CU merge with fixed threshold = 128 |
| 4.3.1.d | JVET-M0170 | Share merge list for translational merge and sub-CU merge with fixed threshold = 256 |
| 4.3.1.e | JVET-M0170 | Share merge list for translational merge and sub-CU merge with fixed threshold = 512 |
| 4.3.2.a | JVET-M0289 | Fixed square merge estimation region for parallel estimation with MER = 8x8 |
| 4.3.2.b | JVET-M0289 | Fixed square merge estimation region for parallel estimation with MER = 16x16 |
| 4.3.2.c | JVET-M0289 | Fixed square merge estimation region for parallel estimation with MER = 32x32 |
| 4.3.2.d | JVET-M0289 | Fixed square merge estimation region for parallel estimation with MER = 64x64 |
| 4.3.2.e | JVET-M0289 | Fixed square MER is applied if all CUs are covered by the MER with MER = 8x8 |
| 4.3.2.f | JVET-M0289 | Fixed square MER is applied if all CUs are covered by the MER with MER = 16x16 |
| 4.3.2.g | JVET-M0289 | Fixed square MER is applied if all CUs are covered by the MER with MER = 32x32 |
| 4.3.2.h | JVET-M0289 | Fixed square MER is applied if all CUs are covered by the MER with MER = 64x64 |
| 4.3.2.i | JVET-M0289 | Rectangular MER is tested with MER = 32 |
| 4.3.2.j | JVET-M0289 | Rectangular MER is tested with MER = 64 |
| 4.3.2.k | JVET-M0289 | Rectangular MER is tested with MER = 128 |
| 4.3.2.l | JVET-M0289 | Rectangular MER is tested with MER = 256 |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.3.1.a | 0.00% | 0.00% | 0.00% | 101% | 102% | 0.02% | -0.05% | -0.04% | 101% | 102% |
| 4.3.1.b | 0.04% | 0.02% | 0.05% | 101% | 104% | 0.08% | -0.04% | 0.15% | 101% | 102% |
| 4.3.1.c | 0.12% | 0.18% | 0.16% | 101% | 102% | 0.14% | 0.22% | 0.28% | 102% | 102% |
| 4.3.1.d | 0.44% | 0.52% | 0.66% | 102% | 103% | 0.63% | 0.68% | 0.69% | 103% | 102% |
| 4.3.1.e | 0.68% | 0.85% | 0.92% | 102% | 102% | 0.93% | 0.86% | 1.09% | 103% | 101% |
| 4.3.2.a | 0.16% | 0.27% | 0.25% | 100% | 101% | 0.14% | 0.13% | 0.36% | 100% | 101% |
| 4.3.2.b | 0.55% | 0.67% | 0.78% | 101% | 101% | 0.56% | 0.66% | 0.70% | 101% | 102% |
| 4.3.2.c | 1.41% | 1.64% | 1.69% | 104% | 100% | 1.41% | 1.38% | 1.54% | 103% | 100% |
| 4.3.2.d | 2.62% | 2.72% | 2.97% | 106% | 100% | 2.51% | 2.56% | 2.61% | 104% | 100% |
| 4.3.2.e | 0.02% | 0.01% | 0.03% | 99% | 100% | 0.01% | 0.08% | 0.04% | 100% | 101% |
| 4.3.2.f | 0.16% | 0.24% | 0.26% | 100% | 99% | 0.16% | 0.11% | 0.18% | 101% | 101% |
| 4.3.2.g | 1.09% | 1.27% | 1.44% | 102% | 99% | 1.13% | 1.20% | 1.20% | 102% | 100% |
| 4.3.2.h | 2.59% | 2.66% | 2.92% | 105% | 98% | 2.44% | 2.45% | 2.51% | 103% | 96% |
| 4.3.2.i | 0.00% | 0.07% | 0.04% | 100% | 101% | 0.00% | 0.04% | 0.04% | 101% | 104% |
| 4.3.2.j | 0.03% | 0.06% | 0.06% | 101% | 102% | 0.01% | 0.04% | -0.17% | 101% | 105% |
| 4.3.2.k | 0.15% | 0.20% | 0.27% | 101% | 102% | 0.15% | 0.22% | 0.06% | 102% | 104% |
| 4.3.2.l | 0.35% | 0.47% | 0.55% | 102% | 101% | 0.37% | 0.31% | 0.50% | 102% | 104% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.3.1.a | Y. Han (Qualcomm) | 1 | F | Y | M |
| 4.3.1.b | Y. Han (Qualcomm) | 1 | F | Y | M |
| 4.3.1.c | Y. Han (Qualcomm) | 1 | F | Y | M |
| 4.3.1.d | Y. Han (Qualcomm) | 1 | F | Y | M |
| 4.3.1.e | Y. Han (Qualcomm) | 1 | F | Y | M |
| 4.3.2.a | J. Ma (HHI) | 1 | F | Y | M |
| 4.3.2.b | J. Ma (HHI) | 1 | F | Y | M |
| 4.3.2.c | J. Ma (HHI) | 1 | F | Y | M |
| 4.3.2.d | J. Ma (HHI) | 1 | F | Y | M |
| 4.3.2.e | J. Ma (HHI) | 1 | F | Y | M |
| 4.3.2.f | J. Ma (HHI) | 1 | F | Y | M |
| 4.3.2.g | J. Ma (HHI) | 1 | F | Y | M |
| 4.3.2.h | J. Ma (HHI) | 1 | F | Y | M |
| 4.3.2.i | J. Ma (HHI) | 1 | F | Y | M |
| 4.3.2.j | J. Ma (HHI) | 1 | F | Y | M |
| 4.3.2.k | J. Ma (HHI) | 1 | F | Y | M |
| 4.3.2.l | J. Ma (HHI) | 1 | F | Y | M |

Detailed description of tests

CE4.3.1 in JVET-M0170 (Qualcomm)

It is proposed to share the same merging candidate list for all leaf CUs of one ancestor node in the CU split tree for enabling parallel processing of small skip/merge-coded CUs. The ancestor node is named merge sharing node. The shared merging candidate list is generated at the merge sharing node pretending the merge sharing node is a leaf CU.

There are 2 types of size threshold definitions, which are denoted as Type-1 and Type-2 definitions. For Type-1 definition, the merge sharing node will be decided for each CU inside a CTU during parsing stage of decoding; moreover, the merge sharing node is the largest ancestor node among all the ancestor nodes of the leaf CUs satisfying the following two criteria.

- The merge sharing node size is equal to or smaller than the size threshold

- No samples of the merge sharing node are outside the picture boundary.

For Type-2 definition, the merge sharing node will be decided for each CU inside a CTU during parsing stage of decoding; moreover, the merge sharing node is an ancestor node of leaf CU which must satisfy the following 2 criteria:

- The merge sharing node size is equal to or larger than the size threshold

- In the merge sharing node, one of the child CU size is smaller than the size threshold

The proposed shared merging candidate list algorithm supports translational merge (including merge mode and triangle merge mode, history-based candidate is also supported) and subblock-based merge mode. For all kinds of merge mode, the behavior of shared merging candidate list algorithm looks basically the same, and it just generates candidates at the merge sharing node pretending the merge sharing node is a leaf CU.

Besides, it is proposed to add new syntax elements to sequence parameter set (SPS) and picture parameter set (PPS).

CE4.3.2 in JVET-M0289 (Huawei)

Solution 1

In the solution 1, the MER is defined as a fixed non-overlapped square grid.

According to the proposal the rule for setting a neighbor unavailable for merge list construction is changed as follows.

New parallel merge estimation rule: “A neighbor coding block is marked as unavailable if its bottom right coordinate falls in the extended merge estimation region of the current block”. The extended MER is depicted in the following figure:

Assume that the bottom right coordinate a neighbor block is given by (XNb, YNb) and the top-left coordinate of the current block is given by (XCt, YCt). Moreover assume that the binary logarithm of MER size is given by N. The neighbor block is marked as unavailable for prediction if;

(XNb >> N is equal to XCt >> N) AND (YNb >> N smaller than or equal to YCt >> N)

OR

(YNb >> N is equal to YCt >> N) AND (XNb >> N smaller than or equal to XCt >> N)

Solution 2

In the solution 2, the MER is defined as a fixed non-overlapped square grid by QT depth.

According to the proposal the rule for setting a neighbor unavailable for merge list construction is changed as follows.

New parallel merge estimation rule: “A neighbor coding block is marked as unavailable if its bottom right coordinate falls in the estimation region of the current block and the QT depth of the current block is equal to or greater than the value of CTU size divided by MER size ”.

Assume that the CTU size is given by CTUSize, the bottom right coordinate a neighbor block is given by (XNb, YNb), the top-left coordinate of the current block is given by (XCt, YCt) and the QT Depth of the current block is given by CuQtDepth. Moreover assume that the binary logarithm of MER size is given by N. The neighbor block is marked as unavailable for prediction if;

(CTUSize >> CuQtDepth smaller than or equal to 1<< N)

AND

(YNb >> N is equal to YCt >> N) AND (XNb >> N equal to XCt >> N)

Solution 3

- Definition of rectangular MER

In the solution 3, the MER is defined rectangular block with same number of luma samples.

According to the proposal the rule for setting a neighbor unavailable for merge list construction is changed as follows.

New parallel merge estimation rule: “A neighbor coding block is marked as unavailable if its bottom right coordinate falls in the rectangular merge estimation region of the current block”. The MER of the current block is defined in the follow:

Assuming the MER number of luma samples is the MER threshold, an ancestor block of the current block is equal to the threshold. Then the region of the ancestor is defined as the MER of the current block.

For example in the Figure 1, if the current block is block D with size 4x8, the MER threshold is 256, based on the definition of the MER, the whole Figure 1 is defined as the MER of block D. Therefore, block A, B, C and E are marked as unavailable for merge estimation.

- Worst Case Guarantee

Worst case of parallel merge estimation is defined as how many sets of parallel processing merge per MER threshold number of luma samples.

For example if the current block is 4x4, the MER threshold is 32 samples and the parent of the current block is 4x8. Then the 4x8 region is defined as MER based on 3.3.1, and per 32 samples 1 set of parallel processing merge.

However, based on the MER definition in 3.3.1, in some special case, the worst case is not guaranteed by 1 set of parallel processing merge.

For example, if the current block is 4x4, the MER threshold is 32 and the current block is split from QT which means the parent block size is 8x8. Then MER is not defined, and per 64 samples 4 sets of (parallel processing) merge, which equivalents to per 32 samples 2 sets of (parallel processing) merge

In another example, if the current block is 4x4, the MER threshold is 32 and the current block is split from vertical TT which means the parent block size is 4x16. Then MER is not defined, and per 64 samples 3 sets of (parallel processing) merge, which equivalents to per 32 samples 1.5 sets of (parallel processing) merge

To guarantee the worst case, on top of 3.3.1, the MER of current block is additional defined as follow:

Assuming the MER number of luma samples is the MER threshold, an ancestor block of the current block is equal to the threshold multiplied by 2 and the current block is split from QT or TT. Then the region of the ancestor is defined as the MER of the current block.

CE4.4: Motion vector coding

Test specification

| Test# | Source | Description |
| --- | --- | --- |
| 4.4.1.a | JVET-M0403 | Unified MVD coding with reduced context coded bins |
| 4.4.1.b | JVET-M0403 | MVD coding with reduced context coded bins |
| 4.4.3 | JVET-M0481 | Symmetrical MVD mode |
| 4.4.4.a | JVET-M0060 | Diagonal direction candidate for MMVD |
| 4.4.4.b | JVET-M0060 | Multiple distance lists for MMVD |
| 4.4.4.c | JVET-M0060 | Combination of directional candidates and multiple distance list |
| 4.4.4.d | JVET-M0060 | Combination of directional candidates and multiple distance list with full-pel position for large range |
| 4.4.4.e | JVET-M0061 | Combination of 4.4.4.a and 4.4.5.c |
| 4.4.5.a | JVET-M0312 | MMVD with adaptive distance table based on occurrences |
| 4.4.5.b | JVET-M0312 | MMVD with adaptive distance table based on picture resolution |
| 4.4.5.c | JVET-M0312 | 4.4.5.a + 4.4.5.b |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.4.1.a | 0.01% | 0.02% | 0.02% | 96% | 96% | 0.03% | -0.09% | -0.11% | 97% | 92% |
| 4.4.1.b | 0.00% | 0.04% | 0.02% | 96% | 96% | -0.01% | 0.01% | -0.02% | 97% | 92% |
| 4.4.3 | -0.16% | -0.12% | -0.16% | 105% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| 4.4.4.a | -0.13% | -0.07% | -0.08% | 106% | 100% | -0.05% | 0.03% | 0.00% | 104% | 100% |
| 4.4.4.b | -0.05% | 0.03% | 0.01% | 102% | 100% | 0.09% | 0.09% | 0.11% | 101% | 100% |
| 4.4.4.c | -0.28% | -0.16% | -0.19% | 106% | 100% | 0.01% | 0.14% | 0.25% | 104% | 100% |
| 4.4.4.d | -0.28% | -0.17% | -0.20% | 106% | 100% | 0.00% | 0.28% | 0.07% | 103% | 100% |
| 4.4.4.e | -0.38% | -0.39% | -0.48% | 101% | 100% | -0.15% | -0.11% | -0.01% | 102% | 100% |
| 4.4.5.a | -0.22% | -0.33% | -0.35% | 103% | 98% | -0.13% | -0.10% | -0.04% | 105% | 100% |
| 4.4.5.b | -0.21% | -0.34% | -0.41% | 103% | 98% | -0.08% | -0.11% | -0.10% | 105% | 100% |
| 4.4.5.c | -0.24% | -0.43% | -0.46% | 103% | 98% | -0.13% | -0.10% | -0.04% | 105% | 100% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.4.1.a | L.P. Van (Qualcomm) | 2 | F | Y | G |
| 4.4.1.b | L.P. Van (Qualcomm) | 2 | F | Y | G |
| 4.4.3 | G. Li (Tencent) | 2 | F | Y | M |
| 4.4.4.a | J. Li (Panasonic) | 2 | F | Y | M |
| 4.4.4.b | J. Li (Panasonic) | 2 | F | Y | M |
| 4.4.4.c | J. Li (Panasonic) | 2 | F | Y | M |
| 4.4.4.d | J. Li (Panasonic) A. Karabutov (Huawei) |  |  |  |  |
| 4.4.4.e | L. Xu (Hikvision) | 1 | F | Y | M |
| 4.4.5.a | T. Hashimoto (Sharp) | 1 | F | Y | M |
| 4.4.5.b | T. Hashimoto (Sharp) | 1 | F | Y | M |
| 4.4.5.c | T. Hashimoto (Sharp) A. Karabutov (Huawei) | 1 | F | Y | M |

The crosscheck of test 4.4.1 reported that the encoding and decoding time from the crosschecker’s side is not reliable.

Detailed description of tests

CE4.4.1 in JVET-M0403 (LGE)

Modifications to the algorithm have been carried out to reduce the worst case use of context coded bins from 24 to 6 by coding MVDx and MVDy components together. MVD (x,y) is coded using a combination of Layer and index information, wherein the index identifies the MVD(x,y) combination within a group of MVDs (i.e., a Layer). Both the tests differ in their groupings of the layers. In the former, all the layers greater than 0 are grouped together, while in the later test, layers greater than or equal to 3 are grouped together while layers 1 and 2 are grouped separately.

CE4.4.3 in JVET-M0481 (Huawei)

In slice level, variables BiDirPredFlag, RefIdxSymL0 and RefIdxSymL1 are derived by searching the reference pictures in List 0 and List 1.

In CU level, a symmetrical mode flag indicating whether symmetrical mode is used or not is explicitly signaled if the prediction direction for the CU is bi-prediction and BiDirPredFlag is equal to 1.

When the flag is true, only mvp_l0_flag, mvp_l1_flag and MVD0 are explicitly signaled. The reference indices are set equal to RefIdxSymL0, RefIdxSymL1 for list 0 and list 1, respectively. MVD1 is just set equal to –MVD0. The final motion vectors are shown in below formula.

mvx0, mvy0=(mvpx0+mvdx0,mvpy0+mvdy0) mvx1, mvy1=(mvpx1-mvdx0,mvpy1-mvdy0)

CE4.4.4.a ~ CE4.4.4.a in JVET-M0060 (Sharp)

Directional candidates

Directional candidates are used in addition to horizontal / vertical candidates of VTM-3.0. The directional candidates are shown as below.

Table 1. Motion direction (proposal)

| Direction IDX | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-axis | +2 | –2 | 0 | 0 | +1 | -1 | -1 | +1 |
| y-axis | 0 | 0 | +2 | –2 | +1 | -1 | +1 | -1 |

Note: the value of x-axis and y-axis of diagonal direction are half of that of horizontal and vertical direction respectively.

Multiple distance lists

Two set of distance lists are proposed. The selection flag is context coded so that it applies in an optimal way. The distance list is shown as follows.

Table 2. First distance list

| Distance IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 3/4-pel | 5/4-pel |

Table 3. Second distance list

| Distance IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Pixel distance | 1-pel | 2-pel | 4-pel | 8-pel |

Full-pel position for large range

In test d, motion vectors are converted into an integer position if the distance is large. Since integer position doesn’t need additional pixels for interpolation, necessary area size for MMVD encoding / decoding decreases, which would reduce memory band requirement. When the second distance list is selected and distance index is larger than 1, the motion vectors are converted into an integer position.

CE4.4.4.e in JVET-M0061 (Sharp & Panasonic)

A combination of CE4.4.4.a and CE4.4.5.c.

CE4.4.5 in JVET-M0312 (Panasonic)

Adaptive distance table is proposed to improve coding gain of MMVD.

Test 1

Use adaptive distance table based on occurrence-based distance table reordering. Distance index is reordered according to the occurrence of usage of each distance in previous coded picture in list 0 and reference index 0.

Test 3

Use adaptive distance table based on picture resolution, i.e., if picture resolution is not larger than 2K, i.e., 1920×1080, Table 2 is used as base distance table; Otherwise, Table 3 is used as base distance table.

Table 2: MMVD distance table candidate

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

Table 3: MMVD distance table

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel | 64-pel | 128-pel |

Test 3 : Test 1 + Test 2

CE4.5: Motion compensation constraints for complexity reduction

Test specification

| Test# | Source | Description |
| --- | --- | --- |
| 4.5.1 | JVET-M0313 | Disabling bi-prediction for 4x8 and 8x4 CUs |
| 4.5.2 | JVET-M0313 | Disabling bi-prediction or inter prediction for CU which memory bandwidth exceeds that of 8x8 bi-prediction |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.5.1 | 0.09% | 0.19% | 0.25% | 99% | 96% | 0.13% | 0.08% | 0.31% | 99% | 105% |
| 4.5.2 | 0.32% | 0.71% | 0.81% | 99% | 97% | 0.49% | 0.88% | 0.80% | 98% | 99% |

Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) |
| --- | --- | --- | --- | --- | --- |
| 4.5.1 | S. Sethuraman (Ittiam) |  |  |  |  |
| 4.5.2 | S. Sethuraman (Ittiam) |  |  |  |  |

Detailed description of tests

CE4.5.1 in JVET-M0313 (Panasonic)

To guarantee that the worst-case memory bandwidth is not exceeded that of 8×8 bi-prediction, bi-prediction is disabled for small CU. For the CU coded in merge mode, the motion vector from L0 direction of a bi-prediction motion vector candidate is used to predict the CU, which is the same situation in HEVC. For the CU coded in inter mode, only uni-prediction is allowed and the first bin of the inter direction which indicates uni-prediction or bi-prediction is not signaled.

CE4 related contributions

| JVET-VC number | Title | Authors |
| --- | --- | --- |
| JVET-M0067 | Non-CE4: Weighted prediction with BDOF and bi-prediction with CU weights harmonization | T. Hashimoto, T. Chujoh, T. Ikai, E. Sasaki (Sharp) |
| JVET-M0068 | Non-CE4: MMVD scaling fix | E. Sasaki, T. Ikai (Sharp), |
| JVET-M0069 | Non-CE4: Syntax change of MMVD | E. Sasaki, T. Chujoh, T. Ikai (Sharp) |
| JVET-M0081 | Non-CE4: Simplification of AMVP list generation in AMVR | Y. Kato, K. Abe, T. Toma (Panasonic) |
| JVET-M0117 | CE4-related: On MVP candidate list generation for AMVP | R. Yu, D. Liu, K. Andersson, P. Wennersten, J. Strom, R. Sjoberg (Ericsson) |
| JVET-M0127 | CE4-related: Modification on Merge List | Y. Han, W.-J. Chien, D. Rusanovskyy, Y.-H. Chao, C.-C. Chen, H. Wang, H. Huang, C.-H. Hung, Y. Zhang, M. Karczewicz (Qualcomm) |
| JVET-M0171 | CE4-related: MMVD cleanups | C.-Y. Lai, T.-D. Chuang, Y.-L. Hsiao, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-M0193 | CE4-related: Pairwise Average Candidate Reduction | A. Tamse, M. W. Park, K. Choi (Samsung) |
| JVET-M0205 | CE4-related: MMVD optimizations | S. Jeong, K. Choi (Samsung) |
| JVET-M0206 | CE4-related: MMVD improvements | S. Jeong, M. W. Park, K. Choi (Samsung) |
| JVET-M0220 | Non-CE4: Subjective quality analysis of non-sub-block ATMVP | Y.-H Chao, W.-J Chien, M. Karczewicz (Qualcomm) |
| JVET-M0230 | Non-CE4: Temporal MV buffer reduction | Y.-W. Chen, X. Wang (Kwai Inc.) |
| JVET-M0231 | Non-CE4: Regular merge flag coding | Y.-W. Chen, X. Wang (Kwai Inc.) |
| JVET-M0264 | Non-CE4: Harmonization between HMVP and GBi | J. Li, S. Wang, W. Gao(Peking University), L. Zhang, K. Zhang, H. Liu, J. Xu(Bytedance), X. Xiu, J. Luo, Y. He(InterDigital), |
| JVET-M0267 | Non-CE4: Harmonization of MMVD and AMVR | K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong(Bytedance) |
| JVET-M0272 | CE4-related: Restrictions on History-based Motion Vector Prediction | L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong(Bytedance) |
| JVET-M0286 | Non-CE4: Simplifications for triangular prediction mode | T. Solovyev, S. Esenlik, S. Ikonin, J. Chen (Huawei) |
| JVET-M0300 | CE4-related: HMVP and parallel processing with tiles and tile groups | A.M. Kotra, J. Chen, B. Wang, S. Esenlik, H. Gao (Huawei) |
| JVET-M0307 | CE4-related : Candidates optimization on MMVD | N. Park, H. Jang, J. Nam, J. Lim, S. Kim (LGE) |
| JVET-M0308 | Non-CE4: MMVD simplification | X. Chen, J. Zheng (HiSilicon) |
| JVET-M0314 | CE4-related: MMVD improving with signaling distance table | J. Li, R.-L. Liao, C. S. Lim (Panasonic) |
| JVET-M0315 | Non-CE4: MMVD scaling simplification | J. Li, R.-L. Liao, C. S. Lim (Panasonic) |
| JVET-M0330 | CE4-related: Simplification of candidate list derivation for MMVD mode | L. Xu, F. Chen, L. Wang (Hikvision), |
| JVET-M0345 | CE4-related: Remove redundancy between TMVP and ATMVP | S.H. Wang (Peking Univerisity), X. Zheng (DJI), S.S. Wang, S.W. Ma |
| JVET-M0346 | CE4-related: Non-square compression unit for temporal motion data storage | S.H. Wang (Peking Univerisity), X. Zheng (DJI), S.S. Wang, S.W. Ma |
| JVET-M0348 | CE4-related: Further reducing VVC memory bandwidth worst case by combining 4x4/4x8/8x4 bi-prediction with AMVR | X.W. Meng (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma |
| JVET-M0350 | CE4-related: CE4-related: Quadtree-based Merge Estimation Region for VVC | T.L. Fu (Peking University), X. Zheng (DJI), S.S Wang, S.W. Ma |
| JVET-M0359 | Non-CE4: Modification of merge data syntax | G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS) |
| JVET-M0369 | CE4-related: Syntax changes of merge data | Y. Ahn, D. Sim (Digital Insights) |
| JVET-M0405 | CE4-related: Simplified merge candidate list for small blocks | X. Xu, X. Li, S. Liu (Tencent) |
| JVET-M0406 | CE2/4-related: Unified merge list size for block and sub-block merge modes | X. Xu, X. Li, S. Liu (Tencent) |
| JVET-M0422 | CE4-related: Simplified MVD coding | X. Li, X. Xu, X. Zhao, S. Liu (Tencent) |
| JVET-M0433 | CE4-related: Constraint on GBi index inheritance in Merge Mode | G. Li, X. Xu, X. Li, S. Liu (Tencent) |
| JVET-M0435 | CE4-related: MMVD offset table signaling | G. Li, X. Xu, X. Li, S. Liu (Tencent) |
| JVET-M0437 | Non-CE4: Size constraint on MMVD | J. Zhao, S. Kim (LGE) |
| JVET-M0444 | CE4-related: Simplified symmetric MVD based on CE4.4.3 | J. Luo, Y. He (InterDigital) |
| JVET-M0448 | CE4-related: Triangle merge index signaling | M. Xu, X. Li, S. Liu (Tencent) |
| JVET-M0479 | Non-CE4: On clipping of scaled motion vectors | K. Misra, F. Bossen (Sharp) |
| JVET-M0484 | Non-CE4: Line buffer size reduction method for generalized bi prediction | T. Solovyev, H. Gao, S. Esenlik, S. Ikonin, J.Chen (Huawei) |
| JVET-M0502 | CE4-related: Improved context for prediction mode flag | X. Zhao, X. Li, S. Liu (Tencent) |
| JVET-M0507 | CE4-related: Hybrid Merge Estimation Region | H. Wang, V. Seregin, W.-J. Chien, T. Hsieh, Y. Han, M. Karczewicz (Qualcomm) |
| JVET-M0512 | Non-CE4: On Temporal Motion Buffer Compression | F. Bossen, K. Misra, A. Segall (Sharp Labs of America) |
| JVET-M0518 | CE4-related: Supplemental results on STMVP design of CE4.2.3.a and combination with methods of JVET-M0127 (CE4.1.2.a) and JVET-M0127. | D. Rusanovskyy, Y.-H. Chao, Y. Han, W.-J. Chien, M. Karczewicz |

Recommendations

Review CE4 document if needed. Review all CE4-related documents.