| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0029-v2 |
| --- | --- |

| Title: | CE9: Summary Report on Decoder Side MV Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu Semih Esenlik | Tel: Email: | xiaoyuxiu@kwai.com semih.esenlik@huawei.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

The tools in the scope of this CE include bi-directional optical flow (BDOF) and decoder side motion vector refinement (DMVR).

The core experiment summary report is organized into 2 sub-tests as follows:

- CE9.1: BDOF design (3 tests)

- CE9.2: DMVR design (24 tests)

This report summarises the status of each experiment. Crosscheck results are integrated in the report.

# Introduction / Problem Statement / …

The description of Core Experiment 9 (CE9) on Decoder Side Motion Vector Derivation [1] has been finalized on Nov. 2nd, 2018. A total 11 tests are defined in CE9 description. After the finalization of the CE description document, 3 tests were withdrawn by proponent, therefore results of 8 tests are reported in the summary report.  The goal of this Core Experiment (CE) is to further investigate the techniques of decoder side motion vector derivation techniques. Tested solutions in the CE9 were evaluated according to BD-rate gain, runtime and complexity assessment.

# Experimental Conditions and Evaluation

## Software

The VTM-3.0 software has been used (SHA-1: ae9412fc2ef56a20bef0a481ff72b5eaeaa5a350) in the CE9. Additionally, the combination of the techniques described in JVET-L0029 and the techniques described in JVET-L0173 with additional CU size constraints of disabling DMVR described in JVET-L0098 is used as the software base for the CE tests in the DMVR category.

## Anchors

- VTM-3.0 is used as the anchors of the CE9

## Test Sequences, Bit Rates and Coding Conditions

CTC as described in [2].

## Reporting of the Results

All test results and crosscheck results were uploaded to git servers and included in the CE9 summary report. The BD-rate, the encoding and decoding with the same SIMD setting for both anchor and test are evaluated.

## Additional evaluation of DMVR tests

The following requirements and the complexity characteristics are analyzed as part of DMVR tests in the CE summary report.

### Requirements

- Reference block pre-fetches shall be guaranteed at least for a region of 128x128

- Refined motion vectors of left 128x128 CTU shall not be used for reference data pre-fetch by the DMVR of the current CTU

- Reconstructed samples of neighboring blocks (except for the top CTU boundary neighboring blocks) shall not be used for template matching of the DMVR

- Wavefront parallelism shall be possible. Refined motion vectors or reconstructed samples from the wavefront diagonal or from the right of the wavefront diagonal shall not be used (in particular the top-right CTU shall not be used).

### Worst case latency characteristics

- Description of processing steps (parallel/sequential)

- Description of data dependency within a 128x128 region of DMVR

### Memory storage

- Intermediate buffers

- Description of the usage of refined motion vectors after consumed by MC

### Complexity in terms of operations

- Number of multiplies and additions

- Number of comparisons

- Number of on-chip memory accesses

### Memory BW increase

- Description of reference block sizes

# CE9.1: BDOF design

## Test specifications

| # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 9.1.1.a | 1. use 8-tap DCTIF filters to generate the prediction samples in extended region and inside CU 2. padding for reference samples outside [w+7, h+7] for final MC (DCTIF) | Xiu, Xiaoyu | H. Liu |
| 9.1.1.b | 1. use integer filters to generate the prediction samples in extended region 2. use 8-tap DCTIF filters to generate the prediction samples inside CU | Xiu, Xiaoyu | H. Liu |
| 9.1.1.c | 1. apply different gradient calculation method for prediciton samples on the CU boundaries and inside CU | Xiu, Xiaoyu | H. Liu |

## Summary of results

|  |  |  | VTM |  |  |  |  | Crosscheck |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Document | Crosschecker | Y | U | V | EncT | DecT | EncT | DecT |
| 9.1.1 a | JVET-M0487 | H. Liu | -0.01% | 0.03% | 0.02% | 103% | 102% | 105% | 104% |
| 9.1.1 b |  | H. Liu | 0.05% | 0.02% | 0.02% | 99% | 98% | 99% | 100% |
| 9.1.1 c |  | H. Liu | 0.10% | 0.03% | 0.01% | 99% | 98% | 100% | 100% |

## Related Contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-M0487 | CE9.1.1 | CE9: Simplifications on bi-directional optical flow (BDOF) (test 9.1.1) |

# CE9.1: DMVR design

## Test specifications

| # | Description | Document |
| --- | --- | --- |
| 9.2.1 a DMVR base s/w | 1. padding for reference samples outside [w+7, h+7] for final MC (DCTIF) 2. use bi-linear filter to generate prediction samples [w+4, h+4] for motion refinement 3a. MRSAD is used as metric for motion refinement 3b. MRSAD calculation on every other row 4. error surface based sub-pixel refinement 5a. DMVR early termination based on MV distance 5b. DMVR early termination based on sample difference Block size criteria: CUs with height >= 8 and size > 64 and CUs with size <= 1024 Refined MV: MC, DBF, TMVP, spatial MV prediction from top and top-left CTU | JVET-M0147 (S. Sethuraman) |
| 9.2.1 b0 | CE9.2.1a + Integer DMVR, SAD, and refined MV used only for MC |  |
| 9.2.1 b | CE9.2.1b0 + Enable 32x32 forced split + enable DMVR for w*h > 1024 + Replace CU level early termination to Sub-PU level |  |
| 9.2.1 c | CE9.2.1b + Replace SAD with MR-SAD |  |
| 9.2.1 d | CE9.2.1c + Replace Integer DMVR with Bilinear MC |  |
| 9.2.1 e | CE9.2.1d + use of refined MV for deblocking, TMVP and for spatial MV prediction from top/top-left CTU neighbors |  |
| 9.2.1 f | CE9.2.1e + use of refined MV also within CTU row from VPDUs that were processed 2 VPDUs behind current VPDU |  |
| 9.2.1 f1 | CE9.2.1f + MC restricted to use samples within (64+7)*(64+7) for VPDU of CUs larger than 64x64 |  |
| 9.2.1 g | CE9.2.1e + replace sub-PU size to 16x16 from 32x32 |  |
| 9.2.2 c | CE9.2.1g + replace MR-SAD with PR-MR-SAD | JVET-M0147 (S. Sethuraman) |
| 9.2.2 d | CE9.2.2c + Change sub-CU level early termination threshold (3 times) to know DecT impact |  |
| *9.2.2 f | Results for CE9.2.2c when BDOF is disabled in both VTM3.0 and with DMVR + DMVR is enabled for ATMVP and MMVD |  |
| 9.2.2 f1 | Disabling refined MV usage for anything other than MC in CE9.2.2c |  |
| 9.2.2 f2 | Replacing PR-MR-SAD with SAD in CE9.2.2c |  |
| 9.2.3 a | No diagonal direction checking when all cross points' cost value not less than center point cost value | JVET-M0306 (X.Chen) |
| 9.2.3 b | Down sampling for MRSAD's mean value calculation |  |
| 9.2.3 c | Replacing MRSAD with SAD in CE9.2.1a |  |
| 9.2.4 a | CE9.2.1a + allow inter reconstruction to be used as intra reference | JVET-M0447 (M. Xu) |
| 9.2.4 b | CE9.2.1a + don't allow inter reconstruction to be used as intra reference |  |
| 9.2.4 c | CE9.2.1a + don't allow DMVR reconstruction to be used as intra reference |  |
| 9.2.5 a | CE9.2.1a is changed as follows: minCost equal to zero replaces minCost less than (w*h*(1<<(Max(2, 14 – Bit Depth) + (Bit Depth-8)+0)) | JVET-M0062 (T. Chujoh ) |
| 9.2.5 b | CE9.2.1a is changed as follows: minCost equal to zero replaces minCost less than (w*h*(1<<(Max(2, 14 – Bit Depth) + (Bit Depth-8)-1)) |  |
| 9.2.6 | CE9.2.1a is changed as follows: Disable DMVR for CUs with size > 4096 | JVET-M0076 (K. Unno) |
| 9.2.7 | CE9.2.1a is changed as follows: Bilinear filter is replaced by no interpolation (rounding to nearest integer sample) | JVET-M0287 (S. Esenlik) |

## * CE9.2.2.f result is reported compared to BDOF off anchor.

## Summary of results

|  |  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Description | Y | U | V | EncT | DecT |
| 9.2.1 a DMVR base s/w | 1. padding for reference samples outside [w+7, h+7] for final MC (DCTIF) 2. use bi-linear filter to generate prediction samples [w+4, h+4] for motion refinement 3a. MRSAD is used as metric for motion refinement 3b. MRSAD calculation on every other row 4. error surface based sub-pixel refinement 5a. DMVR early termination based on MV distance 5b. DMVR early termination based on sample difference Block size criteria: CUs with height >= 8 and size > 64 and CUs with size <= 1024 Refined MV: MC, DBF, TMVP, spatial MV prediction from top and top-left CTU | -0.71% | -0.81% | -0.87% | 110% | 118% |
| 9.2.1 b0 | CE9.2.1a + Integer DMVR, SAD, and refined MV used only for MC | -0.30% | -0.39% | -0.40% | 101% | 103% |
| 9.2.1 b | CE9.2.1b0 + Enable 32x32 forced split + enable DMVR for w*h > 1024 + Replace CU level early termination to Sub-PU level | -0.58% | -0.86% | -0.86% | 102% | 106% |
| 9.2.1 c | CE9.2.1b + Replace SAD with MR-SAD | -0.66% | -0.88% | -0.92% | 102% | 108% |
| 9.2.1 d | CE9.2.1c + Replace Integer DMVR with Bilinear MC | -0.74% | -1.02% | -1.07% | 102% | 111% |
| 9.2.1 e | CE9.2.1d + use of refined MV for deblocking, TMVP and for spatial MV prediction from top/top-left CTU neighbors | -1.12% | -1.25% | -1.30% | 103% | 114% |
| 9.2.1 f | CE9.2.1e + use of refined MV also within CTU row from VPDUs that were processed 2 VPDUs behind current VPDU | -1.25% | -1.33% | -1.43% | 103% | 114% |
| 9.2.1 f1 | CE9.2.1f + MC restricted to use samples within (64+7)*(64+7) for VPDU of CUs larger than 64x64 | -1.25% | -1.33% | -1.42% | 103% | 114% |
| 9.2.1 g | CE9.2.1e + replace sub-PU size to 16x16 from 32x32 | -1.13% | -1.33% | -1.44% | 103% | 118% |
| 9.2.2 c | CE9.2.1g + replace MR-SAD with PR-MR-SAD | -1.07% | -1.27% | -1.36% | 103% | 118% |
| 9.2.2 d | CE9.2.2c + Change sub-CU level early termination threshold (3 times) to know DecT impact | -1.04% | -1.27% | -1.36% | 103% | 117% |
| *9.2.2 f | Results for CE9.2.2c when BDOF is disabled in both VTM3.0 and with DMVR + DMVR is enabled for ATMVP and MMVD | -1.51% | -1.58% | -1.59% | 116% | 126% |
| 9.2.2 f1 | Disabling refined MV usage for anything other than MC in CE9.2.2c | -0.72% | -1.05% | -1.11% | 102% | 115% |
| 9.2.2 f2 | Replacing PR-MR-SAD with SAD in CE9.2.2c | -1.02% | -1.29% | -1.35% | 103% | 115% |
| 9.2.3 a | No diagonal direction checking when all cross points' cost value not less than center point cost value | -0.71% | -0.84% | -0.90% | 108% | 116% |
| 9.2.3 b | Down sampling for MRSAD's mean value calculation | -0.68% | -0.84% | -0.89% | 108% | 115% |
| 9.2.3 c | Replacing MRSAD with SAD in CE9.2.1a | -0.60% | -0.78% | -0.84% | 107% | 115% |
| 9.2.4 a | CE9.2.1a + allow inter reconstruction to be used as intra reference | -0.71% | -0.81% | -0.87% | 108% | 115% |
| 9.2.4 b | CE9.2.1a + don't allow inter reconstruction to be used as intra reference | 3.87% | 8.82% | 9.62% | 103% | 113% |
| 9.2.4 c | CE9.2.1a + don't allow DMVR reconstruction to be used as intra reference | -0.46% | -0.34% | -0.36% | 108% | 115% |
| 9.2.5 a | CE9.2.1a is changed as follows: minCost equal to zero replaces minCost less than (w*h*(1<<(Max(2, 14 – Bit Depth) + (Bit Depth-8)+0)) | -0.67% | -0.80% | -0.86% | 109% | 118% |
| 9.2.5 b | CE9.2.1a is changed as follows: minCost equal to zero replaces minCost less than (w*h*(1<<(Max(2, 14 – Bit Depth) + (Bit Depth-8)-1)) | -0.70% | -0.81% | -0.87% | 109% | 119% |
| 9.2.6 | CE9.2.1a is changed as follows: Disable DMVR for CUs with size > 4096 | -0.89% | -0.99% | -1.08% | 111% | 119% |
| 9.2.7 | CE9.2.1a is changed as follows: Bilinear filter is replaced by no interpolation (rounding to nearest integer sample) | -0.58% | -0.69% | -0.74% | 109% | 115% |

## * CE9.2.2.f result is reported compared to BDOF off anchor.

## Complexity evaluation

| Test | Cost function | Usage of refined motion vectors | # of operations for Interpolation (for 8x8 reference block) | # of operations for SAD/MRSAD (for 8x8 reference block) | Additional operations (for 8x8 reference block) | Block size criteria |
| --- | --- | --- | --- | --- | --- | --- |
| 9.2.1 a | MRSAD | Motion compensation, from neighbor Cus in top left and top CTUs for merge list and AMVP, Deblocking and temporal MV prediction | Adds: 4272(includes rounding) Mult: 4144 | Adds: 1760 | (1) 5 comparisons for early termination based on MV distance (2) 13 comparisons for cost (3) 128 addition 1 comparison for early termination based on sample (4) 20Additions,14 shifts and 18 conditions for error surface | CUs with height >= 8 and size > 64 CUs with size <= 1024 |
| 9.2.1 b0 | SAD | Motion compensation | Adds: 3072 Mult: 2944 | Adds: 832 | same as base 9.2.1a | CUs with height >= 8 and size > 64 CUs with size <= 1024 |
| 9.2.1 b | SAD | Motion compensation | Adds: 3072 Mult: 2944 | Adds: 832 | (1) 5 comparisons for early termination based on MV distance (2) 13 comparisons for cost (3) 20Additions,14 shifts and 18 conditions for error surface | CUs with height >= 8 and size > 64 32x32 Sub-PU |
| 9.2.1 c | MRSAD | Motion compensation | Adds: 3072 Mult: 2944 | Adds: 1760 | same as CE9.2.1b | CUs with height >= 8 and size > 64 32x32 Sub-PU |
| 9.2.1 d | MRSAD | Motion compensation | Adds: 4272 Mult: 4144 | Adds: 1760 | same as CE9.2.1b | CUs with height >= 8 and size > 64 32x32 Sub-PU |
| 9.2.1 e | MRSAD | same as base 9.2.1a at VPDU level instead of CTU level | Adds: 4272 Mult: 4144 | Adds: 1760 | same as CE9.2.1b | CUs with height >= 8 and size > 64 32x32 Sub-PU |
| 9.2.1 f | MRSAD | same as base 9.2.1a at VPDU level instead of CTU level | Adds: 4272 Mult: 4144 | Adds: 1760 | same as CE9.2.1b | CUs with height >= 8 and size > 64 32x32 Sub-PU |
| 9.2.1 f1 | MRSAD | same as base 9.2.1a | Adds: 4272 Mult: 4144 | Adds: 1760 | same as CE9.2.1b | CUs with height >= 8 and size > 64 32x32 Sub-PU |
| 9.2.1 g | MRSAD | same as base 9.2.1a | Adds: 4272 Mult: 4144 | Adds: 1760 | same as CE9.2.1b | CUs with height >= 8 and size > 64 16x16 Sub-PU |
| 9.2.2 c | PR-MRSAD | same as base 9.2.1a | Adds: 4272 Mult: 4144 | Adds: 1632 | same as CE9.2.1b | CUs with height >= 8 and size > 64 16x16 Sub-PU |
| 9.2.2 d | PR-MRSAD | same as base 9.2.1a | Adds: 4272 Mult: 4144 | Adds: 1632 | same as CE9.2.1b | CUs with height >= 8 and size > 64 16x16 Sub-PU |
| *9.2.2 f | PR-MRSAD | same as base 9.2.1a | Adds: 4272 Mult: 4144 | Adds: 1632 | same as CE9.2.1b | CUs with height >= 8 and size > 64 16x16 Sub-PU |
| 9.2.2 f1 | PR-MRSAD | same as base 9.2.1a | Adds: 4272 Mult: 4144 | Adds: 1632 | same as CE9.2.1b | CUs with height >= 8 and size > 64 16x16 Sub-PU |
| 9.2.2 f2 | SAD | same as base 9.2.1a | Adds: 4272 Mult: 4144 | Adds: 832 | same as CE9.2.1b | CUs with height >= 8 and size > 64 16x16 Sub-PU |
| 9.2.3 a | MRSAD | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | Cost value comparison between cross point and center point | CUs with height >= 8 and size > 64 CUs with size <= 1024 |
| 9.2.3 b | MRSAD | same as base 9.2.1a | same as base 9.2.1a | Adds: 1296 | same as base 9.2.1a | CUs with height >= 8 and size > 64 CUs with size <= 1024 |
| 9.2.3 c | SAD | same as base 9.2.1a | same as base 9.2.1a | Adds: 832 | same as base 9.2.1a | CUs with height >= 8 and size > 64 CUs with size <= 1024 |
| 9.2.4 a | MRSAD | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | CUs with height >= 8 and size > 64 CUs with size <= 1024 |
| 9.2.4 b | MRSAD | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | CUs with height >= 8 and size > 64 CUs with size <= 1024 |
| 9.2.4 c | MRSAD | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | CUs with height >= 8 and size > 64 CUs with size <= 1024 |
| 9.2.5 a | MRSAD | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | CUs with height >= 8 and size > 64 CUs with size <= 1024 |
| 9.2.5 b | MRSAD | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | CUs with height >= 8 and size > 64 CUs with size <= 1024 |
| 9.2.6 | MRSAD | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | same as base 9.2.1a | CUs with height >= 8 and size > 64 CUs with size <= 4096 |
| 9.2.7 | MRSAD | same as base 9.2.1a | Adds: 2576 Mult: 2944 (same as bi-prediction) | same as base 9.2.1a | same as base 9.2.1a | CUs with height >= 8 and size > 64 CUs with size <= 1024 |

## Crosscheck Results

|  |  |  | Tester |  | Crosscheck |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Document | Crosschecker | EncT | DecT | EncT | DecT |
| 9.2.1 a DMVR base s/w | JVET-M0147 | J. Li | 110% | 118% | 111% | 120% |
| 9.2.1 b0 |  | C.-C. Chen | 101% | 103% | 103% | 105% |
| 9.2.1 b |  | C.-C. Chen | 102% | 106% | 103% | 109% |
| 9.2.1 c |  | J. Li | 102% | 108% | 102% | 110% |
| 9.2.1 d |  | J. Li | 102% | 111% | 102% | 112% |
| 9.2.1 e |  | C.-C. Chen | 103% | 114% | 104% | 116% |
| 9.2.1 f |  | C.-C. Chen | 103% | 114% | 104% | 117% |
| 9.2.1 f1 |  | C.-C. Chen | 103% | 114% | 103% | 116% |
| 9.2.1 g |  | C.-C. Chen | 103% | 118% | 104% | 121% |
| 9.2.2 c |  | T. Chujoh | 103% | 118% | 103% | 123% |
| 9.2.2 d |  | T. Chujoh | 103% | 117% | 103% | 123% |
| *9.2.2 f |  | T. Chujoh | 116% | 126% | 116% | 134% |
| 9.2.2 f1 |  | T. Chujoh | 102% | 115% | 102% | 122% |
| 9.2.2 f2 |  | T. Chujoh | 103% | 115% | 103% | 121% |
| 9.2.3 a | JVET-M0306 | T. Ikai | 108% | 116% | 109% | 118% |
| 9.2.3 b |  | T. Ikai | 108% | 115% | 109% | 118% |
| 9.2.3 c |  | T. Ikai | 107% | 115% | 108% | 115% |
| 9.2.4 a | JVET-M0447 | J. Ma | 108% | 115% | 109% | 118% |
| 9.2.4 b |  | J. Ma | 103% | 113% | 104% | 117% |
| 9.2.4 c |  | J. Ma | 108% | 115% | 109% | 117% |
| 9.2.5 a | JVET-M0062 | K. Unno | 109% | 118% | 109% | 116% |
| 9.2.5 b |  | K. Unno | 109% | 119% | 109% | 117% |
| 9.2.6 | JVET-M0076 | T. Chujoh | 111% | 119% | 110% | 122% |
| 9.2.7 | JVET-M0287 | T. Chujoh | 109% | 115% | 106% | 116% |

## Related Contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-M0147 | 9.2.1 and 9.2.2 | CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2 |
| JVET-M0306 | 9.2.3 | CE9: DMVR Simplifications (Test 9.2.3) |
| JVET-M0447 | 9.2.4 | CE9: Constrained intra prediction with DMVR (test 9.2.4) |
| JVET-M0062 | 9.2.5 | CE9: An early termination of DMVR (Test 9.2.5) |
| JVET-M0076 | 9.2.6 | CE9: Block size restriction for DMVR (test 9.2.6) |
| JVET-M0287 | 9.2.7 | CE9: Integer DMVR (Test 9.2.7) |

# Recommendations

- Review all CE tests’ contributions and cross-checks.

- Make decisions on each part on technology under consideration by reviewing CE results.

# Acknowledgement

- Thanks to all proponents for providing CE s/w in time with proper announcement and explanation about modification in the CE9 reflector.

- Thanks to all volunteers for providing the comprehensive cross-checks.

# References

- “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, JVET-L1029, X.Xiu, S. Esenlik.

- “JVET common test conditions and software reference configurations”, JVET-L1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring.