| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0033-v1 |
| --- | --- |

| Title: | CE13: Summary report on coding tools for 360° omnidirectional video |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Philippe Hanhart Jian-Liang Lin Chirag Pujara | Email: | Philippe.Hanhart@InterDigital.com jl.lin@mediatek.com chirag.p@samsung.com |
| Source: | CE13 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 13 on coding tools for 360° omnidirectional video. Different methods to remove subjective artifacts at face boundaries are investigated in CE13: 1) modified in-loop filters, 2) inter prediction using geometry padding, 3) post-filtering, and 4) projection format adaptation. Combinations of different methods is also investigated. All these methods are implemented for HEC, with or without padding. Test results against PHEC and PHEC with in-loop filters disabled across face discontinuities are provided to show the coding efficiency of each normative decoding tool and non-normative pre- and post-processing. BD-rates are computed based on the end-to-end WS-PSNR metric. Crosschecking results are integrated in this contribution.

# CE13.1: In-loop filters

Test 1.2 is withdrawn since tiles have not been implemented in VTM-3.0.

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 13.1.1 | In-loop filters disabled across face discontinuities - HEC - PHEC (padding of 4 samples around face row) | JVET-M0362 (MediaTek/InterDigital) |
| 13.1.3 | HEC with in-loop filters using spherical neighbors - Deblocking using spherical neighbors, SAO and ALF disabled across face discontinuities - Deblocking and ALF using spherical neighbors, SAO disabled across face discontinuities - All in-loop filters using spherical neighbors | JVET-M0363 (MediaTek) |
| 13.1.4 | HEC with deblocking using spherical neighbors, SAO and ALF disabled across face discontinuities - Boundary strength fixed to 1 - Boundary strength fixed to 2 - Boundary strength determined based on the prediction modes | JVET-M0320 (Zhejiang University) |

## Test results

### Compared to PHEC

|  | VTM (end-to-end WS-PSNR) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.1.1.a | -0.57% | -0.58% | -0.37% | 100% | 102% |
| 13.1.1.b | -0.01% | -0.29% | -0.13% | 100% | 102% |
| 13.1.3.a | -0.62% | -0.92% | -0.69% | 100% | 103% |
| 13.1.3.b | -0.65% | -0.96% | -0.77% | 100% | 101% |
| 13.1.3.c | -0.64% | -0.93% | -0.74% | 100% | 101% |
| 13.1.4.a | -0.60% | -0.60% | -0.40% |  |  |
| 13.1.4.b | -0.59% | -0.78% | -0.58% |  |  |
| 13.1.4.c | -0.60% | -0.70% | -0.51% |  |  |
| M0322 | -0.36% | -0.57% | -0.40% |  |  |
| M0323 | 2.39% | -1.92% | -1.31% |  |  |

|  | VTM (codec PSNR) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.1.1.b | -0.32% | -1.41% | -1.23% | 100% | 102% |

### Compared to PHEC with in-loop filters disabled across face discontinuities

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.1.1.a | -0.56% | -0.30% | -0.23% | 100% | 100% |
| 13.1.1.b | - | - | - | - | - |
| 13.1.3.a | -0.61% | -0.63% | -0.56% | 100% | 101% |
| 13.1.3.b | -0.64% | -0.67% | -0.63% | 100% | 99% |
| 13.1.3.c | -0.63% | -0.64% | -0.61% | 100% | 99% |
| 13.1.4.a | -0.59% | -0.32% | -0.26% |  |  |
| 13.1.4.b | -0.58% | -0.49% | -0.45% |  |  |
| 13.1.4.c | -0.59% | -0.41% | -0.38% |  |  |
| M0322 | -0.35% | -0.28% | -0.27% |  |  |
| M0323 | 2.40% | -1.63% | -1.18% |  |  |

### Compared to HEC with in-loop filters disabled across face discontinuities

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.1.1.a | - | - | - | - | - |
| 13.1.1.b | 0.57% | 0.30% | 0.23% | 100% | 100% |
| 13.1.3.a | -0.05% | -0.34% | -0.33% | 100% | 101% |
| 13.1.3.b | -0.07% | -0.38% | -0.40% | 100% | 99% |
| 13.1.3.c | -0.07% | -0.35% | -0.38% | 100% | 99% |
| 13.1.4.a | -0.02% | -0.02% | -0.03% |  |  |
| 13.1.4.b | -0.02% | -0.20% | -0.21% |  |  |
| 13.1.4.c | -0.02% | -0.12% | -0.15% |  |  |

## Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 13.1.1.a |  |  |  |  |  |  |
| 13.1.1.b |  |  |  |  |  | CE13 anchor |
| 13.1.3.a |  |  |  |  |  |  |
| 13.1.3.b |  |  |  |  |  |  |
| 13.1.3.c |  |  |  |  |  |  |
| 13.1.4.a | P. Hanhart | 2 | F | Y* | -/105% | Some mismatch due to platform difference (Linux vs Windows) |
| 13.1.4.b | P. Hanhart | 2 | F | Y* | -/105% |  |
| 13.1.4.c | P. Hanhart | 2 | F | Y* | -/105% |  |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed

Comment: any information that might be helpful to the group

## Brief description of tests

### CE13.1.1 in JVET-M0362 (MediaTek/InterDigital)

### For the de-blocking filter, the filtering process is disabled when the filtered pixels cross the discontinuous edge.

For SAO, when the referenced pixels of current pixel in the SAO filtering process cross the discontinuous edge, the proposed modification disables the SAO filtering process of the current pixel to avoid inadequate filtering process.

For ALF, when filtering a CU, reconstructed sample positions used in filtering are clipped to be within the same side of the face discontinuity to which the current CU belongs.

### CE13.1.3 in JVET-M0363 (MediaTek)

#### De-blocking filter

Based on the concept of spherical continuity, the de-blocking filter fetches the spherical pixels located at another face for filtering when the filtered pixels cross the frame boundary (layout boundary) or the discontinuous edge as illustrated in Figure 1. The spherical pixels are derived directly from the corresponding face without any interpolation and the boundary strength is set to 2 in the de-blocking filtering process.

Figure 1. Spherical pixels from another face for 360°-based de-blocking filter

#### SAO filter

When the referenced neighboring pixels of the current pixel in SAO filtering process cross the discontinuous edge, the proposed implementation uses the spherical pixels located at another face for SAO filtering as illustrated in Figure 2. The pixel data in the spherical positions are copied directly from the corresponding face without any interpolation.

Figure 2. Spherical pixels from another face for 360°-based SAO filter

#### ALF

In ALF, during the determination of the filter coefficients or the filtering process, the accessed pixels crossing the frame boundary (layout boundary) or the discontinuous edge are changed to directly copy the pixels data located at the spherical positions without any interpolation as illustrated in Figure 3.

Figure 3. Spherical pixels from another face for 360°-based ALF filter

### CE13.1.4 in JVET-M0320 (Zhejiang University)

Blocks located at face boundaries are in-loop filtered using their spherical neighbors with de-blocking filter. SAO and ALF are disabled across face the discontinuous edge to avoid inappropriate filtering process (as described in 1.4.1). The derivation process of BS is simplified to reduce line buffer. The coding performance of three cases of different BS values is tested. Three cases of different BS values are listed as follows:

Case a: BS is fixed to1;

Case b: BS is fixed to2;

Case c: BS is determined by the prediction modes of blocks,

Figure 4. The derivation process of BS in Case c

Block P and block Q are blocks which are adjacent on the sphere.

# CE13.2: Inter prediction

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 13.2.1 | Test 1.1.a with face row based geometry padding of reference pictures - Using projection with Lanczos 3/2 interpolation - Using projection with bilinear interpolation - Using projection with nearest-neighbor interpolation - Using unfolding-based padding without interpolation | JVET-M0364 (MediaTek) JVET-M0143 (InterDigital) JVET-M0364 (MediaTek) JVET-M0364 (MediaTek) |
| 13.2.2 | Test 1.1.a with face based geometry padding of reference pictures | JVET-Mxxxx (RWTH Aachen University) |

## Test results

### Compared to PHEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.2.1.a | -2.00% | -2.48% | -2.38% | 101% | 117% |
| 13.2.1.b | -1.97% | -2.49% | -2.38% | 100% | 109% |
| 13.2.1.c | -1.75% | -2.17% | -2.08% | 100% | 105% |
| 13.2.1.d | -1.32% | -1.81% | -1.65% | 100% | 103% |
| 13.2.2 |  |  |  |  |  |

### Compared to PHEC with in-loop filters disabled across face discontinuities

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.2.1.a | -1.99% | -2.20% | -2.25% | 101% | 114% |
| 13.2.1.b | -1.96% | -2.21% | -2.25% | 100% | 105% |
| 13.2.1.c | -1.74% | -1.89% | -1.95% | 100% | 103% |
| 13.2.1.d | -1.31% | -1.53% | -1.52% | 100% | 101% |
| 13.2.2 |  |  |  |  |  |

### Compared to HEC with in-loop filters disabled across face discontinuities

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.2.1.a | -1.43% | -1.91% | -2.02% | 100% | 114% |
| 13.2.1.b | -1.40% | -1.92% | -2.02% | 100% | 105% |
| 13.2.1.c | -1.19% | -1.60% | -1.72% | 100% | 103% |
| 13.2.1.d | -0.75% | -1.24% | -1.29% | 99% | 101% |
| 13.2.2 |  |  |  |  |  |

## Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 13.2.1.a | P. Hanhart | 2 | F | Y | -/122% |  |
| 13.2.1.b |  |  |  |  |  |  |
| 13.2.1.c | P. Hanhart | 2 | F | Y | -/108% |  |
| 13.2.1.d | P. Hanhart | 2 | F | Y | -/106% |  |
| 13.2.2 |  |  |  |  |  |  |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed

Comment: any information that might be helpful to the group

## Brief description of tests

### CE13.2.1 in JVET-M0364 (MediaTek) / JVET-M0143 (InterDigital)

The geometry padding in Figure 5 demonstrates a pixel position qb qbqboutside the Bottom face is represented by its spherical corresponding pixel qa, which is located at the Front face. This spherical corresponding pixel is obtained by using the HEC mapping functions. In Test 13.2.1.a, when the derived coordinate qa is fractional, 6-tap and 4-tap Lanczos interpolation filters are applied for the luminance and chrominance components, respectively. In Test 13.2.1.b, bilinear interpolation is applied for the luminance and chrominance components. In Test 13.2.1.c, nearest neighbor (NN) rounding is applied for the luminance and chrominance components. In Test 13.2.1.d, the padding areas of sub-frame buffers are filled by duplicating spherical adjacent faces without any interpolation.

Figure 5. Geometry padding process

To achieve the geometry padding in inter prediction, the proposed implementation maximally utilizes face continuity by applying geometry padding on extended padding areas of top and bottom face row for each reference frame data. The padding size is the LCU size with additional 16 pixels.

### CE13.2.2 in JVET-Mxxxx (RWTH Aachen University)

# CE13.3: Post-filtering

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 13.3.1 | Test 1.1.a with post-filtering of seam artifacts using deblocking filter - Boundary strength fixed to 1 - Boundary strength fixed to 2 | JVET-M0321 (Zhejiang University) |

## Test results

### Compared to PHEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.3.1.a | -0.61% | -0.58% | -0.37% |  |  |
| 13.3.1.b | -0.61% | -0.78% | -0.56% |  |  |

### Compared to PHEC with in-loop filters disabled across face discontinuities

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.3.1.a | -0.60% | -0.30% | -0.23% |  |  |
| 13.3.1.b | -0.60% | -0.50% | -0.43% |  |  |

### Compared to HEC with in-loop filters disabled across face discontinuities

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.3.1.a | -0.04% | 0.00% | 0.00% |  |  |
| 13.3.1.b | -0.04% | -0.20% | -0.19% |  |  |

## Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 13.3.1.a | P. Hanhart | 2 | F | Y* | -/108% | Some mismatch due to platform difference (Linux vs Windows) |
| 13.3.1.b | P. Hanhart | 2 | F | Y* | -/108% |  |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed

Comment: any information that might be helpful to the group

## Brief description of tests

### CE13.3.1 in JVET-M0321 (Zhejiang University)

Blocks located at face boundaries are post filtered using their spherical neighbors with de-blocking filter. In-loop filters are disabled across face the discontinuous edge to avoid inappropriate filtering process (the same as 1.4.1). The coding performance of different fixed BS values is tested. Two cases of fixed BS values are listed as follows:

Case a: BS is fixed to1;

Case b: BS is fixed to2;

# CE13.4: Projection format adaptation

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 13.4.1 | Test 1.1.a with adaptive frame packing | JVET-M0144 (InterDigital) |
| 13.4.2 | Pre-rotation - Test 1.1.a with pre-rotation - Test 1.1.b with pre-rotation | JVET-M0235 (Samsung) |

## Test results

### Compared to PHEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.4.1 | -0.87% | -0.49% | -0.55% | 100% | 106% |
| 13.4.2.a | -0.96% | -1.54% | -1.33% |  |  |
| 13.4.2.b | -0.40% | -1.18% | -1.32% |  |  |
| M0534 | -1.14% | -1.25% | -1.49% |  |  |

### Compared to PHEC with in-loop filters disabled across face discontinuities

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.4.1 | -0.86% | -0.20% | -0.41% | 100% | 102% |
| 13.4.2.a | -0.95% | -1.25% | -1.20% |  |  |
| 13.4.2.b | -0.39% | -0.89% | -1.19% |  |  |
| M0534 | -1.13% | -0.96% | -1.36% |  |  |

### Compared to HEC with in-loop filters disabled across face discontinuities

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.4.1 | -0.30% | 0.10% | -0.18% | 100% | 102% |
| 13.4.2.a | -0.39% | -0.96% | -0.96% |  |  |
| 13.4.2.b | 0.17% | -0.54% | -1.01% |  |  |
| M0534 | -0.57% | -0.67% | -1.12% |  |  |

## Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 13.4.1 | C. Pujara | 2 | F | Y |  |  |
| 13.4.2.a | P. Hanhart | 2 | F | Y | 100%/104% |  |
| 13.4.2.b | P. Hanhart | 2 | F | Y | 101%/104% |  |
| M0534 | P. Hanhart | 2 | F | Y | - |  |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed

Comment: any information that might be helpful to the group

## Brief description of tests

### CE13.4.1 in JVET-M0144 (InterDigital)

AFP periodically modifies the location of discontinuous face boundaries in the frame packed pictures to improve coding efficiency and reduce the visibility of face seam artifacts. AFP considers four frame packing configurations corresponding to yaw rotations of 0°, 90°, 180°, and 270°. Among the four combinations, the cost of each configuration is calculated based on gradient classification. For the first picture, the best combination is used. For the subsequent IRAP pictures, AFP alternates between the different configurations based on the cost of each combination computed for the given IRAP picture.

The frame packing parameters are determined at the encoder side at every IRAP and signaled in the PPS.

### CE13.4.2 in JVET-M0235 (Samsung)

It has been observed that rotation helps in improving the coding gain for 360° video. Rotation of the content before projection can help in reducing the discontinuities in the complex region (hard to encode) and align the edges in the content more suitable for the conventional video compression. Rotation parameters are derived from the first frame and can be signaled in SEI/VUI.

Input image

36 HEC image

Generation

(Low Resolution)

Compute

Gradient angles

Find the image

with max no. of

vertical and

horizontal gradients

Figure 6. Block diagram of optimal rotation parameter estimation algorithm.

To arrive at the optimal rotation angle without encoding the video we devised the following algorithm steps,

- Downsize the image to 2K size and generate 36 HEC images corresponding to rotation angles multiple of 10 degrees starting from 10 to 360 degrees. (only for X axis rotation)

- Compute gradient magnitudes and angles for each of these 36 images

- Find the image with maximum number of vertical and horizontal gradients from these images. Here we consider gradients between -5 to 5 degrees as horizontal and -85 to 95 degrees as vertical.

- After arriving at the best angle at the granularity of 10 degree these steps can be further repeated to reach to 1 degree resolution.

- This algorithm can also be applied at RAP level.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 7. Frame from Gaslamp sequence and its edge image. (a) Image with no rotation, (b) gradient image of a, (c) image with optimal rotation (140 degrees), (d) gradient image of c

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 7 shows the image from the Gaslamp sequence with no rotation and with optimal rotation. It can be seen that the gradients are aligned to vertical and horizontal directions for the later case.

Early termination based on Motion estimation

As this algorithm analyses only first frame, we added an additional check if the rotation should be performed or not based on the motion estimation from the I frame of first and second RAP. If the SAD between these two frames is above some threshold then we do not perform rotation. With this condition, we can avoid the compression losses for 6k sequences.

# CE13.5: Combinations

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 13.5.1 | Face row based geometry padding of reference pictures and in-loop filters using spherical neighbors - 1.3.a + 2.1.a - 1.3.b + 2.1.a - 1.3.c + 2.1.a - 1.3.a + 2.1.d - 1.3.b + 2.1.d - 1.3.c + 2.1.d | JVET-M0367 (MediaTek) |
| 13.5.2 | Face based geometry padding of reference pictures and in-loop filters using spherical neighbors - Test 2.2 + DBF using spherical neighbors, SAO and ALF disabled across face boundaries - Test 2.2 + in-loop filters using spherical neighbors | JVET-Mxxxx (RWTH Aachen University) |

## Test results

### Compared to PHEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.5.1.a | -2.02% | -2.61% | -2.49% | 101% | 118% |
| 13.5.1.b | -2.02% | -2.66% | -2.53% | 101% | 116% |
| 13.5.1.c | -2.03% | -2.65% | -2.51% | 101% | 116% |
| 13.5.1.d | -1.34% | -1.95% | -1.79% | 100% | 104% |
| 13.5.1.e | -1.35% | -1.97% | -1.84% | 100% | 102% |
| 13.5.1.f | -1.36% | -1.94% | -1.79% | 100% | 102% |
| 13.5.2.a |  |  |  |  |  |
| 13.5.2.b |  |  |  |  |  |

### Compared to PHEC with in-loop filters disabled across face discontinuities

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.5.1.a | -2.01% | -2.33% | -2.36% | 101% | 115% |
| 13.5.1.b | -2.01% | -2.38% | -2.40% | 101% | 113% |
| 13.5.1.c | -2.02% | -2.37% | -2.38% | 101% | 113% |
| 13.5.1.d | -1.33% | -1.67% | -1.65% | 100% | 102% |
| 13.5.1.e | -1.34% | -1.69% | -1.71% | 100% | 100% |
| 13.5.1.f | -1.35% | -1.66% | -1.66% | 100% | 100% |
| 13.5.2.a |  |  |  |  |  |
| 13.5.2.b |  |  |  |  |  |

### Compared to HEC with in-loop filters disabled across face discontinuities

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.5.1.a | -1.45% | -2.04% | -2.13% | 101% | 115% |
| 13.5.1.b | -1.45% | -2.09% | -2.17% | 101% | 113% |
| 13.5.1.c | -1.46% | -2.08% | -2.15% | 101% | 113% |
| 13.5.1.d | -0.77% | -1.37% | -1.42% | 100% | 102% |
| 13.5.1.e | -0.78% | -1.39% | -1.48% | 100% | 100% |
| 13.5.1.f | -0.79% | -1.37% | -1.43% | 99% | 100% |
| 13.5.2.a |  |  |  |  |  |
| 13.5.2.b |  |  |  |  |  |

## Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 13.5.1.a |  |  |  |  |  |  |
| 13.5.1.b |  |  |  |  |  |  |
| 13.5.1.c |  |  |  |  |  |  |
| 13.5.1.d |  |  |  |  |  |  |
| 13.5.1.e |  |  |  |  |  |  |
| 13.5.1.f |  |  |  |  |  |  |
| 13.5.2.a |  |  |  |  |  |  |
| 13.5.2.b |  |  |  |  |  |  |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed

Comment: any information that might be helpful to the group

## Brief description of tests

### CE13.5.1 in JVET-M0367 (MediaTek)

Combinations of tests 1.3 (see Section 1.4.2), and 2.1 (see Section 2.4.1).

### CE13.5.2 in JVET-Mxxxx (RWTH Aachen University)

# Related contributions

## CE13-related: In-loop filters disabled across face discontinuities on PHEC with 2-pixel padding (JVET-M0322)

This proposal uses in-loop filters disabled across face discontinuities on PHEC (padding of 2 samples around face row) to solve the subjective issue of HEC. Test results show that proposed method yields -0.35% in BD-rate (using end-to-end Y-WS-PSNR) reduction on average in comparison to that of in-loop filters disabled across face discontinuities on PHEC (padding of 4 samples around face row) while keeping similar subjective results.

## CE13-related: Adaptive QP to improve subjective quality for PHEC (JVET-M0323)

This contribution proposes a geometry based adaptive QP method together with in-loop filters disabled across face discontinuities and padding (padding of 2 samples around face row) on PHEC. Boundary areas are coded with better quality by using lower QP to further reduce visible artifacts. The experimental results show that the proposed method can significantly improve subjective quality with some coding loss in objective performance.

## CE13-related: HEC with Pre-rotation + Adaptive Frame Packing(Test 4.2.a+4.1) (JVET-M0534)

This contribution reports simulation results of Pre-rotation followed by Adaptive Frame Packing(AFP) on Hybrid Equiangular Cubemap (HEC) implementation over 360Lib8.0 merged with VTM3.0. This test is implemented over test 1.1a (HEC with in-loop filters disabled) which shows gain of {-1.14%, -1.25%, -1.49%} over CTC anchor and {-1.13%, -0.96%, -1.36%} over CE anchor (test 1.1b), while it shows a gain of {-0.57%, -0.67%, -1.12%} over test 1.1a under 360 Common Test Condition (CTC) for E2EWSPSNR. Maximum gain is observed for Gaslamp sequence, which is around -2.78%.