| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0054 |
| --- | --- |

| Title: | CE2: Modified affine inheritance from above CTU (test 2.4.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, and Marta Karczewicz 10160 Pacific Mesa Blvd, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the result of CE2.4.6. In which, affine inheritance from 6-paramter affine neighboring coded block in the above CTU is modified. The bottom-left CPMV is predicted from a subblock MV locates in the left or bellow left of the current block.  The BD-rates in random access common test condition are -0.01%, 0.01%, -0.02%, and are 0.02%, -0.03%, 0.02% in low delay B.

# Introduction

In CE2.4.6, deriving a 6-parameter affine inherited candidate from a neighboring block in the above CTU is illustrated in Fig. 1. The top-left CPMV v0 and top-right CPMVs v1 are derived by the the bottom-left MV vLB and bottom-right MV vRB of the neighboring candidate block:

v0=vRB-vLBneiW*posCurX-posNeiX+vLB

v1=vRB-vLBneiW*posCurX+curW-posNeiX+vLB

Wherein neiW is the width of the neighboring block, curW is the width of current block, posNeiX is the x coordinate of the top-left pixel of neighboring block, and posCurX is the x coordinate of the top-left pixel of current block.

The bottom-left CPMV v2 is derived as follows:

If the MV from left neighbor vleft is available and has the same reference picture as  vLB

v2 =vleft;

Else if the MV from bellow-left neighbor vbellowleft is available and has the same reference 
      picture as  vLB

v2 =vbellowleft;

Else

deltaHorX,deltaHorY=vRB-vLBneiW

∇vVer=(-deltaHorY,deltaHorX)

v2 =vRB-vLBneiW*posCurX-posNeiX+∇vVer*curH+vLB

Wherein curH is the height of current block.

Fig. 1

# Test results

The random access (RA) and low delay B (LB) results under common test condition are reported in Table 1. The BD-rates are -0.01%, 0.01%, -0.02% in RA, and are 0.02%, -0.03%, 0.02% in LB. The runtime remains the same as in VTM-3.0.

Table 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.03% | 99% | 101% |
| Class A2 | -0.03% | -0.02% | 0.01% | 100% | 100% |
| Class B | -0.01% | 0.05% | -0.07% | 100% | 100% |
| Class C | 0.01% | -0.02% | 0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | -0.02% | 100% | 100% |
| Class D | 0.01% | -0.03% | 0.01% | 100% | 100% |
| Class F | -0.02% | -0.01% | -0.01% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.23% | -0.08% | 100% | 102% |
| Class C | -0.01% | 0.20% | 0.14% | 100% | 97% |
| Class E | 0.05% | -0.01% | 0.04% | 99% | 96% |
| Overall | 0.02% | -0.03% | 0.02% | 100% | 99% |
| Class D | 0.00% | -0.02% | 0.37% | 101% | 99% |
| Class F | 0.05% | -0.26% | 0.25% | 101% | 100% |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).