| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0062-v1 |
| --- | --- |

| Title: | CE9: An early termination of DMVR (Test 9.2.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-43-299-8526 chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution is a report of CE9.2.5. In this contribution, a result of an early termination of DMVR (Decoder Motion Vector Refinement) has been reported. DMVR is a method of improving coding efficiency without explicitly sending overheads by motion vectors refinement of the merge mode on the both encoder and decoder side. One of the tasks is to reduce the complexity of the decoder side. In this contribution, an early termination of DMVR is improved. As experimental results, the decoding time was reduced by 2% without additional SIMD, and the coding loss is 0.04%.

Introduction

As CE9.2 [1], several problems of the DMVR have been discussed. One of the problem is the complexity of decoder side. In the case of software implementation, the method of early termination is a technique often used for speeding up motion vector search and mode selection. In this contribution, we have tested a result of a simple early termination for the DMVR. In the following section, the method and experimental results have been reported.

Early termination

Current cost determination method of DMVR on CE9.2 base software is as follows:

Step 1: Cost determination of initial MV

If initial matching cost is equal to zero, then the DMVR processing is terminated, else go to step 2.

Step 2: Cost determination of refinement MV

A motion vector with the minimum cost of the search range is searched.

As the default matching cost of DMVR, MRSAD (Mean Removed Sum of Absolute Differences) is used. The bit precision of cost is equal to Max(2, 14 – BitDepth) + BitDepth, because the interpolated prediction samples are designed with computational precision of more than two decimal places. Additionally, current cost is not considered the block size. Therefore, since the probability of cost equal to zero is very small actually, another threshold should be designed.

In this contribution, the cost determination of initial MV is improved as follow.

- If the initial matching cost of step 1 is less than (w*h*(1<<(Max(2, 14 – BitDepth) + (BitDepth-8)+TH)), then the cost is regarded as zero and the DMVR processing is terminated, else go to step 2.

Here, w is the width of block, h is the height of block, BitDepth is the bit depth of samples and TH is the parameter of early termination. As a preliminary experimental result, 0 was selected for the value of TH.

In our proposal, only one line is replaced on CE9.2 base software and any additional SIMD code is not used.

Experimental results

The simulation based on JVET-L1010 [2] for SDR sequences has been conducted. According to the suggestion of CE9, the following test conditions have been used.

- The software used in CE9 is: CE9.2_base_sw.

- Anchor for Mandatory simulations: CE9.2_base_sw.

- Test for mandatory simulations: Anchor + DMVR on.

The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

Table 1 Summary of experimental results (TH=0) compared to CE9.2 base software

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0_CE9.2_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.02% | 0.09% | 100% | 97% |
| Class A2 | 0.06% | 0.06% | 0.08% | 100% | 98% |
| Class B | 0.02% | -0.06% | 0.00% | 100% | 98% |
| Class C | 0.03% | 0.03% | -0.09% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.01% | 0.01% | 100% | 98% |
| Class D | 0.02% | -0.05% | 0.10% | 100% | 99% |
| Class F (optional) | 0.00% | 0.00% | 0.01% | 100% | 99% |

Table 2 Summary of experimental results (TH=-1) compared to CE9.2 base software

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0_CE9.2_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.04% | 100% | 98% |
| Class A2 | 0.01% | 0.02% | 0.04% | 99% | 98% |
| Class B | 0.02% | -0.06% | -0.03% | 100% | 99% |
| Class C | 0.02% | 0.05% | -0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.00% | 100% | 99% |
| Class D | 0.01% | -0.02% | 0.03% | 100% | 99% |
| Class F (optional) | -0.02% | 0.02% | -0.01% | 100% | 99% |

Conclusion

In this contribution, a result of CE9.2.5 been reported. As experimental results, the decoding time was reduced by 2% without additional SIMD, and the coding loss is 0.04%..

Reference

[1]	X. Xiu and S. Esenlik, Description of Core Experiment 9 (CE9): Decoder-side motion derivation, Joint Video Experts Team Document, JVET-L1029, Macao, Oct. 2018.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-L1010, Ljubljana, July 2018.

[3]	T Chujoh and T. Ikai, CE9-related: An early termination of DMVR, Joint Video Experts Team Document, JVET-L0367, Macao, Oct. 2018.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).