| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0064 |
| --- | --- |

| Title: | Non-CE3: CCLM table reduction and bit range control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Frank Bossen† Eiichi Sasaki | Tel: Email: | +81-43-299-8526 yasugi.yukinobu@sharp.co.jp frank@bossentech.com eiichi.sasaki@sharp.co.jp |
| Source: | Sharp Corporation, †Sharp Electronics of Canada |  |  |

_____________________________

# Abstract

This contribution proposes a method to reduce the CCLM bit range and table size.  It is asserted that the VTM3.0 CCLM has two problems: (i) that the current design requires the use of two, large tables and (ii) that the precision of the entries in these tables necessitates high bit-depth multiplication.  Specifically, it is observed that the VTM-3.0 CCLM employs two tables, each with 512 elements and where each element is 17 bits.  This table is designed to avoid a division operation during the derivation process.  However, when the internal bit depth of the decoded sample values is 10 bits, it is asserted that this design results in a 28 bit multiplication and 37 bit accumulation for the gradient a and intercept b, respectively, in the CCLM algorithm.  It is argued that it would be desirable to reduce the calculation bit-depth to fit within 16 bits, which would have a benefit of both reducing the complexity of the calculation as well as the reducing the memory required for storing any needed tables.

Here, it is proposed to use a single table with 16 elements and with 3 bits per element.  Furthermore, experimental results show that the proposed change preserved the almost same compression efficiency as the current VTM-3.0.

# Summary of VTM-3.0 CCLM and proposed CCLM method

The following table shows a comparison between the VTM-3.0 our proposal.

As can be seen form the analysis, we observe that the VTM 3.0 needs a 16896-bit table that results in 27-bit multiplications in the critical path, e.g. the intra loop.

Conversely, our solution employs a 48-bit table and results in 15-bit multiplication in the critical path.

|  | VTM-3.0 | Proposal |
| --- | --- | --- |
| Table | Floor( 232 / diff ) (g_aiLMDivTableHigh[[]] Floor( 232 / diff ) − Floor( 216 / diff ) * 216 (g_aiLMDivTableLow[]) 17 bits * 512 elements + 16 bits * 512 elements = 16896 bits | DivSigTable[] 3 bits * 16 elements = 48 bits |

|  | VTM30 | Proposal |
| --- | --- | --- |
| Derivation for a | ( ( maxC − minC ) * Floor( 216 / diff ) + div + add ) >> shift 27 bits ( -1023 * 65536 ) (26 bits after right-shift) | ( ( maxC − minC ) * ( DivSigTable [ diff >> sc ] ¦ 8 ) + add ) >> expC 11 bits * 4 bits = 15 bits |
| Derivation for b | minC − ( ( a * minY ) >> k ) 26 bits * 10 bits = 36 bits | minC – ( k < 0 ? ( a * minY ) << (-k) : ( a * minY ) >> k 5 bits * 10 bits = 15 bits |
| Prediction (per pixel) | predSamples[ x ][ y ] = Clip1C( ( ( pDsY[ x ][ y ] * a ) >> k ) + b ) 10 bits * 26 bits = 36 bits | predSamples[ x ][ y ] = Clip1C( ( ( pDsY[ x ][ y ] * a ) >> k ) + b ) 10 bits * 5 bits = 15 bits |

# VTM3.0 CCLM calculation

In the current VTM-3.0, the derivation of the linear model parameters (a and b) of CCLM is described as follows:

The variables a, b, and k are derived as follows:

    - If numSampL is greater than 0, and numSampT is greater than 0, the following applies:

k = 0			(8-150)

a = 0			(8-151)

b = 1 << ( BitDepthC − 1) 		(8-152)

    - Otherwise, the following applies:

shift = ( BitDepthC > 8 )  ?  BitDepthC − 9  :  0		(8-153)

add = shift  ?  1 << ( shift − 1)  :  0		(8-154)

diff = ( maxY − minY + add ) >> shift		(8-155)

k = 16			(8-156)

    - If diff is greater than 0, the following applies:

div = ( ( maxC − minC ) * ( Floor( 232 / diff ) − Floor( 216 / diff ) * 216 ) + 215 ) >> 16	(8-157)

a = ( ( maxC − minC ) * Floor( 216 / diff ) + div + add ) >> shift	(8-158)

    - Otherwise, the following applies:

a = 0	(8-159)

b = minC − ( ( a * minY ) >> k )		(8-160)

In this current implementation, “( Floor( 232 / diff )  − Floor( 216 / diff ) * 216 )” and “Floor( 216 / diff )” are used instead of division operator (1/diff), and they are implemented as two look-up tables (g_aiLMDivTableLow[] and g_aiLMDivTableHigh[]). However, the Low and High tables have 512 elements of 16- and 17-bit integer at maximum, respectively. Consequently, we observe that the total size of the tables exceeds 16KB, to say precisely, it is 16896 bits (512*16 + 512*17). Note that 17bit integer (65536) is only needed for diff=1.

In addition, there is also problem with the bit width of multiplication. When the internal bit depth is 10 bits, the bit width of the multiplication during derivation of a and b are 26 bits and 36(10 + 26) bits in the worst case, respectively. Here 26-bits are needed for the worst case of -1023 * 65536, 11 for difference between chroma values and 26 for the maximum width of a after the right shift by “shift”.

# Proposed method

Here, we propose an alternative method for calucating the a and b variables.  The proposed method has two, key features:

- Removal of the table for lower digits

- Reduction of the table size and the width of multiplications

## Removal of the table for lower digits

The performance of VTM-3.0 was not changed significantly regardless of whether the table for lower digits (g_aiLMDivTableLow) is used, according to our experiments. Hence, the proposed method removed the table.

## Reduction of the table size and the width of multiplications

In the proposed method, the difference between pixel values (diff and diffC) and the gradient (a) are expressed by an exponential notation. For example, ‘diff’ is approximated with a 4-bit significant part and an exponent. Consequently, the table for 1/diff is reduced into 16 elements for 16 values of the significand as follows:

DivSigTable [ ] = { 0, 7, 6, 5, 5, 4, 4, 3, 3, 2, 2, 1, 1, 1, 1, 0 }

Note that a 4-bit significand is always more than or equal to 8 because it expresses the four most significant bits. The table therefore omits the MSB bit and each element needs only 3 bits. The total size of the table is 3*16=48 bits.

The 4-bit significant part also reduces the maximum bit width of the multiplication in the calculation of ‘a’ into 15 bits in the worst case, that ‘diffC’ is 11-bit.

The gradient ‘a’ is also expressed by the exponential notation with a 5-bit significant part which means 4 bits from 1/diff and 1 for sign of ‘diffC’. The exponent part of ‘a’ is derived from the bit width of ‘diffC’. The exponent part of the resulting ‘a’, which is used as a shift amount represented by ‘k’ in the calculation of ‘b’ and predicted samples later, is determined based on the value of those exponents.

Since the bit width of the significant part of ‘a’ is five, the maximum width of multiplication in the calculation of ‘b’ is also limited to 15 bits.

Additionally, the proposed method restricts the actual range of the gradient based on the exponent part of ‘a’, or the shift amount ‘k’, so that the range of the intercept b does not become too wide. Specifically, the range of ‘k’ is restricted to be greater than 0 and the range of the gradient ‘a’ is limited from -15*2^-1 to 15*2^-1. This limitation guarantees that the shift operation for the intercept ‘b’ and predSamples is always a right shift.

## Draft text

The variables a, b, and k are derived as follows:

    - If numSampL is greater thanequal to 0, and numSampT is greater thanequal to 0, the following applies:

k = 0			(8-150)

a = 0			(8-151)

b = 1 << ( BitDepthC − 1) 		(8-152)

    - Otherwise, the following applies:

if ( diff == 0 ) {

a = 0

b = minC

} else {

diff = maxY – minY

diffC = maxC − minC

x = Floor( Log2( diff ) )

normDiff = (diff << 4 >> x ) & 15

x += ( normDiff != 0 ) ? 1 : 0

y = Floor( Log2( Abs( diffC ) ) ) + 1

a = ( diffC * (DivSigTable[ normDiff ] | 8 ) + 2y-1 ) >> y

k = 3 + x - y

if ( k < 1 ) {

k = 1

a = Sign(a) * 15

}

b = minC - ( ( a * minY ) >> k )

}

The array DivSigTable [ ] is specified as follows:

DivSigTable [ ] = { 0, 7, 6, 5, 5, 4, 4, 3, 3, 2, 2, 1, 1, 1, 1, 0 }	(8-xxx)

The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0.. nTbH − 1 are derived as follows:

predSamples[ x ][ y ] = Clip1C( ( ( pDsY[ x ][ y ] * a ) >> k ) + b )	(8-161)

# Experimental results

# Conclusion

We propose a method to reduce the table size and maximum bit width of multiplication in CCLM.  The test results shows that the proposed method has negligible impact on BD rate performance and processing time, while allowing the CCLM method to be implemented in 16-bit arithmetic and reducing the table size form 16896 bits to 48 bits. We propose that the method be adopted into the VVC Working Draft.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).