| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0071-v2 |
| --- | --- |

| Title: | AHG12: Improved parallel processing capability with WPP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuji Fujimoto, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 Yuji.Fujimoto@sony.com Masaru.Ikeda@sony.com teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In this contribution, VPDU (Virtual Pipeline Data Unit) based WPP (Wavefront Parallel Processing) is proposed. In VVC, due to the increase in max CTU size from 64x64 (HEVC) to 128x18, the number of process in parallel (threads) of HEVC style WPP becomes half of HEVC. VPDU based WPP (proposed VVC WPP) can improve 30% parallel level. This contribution also confirmed that coding efficiency loss is (AI 0.02%, RA 0.15%, LB 0.06%, LP 0.04%), while coding efficiency loss of HEVC style WPP is (AI 0.39%, RA 2.62%, LB 3.05%, LP 2.80%).

Introduction

WPP in HEVC

WPP (Wavefront Parallel Processing) is introduced in HEVC. It enables to decode CABAC in parallel for each CTU row. Considering dependency between blocks, current block can be decoded when neighbouring blocks indicated by arrow finished decoding as shown in Figure 1.

Figure 1: Block dependency

WPP enables to process CTUs same number of at the same time in parallel as shown in Figure 2. At the beginning of every CTU row, CABAC is reset using context of the end of above right CTU. Then CABAC can be processed in parallel. Coding efficiency loss due to CABAC initialization at each CTU raw can be reduced by using updated contexts in the upper CTU row.

Figure 2: Example of WPP in HEVC

The example of parallel processing for the example in Figure 2 is shown in Figure 3.

Figure 3: Example of parallel processing

Problems in VVC

In VVC, Max CTU size is increased to 128x128, while max CTU size in HEVC is 64x64. It drastically decreases parallel processing capability. For example,

- The number of process in parallel (threads) is reduced, since the number of CTU lines are decreased (for the same picture resolution)

- The pipeline delay of WPP is longer due to the increase of CTU size.

HEVC style CTU based WPP can be implemented in VVC as shown in Figure 4. Blue lines and grey lines indicate CTU boundary and VPDU boundary respectively.

Figure 4: HEVC style CTU based WPP in VVC

In this case, the pipelines delay of HEVC style CTU based WPP in VVC is as shown in Figure 5. Blue lines and grey lines indicate CTU and VPDU respectively.

Figure 5: The pipeline delay of HEVC style CTU based WPP in VVC

In this case, the number of process in parallel (threads) is 15 (the average number of threads is 10.6) for 4K (3840x2160).

Proposal

VPDU (Virtual Pipeline Data Unit)

In VVC, the concept of VPDU (Virtual Pipeline Data Unit) is introduced for efficient pipeline designing.

Figure 6: Example of VPDU pipeline in VVC

Constraints were proposed to enable 64x64-L/32x32-C pipeline design [1], [2]. 1) the maximum Intra block size are 64x64 and 32x32 for Luma and Chroma, respectively as shown in Figure 7.

Figure 7: The constraint on maximum block size for Intra

2) For inter block, the following BT/TT splits in red lines are not allowed.

Figure 8: The constraints on BT and TT splits

3) The maximum transform size is restricted to 64x64.

VPDU based WPP

By using VPDU concept, parallel processing capability of WPP can be improved. In this contribution, VPDU-CTU based WPP is proposed for parallel processing of VVC. The points of the proposal are,

- The pipeline delay of WPP can be reduced from 2 CTUs to 4 VPDUs (1 CTU)

- The upper right CTU is unavailable for pixel and MV reference (break dependency off)

- According to the block partition restrictions for VPDU, the upper right CTU is referred only in limited cases. (not allowed to refer the upper right CTU for Inter 128xN)

The changes to VTM3 are small. The example of VPDU based WPP is shown in Figure 9.

Figure 9: Proposed Method (VVC WPP)

The constraints on reference (breaking dependency off) are summarized as follows.

Figure 10: Constraints on reference (CU size is smaller than VPDU)

Figure 11: Constraints on reference (CU size is larger than VPDU)

The references marked “x” are restricted and the restriction on reference is limited cases.

In this case, the pipelines delay of VPDU based WPP in VVC is as shown in Figure 12. Grey lines indicate VPDU.

Figure 12: The pipeline delay of VPDU based WPP in VVC

In this case, the number of process in parallel (threads) is 17 (the average number of threads is 13.8) for 4K (3840x2160).

Simulation Results

The performance of proposed method is evaluated by implementing on top of VTM 3.0. Test conditions are as follows.

- VTM3.0

-   #define HEVC_TILES_WPP  1

-   --WaveFrontSynchro=1

- Anchor : HEVC style WPP

- Tested : proposed method

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM-3.0 HEVC_TILES_WPP |  |  |
|  | Y | U | V |
| Class A1 | 0.07% | 0.14% | 0.12% |
| Class A2 | 0.02% | 0.08% | 0.12% |
| Class B | 0.02% | -0.06% | -0.02% |
| Class C | -0.04% | 0.01% | -0.04% |
| Class E | 0.05% | 0.03% | -0.02% |
| Overall | 0.02% | 0.03% | 0.02% |

Table 1: The results of All Intra Main10

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM-3.0 HEVC_TILES_WPP |  |  |
|  | Y | U | V |
| Class A1 | 0.26% | 0.49% | 0.44% |
| Class A2 | 0.19% | 0.21% | 0.26% |
| Class B | 0.14% | -0.09% | 0.20% |
| Class C | 0.05% | -0.04% | 0.10% |
| Overall | 0.15% | 0.10% | 0.24% |

Table 2: The results of Random access Man10

|  | Low delay B Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM-3.0 HEVC_TILES_WPP |  |  |
|  | Y | U | V |
| Class B | 0.11% | -0.02% | 0.19% |
| Class C | 0.09% | -0.06% | 0.00% |
| Class E | -0.06% | 0.15% | 0.44% |
| Overall | 0.06% | 0.01% | 0.19% |

Table 3: The results of Low delay B Main10

|  | Low delay P Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM-3.0 HEVC_TILES_WPP |  |  |
|  | Y | U | V |
| Class B | 0.09% | 0.00% | 0.16% |
| Class C | 0.04% | -0.20% | -0.04% |
| Class E | -0.05% | 0.44% | -0.13% |
| Overall | 0.04% | 0.04% | 0.02% |

Table 4: The results of Low delay P Main10

The results of HEVC style WPP (CTU128(anchor) vs CTU64) are summarized below.

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM-3.0 HEVC_TILES_WPP |  |  |
|  | Y | U | V |
| Class A1 | 0.44% | 0.65% | 0.94% |
| Class A2 | 0.42% | 0.82% | 0.94% |
| Class B | 0.37% | 0.59% | 1.03% |
| Class C | 0.29% | 1.00% | 1.08% |
| Class E | 0.50% | 0.38% | 0.30% |
| Overall | 0.39% | 0.69% | 0.89% |

Table 5: The results of All Intra Main10

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM-3.0 HEVC_TILES_WPP |  |  |
|  | Y | U | V |
| Class A1 | 4.09% | 4.14% | 4.63% |
| Class A2 | 3.67% | 5.12% | 4.62% |
| Class B | 2.43% | 3.09% | 3.50% |
| Class C | 0.96% | 2.00% | 2.52% |
| Overall | 2.62% | 3.42% | 3.69% |

Table 6: The results of Random access Man10

|  | Low delay B Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM-3.0 HEVC_TILES_WPP |  |  |
|  | Y | U | V |
| Class B | 2.42% | 4.60% | 5.57% |
| Class C | 1.15% | 3.41% | 4.37% |
| Class E | 6.64% | 7.67% | 7.44% |
| Overall | 3.05% | 4.97% | 5.63% |

Table 7: The results of Low delay B Main10

|  | Low delay P Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM-3.0 HEVC_TILES_WPP |  |  |
|  | Y | U | V |
| Class B | 2.16% | 4.65% | 6.02% |
| Class C | 1.06% | 3.70% | 4.65% |
| Class E | 6.17% | 9.12% | 7.44% |
| Overall | 2.80% | 5.45% | 5.92% |

Table 8: The results of Low delay P Main10

Discussions

WPP parallel level in ideally condition (All CTUs have the same processing time) can be estimated by the following calculations. The results are summarized Table 9.

- Maximum parallel level of HEVC style WPP = min (ceil (CTUw/2), CTUh)

- Maximum parallel level of proposed VVC WPP = min (CTUw, CTUh)

- Average parallel level = Σ p(x, y)/(CTUw * CTUh)

- CTUw : The number of CTUs in horizontal

- CTUh : The number of CTUs in vertical

- p(x, y) : Related to the number of parallel levels when CTU(x, y) is being processed. (The number of CTUs can be processed at that time)

|  | Max CTU size | CTUw, CTUh | Maximum parallel level | Average parallel level |
| --- | --- | --- | --- | --- |
| HEVC | 64x64 | 60, 34 | 30 | 21.2 |
| VVC (HEVC style WPP) | 128x128 | 30, 17 | 15 | 10.6 |
| VVC (Proposed VVC WPP) | 128x128 | 30, 17 | 17 | 13.8 |

Table 9: WPP parallel level (for 4K(3840x2160))

It can be confirmed that VVC (HEVC style WPP) parallel level is half of HEVC (HEVC style WPP) parallel level. VVC (proposed VVC WPP) can on average improve by 30% parallel level

Coding efficiency loss with HEVC style WPP is summarized in Table 5, Table 6, Table 7 and Table 8. In these cases, the average coding efficiency loss is large (AI 0.39%, RA 2.62%, LB 3.05%, LP 2.80%). While coding efficiency impact of proposed VVC WPP is small (AI 0.02%, RA 0.15%, LB 0.06%, LP 0.04%).

Conclusions

In this contribution, VPDU (Virtual Pipeline Data Unit) based WPP (Wavefront Parallel Processing) is proposed. In VVC, the number of process in parallel (threads) of HEVC style WPP is half of HEVC. VPDU based WPP (proposed VVC WPP) can improve by 30% average parallel level. This contribution also confirmed that coding efficiency loss is (AI 0.02%, RA 0.15%, LB 0.06%, LP 0.04%), while coding efficiency loss of HEVC style WPP is (AI 0.39%, RA 2.62%, LB 3.05%, LP 2.80%).

This contribution proposes to study further parallel processing capability with WPP by CE or AHG.

References

[1] JVET-K0230, “CE1-related: Separate tree partitioning at 64x64-luma/32x32-chroma unit level”, T. D.  Chuang, C. Y. Chen, Y. W. Huang, S. M. Lei (MediaTek Inc.)

[2] JVET-L0081, “CE1.2.1: Constraint for binary and ternary partitions”, C. M. Tsai, C. W. Hsu, T. D.  Chuang, C. Y. Chen, Y. W. Huang, S. M. Lei (MediaTek Inc.)

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).