| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0072-v2 |
| --- | --- |

| Title: | Non-CE6: On transform skip for larger block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to allow transform skip for larger transform size, and proposes to add signaling to indicate the maximum size of transform skip. Simulation results show that bd-rate changes are -0.03%, -0.05% and -0.07% up to 8x8 transform skip size for AI, RA and LB, respectively; -0.03%, -0.09% and -0.15% up to 16x16 transform skip size for AI, RA, and LB, respectively; -0.03%, -0.11% and -0.21% up to 32x32 transform skip size for AI, RA, and LB, respectively; -0.03%, -0.10% and -0.19% up to 64x64 transform skip size for AI, RA, and LB, respectively. Especially, in Class F, up to 16x16 transform skip size provides bd-rate reduction by -2.15%, -2.36% and -2.53 % with 124%, 113% and 111% Encoding time for AI, RA and LB, respectively. The proponents of this contribution believe that the maximum size of transform skip should be 16x16. As conclusion, this contribution proposes to add the syntax and semantics to signal the maximum size of transform skip in the WD, and proposes to study further the appropriate maximum transform skip size by CE or AHG.

In v2, results are updated.

Introduction

Transform skip is introduced as an effective coding tool to preserve edges in screen contents and/or mixed contents. In HEVC [2], transform skip is available for 4x4 to 32x32 transform blocks. The size of transform skip is signaled by TransformSkipLog2MaxSize (log2_max_transform_skip_block_size_minus2) in PPS. On the other hand, in VVC WD [1] or JVET-L0111 [4], transform skip is allowed for the 2x4/4x2/4x4 transform blocks. The related syntax in WD is shown below (highlighted in yellow):

| if( transform_skip_enabled_flag && ( cIdx ! = 0 ¦ ¦ tu_mts_flag[ x0 ][ y0 ] = = 0 ) && ( log2TbWidth <= 2 ) && ( log2TbHeight <= 2 ) ) |  |
| --- | --- |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |

In VTM SW [3], the functionality of larger transform skip size exists in the source code, but assertion failed occurs when TransformSkipLog2MaxSize is set to a value larger than 2.

This contribution proposes to allow transform skip for larger transform size, and proposes to add signaling to indicate the maximum size of transform skip. The coding performance of larger transform skip size is also reported.

Proposal

Changes in VTM SW

In VTM SW, the following changes highlighted in yellow are proposed.

Note that signaling transform skip size (log2_max_transform_skip_block_size_minus2) has been implemented in VTM.

// VTM-3.0/source/App/EncoderApp/EncAppCfg.cpp#L2118

bool EncAppCfg::xCheckParameter()

{

…

xConfirmPara(m_log2MaxTransformSkipBlockSize>6, "Transform Skip Log2 Max Size must be less than or equal to 6.");

xConfirmPara(m_log2MaxTransformSkipBlockSize!=2, "Transform Skip Log2 Max Size must be 2 for V1 profiles.");

…

}

Changes in WD

In WD text, the following changes highlighted in yellow are proposed.

7.3.2.2	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| transform_skip_enabled_flag | u(1) |
| if( transform_skip_flag_enabled_flag) |  |
| log2_max_transform_skip_block_size_minus2 | ue(v) |
| … |  |
| } |  |

log2_max_transform_skip_block_size_minus2 plus 2 specifies the maximum transform block size for which transform_skip_flag may be present in coded pictures referring to the PPS. When not present, the value of log2_max_transform_skip_block_size_minus2 is inferred to be equal to 0. When present, the value of log2_max_transform_skip_block_size_minus2 shall be less than or equal to MaxTbLog2SizeY – 2.

The variable Log2MaxTransformSkipSize is derived as follows:

Log2MaxTransformSkipSize = log2_max_transform_skip_block_size_minus2 + 2	(X -XX)

7.3.4.12	Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && ( cIdx ! = 0 ¦ ¦ tu_mts_flag[ x0 ][ y0 ] = = 0 ) && ( log2TbWidth <= Log2MaxTransformSkipSize ) && ( log2TbHeight <= Log2MaxTransformSkipSize ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |

Simulation results

Following tests were carried out by using the VTM-3.0 software under the common test conditions (CTC) [5]:

Test#1: Transform Skip for up to 8x8 Transform blocks

Test#2: Transform Skip for up to 16x16 Transform blocks

Test#3: Transform Skip for up to 32x32 Transform blocks

Test#4: Transform Skip for up to 64x64 Transform blocks

Note that command options for transform skip size extension are used as below:

Encoder Option: --TransformSkipLog2MaxSize=N (N=3,4,5,6)

Table 3-1 shows the summary of coding performances for Class A1/A2/B/C/E. Table 3-2 shows the summary of the coding performances for Class F. Figure 3-1 shows trade-off of coding performance (BD%) and complexity (Enctime) for Class F in RA. Table 3-3 to 3-6 shows results of each tests, respectively.

As can be seen in Figure 3-1, larger transform skip size improves coding performances by up to 2.5% for class F. Especially, transform skip size for up to 16x16 TBs is reasonable setting in terms of trade-off between gain and complexity. The results show 2.2% gain with 113% Encoding time for class F in RA.

Detail results are available in the attached spreadsheets.

Table 3-1: Summary of coding performances for Class A1/A2/B/C/E (TBD)

| All Intra | Y | U | V | Enctime [%] | Dectime [%] |
| --- | --- | --- | --- | --- | --- |
| Test#1: TS for up to 8x8 TBs | -0.03% | -0.04% | -0.03% | 106% | 100% |
| Test#2: TS for up to 16x16 TBs | -0.03% | -0.04% | -0.05% | 120% | 100% |
| Test#3: TS for up to 32x32 TBs | -0.03% | -0.09% | -0.12% | 128% | 100% |
| Test#4: TS for up to 64x64 TBs | -0.03% | -0.08% | -0.16% | 130% | 100% |
| Random Access | Y | U | V | Enctime [%] | Dectime [%] |
| Test#1: TS for up to 8x8 TBs | -0.05% | 0.06% | 0.06% | 103% | 101% |
| Test#2: TS for up to 16x16 TBs | -0.09% | 0.09% | 0.07% | 110% | 100% |
| Test#3: TS for up to 32x32 TBs | -0.11% | 0.09% | 0.08% | 119% | 101% |
| Test#4: TS for up to 64x64 TBs | -0.10% | 0.13% | 0.03% | 126% | 100% |
| Low delay B | Y | U | V | Enctime [%] | Dectime [%] |
| Test#1: TS for up to 8x8 TBs | -0.07% | 0.26% | 0.04% | 103% | 100% |
| Test#2: TS for up to 16x16 TBs | -0.15% | 0.18% | 0.41% | 111% | 100% |
| Test#3: TS for up to 32x32 TBs | -0.21% | 0.42% | 0.03% | 120% | 100% |
| Test#4: TS for up to 64x64 TBs | -0.19% | 0.27% | -0.14% | 128% | 100% |

Table 3-2: Summary of coding performances for Class F

| All Intra | Y | U | V | Enctime [%] | Dectime [%] |
| --- | --- | --- | --- | --- | --- |
| Test#1: TS for up to 8x8 TBs | -1.88% | -1.55% | -1.54% | 116% | 99% |
| Test#2: TS for up to 16x16 TBs | -2.15% | -1.79% | -1.82% | 124% | 96% |
| Test#3: TS for up to 32x32 TBs | -2.14% | -1.79% | -1.98% | 131% | 96% |
| Test#4: TS for up to 64x64 TBs | -2.14% | -1.90% | -1.95% | 132% | 96% |
| Random Access | Y | U | V | Enctime [%] | Dectime [%] |
| Test#1: TS for up to 8x8 TBs | -1.83% | -1.31% | -1.48% | 106% | 100% |
| Test#2: TS for up to 16x16 TBs | -2.21% | -1.60% | -1.92% | 113% | 99% |
| Test#3: TS for up to 32x32 TBs | -2.36% | -1.80% | -2.11% | 123% | 99% |
| Test#4: TS for up to 64x64 TBs | -2.37% | -1.80% | -2.01% | 130% | 99% |
| Low delay B | Y | U | V | Enctime [%] | Dectime [%] |
| Test#1: TS for up to 8x8 TBs | -1.69% | -1.09% | -1.73% | 104% | 99% |
| Test#2: TS for up to 16x16 TBs | -2.53% | -1.64% | -2.38% | 111% | 97% |
| Test#3: TS for up to 32x32 TBs | -3.12% | -2.41% | -2.41% | 122% | 97% |
| Test#4: TS for up to 64x64 TBs | -3.20% | -2.01% | -2.53% | 133% | 98% |

Figure 3-1: Trade-off of BD% vs Enctime[%] of Class F in RA

Table 3-3: Results of Test#1: TS for up to 8x8 TBs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.03% | 0.10% | 103% | 99% |
| Class A2 | 0.03% | 0.03% | 0.00% | 106% | 101% |
| Class B | 0.00% | -0.02% | 0.00% | 106% | 100% |
| Class C | -0.12% | -0.14% | -0.26% | 109% | 99% |
| Class E | 0.01% | -0.02% | 0.05% | 105% | 99% |
| Overall | -0.03% | -0.04% | -0.03% | 106% | 100% |
| Class D | -0.11% | -0.17% | -0.17% | 110% | 98% |
| Class F | -1.88% | -1.55% | -1.54% | 116% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.23% | 0.21% | 102% | 100% |
| Class A2 | 0.03% | -0.08% | -0.03% | 103% | 101% |
| Class B | -0.04% | 0.16% | -0.05% | 103% | 101% |
| Class C | -0.18% | -0.09% | 0.16% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.06% | 0.06% | 103% | 101% |
| Class D | -0.34% | -0.43% | -0.10% | 105% | 100% |
| Class F | -1.83% | -1.31% | -1.48% | 106% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.29% | -0.22% | 103% | 101% |
| Class C | -0.21% | 0.10% | 0.34% | 104% | 99% |
| Class E | 0.06% | 1.38% | 0.09% | 100% | 100% |
| Overall | -0.07% | 0.26% | 0.04% | 103% | 100% |
| Class D | -0.38% | 0.11% | 0.26% | 105% | 99% |
| Class F | -1.69% | -1.09% | -1.73% | 104% | 99% |

Table 3-4: Results of Test#2: TS for up to 16x16 TBs (TBD)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.02% | 0.05% | 119% | 100% |
| Class A2 | 0.04% | -0.03% | -0.07% | 122% | 101% |
| Class B | 0.00% | 0.01% | 0.01% | 119% | 100% |
| Class C | -0.16% | -0.15% | -0.24% | 121% | 99% |
| Class E | 0.00% | -0.01% | 0.01% | 119% | 99% |
| Overall | -0.03% | -0.04% | -0.05% | 120% | 100% |
| Class D | -0.11% | -0.33% | -0.35% | 122% | 100% |
| Class F | -2.15% | -1.79% | -1.82% | 124% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.11% | 0.27% | 108% | 101% |
| Class A2 | 0.06% | 0.19% | 0.01% | 109% | 101% |
| Class B | -0.10% | 0.19% | -0.01% | 110% | 100% |
| Class C | -0.27% | -0.13% | 0.05% | 112% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.09% | 0.07% | 110% | 100% |
| Class D | -0.49% | -0.31% | -0.06% | 114% | 99% |
| Class F | -2.21% | -1.60% | -1.92% | 113% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.18% | 0.42% | 111% | 100% |
| Class C | -0.46% | 0.10% | 0.29% | 116% | 100% |
| Class E | 0.14% | 0.89% | 0.53% | 104% | 100% |
| Overall | -0.15% | 0.18% | 0.41% | 111% | 100% |
| Class D | -0.76% | -0.38% | 0.01% | 117% | 98% |
| Class F | -2.53% | -1.64% | -2.38% | 111% | 97% |

Table 3-5: Results of Test#3: TS for up to 32x32 TBs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | 0.08% | 129% | 99% |
| Class A2 | 0.05% | 0.00% | -0.16% | 132% | 101% |
| Class B | -0.01% | -0.04% | -0.06% | 127% | 100% |
| Class C | -0.17% | -0.24% | -0.31% | 128% | 100% |
| Class E | 0.02% | -0.13% | -0.13% | 126% | 98% |
| Overall | -0.03% | -0.09% | -0.12% | 128% | 100% |
| Class D | -0.12% | -0.25% | -0.43% | 127% | 100% |
| Class F | -2.14% | -1.79% | -1.98% | 131% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.04% | 0.38% | 117% | 100% |
| Class A2 | 0.08% | 0.08% | 0.04% | 118% | 101% |
| Class B | -0.13% | 0.27% | -0.04% | 119% | 101% |
| Class C | -0.33% | -0.11% | 0.02% | 122% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | 0.09% | 0.08% | 119% | 101% |
| Class D | -0.58% | -0.26% | 0.06% | 124% | 100% |
| Class F | -2.36% | -1.80% | -2.11% | 123% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | 0.18% | -0.14% | 123% | 101% |
| Class C | -0.59% | 0.25% | 0.17% | 124% | 98% |
| Class E | 0.13% | 1.05% | 0.15% | 111% | 99% |
| Overall | -0.21% | 0.42% | 0.03% | 120% | 100% |
| Class D | -0.90% | -0.36% | -0.24% | 126% | 97% |
| Class F | -3.12% | -2.41% | -2.41% | 122% | 97% |

Table 3-6: Results of Test#4: TS for up to 64x64 TBs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | 0.10% | 132% | 99% |
| Class A2 | 0.06% | 0.03% | -0.15% | 133% | 101% |
| Class B | 0.00% | -0.06% | -0.14% | 128% | 99% |
| Class C | -0.16% | -0.26% | -0.31% | 129% | 100% |
| Class E | 0.01% | -0.09% | -0.23% | 128% | 99% |
| Overall | -0.03% | -0.08% | -0.16% | 130% | 100% |
| Class D | -0.12% | -0.30% | -0.44% | 127% | 99% |
| Class F | -2.14% | -1.90% | -1.95% | 132% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.20% | 0.32% | 124% | 100% |
| Class A2 | 0.09% | 0.15% | -0.01% | 125% | 101% |
| Class B | -0.11% | 0.21% | -0.11% | 126% | 101% |
| Class C | -0.33% | -0.04% | 0.02% | 129% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | 0.13% | 0.03% | 126% | 100% |
| Class D | -0.63% | -0.31% | 0.13% | 130% | 100% |
| Class F | -2.37% | -1.80% | -2.01% | 130% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.14% | -0.05% | 129% | 100% |
| Class C | -0.54% | 0.32% | 0.11% | 130% | 98% |
| Class E | 0.15% | 0.88% | -0.63% | 122% | 100% |
| Overall | -0.19% | 0.27% | -0.14% | 128% | 100% |
| Class D | -0.89% | 0.02% | -0.08% | 132% | 97% |
| Class F | -3.20% | -2.01% | -2.53% | 133% | 98% |

Conclusion

This contribution proposes,

- To add the syntax and semantics to signal the maximum size of transform skip in the WD

- To set TransformSkipLog2MaxSize equal to 4 in CTC, which enables up to 16x16 transform skip size.

- To study further the appropriate maximum transform skip size by CE or AHG.

References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, Oct. 2018

[2] R. Joshi, S. Liu, G. Sullivan, G. Tech, Y.-K. Wang, J. Xu, Y. Ye, "HEVC Screen Content Coding Draft Text 6," JCTVC-W1005, Feb. 2016

[3] VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

[4] J. Park, B. Jeon, “CE6-related: Transform Skip Condition on Transform Block size,” JVET-L0111, Oct. 2018

[5] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Oct. 2018

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).