| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0076 |
| --- | --- |

| Title: | CE9: Block size restriction for DMVR (test 9.2.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

In this contribution, it is tested that coding performances with block size restriction for decoder-side motion vector refinement (DMVR). Threshold 4096 pixels for block size restriction is used in proposed method. On the other hand, threshold 1024 pixels is used in CE9.2 base software (same as CE9.2.1a). BD-rate for luma is -0.89% with threshold 4096 compared with VTM-3.0.

Introduction

Core Experiment 9 (CE9) related to decoder-side motion vector derivation is established in 12th meeting at Macao on October, 2018 [1]. This contribution presents a proposal of block size restriction for decoder-side motion vector refinement (DMVR). The proposal is related to CE9.2. The remainder of this contribution is divided into two parts;

- Proposed method, and

- Experimental results under the CTC.

Proposed Method

In 12th meeting at Macao, KDDI and MediaTek proposed block size restriction for DMVR. The threshold for block size restriction was 4096 pixels [2] or 64x64 [3]. On the other hand, the threshold adopted to the CE9.2 base software was 1024 pixels. Therefor, in this contribution, coding performances with block size restrictions for DMVR are tested. The test conditions are described in the following table, consisting of the number of samples threshold TH. The threshold value 4096 pixels is set to meet VPDU constraints.

Table 1 Test conditions for block size thresholds.

| Test | TH |
| --- | --- |
| CE9.2.6 | 4096 |
| CE9.2 base software (CE9.2.1a) | 1024 |

Each threshold is used like following code.

bool PU::checkDMVRCondition(const PredictionUnit& pu)

{

return pu.mergeFlag

&& pu.mergeType == MRG_TYPE_DEFAULT_N

&& !pu.cu->affine

#if !ENABLE_DMVR_FOR_MMVD

&& !pu.mmvdMergeFlag

&& !pu.cu->mmvdSkip

#endif

&& PU::isBiPredFromDifferentDirEqDistPoc(pu)

#if DMVR_CU_RESTRICTION

&& (pu.lheight() >= 8)

&& ((pu.lheight() * pu.lwidth()) >= 64)

&& ((pu.lheight() * pu.lwidth()) <= TH)

#endif

;

}

Experimental Results

The proposed method is implemented on top of the CE9.2 base software. In order to verify the performance of the proposed method, we have run simulations according to the common test conditions of JVET-L1010 [4].

Results under the common test conditions

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Test results

Table 2 shows the experimental results with threshold 4096. It can be seen that BD-rate for luma is -0.89% for RA in comparison with VTM-3.0. BD-rate for luma of the CE9.2 base software (threshold 1024) is -0.71% [5]. Therefore, BD-rates gain is -0.18% with threshold 4096 compared with in case of using threshold 1024.

Table 2 Experimental results of TH=4096 case (CE9.2.6) for RA condition compared with VTM-3.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.93% | -0.89% | -1.19% | 109% | 115% |
| Class A2 | -1.35% | -1.47% | -1.48% | 111% | 120% |
| Class B | -0.67% | -0.79% | -0.88% | 111% | 119% |
| Class C | -0.80% | -0.93% | -0.95% | 111% | 122% |
| Class E |  |  |  |  |  |
| Overall | -0.89% | -0.99% | -1.08% | 111% | 119% |
| Class D | -0.72% | -0.97% | -0.96% | 113% | 124% |
| Class F | -0.35% | -0.46% | -0.42% | 109% | 120% |

Conclusion

This contribution presented coding performances with block size restriction. Threshold 4096 pixels for block size restriction was used in proposed method. On the other hand, threshold 1024 pixels was used in CE9.2 base software (same as CE9.2.1a). BD-rates for luma were -0.89% with threshold 4096 and -0.71% with threshold 1024 compared with VTM-3.0. Therefore, BD-rate gain was -0.18% with threshold 4096 compared with in case of using threshold 1024.

- Acknowledgment

We would like to thank Sharp for doing crosschecking of our proposal.

References

- X. Xiu and S. Esenlik, “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, JVET-L1029, Macao, CN, Oct. 2018.

- K. Unno, K. Kawamura, and S. Naito, “CE9-related : DMVR with Coarse-to-Fine Search and Block Size Limit”, JVET-L0382, Macao, CN, Oct. 2018.

- C. –C. Chen, C. –W. Hsu, Y. –W. Huang and S. –M. Lei, “CE9-related: Simplified DMVR with reduced internal memory”, JVET-L0098, Macao, CN, Oct. 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-L1010, Macao, CN, Oct. 2018.

- S. Sethuraman, “CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2”, JVET-M0147, Marrakech, MA, Jan. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).