| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0077 |
| --- | --- |

| Title: | CE9-related: Relaxation of block size restriction for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution presents relaxation method of block size restriction for decoder-side motion vector refinement (DMVR). The CE9.2 base software (same as CE9.2.1a) restricts DMVR for blocks larger than 1024 pixels. In this contribution, it is proposed that only central 1024 pixels region is used for DMVR in case of block size is larger than 1024 pixels. And it is also proposed that DMVR is restricted in case of block size is larger than 4096 pixels to meet VPDU constraints (same as CE9.2.6). BD-rate for luma is 0.81% compared with VTM-3.0.

Introduction

Core Experiment 9 (CE9) related to decoder-side motion vector derivation is established in 12th meeting at Macao on October, 2018 [1]. This contribution presents a proposal of block size restriction for decoder-side motion vector refinement (DMVR). The proposal is related to CE9.2. The remainder of this contribution is divided into two parts;

- Proposed method, and

- Experimental results under the CTC.

Proposed Method

In the 12th meeting at Macao, it was adopted that block size restriction for DMVR with threshold 1024 pixels. And different threshold (4096 pixels) is proposed in CE9.2.6 [2]. It is reported that BD-rate gains -0.18% compared with CE9.2 base software (same as CE9.2.1a) by using larger threshold in [2]. Larger threshold is enable to get more BD-rate gains but needs more memories for reference pixels and larger circuits for calculation of MRSAD. In this contribution, it is proposed that only 1024 pixels region is used for DMVR at a large block. The proposed method enables to use larger threshold without increasing the worst case of memories and circuits.

Figure 1 shows processing flow of the proposed method. As shown in Fig, proposed method has two features.

- Restrict DMVR at blocks larger than 4096 pixels (same as CE9.2.6) to meet VPDU constraints.

- Only central 1024 pixels region is used for DMVR at blocks larger than 1024 pixels.

Figure 1 Flow chart of proposed method.

The “central 1024 pixels region” in (b) is decided as following pseudo code.

validWidth = puWidth;		// validWidth, validHeight : width and height of the region for DMVR

validHeight = puHeight;		// puWidth, puHeight : original block size

offsetX = 0;					// offsetX, offsetY : left upper point of the region for DMVR

offsetY = 0;

while (validWidth*validHeight > 1024)

{

if (validWidth > validHeight)

{

validWidth >>= 1;

offsetX += (validWidth >> 1);

}

else

{

validHeight >>= 1;

offsetY += (validHeight >> 1);

}

}

Some examples of the central regions are shown in Figure 2.

Figure 2 Examples of the central regions for DMVR.

When calculating MRSAD, only even line pixels are used in the CE9.2 base software and the proposed method is implemented on top of the CE9.2 base software. Therefore, only 512 pixels (even line pixels in the central region) are used to calculate MRSAD in the proposed method.

Experimental Results

The proposed method is implemented on top of the CE9.2 base software. In order to verify the performance of the proposed method, we have run simulations according to the common test conditions of JVET-L1010 [4].

Results under the common test conditions

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Test results

Table 2 shows the experimental results with threshold 4096. It can be seen that BD-rate for luma is -0.81% for RA in comparison with VTM-3.0. BD-rate for luma of the CE9.2 base software is -0.71% [4]. Therefore, the proposed method has -0.10% BD-rate gain compared with the CE9.2 base software.

Table 2 Experimental results for RA condition compared with VTM-3.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.79% | -0.78% | -1.02% | 109% | 115% |
| Class A2 | -1.18% | -1.31% | -1.33% | 112% | 120% |
| Class B | -0.62% | -0.77% | -0.84% | 111% | 119% |
| Class C | -0.78% | -0.86% | -0.91% | 111% | 122% |
| Class E |  |  |  |  |  |
| Overall | -0.81% | -0.90% | -1.00% | 111% | 119% |
| Class D | -0.72% | -0.92% | -0.91% | 113% | 124% |
| Class F | -0.32% | -0.48% | -0.41% | 109% | 120% |

Conclusion

This contribution presented relaxation method of block size restriction for decoder-side motion vector refinement (DMVR). The CE9.2 base software (same as CE9.2.1a) restricted DMVR for blocks larger than 1024 pixels. In this contribution, it was proposed that only central 1024 pixels region was used for DMVR in case of block size was larger than 1024 pixels. And it is also proposed that DMVR was restricted in case of block size was larger than 4096 pixels (same as CE9.2.6). BD-rate gain for luma was -0.81% compared with VTM-3.0. On the other hand, BD-rate gain for luma of the CE9.2 base software (same as CE9.2.1a) was -0.71%. Therefore, the proposed method had -0.10% BD-rate gain compared with the CE9.2 base software without increasing the worst case of memories and circuits.

- Acknowledgment

We would like to thank Sharp for doing crosschecking of our proposal.

References

- X. Xiu and S. Esenlik, “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, JVET-L1029, Macao, CN, Oct. 2018.

- K. Unno, K. Kawamura, and S. Naito, “CE9: Block size restriction for DMVR (test 9.2.6)”, JVET-M0076, Marrakech, MA, Jan. 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-L1010, Macao, CN, Oct. 2018.

- S. Sethuraman, “CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2”, JVET-M0147, Marrakech, MA, Jan. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).