| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET- M0085-v2 |
| --- | --- |

| Title: | CE6-related: Fast algorithm for DST-4/DCT-4 as alternative transforms for MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tadamasa Toma, Kiyofumi Abe (Panasonic) Masaru Ikeda, Takeshi Tsukuba (Sony) | Tel: Email: | toma.tadamasa@jp.panasonic.com abe.kiyo@jp.panasonic.com masaru.Ikeda@sony.com takeshi.tsukuba@sony.com |
| Source: | Panasonic, Sony |  |  |

_____________________________

- Abstract

A new set of MTS transforms, i.e. replacing DST-7 and DCT-8 by DST-4 and DCT-4, was proposed in JVET-L0262 [5] in Macao meeting. Since fast algorithm for DCT-2 is widely known, it was discussed in the Macao meeting if fast algorithm is also available for non-DCT-2 transform basis functions that are used in MTS. This contribution proposes fast algorithm for DST-4 and DCT-4. By utilizing symmetric property of matrix coefficient, the number of multiplication operations is reduced to 75 % as compared to matrix multiplication.

Also, following additional evaluations related to JVET-L0262 are conducted and these results are provided. It is generally observed that BD-rate loss of DST-4 / DCT-4 approach compared to DST-7 / DCT-8 gets smaller if the maximum MTS size is restricted.

- Combination with CE6-3.2b and CE6-3.2c: Restrict maximum MTS size

- Combination with CE6-4.1d: Sub-block Transform (SBT) for inter blocks

- Combination with JVET-M0046 [6]: Keep non-zero coefficients only in the top-left 16x16 region

In v2, simulation results of the additional results in section 4 are updated and summary of the additional results is added.

Fast algorithm for DST-4 / DCT-4

Fast algorithm is proposed for DST-4 and DCT-4 that utilizes symmetric property of matrix coefficients. The number of multiplications can be reduced to 75%. Since DST-4 can be derived by flipping and sign changing of DCT-4, the same algorithm can be shared between DST-4 and DCT-4. In this section, the fast algorithm is described taking DCT-4 as an example.

Figure 1 shows symmetric property of DCT-4’s coefficients. The coefficients can be decomposed into 2x2 matrices that have symmetric property and the first algorithm can be obtained based on this symmetry. 16-points and 32- points DCT-4 have the same property as well.

Figure 1  Symmetric property of DCT-4’s coefficients

There are eight types of 2x2 matrices as shown in Figure 2 and DCT-4 matrix can be decomposed using these 2x2 matrices.

Figure 2  Eight types of 2x2 symmetric matrices

An example of fast algorithm for matrix multiplication is described below using 2x2 matrix of Type 1 in Figure 2. By introducing intermediate variable t, total number of multiplications can be reduced from four to three.

t=b*x0+x1                              (Equation 2-1)

y0=t-x0* b-a =a*x0+ b*x1           (Equation 2-2)

y1=t-x1* b+a =b*x0- a*x1           (Equation 2-3)

Table 1 shows the number of multiplication and adding operations that is needed for the fast algorithm as compared to the matrix multiplication. The number of multiplication operations is reduced to 75%.

Table 1  Number of multiplication and adding operations for matrix multiplication and fast algorithm

|  | Matrix multiplication |  | Fast algorithm |  |
| --- | --- | --- | --- | --- |
| N-points | Mul | Add | Mul | Add |
| 4 | 16 | 12 | 12 | 14 |
| 8 | 64 | 56 | 48 | 60 |
| 16 | 256 | 240 | 192 | 248 |
| 32 | 1024 | 992 | 768 | 1008 |

Experimental Results

The proposed fast algorithm was implemented in VTM 3.0 and tests were conducted using the CTC with inter MTS enabled.

CTC with inter MTS enabled
(Anchor: MTS with DST-4/DCT-4 on VTM 3.0)

The following tables summarize the EncT and DecT reduction resulting from the fast algorithm. Here, anchor is the modified VTM 3.0 that replaces MTS transform basis functions to DST-4 and DCT-4. DecT is reduced by 5 % in AI, 2 % in RA and 3 % in LDB, respectively. Since the fast algorithm does not affect the transform results, BD-rate is the same.

Table 2  BD-rate results of All Intra

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | 0.00% | 0.00% | 96% | 92% |
| Class A2 | 0.00% | 0.00% | 0.00% | 97% | 94% |
| Class B | 0.00% | 0.00% | 0.00% | 97% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 95% |
| Class E | 0.00% | 0.00% | 0.00% | 97% | 95% |
| Overall | 0.00% | 0.00% | 0.00% | 97% | 95% |
| Class D | 0.00% | 0.00% | 0.00% | 97% | 91% |
| Class F | 0.00% | 0.00% | 0.00% | 97% | 100% |

Table 3  BD-rate results of Random Access

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 96% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 99% |

Table 4  BD-rate results of Low Delay B

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 102% |

CTC with inter MTS enabled (Anchor: VTM 3.0)

The following tables summarize the EncT and DecT reduction resulting from the fast algorithm. Anchor is VTM 3.0. DecT is reduced by 7 % in AI, 2 % in RA and 3 % in LDB, respectively.

Table 5  BD-rate results of All Intra

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.66% | 0.55% | 0.71% | 101% | 89% |
| Class A2 | 0.50% | 0.23% | 0.21% | 99% | 91% |
| Class B | 0.33% | -0.01% | 0.04% | 100% | 95% |
| Class C | -0.24% | -0.27% | -0.32% | 100% | 95% |
| Class E | -0.01% | -0.24% | -0.17% | 100% | 94% |
| Overall | 0.23% | 0.03% | 0.07% | 100% | 93% |
| Class D | -0.27% | -0.42% | -0.35% | 100% | 95% |
| Class F | -0.12% | -0.21% | -0.04% | 100% | 99% |

Table 6  BD-rate results of Random Access

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.32% | 0.27% | 0.44% | 101% | 98% |
| Class A2 | 0.29% | 0.24% | 0.16% | 100% | 99% |
| Class B | 0.18% | 0.14% | 0.01% | 100% | 98% |
| Class C | -0.18% | 0.01% | -0.05% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | 0.15% | 0.11% | 100% | 98% |
| Class D | -0.24% | -0.48% | -0.36% | 100% | 96% |
| Class F | -0.03% | -0.12% | -0.05% | 100% | 99% |

Table 7  BD-rate results of Low Delay B

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class B | -0.06% | -0.02% | -0.49% | 100% | 97% |
| Class C | -0.29% | -0.45% | -0.09% | 100% | 97% |
| Class E | -0.14% | -0.19% | -1.03% | 100% | 99% |
| Overall | -0.16% | -0.20% | -0.49% | 100% | 97% |
| Class D | -0.29% | -0.68% | -1.02% | 100% | 97% |
| Class F | -0.39% | -0.13% | -0.78% | 100% | 101% |

Conclusion

This contribution proposes the fast algorithm for DST-4 and DCT-4. The proposed algorithm reduces the number of multiplication operations to 75 %. The algorithm is implemented in VTM 3.0 by replacing transform basis functions of MTS by DST-4 / DCT-4 and evaluation was conducted in CTC with inter MTS enabled. The results reportedly show that DecT is reduced by 5 % in AI, 2 % in RA and 3 % in LDB, respectively as compared to the modified VTM 3.0 that uses DST-4 / DCT-4 for MTS without the fast algorithm.

Additional results

Additional evaluations related to DST-4 / DCT-4 approach are conducted. The results of restricting maximum MTS size with DST-4 / DCT-4 and replacing DST-7 / DCT-8 by DST-4 / DCT-4 in JVET-M0046 [6] are provided. In these evaluations, the fast algorithm for DST-4 and DCT-4 is not enabled.

Combination with CE 6-3.2b

CE6-3.2b incorporates DST-7 / DCT-8 for shorter side of Nx32 and 32xN for N<32 with max. MTS size equals to 16. The combination of DST-4 / DCT-4 with CE6-3.2b is evaluated by replacing DST-7 / DCT-8 with DST-4 / DCT-4, respectively. Experimental results with inter MTS enabled under CTC are shown as follows. Anchor is VTM 3.0 with inter MTS enabled.

Table 8  BD-rate results of All Intra

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 1.11% | 1.47% | 1.54% | 92% | 86% |
| Class A2 | 0.67% | 0.53% | 0.48% | 94% | 91% |
| Class B | 0.53% | 0.31% | 0.41% | 95% | 91% |
| Class C | -0.14% | -0.27% | -0.23% | 96% | 97% |
| Class E | 0.16% | 0.04% | 0.14% | 95% | 93% |
| Overall | 0.44% | 0.37% | 0.42% | 94% | 91% |
| Class D | -0.21% | -0.39% | -0.34% | 97% | 98% |
| Class F | -0.07% | -0.11% | 0.02% | 96% | 99% |

Table 9  BD-rate results of Random Access

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.68% | 0.89% | 0.92% | 92% | 98% |
| Class A2 | 0.50% | 0.35% | 0.27% | 93% | 99% |
| Class B | 0.41% | 0.40% | 0.28% | 92% | 97% |
| Class C | -0.08% | 0.07% | 0.02% | 92% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.35% | 0.40% | 0.34% | 92% | 98% |
| Class D | -0.22% | -0.41% | -0.47% | 94% | 98% |
| Class F | 0.01% | -0.01% | -0.02% | 93% | 99% |

Table 10  BD-rate results of Low Delay B

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class B | 0.26% | 0.20% | -0.59% | 90% | 96% |
| Class C | -0.12% | -0.22% | -0.26% | 91% | 99% |
| Class E | 0.05% | 0.01% | -0.80% | 92% | 97% |
| Overall | 0.08% | 0.01% | -0.54% | 91% | 97% |
| Class D | -0.21% | -0.08% | -0.53% | 94% | 98% |
| Class F | -0.21% | -0.21% | -0.71% | 92% | 99% |

Combination with CE 6-3.2c

There is another CE related to CE6-3.2b, which restricts max MTS size equals to 16. Combination with CE6-3.2b is also evaluated by replacing DST-7 and DCT-8 with DST-4 and DCT-4, respectively. Experimental results with inter MTS enabled under CTC are shown as follows. Anchor is VTM 3.0 with inter MTS enabled.

Table 11  BD-rate results of All Intra

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 1.43% | 1.73% | 1.88% | 85% | 86% |
| Class A2 | 0.89% | 0.72% | 0.65% | 88% | 91% |
| Class B | 0.75% | 0.48% | 0.57% | 89% | 91% |
| Class C | -0.07% | -0.10% | -0.28% | 91% | 98% |
| Class E | 0.37% | 0.13% | 0.33% | 89% | 93% |
| Overall | 0.64% | 0.54% | 0.57% | 89% | 92% |
| Class D | -0.18% | -0.51% | -0.21% | 93% | 98% |
| Class F | 0.00% | -0.08% | -0.03% | 92% | 96% |

Table 12  BD-rate results of Random Access

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.87% | 0.89% | 1.14% | 86% | 97% |
| Class A2 | 0.63% | 0.36% | 0.34% | 87% | 98% |
| Class B | 0.61% | 0.54% | 0.51% | 87% | 98% |
| Class C | -0.04% | 0.01% | 0.04% | 87% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.49% | 0.43% | 0.48% | 87% | 98% |
| Class D | -0.17% | -0.36% | -0.38% | 89% | 97% |
| Class F | 0.08% | 0.04% | -0.01% | 89% | 99% |

Table 13  BD-rate results of Low Delay B

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class B | 0.45% | 0.16% | -0.22% | 83% | 96% |
| Class C | -0.09% | -0.34% | -0.23% | 85% | 97% |
| Class E | 0.17% | 0.68% | -0.20% | 88% | 97% |
| Overall | 0.20% | 0.12% | -0.22% | 85% | 97% |
| Class D | -0.10% | -0.46% | -0.92% | 89% | 98% |
| Class F | -0.20% | -0.55% | -0.50% | 86% | 99% |

Combination with CE 6-4.1d

CE6-4.1d is a sub-block transform for inter blocks in which transform type of residual TU is signaled with four transform candidates per TU. The combination of DST-4 / DCT-4 with CE6-4.1d is evaluated with the maximum MTS size which is restricted to 16. Experimental results with inter MTS enabled under CTC are shown as follows. Anchor is VTM 3.0 with inter MTS enabled.

Table 14  BD-rate results of All Intra

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 1.43% | 1.73% | 1.88% | 85% | 81% |
| Class A2 | 0.89% | 0.72% | 0.65% | 88% | 86% |
| Class B | 0.75% | 0.48% | 0.57% | 88% | 88% |
| Class C | -0.07% | -0.10% | -0.28% | 91% | 97% |
| Class E | 0.37% | 0.13% | 0.33% | 89% | 90% |
| Overall | 0.64% | 0.54% | 0.57% | 89% | 89% |
| Class D | -0.18% | -0.51% | -0.21% | 93% | 99% |
| Class F | 0.00% | -0.08% | -0.03% | 91% | 96% |

Table 15  BD-rate results of Random Access

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.72% | 0.75% | 1.09% | 89% | 95% |
| Class A2 | 0.20% | 0.10% | 0.11% | 95% | 97% |
| Class B | 0.19% | 0.36% | 0.70% | 93% | 96% |
| Class C | -0.42% | -0.07% | 0.07% | 92% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | 0.27% | 0.49% | 92% | 96% |
| Class D | -0.67% | -0.33% | -0.32% | 96% | 101% |
| Class F | -0.20% | -0.11% | -0.14% | 92% | 99% |

Table 16  BD-rate results of Low Delay B

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class B | -0.25% | -0.89% | 0.07% | 94% | 94% |
| Class C | -0.84% | -1.25% | -0.41% | 93% | 95% |
| Class E | -0.31% | -0.14% | -0.50% | 93% | 96% |
| Overall | -0.46% | -0.82% | -0.23% | 93% | 95% |
| Class D | -0.96% | -1.44% | -0.42% | 100% | 99% |
| Class F | -1.32% | -0.82% | -1.49% | 91% | 98% |

Combination with JVET-M0046 [6]

New complexity reduction method of MTS is proposed in JVET-M0046 [6]. In this proposal, non-zero coefficients are kept only in the top-left 16x16 region and the rest of coefficients are zero-out in a 32x32/32x16 or 16x32 DST-7/DCT-8 based transform unit. Combination with JVET-M0046 is also evaluated by replacing DST-7 and DCT-8 with DST-4 and DCT-4, respectively. Anchor is VTM 3.0 with inter MTS enabled.

Table 17  BD-rate results of All Intra

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.74% | 0.48% | 0.71% | 98% | 89% |
| Class A2 | 0.60% | 0.22% | 0.23% | 98% | 92% |
| Class B | 0.41% | 0.01% | 0.05% | 98% | 92% |
| Class C | -0.18% | -0.21% | -0.25% | 99% | 98% |
| Class E | 0.08% | -0.22% | 0.00% | 99% | 94% |
| Overall | 0.31% | 0.04% | 0.12% | 98% | 93% |
| Class D | -0.22% | -0.42% | -0.16% | 98% | 98% |
| Class F | -0.11% | -0.15% | -0.07% | 98% | 97% |

Table 18  BD-rate results of Random Access

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.36% | 0.28% | 0.29% | 96% | 98% |
| Class A2 | 0.29% | 0.04% | 0.05% | 96% | 99% |
| Class B | 0.26% | 0.04% | 0.26% | 96% | 98% |
| Class C | -0.13% | -0.04% | 0.01% | 96% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.18% | 0.07% | 0.16% | 96% | 98% |
| Class D | -0.17% | -0.37% | -0.46% | 97% | 99% |
| Class F | -0.05% | -0.11% | 0.06% | 97% | 100% |

Table 19  BD-rate results of Low Delay B

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class B | 0.05% | -0.07% | -0.33% | 96% | 97% |
| Class C | -0.18% | -0.21% | 0.30% | 95% | 97% |
| Class E | 0.10% | 0.75% | -0.45% | 96% | 98% |
| Overall | -0.01% | 0.09% | -0.15% | 96% | 97% |
| Class D | -0.17% | -0.18% | -0.33% | 97% | 98% |
| Class F | -0.18% | -0.03% | -0.49% | 96% | 99% |

Summary of additional results

Table 20 summarizes the additional results of the combination tests in terms of BD-rate together with the results that are used as reference methods in each combination test. Anchor is all VTM 3.0 with inter-MTS enabled. It is generally observed that BD-rate loss of DST-4 / DCT-4 approach compared to DST-7 / DCT-8 gets smaller if the maximum MTS size is restricted. Note that the maximum MTS size is additionally restricted to 16 in No. 4 that is the combination with CE 6-4.1d.

Table 20  Summary of additional results

|  |  |  | AI, w/ InterMTS |  |  | RA, w/ InterMTS |  |  | LDB, w/ InterMTS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Doc. | Test | Y | U | V | Y | U | V | Y | U | V |
| 1 | JVET-M0085 | Sec. 2.2 | 0.23% | 0.03% | 0.07% | 0.13% | 0.15% | 0.11% | -0.16% | -0.20% | -0.49% |
| 2 | JVET-M0085 | Sec. 4.1 | 0.44% | 0.37% | 0.42% | 0.35% | 0.40% | 0.34% | 0.08% | 0.01% | -0.54% |
| 3 | JVET-M0085 | Sec. 4.2 | 0.64% | 0.54% | 0.57% | 0.49% | 0.43% | 0.48% | 0.20% | 0.12% | -0.22% |
| 4 | JVET-M0085 | Sec. 4.3 | 0.64% | 0.54% | 0.57% | 0.13% | 0.27% | 0.49% | -0.46% | -0.82% | -0.23% |
| 5 | JVET-M0085 | Sec. 4.4 | 0.31% | 0.04% | 0.12% | 0.18% | 0.07% | 0.16% | -0.17% | -0.18% | -0.33% |
| 6 | JVET-M0522 [7] | CE6-3.2b | 0.33% | 0.40% | 0.38% | 0.28% | 0.30% | 0.38% | 0.26% | 0.20% | -0.01% |
| 7 | JVET-M0522 | CE6-3.2c | 0.55% | 0.62% | 0.63% | 0.45% | 0.43% | 0.43% | 0.35% | 0.15% | -0.10% |
| 8 | JVET-M0140 [8] | CE6-4.1d |  |  |  | -0.29% | -0.14% | 0.03% | -0.47% | -0.64% | -0.06% |
| 9 | JVET-M0046 [6] | Test 5 | 0.12% | 0.05% | 0.02% | 0.05% | 0.01% | 0.10% | 0.14% | 0.10% | -0.05% |

Reference

- K. Abe, T. Toma, “CE6-related: Type4 only AMT” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-K0130.

- K. Naser, F. Le Léannec, E. François, “CE6-related: Type4 only AMT” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-K0265.

- T. Tsukuba, M. Ikeda, T. Suzuki, “CE6-related: AMT with only Type2/Type4 DCT/DST” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-K0394

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1002

- K. Abe, T. Toma, M. Ikeda, T. Tsukuba, K. Naser, F. L. Leannec, E. Francois, “CE 6-1.7a: MTS simplification by reusing DCT-2 partial butterfly - Change MTS transform to DST-4/DCT-4” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-L0262.

- M. Zhou and Y. Hu, “CE6-related: A study of primary transforms” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-M0046.

- H. E, Egilmez, V. Seregin, A. Said, M. Karczewicz, “CE6: MTS support for large rectangular blocks (Test 6.3.2)” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-M0522.

- Y. Zhao, H. Gao, H. Yang, J. Chen, “CE6: Sub-block transform for inter blocks (Test 6.4.1)” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11, JVET-M0140.

Patent rights declarations

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).