| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0099-v2 |
| --- | --- |

| Title: | Non-CE3: Partial sorting for non-MPM modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Geert Van der Auwera Ted Hsieh Vadim Seregin Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Two modifications to the non-MPM mode derivation and signalling are proposed. Firstly, it is proposed to change the signalling of non-MPM modes from truncated binary to 6-bit fixed length. The second modification involves replacing the 6 MPM sorting algorithm currently used with a partial sorting algorithm. It is asserted that the proposed partial sorting method simplifies the non-MPM mode derivation by reducing the number of operations (conditions, comparisons, swaps, and increments/additions) as follows: from (11, 11, 6, 4) to (1, 1, 0, 1) in the worst case. The BD-rate for the proposed method is (0.04%, 0.01%, 0.02%) for (Y, Cb, Cr) with runtimes of 98% EncT and 97% DecT for the AI configuration and (0.00%, 0.09%, 0.02%) for (Y, Cb, Cr) with runtimes of 99% EncT and 98% DecT for the RA configuration. It is asserted that the loss observed is only attributable to the change of truncated binary to fixed length coding.

In version 2 (v2) of this document, a small typo is corrected in the Section 4 (Changes to the Specification).

# Introduction

The intra mode used to code luma blocks are indicated by either the MPM index or as a non-MPM mode. For reference lines with line index (MRL index) greater than 0, only the MPM (using a different derivation) are used for signalling the intra mode. This proposal pertains to the non-MPM mode signalling when MRL index is equal to 0.

In VTM3, when an intra mode is signalled as a non-MPM mode, the MPM list is sorted and the remaining entries are re-indexed such that the non-MPM modes may be signalled with an index in the range of 0 to 60, inclusive. The truncated binary code is used for this syntax element.

The sorting operation and the subsequent re-indexing are performed both at the encoder and the decoder and comprise of several conditions, comparisons, swaps and increments. For an MPM list of size 6, 10 – 12 conditional checks (if condition with comparison) must be performed for sorting the list, and in the worst case as many swaps. Even taking into account the fact that the PL and DC modes are always present in the MPM list, six conditional checks must be performed for sorting the list. Subsequently, the re-indexing involves “re-numbering” the modes from 0 to 66, inclusive, to 0 to 60, inclusive. In the worst case, this involves at most four conditional checks and four increment operations at the decoder for each coding block. Finally, the truncated binary decoding also involves a conditional check in parsing the codeword.

This document proposes to simplify the non-MPM mode signalling with two simplifications: replacing the truncated binary coding to use 6-bit fixed-length coding for the non-MPM mode, and using a partial sorting algorithm by just identifying one mode (neither PL nor DC) to enable re-indexing without full sorting.

# Proposal

The proposal involves two simplifications to the non-MPM mode derivation and signalling:

- Change the binarization of the non-MPM modes from truncated binary to 6-bit fixed-length signalling.

- Remove the 6 MPM sorting mechanism to re-index the non-MPM modes. Instead, within the existing MPM derivation, one mode (M) that is neither PL nor DC is identified. The choice of M is integrated in the derivation of the MPM, and hence no additional conditional checks are needed. The modes PL, DC and M are used to re-index the non-MPM modes with a simple comparison and addition. Details of the partial sorting mechanism are given in Section 4. The re-indexed non-MPM mode is in the range of 0 to 63, inclusive, and is signalled using a 6-bit fixed-length codeword. It may be noted since full sorting is not performed, there would be three modes which could be signalled either as an MPM or as a non-MPM mode.

Following is an analysis of the complexity of the proposed method compared to the anchor VTM3:

Table 1: Analysis of worst-complexity of non-MPM mode derivation for anchor (VTM3) and the proposed method.

| Step | Operations | Anchor (VTM3) | Proposed |
| --- | --- | --- | --- |
| Sorting (four non-PL non-DC modes) | If conditions | 6 | 0 |
|  | Comparisons | 6 | 0 |
|  | Swaps | 6 | 0 |
| Re-indexing (worst case) | If conditions | 4 | 1 |
|  | Comparisons | 4 | 1 |
|  | Increments/additions | 4 | 1 |
| Coding | If conditions | 1 | 0 |
|  | Comparisons | 1 | 0 |
| Total | If conditions | 11 | 1 |
|  | Comparisons | 11 | 1 |
|  | Swaps | 6 | 0 |
|  | Increments/additions | 4 | 1 |

# Results

The results of the proposed method on top of VTM3 are shown in Table 1. The BD-rate for the proposed method is (0.04%, 0.01%, 0.02%) for (Y, Cb, Cr) with runtimes of 98% EncT and 97% DecT for the AI configuration and (0.00%, 0.09%, 0.02%) for (Y, Cb, Cr) with runtimes of 99% EncT and 98% DecT for the RA configuration. The minor loss observed is attributable to the change of truncated binary to fixed length coding.

Table 2: Results for partial sorting for non-MPM modes with fixed length coding

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.05% | 0.08% | 98% | 100% |
| Class A2 | 0.03% | 0.05% | 0.01% | 98% | 98% |
| Class B | 0.03% | -0.01% | -0.01% | 98% | 96% |
| Class C | 0.03% | -0.01% | -0.04% | 99% | 98% |
| Class E | 0.05% | -0.03% | 0.08% | 98% | 94% |
| Overall | 0.04% | 0.01% | 0.02% | 98% | 97% |
| Class D | 0.04% | 0.06% | 0.05% | 98% | 97% |
| Class F | 0.07% | -0.03% | 0.08% | 99% | 94% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.21% | 97% | 99% |
| Class A2 | 0.00% | 0.04% | 0.07% | 98% | 97% |
| Class B | -0.01% | 0.23% | -0.14% | 99% | 98% |
| Class C | 0.02% | 0.01% | 0.03% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.09% | 0.02% | 99% | 98% |
| Class D | -0.01% | -0.47% | 0.12% | 100% | 99% |
| Class F | -0.01% | 0.14% | 0.10% | 99% | 100% |

# Changes to specification text

The following changes are made to the intra mode derivation, in Section 8.2.2 of JVET-L1001-v4, with additions highlighted in yellow and removals struck-through.

### Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-1 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

NOTE – : The intra prediction modes INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM are only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb + cbWidth − 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA.

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..5 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-4)

candModeList[ 1 ] = INTRA_PLANAR	(8-5)

candModeList[ 2 ] = INTRA_DC		(8-6)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-7)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-8)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-9)

maxModeVal = candIntraPredModeA

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-10)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-11)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-12)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-13)

candModeList[ 4 ] = 2 + ( candIntraPredModeA % 64 )	(8-14)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 59 ) % 64 )	(8-15)

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-16)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-17)

maxModeVal = maxAB

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-18)

candModeList[ 1 ] = candIntraPredModeB	(8-19)

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 2 ] = INTRA_PLANAR	(8-20)

candModeList[ 3 ] = INTRA_DC		(8-21)

  - If maxAB − minAB is in the range of 2 to 62, inclusive, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-22)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-23)

  - Otherwise, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-24)

candModeList[ 5 ] = 2 + ( ( maxAB ) % 64 )	(8-25)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

  - If maxAB − minAB is equal to 1, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-26)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-27)

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-28)

candModeList[ 5 ] = 2 + ( maxAB % 64 )	(8-29)

  - Otherwise if maxAB − minAB is equal to 2, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-30)

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-31)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-32)

candModeList[ 5 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-33)

  - Otherwise if maxAB − minAB is greater than 61, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-34)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-35)

candModeList[ 4 ] = 2 + ( minAB % 64 )	(8-36)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-37)

  - Otherwise, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-38)

candModeList[ 3 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-39)

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-40)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-41)

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-42)

candModeList[ 1 ] = candIntraPredModeB	(8-43)

candModeList[ 2 ] = 1 − minAB		(8-44)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-45)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-46)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-47)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

candModeList[ 0 ] = maxAB		(8-48)

candModeList[ 1 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-49)

candModeList[ 2 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-50)

candModeList[ 3 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-51)

candModeList[ 4 ] = 2 + ( maxAB % 64 )	(8-52)

candModeList[ 5 ] = 2 + ( ( maxAB + 59 ) % 64 )	(8-53)

- Otherwise, the following applies:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-54)

candModeList[ 1 ] = ( candModeList[0]  = =  INTRA_PLANAR )  ?  INTRA_DC  :  	(8-55) 
		INTRA_PLANAR

candModeList[ 2 ] = INTRA_ANGULAR50	(8-56)

candModeList[ 3 ] = INTRA_ANGULAR18	(8-57)

candModeList[ 4 ] = INTRA_ANGULAR46	(8-58)

candModeList[ 5 ] = INTRA_ANGULAR54	(8-59)

maxModeVal = INTRA_ANGULAR50

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

candModeList[ 0 ] = INTRA_ANGULAR50	(8-60)

candModeList[ 1 ] = INTRA_ANGULAR18	(8-61)

candModeList[ 2 ] = INTRA_ANGULAR2	(8-62)

candModeList[ 3 ] = INTRA_ANGULAR34	(8-63)

candModeList[ 4 ] = INTRA_ANGULAR66	(8-64)

candModeList[ 5 ] = INTRA_ANGULAR26	(8-65)

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived as followsby applying the following ordered steps:

IntraPredModeY[ xCb ][ yCb ] = intra_luma_mpm_remainder + 2 +
		( intra_luma_mpm_remainder  >=  maxModeVal − 2 ) ? 1 : 0

- When candModeList[ i ] is greater than candModeList[ j ] for i = 0..4 and for each i, j = ( i + 1 )..5, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )	(8-66)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ].

  - For i equal to 0 to 5, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].The following changes are made to Table 9-4.

| coding_unit( ) | cu_skip_flag[ ][ ] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | pred_mode_flag | FL | cMax = 1 |
|  | intra_luma_ref_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | intra_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_mpm_idx[ ][ ] | TR | cMax = 5, cRiceParam = 0 |
|  | intra_luma_mpm_remainder[ ][ ] | TBFL | cMax = 603 |
|  | intra_chroma_pred_mode[ ][ ] | 9.5.3.7 | - |

# Conclusions

A simplified derivation of non-MPM modes is proposed with 6-bit fixed length signalling. The number of conditional checks (used in sorting and re-indexing) is reduced with the proposed changed with minor loss in BD-rate performance. It is suggested to adopt this change to the VTM specification.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).