| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 January 2019 | Document: JVET-M0103-v1 |
| --- | --- |

| Title: | Deblocking for multi-hypothesis intra inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Enhorn, R. Yu, Z. Zhang, R. Sjöberg | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

Multi-hypothesis intra inter prediction can produce boundaries on both CU and sub-block level due to uniform scaling of intra and inter prediction samples. Currently no deblocking is applied for such boundaries in VVC when transform, motion or intra deblocking criterions are false. This contribution proposes to set the boundary strength to 1 for boundaries of  multi-hypothesis intra inter prediction on an 8x8 grid such that the boundaries will be deblocked as other inter modes.

The proposed fix is claimed to produce better subjective quality especially noticeable at low bitrates. Reported Y/U/V BD-rates are -0.02%/-0.02%/0.03% for random access, 0.00%/-0.01%/-0.09% for low delay B and -0.05%/-0.07%/-0.34% for low delay P over VTM-3.0.

# Introduction

At the 12th JVET meeting in Ljubljana the multi-hypothesis intra inter prediction [1] was adopted introducing intra and inter modes combined with superimposition. Good objective coding gains were achieved but, in some cases, introducing blocking artifacts. This document includes adjustments to the deblocking filter both for coding unit boundaries and sub-block boundaries of multi-hypothesis intra inter prediction.

# Proposed methods

## Coding Unit boundaries

Even though multi-hypothesis intra inter prediction includes an intra prediction part it is not recognized as an intra prediction by the deblocking filter with the otherwise boundary strength set to the value 2. Currently the multi-hypothesis intra inter CU boundaries will only be deblocking filtered if there are non-zero transform coefficients or the inter prediction have different motion vectors from adjacent CU or one CU is intra coded. When these conditions are not met there can still be visible artifacts due to the addition of intra prediction, however due to superimposition of the inter and intra prediction, the artifact is not as strong as for intra prediction.

The proposed method is to set the boundary strength parameter to 1 for boundaries where current or adjacent block are coded using multi-hypothesis intra inter prediction and none of the blocks are intra coded.

## Sub-block boundaries

When the selected intra prediction mode for multi-hypothesis intra inter prediction is an angular mode, the superimposition uses larger weights for intra predicted samples closer to intra reference samples used while gradually, in four evenly spaced intervals, shift emphasis to the inter predicted samples. The stepping used while efficiently and at low complexity compared to finer granular gradient may create blocking artifacts at the boundaries for the intervals.

Proposed method for deblocking filter is to set the boundary strength to 1 for the boundaries of the intervals with different weights.

To avoid overlap and allow parallelization of deblocking filtering we apply the deblocking of boundaries from multi-hypothesis intra inter prediction on an 8x8 grid as VVC.

Figure 1 – Deblocking filtered internal edges (dashed) of a block of width respectively height 32

# Results

Proposed method was implemented in and compared to VTM-3.0. As the method is applicable to inter pictures only the All Intra test case is redundant.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,01% | 0,00% | 100% | 100% |
| Class A2 | -0,01% | -0,05% | 0,09% | 100% | 102% |
| Class B | -0,02% | -0,02% | 0,02% | 100% | 100% |
| Class C | -0,01% | 0,00% | 0,04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,02% | -0,02% | 0,03% | 100% | 101% |
| Class D | 0,02% | -0,07% | 0,02% | 100% | 100% |
| Class F | 0,00% | 0,03% | 0,04% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,02% | -0,20% | 0,04% | 100% | 96% |
| Class C | 0,02% | 0,19% | 0,01% | 100% | 100% |
| Class E | -0,06% | 0,04% | -0,44% | 99% | 104% |
| Overall | 0,00% | -0,01% | -0,09% | 100% | 99% |
| Class D | -0,02% | 0,38% | -0,02% | 100% | 100% |
| Class F | 0,00% | 0,31% | -0,14% | 99% | 97% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,03% | 0,05% | -0,05% | 100% | 100% |
| Class C | -0,02% | 0,04% | -0,09% | 100% | 101% |
| Class E | -0,12% | -0,41% | -1,15% | 99% | 100% |
| Overall | -0,05% | -0,07% | -0,34% | 100% | 100% |
| Class D | 0,01% | -0,28% | -0,24% | 99% | 103% |
| Class F | 0,08% | -0,50% | -0,26% | 100% | 99% |

Informal subjective evaluation was performed using CTC and CE11.1 [2] test set and conditions. Noticeable improvements were observed on the Kimono, FoodMarket and Cactus sequences. Following examples shows VTM-3.0 anchor and proposed method applied at decoding for a single frame of VTM-3.0 bitstream to not alter mode selection.

Figure 2 – Kimono1 encoded at QP 34 with VTM-3.0 (ALF=0)

Figure 3 – Kimono1 encoded at QP 34 with VTM-3.0 (ALF=0) with proposed method

Figure 4 - Cactus encoded at QP 32 with VTM-3.0

Figure 5 - Cactus encoded at QP 32 with VTM-3.0 with proposed method

# Conclusion

In this document a method for deblocking filtering of boundaries from the multi-hypothesis intra inter prediction was presented providing both objective and subjective benefits. We recommend adopting the proposed method into next VTM and VVC working draft.

# References

[1] M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode”, Document of Joint Video Experts Team, JVET-L0100, October 2018

[2] A. Norkin, A. M. Kotra, ”Description of Core Experiment 11 (CE11): Deblocking”, Document of Joint Video Experts Team, JVET-L1031, October 2018

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).