| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0105-v2 |
| --- | --- |

| Title: | Non-CE7: Delta QP for Chroma CU |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Roman Chernyak Sergey Ikonin Jianle Chen | Tel: Email: | +7-925-008-12-86 chernyak.roman@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a scheme of applying delta QP for Chroma CU into VVC codec, taking into account separate tree mechanism.

# Introduction

The contribution JVET-L0428 adopted in Macao proposed a signalling mechanism and derivation process for delta QP for Luma CU for Dual Tree case. Figure 1 illustrates the signalling approach.

Figure 1 – illustration of signalling process for delta QP

It was proposed to signal delta QP only for Luma CU in case of Dual Tree and derive QP value for Chroma CU based on QP value of correspondent Luma CU. During the discussion, it was mentioned that some mechanism of usage delta QP for Chroma CU is also desirable.

In the current specification draft JVET-L1001-v7, QP derivation process for Chroma components is performed in following way.

The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, are derived as follows:

Qp′Cb = qPCb + QpBdOffsetC

Qp′Cr = qPCr + QpBdOffsetC

Where, the variables qPCb and qPCr are set equal to the value of QpC as specified in mapping table (Table 1) based on the index qPi equal to qPiCb and qPiCr, respectively.

Table 1 - Specification of QpC as a function of qPi

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi - 6 |

Where, indexes qPiCb and qPiCr are calculated according to following formulas:

qPiCb = Clip3( −QpBdOffsetC, 69, QpY + pps_cb_qp_offset + slice_cb_qp_offset )

qPiCr = Clip3( −QpBdOffsetC, 69, QpY + pps_cr_qp_offset + slice_cr_qp_offset )

# Proposed method

Current contribution proposes to introduce PPS flag (cu_chroma_qp_delta_enabled_flag) which indicates whether delta QP for Chroma CU is used. If cu_chroma_qp_delta_enabled_flag is equals to one, then delta QP for Chroma CU is signalled in the bitstream and further used in Chroma QP derivation process. Figure 2 illustrates the signalling approach.

Figure 2

In order to save signalling for delta QP for Chroma CU it is proposed to modify derivation process in a following way.

The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, are derived as follows:

Qp′Cb = qPCb + QpBdOffsetC

Qp′Cr = qPCr + QpBdOffsetC

Where the variables qPCb and qPCr are set equal to the value of QpC as specified in mapping table (Table 1) based on the index qPi equal to qPiCb and qPiCr, respectively, where

qPiCb = Clip3( -QpBdOffsetC, 69, QpY + pps_cb_qp_offset + slice_cb_offset + qPiCb_A-QpY_A_Cb + CuCbQpDeltaVal )

qPiCr = Clip3( -QpBdOffsetC, 69, QpY + pps_cb_qp_offset + slice_cb_offset + qPiCr_A-QpY_A_Cr  + CuCrQpDeltaVal )

Where qPiCb_A and qPiCr_A are set equal to qPiCb  and qPiCr  values corresponding to left neighboring block to current Chroma CU;

QpY_A_Cb  = QpY_A+pps_cb_qp_offset + slice_cb_offset

QpY_A_Cr =QpY_A+pps_cr_qp_offset + slice_cr_offset

Where QpY_A is set equal to the luma quantization parameter QpY of the luma coding unit that covers central sample of the left neighboring block to current Chroma CU.

Figure 3 – An example of usage the method

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).