| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0108 |
| --- | --- |

| Title: | CE3-related: Reducing the number of reference samples and table size in LM Chroma process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Tangi Poirier, Fabrice Le Léannec | Email: | edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

In this contribution two modifications of the LM chroma process are proposed. The first modification consists in reducing the number of reference samples used to compute the cross-component linear model parameters to up to 6. The second modification consists in reducing the size of the table required to perform the division for computing the linear model parameters from 2×512 to 64. The first modification is reported to result in PSNR-Y, U, V BD-rate variations of 0.0%, 0.1%, 0.1% in AI configuration, 0.0%, 0.1%, 0.1% in RA configuration, while reducing the number of checks to derive the cross-component linear model parameters. The second modification is reported to in PSNR-Y, U, V BD-rate variations of 0.0%, 0.0%, 0.0% in AI configuration, 0.0%, 0.0%, 0.1% in RA configuration, while reducing the memory size from 2048 bytes to 128 bytes. The combination of both modifications results in PSNR-Y, U, V BD-rate variations of 0.0%, 0.1%, 0.1% in AI configuration, 0.0%, 0.0%, 0.0% in RA configuration.

# Introduction

LM chroma mode consists in deriving linear parameters (a, b) linking the chroma samples to the luma samples from neighboring samples (upper line, left column) of the current block. These linear parameters are then applied to compute a prediction of the chroma block using the reconstructed luma samples. Two modes are supported, CCLM and MDLM. CCLM makes use of the upper line and left column samples, while MDLM makes use of either the upper line or left column samples.

Figure 1. Locations of the samples used for the derivation of a and b.

The chroma prediction samples predC(i,j) at location (i,j) in the block are computed as follows from the filtered luma reconstructed samples recL(i,j):

predC(i,j) = a.recL’(i,j) + b

## Issue 1 – number of checks for deriving the linear parameters

In VTM3.0, for deriving the linear parameters (a, b), the minimum and maximum values of the reference luma samples are first computed. Let (minY, minC) be the couple of luma and chroma values, in the neighbouring reconstructed samples, for which minY has the minimum value among all the luma values, and (maxY, maxC) is the couple of luma and chroma values, in the neighbouring reconstructed samples, for which maxY has the maximum value among all the luma values.

a and b values are derived as follows:

a = ( maxC – minC ) / ( maxY – minY )	(eq. 1)

b = minC – a × minY	(eq. 2)

For a N×M chroma block, the computation of the minimum and maximum luma samples values require up to 2 × ( N + M – 1 ) checks.

## Issue 2 – tables size for the division for deriving the linear parameters

The division applied in (eq .1) is described as follows in the current VTM specification (JVET-L1001):

shift = ( BitDepthC > 8 )  ?  BitDepthC − 9  :  0		(8-153)

add = shift  ?  1 << ( shift − 1)  :  0		(8-154)

diff = ( maxY − minY + add ) >> shift		(8-155)

div = ( ( maxC − minC ) * ( Floor( 232 / diff ) − Floor( 216 / diff ) * 216 ) + 215 ) >> 16	(8-157)

a = ( ( maxC − minC ) * Floor( 216 / diff ) + div + add ) >> shift	(8-158)

In practice, equations (8-157 and 8-158) requires using two look-up-tables, one to produce the operation:

g_aiLMDivTableLow[diff - 1] = ( Floor( 232 / diff ) − Floor( 216 / diff ) * 216 )

and one to produce the operation:

g_aiLMDivTableHigh[diff - 1] = Floor( 216 / diff )

In the VTM software, those tables are of size 512 elements of 16 bits. This leads to a memory of 2048 bytes.

# Proposed simplifications

## Reducing the number of neighboring samples to derive the linear parameters

It is first proposed to identify the minimum and maximum neighboring luma values from a reduced set of up to 6 samples, selected as following positions (illustrated in Figure 2):

- Upper line: 3 locations (i – 1, 0), (i – 1, N – 1), (i – 1, N/2)

- Left column: 3 locations (0, j – 1), (M – 1, j – 1), (M/2, j – 1)

Figure 2. Reduced set of samples used for the derivation of a and b.

For a chroma block of size N×M, the maximum number of checks is reduced from 2 × ( N + M – 1 ) to 11  (e.g. 3 less for a 4x4 block, 19 less for a 8x8 block).

## Reducing the tables size for the division for deriving the linear parameters

The second modification consists in using only one table for producing the division, and in reducing its size. The process works as follows:

shift is set to ( maxY − minY ) >> K

if shift is greater than 0, shift is set to ( 1 + Log2(shift) )

add = shift  ?  1 << ( shift − 1)  :  0

diff = ( maxY − minY + add ) >> shift

a = ( ( maxC − minC ) * Floor( ( 216 + diff / 2 ) / diff ) + add ) >> shift

This requires one single table, of size 2K, derived as follows:

g_aiLMDivTable[diff - 1] = Floor( ( 216 + diff/2 ) / diff )

instead of the 2 tables of size 512 needed in current VTM. In practice, K can be much lower than 9 (e.g. 6), as shown in results below.

# Results

## Test over VTM3.0

The results of the proposed implementation are shown below against the VTM3.0 with as anchor.

### Reduction of the number of reference samples to up to 6

The full results are reported in the attached file “SDR_6samples.xlsm”.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.0% | 0.2% | 0.2% | 99% | 98% |
| Class A2 | 0.1% | 0.3% | 0.3% | 98% | 100% |
| Class B | 0.0% | -0.1% | 0.0% | 98% | 99% |
| Class C | 0.0% | 0.1% | 0.1% | 97% | 97% |
| Class E | 0.0% | 0.2% | 0.1% | 99% | 98% |
| Overall | 0.0% | 0.1% | 0.1% | 98% | 99% |
| Class D | 0.0% | 0.2% | 0.2% | 98% | 98% |
| Class F (optional) | 0.0% | 0.2% | 0.2% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.0% | 0.1% | 0.4% | 100% | 100% |
| Class A2 | 0.0% | 0.2% | 0.3% | 97% | 99% |
| Class B | 0.0% | 0.0% | -0.1% | 99% | 99% |
| Class C | 0.0% | 0.0% | 0.1% | 99% | 99% |
| Overall | 0.0% | 0.1% | 0.1% | 99% | 99% |
| Class D | 0.0% | -0.1% | 0.2% | 99% | 99% |
| Class F (optional) | 0.0% | 0.1% | -0.1% | 99% | 99% |

### Reduction of the tables size

Several table sizes were tested, from 256 to 64 elements. Below we only show this later case (one table of 64 elements). The corresponding attached excel file is “SDR_tableSize64.xlsm”.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.0% | -0.2% | -0.2% | 99% | 101% |
| Class A2 | 0.0% | 0.0% | 0.0% | 98% | 98% |
| Class B | 0.0% | 0.0% | 0.0% | 99% | 99% |
| Class C | 0.0% | 0.0% | -0.2% | 99% | 98% |
| Class E | 0.0% | 0.0% | 0.2% | 99% | 99% |
| Overall | 0.0% | 0.0% | 0.0% | 99% | 99% |
| Class D | 0.0% | 0.0% | 0.1% | 99% | 98% |
| Class F (optional) | 0.0% | 0.0% | 0.2% | 98% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.0% | -0.1% | 0.0% | 99% | 98% |
| Class A2 | 0.0% | 0.0% | 0.1% | 97% | 101% |
| Class B | 0.0% | 0.1% | 0.0% | 100% | 98% |
| Class C | 0.0% | -0.1% | 0.2% | 98% | 99% |
| Overall | 0.0% | 0.0% | 0.1% | 99% | 99% |
| Class D | 0.0% | -0.4% | 0.1% | 98% | 99% |
| Class F (optional) | 0.0% | 0.0% | 0.1% | 99% | 98% |

### Combination of the reduction of the number of reference samples and of the tables size

Combining the number of reference samples to up to 6, and the table size to one table of 64 elements, leads to the results below. The corresponding attached excel file is “SDR_6samples_tableSize64.xlsm”.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.0% | 0.0% | 0.1% | 99% | 98% |
| Class A2 | 0.1% | 0.4% | 0.3% | 98% | 98% |
| Class B | 0.0% | 0.0% | 0.0% | 98% | 99% |
| Class C | 0.0% | 0.0% | 0.1% | 98% | 98% |
| Class E | 0.0% | 0.1% | 0.2% | 99% | 100% |
| Overall | 0.0% | 0.1% | 0.1% | 98% | 99% |
| Class D | 0.0% | 0.1% | 0.3% | 98% | 99% |
| Class F (optional) | 0.0% | 0.2% | 0.2% | 98% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.0% | -0.1% | 0.0% | 99% | 100% |
| Class A2 | 0.0% | 0.4% | 0.2% | 98% | 97% |
| Class B | 0.0% | -0.1% | -0.1% | 100% | 99% |
| Class C | 0.0% | -0.1% | 0.1% | 98% | 99% |
| Overall | 0.0% | 0.0% | 0.0% | 99% | 99% |
| Class D | 0.0% | -0.3% | 0.1% | 98% | 100% |
| Class F (optional) | 0.0% | 0.3% | 0.2% | 101% | 100% |

## Test over CE3.2.4 on HDR chroma sample location indication

Additional tests were done for HDR PQ content, on top of the CE3.2.4 proposal, that investigates the usage of chroma location adapted filtering in LM chroma mode.

Results for various table sizes are provided in attached excel files. We show below only the case of table size 64.

### Reduction of the tables size

Below the results of the table size reduction to one table of 64 elements are shown for the RA case. The anchor is CE3.2.4. The corresponding attached excel file is “HDR_CE324a_tableSize64.xlsm”.

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 WCG_EXT=1 |  |  |  |  |  |  |  |
|  | BD-rate |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | 0.0% | 0.1% | 0.1% | -0.3% | -0.6% | 0.1% | 0.0% | -0.8% |
| SunRise | -1.1% | 0.0% | 0.0% | -0.7% | -0.6% | 0.0% | -0.7% | -0.3% |
| ShowGirl2 | -0.1% | -0.1% | -0.1% | 0.9% | 0.0% | -0.1% | 0.8% | 0.2% |
| BalloonFestival | -0.3% | -0.1% | 0.0% | 0.3% | -0.1% | 0.0% | 0.4% | 0.1% |
| Hurdles | 0.1% | 0.0% | 0.0% | 0.3% | -0.5% | 0.0% | 0.1% | -0.3% |
| Starting | -0.6% | 0.0% | 0.1% | 0.0% | -0.2% | 0.1% | 0.1% | 0.0% |
| Cosmos1 | 0.2% | 0.0% | 0.0% | 0.1% | -0.4% | 0.0% | 0.3% | -0.4% |
| Average HDR-B | -0.2% | 0.0% | 0.0% | 0.1% | -0.3% | 0.0% | 0.1% | -0.2% |
| Enc Time[%] | 98% |  |  |  |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  |  |  |  |

The BD-rate variations over the VTM3.0 are depicted in the table below. The gain from CE3.2.4 proposal is maintained.

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 WCG_EXT=1 |  |  |  |  |  |  |  |
|  | BD-rate |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | -0.1% | 0.0% | 0.1% | -2.5% | -0.5% | 0.0% | -2.1% | -0.5% |
| SunRise | -2.8% | 0.1% | 0.0% | -6.1% | -6.9% | -0.1% | -3.8% | -3.0% |
| ShowGirl2 | -1.0% | -0.1% | 0.0% | -7.5% | -1.8% | -0.1% | -5.2% | -1.7% |
| BalloonFestival | -0.2% | -0.1% | 0.0% | 1.0% | -1.7% | 0.0% | 1.5% | -1.3% |
| Hurdles | -7.6% | -0.6% | -0.4% | -7.3% | -17.7% | -0.4% | -6.1% | -14.5% |
| Starting | -3.0% | -0.5% | -0.3% | -1.2% | -7.4% | -0.3% | -1.2% | -4.9% |
| Cosmos1 | -0.7% | 0.0% | -0.1% | -5.1% | -7.0% | 0.0% | -5.1% | -6.9% |
| Average HDR-B | -2.2% | -0.2% | -0.1% | -4.1% | -6.1% | -0.1% | -3.1% | -4.7% |
| Enc Time[%] | 98% |  |  |  |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  |  |  |  |

### Combination of the reduction of the number of reference samples and of the tables size

Below the results of combining the two simplifications (reference samples reduced to 6, table size reduced to one table of 64 elements) are shown for the RA case. The anchor is CE3.2.4. The corresponding attached excel file is “HDR_CE324a_6samples_tableSize64.xlsm”. The gain from CE3.2.4 proposal is maintained.

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 WCG_EXT=1 |  |  |  |  |  |  |  |
|  | BD-rate |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | 0.0% | 0.1% | 0.1% | 0.3% | -0.3% | 0.1% | 0.4% | -0.4% |
| SunRise | 1.2% | 0.0% | 0.0% | 0.5% | 1.1% | 0.0% | -0.3% | 0.3% |
| ShowGirl2 | -0.1% | 0.0% | 0.0% | -0.5% | -0.9% | 0.0% | -0.6% | -1.6% |
| BalloonFestival | 0.0% | -0.1% | 0.0% | 0.6% | 0.2% | 0.0% | 0.8% | 0.2% |
| Hurdles | 2.3% | 0.1% | 0.1% | 1.2% | 0.3% | 0.1% | 1.0% | 0.4% |
| Starting | 0.3% | 0.1% | 0.1% | 0.5% | 0.6% | 0.1% | 0.3% | 0.6% |
| Cosmos1 | 0.2% | 0.0% | 0.0% | -0.7% | -1.6% | 0.0% | -0.7% | -1.4% |
| Average HDR-B | 0.6% | 0.0% | 0.0% | 0.3% | -0.1% | 0.0% | 0.1% | -0.3% |
| Enc Time[%] | 99% |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |  |  |

The BD-rate variations over the VTM3.0 are depicted in the table below.

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 WCG_EXT=1 |  |  |  |  |  |  |  |
|  | BD-rate |  |  |  |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | 0.0% | 0.0% | 0.0% | -2.0% | -0.2% | 0.0% | -1.6% | -0.1% |
| SunRise | -0.5% | 0.1% | 0.0% | -4.9% | -5.2% | 0.0% | -3.4% | -2.4% |
| ShowGirl2 | -1.0% | 0.0% | 0.0% | -8.8% | -2.7% | 0.0% | -6.6% | -3.4% |
| BalloonFestival | 0.1% | -0.1% | 0.0% | 1.2% | -1.4% | 0.0% | 1.9% | -1.2% |
| Hurdles | -5.6% | -0.5% | -0.4% | -6.4% | -17.0% | -0.4% | -5.2% | -13.9% |
| Starting | -2.2% | -0.4% | -0.3% | -0.8% | -6.7% | -0.3% | -1.0% | -4.4% |
| Cosmos1 | -0.6% | 0.0% | -0.1% | -5.9% | -8.2% | -0.1% | -6.0% | -7.9% |
| Average HDR-B | -1.4% | -0.1% | -0.1% | -3.9% | -5.9% | -0.1% | -3.1% | -4.8% |
| Enc Time[%] | 99% |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |  |  |

# Conclusion

Two simplifications of the LM chroma mode are proposed for deriving the linear parameters:

- Reduction of the number of reference samples to up to 6,

- Reduction of the look-up-tables sizes from 2x512 elements to 64 elements.

No negative impact is observed for both SDR and HDR cases. It is therefore suggested to adopt the proposed changes. The specification changes are provided in section 7 below.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification changes

The changes compared to VTM3.0 specification re indicated in track change mode and highlighted in yellow.

##### 8.2.4.2.8 Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode

…

- The variables minY, maxY, minC and maxC are derived as follows:

    - The variable minY is set equal to 1 << (BitDepthY) + 1 and the variable maxY is set equal to −1.

    - If availT is equal to TRUE, the variables minY, maxY, minC and maxC with x = 0,nS/2,nS − 1 are derived as follows:

    - If minY is greater than pTopDsY[ x * xS ], the following applies:

minY = pTopDsY[ x * xS ] 	(8-142)

minC = p[ x * xS ][ −1 ] 	(8-143)

    - If maxY is less than pTopDsY[ x * xS ], the following applies:

maxY = pTopDsY[ x * xS ] 	(8-144)

maxC = p[ x * xS ][ −1 ] 	(8-145)

    - If availL is equal to TRUE, the variables minY, maxY, minC and maxC with y = 0,nS/2,nS − 1 are derived as follows:

    - If minY is greater than pLeftDsY[ y * yS ], the following applies:

minY = pLeftDsY[ y * yS ] 	(8-146)

minC = p[ −1 ][ y * yS ] 	(8-147)

    - If maxY is less than pLeftDsY[ y * yS ], the following applies:

maxY = pLeftDsY[ y * yS ] 	(8-148)

maxC = p[ −1 ][ y * yS ] 	(8-149)

- The variables a, b, and k are derived as follows:

    - If numSampL is greater than 0, and numSampT is greater than 0, the following applies:

k = 0		(8-150)

a = 0		(8-151)

b = 1 << ( BitDepthC − 1) 		(8-152)

    - Otherwise, the following applies:

shift = ( maxY − minY ) >> 6		(8-153)

if (shift is greater than 0), shift is set to ( 1 + Log2(shift) )

add = shift  ?  1 << ( shift − 1)  :  0		(8-154)

diff = ( maxY − minY + add ) >> shift		(8-155)

k = 16		(8-156)

    - If diff is greater than 0, the following applies:

-

a = ( ( maxC − minC ) * Floor( ( 216 + diff / 2 ) / diff ) + add ) >> shift	(8-158)

    - Otherwise, the following applies:

a = 0	(8-159)

b = minC − ( ( a * minY ) >> k )		(8-160)

…