| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0110 |
| --- | --- |

| Title: | CE2-related: Alignment of affine control-point motion vector and subblock motion vector |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, Vadim Seregin, Hongtao Wang and Marta Karczewicz 10160 Pacific Mesa Blvd, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, vseregin, hongtaow martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes to use the subblock motion vectors as the affine control-point motion vectors (CPMVs). The affine motion inheritance uses the subblock motion vectors from the neighboring coded block, and they are not necessary control-point motion vectors. It’s reported that the BD-rates in random access common test condition are -0.01%, 0.01%, 0.00%, and are x.xx%, x.xx%, x.xx% in low delay B.

# Introduction

In current VTM, the affine motion model is represented by 3 control-point motion vectors (CPMVs) v0=(v0x,v0y), v1=(v1x,v1y), and v2=(v2x,v2y)  at the 3 corners of current block. The affine motion compensation is performed in subblock basis. Each subblock may have a different motion vector. The motion vector for the subblock at ith row and jth column is derived as

| vx=(v1x-v0x)blkW(j*sbW+sbW2)+(v2x-v0x)blkH(i*sbH+sbH2)+v0xvy=(v1y-v0y)blkW(j*sbW+sbW2)+(v2y-v0y)blkH(i*sbH+sbH2)+v0y | (1) |
| --- | --- |

wherein blkW and blkH are the width and height of the current block, and sbW and sbH are the width and height of the subblock, respectively.  An example of affine CPMVs and subblock MVs is shown in Fig. 1. Note that the CPMVs are different from the subblock MVs. In the affine motion inheritance, the CPMVs of a neighboring coded block are used to predict the CPMVs of the current block. Except that if the neighboring coded block locates at the above CTU row then the subblock motion vectors are used for affine motion inheritance to avoid additional line buffer for CPMVs. However, a separate local buffer, in addition to the normal MV buffer, is needed to store the affine CPMVs for affine motion inheritance from current CTU and left CTU. In the affine merge mode, if an affine inherited merge candidate is selected, the predicted CPMVs are first derived from the neighboring coded block, a first rounding process is applied. Then subblock MVs are derived from the predicted CPMVs, a second rounding process is applied.

Fig. 1

# Propose method

In the proposed method, the affine CPMVs are set as shown in Fig. 2. The top-left CPMV v0 is the same as the MV of the top-left subblock. The top-right and bottom-left CPMVs locate at two virtual subblocks at the extended subblock column and subblock row of the current block, such that the distance between v0 and v1 and v0 and v2 remain blkW and blkH (power of 2), respectively. The motion vector for the subblock at ith row and jth column is derived as

| vx=(v1x-v0x)blkW*j*sbW+v2x-v0xblkH*i*sbH+v0xvy=(v1y-v0y)blkW*j*sbW+v2y-v0yblkH*i*sbH+v0y | (2) |
| --- | --- |

In affine motion inheritance, the top-right and bottom-left CPMVs of the neighboring coded block are not available since they are at the virtual subblocks and are not stored. Instead, the bottom-right, bottom-middle and right-middle subblock MVs of neighboring coded block (as shown in Fig. 3) are used to derive the predicted CPMVs of current block, such that the distance between v0 and v1 and v0 and v2 are nW/2 and nH/2, wherein nW/2 and nH/2 are the width and height of the neighboring block and both remain power of 2. If the neighboring block locates at the above CTU, then only v0 and v2 are used for affine motion inheritance and the 4-parameter affine motion model is assumed.

Fig. 2

Fig. 3

In the affine merge mode, the subblock MVs are directly predicted from the neighboring coded block instead of the two-steps process as in VTM-3.0.

# Test results

The random access (RA) and low delay B (LB) results under common test condition are reported in Table 1. The BD-rates are -0.01%, 0.01%, 0.00% in RA, and are x.xx%, x.xx%, x.xx% in LB. The runtime remains the same as in VTM-3.0.

Table 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.02% | 98% | 98% |
| Class A2 | -0.02% | 0.04% | 0.09% | 99% | 96% |
| Class B | -0.02% | 0.03% | -0.05% | 96% | 95% |
| Class C | 0.01% | 0.00% | 0.02% | 96% | 93% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | 0.00% | 97% | 95% |
| Class D | 0.02% | -0.02% | 0.11% | 96% | 92% |
| Class F | -0.04% | -0.03% | 0.02% | 95% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.03% | 0.20% | 0.15% | 100% | 100% |
| Class E | 0.04% | -0.15% | -0.28% | 96% | 91% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.03% | 0.01% | 0.12% | 97% | 93% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).