| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0114 |
| --- | --- |

| Title: | CE7-related: implicit QP-offset based on block size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. de Lagrange E. François F. Le Léannec | Email: | philippe.delagrange@technicolor.com edouard.francois@technicolor.com fabrice.leleannec@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes to add an implicit Quantization Parameter (QP) offset based on block size, to leverage anticipated correlation between block size and QP. It is reported that the approach can help improving the local QP adaptation granularity without significant increase in bit cost. Corresponding modifications to draft text are proposed.

# Introduction

Professional encoders (e.g. broadcast encoders) make intensive use of local QP adjustment in rate control processes, mainly for psychovisual adaptation. Indeed, the quality of local quantization adjustments plays a major role in practical bitrate gains, and in visual quality of compressed videos.

One commonly annoying artefact is often observed in large blocks which contain both simple (low spatial frequency) and complex (highly textured) content. One would expect fine quantization on simple content to retain fine texture, and coarser quantization on high-contrast content, because the eye is sensitive to relative error, not absolute error. Coarse-level QP control cannot satisfy both requirements: either the selected QP is low to favor fine texture and bit cost will be very high for the high-contrast part, or a higher QP is selected, with following artefacts usually very visible and badly judged:

- loss of detail on fine texture

- mosquito noise or ripple coming from neighboring contours if transform size is large

- quality discrepancy with neighboring uniform simple blocks which get lower QP (better quality)

Figure 1 shows an illustrative example.

Figure 1. Example of artefacts related to coarse-grained QP adaptation

Thus, the finer-grained QP-adaptation is, the better (ideally 16x16 or 8x8 blocks), but the bit cost related to QP-deltas can be a problem.

Implicit QP-adaptation can help addressing this issue, by improving the granularity without additional bit cost. Some previous proposals introduced the concept of implicit QP derivation, either based on local (neighboring samples) signal statistics (JVET-J0025), or based on the average value of the luma prediction (JVET-J0026).

# Proposal overview

In the current contribution, the problem of local QP adaptation is addressed differently from these two former solutions. The contribution proposes to add an implicit QP offset based on block size. The proposed implicit adaptation is based on the following observations:

- on the one hand, higher QP is generally used in high-detail (or high-motion) areas and contours, because the human eye is less sensitive to absolute error in complex areas

- on the other hand, smaller blocks are often used in such high-detailed areas to follow contours (or motion)

Consequently, a correlation between QP and block size is often observed. This is illustrated in Figure 2 and Figure 3, where blue shades indicate lower QP and yellow shades indicate higher QP.

Figure 2. Example picture split

Figure 3. Expected QP-delta based on luma variance

The proposal comprises the following aspects:

- specification of a block-size related dQP table, either defined by default or signaled in HLS (preferably in the PPS)

- specification of a CU-dependent index to the dQP table, either defined by the split level, or by the parameter “subdiv” defined in contribution JVET-M0113

- adaptation of the luma QP derivation (QPY) and of the chroma QP (QPC) derivation from QPY

- specification of a scaling factor at CTU-level to further control QPY

# Proposal description: specification of a QP offset table and size index

The proposal consists in adding a QP offset table, indexed by a value based on block size.

The table could be transmitted at picture level (in PPS), with indexing order from small to big blocks, and variable length, because we observed that expected QP offset varies significantly for small blocks but tends to be stable for big blocks, as illustrated in Table 1.

Table 1. Example QP-offset table

| Size index | 0 | 1 | 2 | 3 | 4 | ≥5 |
| --- | --- | --- | --- | --- | --- | --- |
| QP offset | 3 | 2 | 1 | 0 | -2 | -3 |

All blocks with size index higher than the table size should use (repeat) the last element of the table. Size equal to 0 could mean disabling the feature. Size equal to 1 does not make sense and could be forbidden or have a special meaning.

As said, table index should be based on block size, with small index for small blocks. Several options can be considered for defining the index.

Option1: index defined from the split depth of the current CU

blockSizeIndex = maxSplitDepth – cqtDepth – mttDepth

Option2: index defined from the area of the CU (cf JVET-M0113)

The log2 of the block surface seems a better alternative, because it is more consistent with the actual block size. The subdiv value introduced in JVET-M0113 can be reused:

blockSizeIndex = maxSubdiv – cbSubdiv

with maxSubdiv = 2*(log2_ctu_size_minus2 - log2_min_luma_coding_block_size_minus2).

A comparison between these two alternatives is illustrated in Figure 4 and Figure 5.

Figure 4. Example implicit QP offset based on split depth

Figure 5. Example implicit QP offset based on block area (subdiv parameter of JVET-Mxxx)

# Syntax and semantics changes

## PPS Syntax and semantics

The syntax changes related to the dQP table signaling are indicated in the table below.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| num_block_size_qp_offset | ue(v) |
| for ( i = 0; i < num_block_size_qp_offset; i++ ) { |  |
| pps_block_size_qp_offset[ i ] | se(v) |
| } |  |
| […] |  |
| rbsp_trailing_bits( ) |  |
| } |  |

with the following semantics.

num_block_size_qp_offset specifies the number of pps_block_size_qp_offset[ i ] that are present in the PPS. If zero or one, no pps_block_size_qp_offset[ i ] are coded and all values are inferred to zero. The value of num_block_size_qp_offset shall be in the range is 0 to 2*(log2_ctu_size_minus2 - log2_min_luma_coding_block_size_minus2) + 1, inclusive.

pps_block_size_qp_offset[ i ] specifies the offset array used to derive QpY as specified in subclause 8.4.1. After  num_block_size_qp_offset elements are decoded, the array is extended to 2*(log2_ctu_size_minus2 - log2_min_luma_coding_block_size_minus2) + 1 elements by repeating the last decoded value. The value of pps_block_size_qp_offset[ i ] shall be in the range −( 32 + QpBdOffsetY / 2 ) to +( 31 + QpBdOffsetY / 2 ), inclusive.

Table size (num_block_size_qp_offset) can also be defined by default by the specification (e.g. 6, or full length derived from other parameters, e.g. maxSubdiv+1). In that case, a coding flag (pps_block_size_qp_offset_present_flag) must be specified, e.g. as follows.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| pps_block_size_qp_offset_present_flag | u(1) |
| if ( pps_block_size_qp_offset_present_flag ) { |  |
| for ( i = 0; i < num_block_size_qp_offset; i++ ) { |  |
| pps_block_size_qp_offset[ i ] | se(v) |
| } |  |
| } |  |
| […] |  |
| rbsp_trailing_bits( ) |  |
| } |  |

We think that PPS is the best place for the QP offset table, because its content probably depends heavily on picture type, hierarchy level, QP, etc.

## Quantization parameters derivation process changes

This section describes modifications to the quantization parameters derivation process (subclause 8.4.1 in [1]).

In the proposal, the implicit QP offset is not applied for the QP prediction process, because QP prediction and QP-delta coding operates on a QG level, whereas implicit QP offset operates at CU-level. However, the implicit QP offset is used for chroma QP derivation, and for other processes like deblocking.

First, an index based on block size is computed. As discussed above, two options are proposed.

Option1 – using split level

blockSizeIndex = maxSplitDepth - cqtDepth – mttDepth,
with maxSplitDepth the maximum value allowed for diff_cu_qp_delta_depth

Option2 (preferred) – using cbSubdiv from JVET-M0113,

blockSizeIndex = maxSubdiv - cbSubdiv,
with maxSubdiv = 2*(log2_ctu_size_minus2 - log2_min_luma_coding_block_size_minus2)

Next, an implicit QP offset is derived from the table (which is extended up to maxSubdiv index by repeating last element):

blockSizeQpOffset = pps_block_size_qp_offset[ blockSizeIndex ]

Finally, luma QP derivation is modified so that blockSizeQpOffset is ignored for QP prediction but used for other processes. To this aim, we define an intermediate QpY_CODED:

[…]. The variable qPY_A is derived as follows:

[…]

–	Otherwise, qPY_A is set equal to the luma quantization parameter QpY_CODED of the coding unit containing the luma coding block covering ( xQg – 1, yQg ).

The same stands for qPY_B. Then,

The variables QpY_CODED and QpY is are derived as follows:

QpY_CODED = ( ( qPY_PRED + CuQpDeltaVal + 64 + 2 * QpBdOffsetY )%( 64 + QpBdOffsetY ) ) − QpBdOffsetY

QpY = Clip3( − QpBdOffsetY, 63,  QpY_CODED + blockSizeQpOffset )

The rest of the syntax for chroma QP derivation is unchanged.

## CTU-level syntax and semantics

The proposed feature being implicit, it is probably a must to be able to control it at sub-picture level: encoders usually detect specific areas of the picture, like static logos, text banners, faces, grass, etc., where different adaptation rules are applied.

We suggest adding control syntax at the CTU level, conditioned to a higher-level enabling flag (e.g. slice, PPS, or SPS), like slice_qp_offset_scale_enabled_flag.

We propose an index to a scaling factor table, because a scaling factor is probably more flexible than an enable flag, and consistent with what can be done at picture level. Actually, QP dynamic range is key to psychovisual quality and encoders often make use of a scaling factor as part of the adaptation rules, which highly depends on spatial and temporal complexity of the video content.

The scaling factor table is designed so that more frequent factors are coded first (requiring less bits for the index).

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if( slice_qp_offset_scale_enabled_flag) |  |
| ctb_sz_qp_offset_scaler | ae(v) |
| […] |  |
| } |  |

ctb_sz_qp_offset_scaler specifies the scaling factor used when applying block-size-based implicit QP offset.

- When not present, blockSizeQpOffsetScale is inferred to the default value (1 << blockSizeQpOffsetShift)

- When present, blockSizeQpOffsetScale is computed as follows:

blockSizeQpOffsetScale = blockSizeQpOffsetScaleTable[ ctb_sz_qp_offset_scaler ]

ctb_sz_qp_offset_scaler is coded with arithmetic coding, in a way similar to cu_qp_delta_abs.

blockSizeQpOffsetShift is defined by the specification, e.g. 3.

blockSizeQpOffsetScaleTable specifies possible scaling factors and is defined by default in the specification, for example

Specification of blockSizeQpOffsetScaleTable

| ctb_sz_qp_offset_scaler | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| blockSizeQpOffsetScale | 8 | 0 | 4 | 2 | 6 | 1 | 3 | 5 | 7 |

This table could be extended to values greater than (1 << blockSizeQpOffsetShift) to allow greater than 1 scaling factors (e.g. up to 15).

The computation of QpY in section 3.2 is modified to account for this scaling factor:

QpY = Clip3( − QpBdOffsetY, 63,  QpY_CODED + ( ( blockSizeQpOffset * blockSizeQpOffsetScale ) >> blockSizeQpOffsetShift ) )

# References

[1]	B. Bross, J. Chen, S. Liu, "Versatile Video Coding (Draft 3)", JVET-L1001, Macao, CN, October 2018 (version 3 is used here).

[2]	P. de Lagrange, E. François, P. Bordes, “CE7-related: Quantization Group size uniformity”, JVET-M0113, Marrakech, MA, January 2019

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).