| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0118 |
| --- | --- |

| Title: | CE10-related: A fix for merge_triangle_flag signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In VTM-3.0, the merge_triangle_flag is reported to be redundantly signaled when a block uses MMVD (merge with motion vector difference) or multi-hypothesis intra inter prediction. The contribution describes a fix for the redundant signaling. The fix was tested under the VTM-3.0 random access (RA) and low delay B (LDB) common test configurations. The BD-rate results are reported to be -0.07% for RA, -0.14% for LDB. It is proposed to adopt the fix into next VTM.

# Introduction

In VTM-3.0, when a current block uses MMVD or multi-hypothesis intra inter prediction, the merge_triangle_flag is signaled even though it always has a value of 0.

The contribution describes a fix for the redundant signaling. The fix disables the signaling of the merge_triangle_flag when MMVD or multi-hypothesis intra inter prediction is enabled for a block. The redundant signaling issue was reported in the VTM ticket #105 [1]. The exact same fix was also provided in a patch file in the ticket.

The proposed syntax change on top of VVC draft 3 (version 7) [2] is shown as follows. It is noticed that the syntax in the draft already excludes the signaling of the merge_triangle_flag when MMVD is enabled (i.e. when mmvd_flag is on).

| … |  |
| --- | --- |
| if( sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 && mh_intra_flag[ x0 ][ y0 ] = = 0 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| … |  |

# Results

The fix was implemented in the VTM-3.0 reference software [3] and tested using the VTM common test configuration [4]. The encoding time and decoding time are not reliable.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.08% | -0.08% | 90% | 94% |
| Class A2 | -0.10% | -0.09% | -0.02% | 86% | 87% |
| Class B | -0.07% | -0.01% | -0.18% | 79% | 87% |
| Class C | -0.07% | -0.06% | -0.09% | 86% | 88% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.05% | -0.11% | 84% | 89% |
| Class D | -0.10% | -0.20% | -0.07% | 83% | 86% |
| Class F | -0.03% | -0.01% | -0.03% | 79% | 88% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.32% | 0.00% | 99% | 103% |
| Class C | -0.19% | 0.03% | -0.14% | 100% | 99% |
| Class E | -0.13% | 0.03% | -0.17% | 100% | 104% |
| Overall | -0.14% | -0.12% | -0.09% | 99% | 102% |
| Class D | -0.17% | 0.13% | 0.13% | 88% | 90% |
| Class F | 0.00% | -0.14% | -0.54% | 85% | 89% |

# References

[1]. https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/105.

[2]. B. Bross, J. Chen, S. Liu, “	Versatile Video Coding (Draft 3)”, JVET-L1001.

[3]. VTM-3.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0.

[4]. F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010.

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.