| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0119 |
| --- | --- |

| Title: | CE7-related: Modified dequantization scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karl Sharman Steve Keating | Email: | karl.sharman@sony.com steve.keating@sony.com |
| Source: | Sony Europe Ltd |  |  |

_____________________________

# Abstract

In VVC WD3, when the size of a transform block is not a power of 4, the transform coefficients are modified by a √2 factor, to compensate for an implicit scaling by the transform process.

It is proposed that the scaling be applied by a modification to the QP rather than by multiplication by 181/256 (or 181/128). In addition it is proposed that the adjustment is not applied in the dequantization process to transform skip blocks to be subsequently removed by explicit scaling during the inverse transform skip process.

The proposal reportedly produces negligible BD-rate changes. It is independent, but related to JVET-L0095.

# Introduction

The transformation processes require the resulting coefficients to be scaled by

factor= 1√MN

where M and N are the width and height of the transform block.

In VVC, the dimensions of the block are powers of 2, i.e. M=2m and N=2n. This means that when M=N, or indeed when MN=4f, the factor can be applied by a right-shift. However, when MN is not a power of 4, a simple right-shift is not applicable. Instead, in the current VVC WD, the coefficients are scaled by a 181/256 factor as an additional stage of the dequantization process (and similarly for the quantization process).

There are several alternative methods to remove this additional compensation stage. They all relate to the fact that the QP system is logarithmic, and that QP+6 is an additional division by 2, and QP+3 is an additional division by √2.

## Method 1

One method to consider is to modify the QP by +3 for non-rectangular blocks at the start of each quantization stage. Then, the current system is used to work out the ‘QPper’ parameter (the shift required) and the ‘QPrem’ index which looks up the fractional part of the QP to apply from a table of 6 entries.

The table in HEVC & VVC is of the form:

| QP%6 : | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Inverse quantization | 40/64 | 45/64 | 51/64 | 57/64 | 64/64 | 72/64 |
| Quantization (non-normative) | 26214/16384 | 23302/16384 | 20560/16384 | 18396/16384 | 16384/16384 | 14564/16384 |

Table 1 - quantization table in VVC WD and VVC VTM.

## Method 2

Another method to consider is to increase the size of the look-up table. The QPper and QPrem are calculated without regard of the block shape. Then, the QPrem index is used to look-up the inverse quantization scale factor, except that the look-up stage accounts for the block shape. This can be achieved by using a second table for non-power-of-4 blocks:

i.e., when a non-power-of-4 block is encountered, the following table should be used instead:

| QP%6 : | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Inverse quantization | 29/64 | 32/64 | 36/64 | 40/64 | 45/64 | 51/64 |
| Quantization (non-normative) | 36158/16384 | 32768/16384 | 29127/16384 | 26214/16384 | 23302/16384 | 20560/16384 |

Table 2 - Table to be used for a non-power-of-4 block.

An alternative, but equivalent implementation could use a single table, with 9 elements, due to the significant overlap between the two sets of coefficients.

# Proposal

It is proposed that method 2 is used: it would be easier to apply the adjustment by a multiplexor-based method, rather than introduce a subtraction prior to the “QP modulo 6” calculation.

Regarding method 1, due to the luma-chroma QP mapping table, it is currently not possible to apply the QP adjustment by pre-offsetting, say, the slice chroma QP; and in addition there are questions how to cope with QP when it is at its maximum/minimum permitted value.

Method 2 was also discussed during the review of JVET-L0095, with the decision deferred.

In addition, it is proposed that this scale factor is only applied for transformed blocks – since only transformed blocks need to have a compensation factor applied. Currently, in VTM, the scale factor is applied in the inverse quantization stage, and then inverted by an additional multiply per coefficient during the inverse transform stage (and similarly for the forward stages in VTM). In VVC WD3, the transform skip process appears to apply the scale factor in the inverse quantization stage, but does not remove it during the inverse transform stage.

# Experimental results

The proposed method (method 2 in combination with not applying any compensation for transform skip blocks) has been implemented on top of VTM3.0. Experiments have been run using the CTCs as described in JVET-K1010. As expected, the results have negligible impact on BD-rate, and are also similar to those indicated in JVET-L0095 (which was implemented on top of VTM2.0).

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0-Benchmark-VTM-JVET-K1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.22% | 0.21% | 99% | 105% |
| Class A2 | -0.04% | 0.36% | 0.27% | 100% | 103% |
| Class B | -0.02% | 0.24% | 0.21% | 99% | 102% |
| Class C | -0.03% | 0.12% | 0.12% | 99% | 101% |
| Class E | -0.01% | 0.20% | 0.25% | 99% | 102% |
| Overall | -0.03% | 0.23% | 0.21% | 99% | 102% |
| Class D | 0.00% | 0.20% | 0.14% | 101% | 102% |
| Class F (optional) | -0.01% | 0.14% | 0.20% | 99% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM3.0-Benchmark-VTM-JVET-K1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.12% | 0.10% | 100% | 101% |
| Class A2 | -0.02% | 0.09% | 0.25% | 100% | 100% |
| Class B | -0.01% | 0.24% | 0.00% | 100% | 100% |
| Class C | 0.06% | -0.06% | 0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.10% | 0.10% | 100% | 100% |
| Class D | -0.04% | -0.27% | 0.10% | 100% | 100% |
| Class F (optional) | -0.03% | 0.08% | 0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM3.0-Benchmark-VTM-JVET-K1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.02% | -0.05% | 100% | 100% |
| Class C | 0.06% | 0.28% | 0.39% | 100% | 101% |
| Class E | 0.10% | 1.30% | 0.65% | 100% | 101% |
| Overall | 0.04% | 0.41% | 0.27% | 100% | 100% |
| Class D | 0.02% | 0.16% | 0.69% | 100% | 100% |
| Class F (optional) | 0.04% | -0.35% | -0.68% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM3.0-Benchmark-VTM-JVET-K1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.40% | 0.20% | 100% | 100% |
| Class C | 0.03% | 0.42% | 0.22% | 100% | 100% |
| Class E | -0.21% | 0.72% | 0.17% | 100% | 101% |
| Overall | -0.05% | 0.49% | 0.20% | 100% | 100% |
| Class D | 0.01% | -0.22% | 0.28% | 99% | 100% |
| Class F (optional) | 0.09% | -0.04% | -0.23% | 100% | 100% |

# Conclusion

It is proposed that the method to compensate for blocks that are not a power-of-4 in size (in terms of coefficients) is modified, from the additional scaling factor method in VVC WD3 / VTM3.0 to one that modifies the QP. It is also proposed that no such adjustment is made to transform skip blocks. Two different methods are discussed, with the preferred method implemented on top of VTM3.0, whose simulation results indicate a negligible change in BD-rate.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).