The following draft text modifications implement just the removal of the adjustment factor for transform skip blocks. They are relative to JVET-L1001-v9.docx (JVET-L1001-v13).

### 8.4.3	Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- a variable bitDepth specifying the bit depth of the current colour component.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The variable rectNormFlag is derived as follows:

rectNormFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = = 1 ) && 	transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] = = 0

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y		(8-827)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(8-828)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr		(8-829)

The variables bdShift, rectNorm and bdOffset are derived as follows:

bdShift = bitDepth + ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 ) * 8 ( rectNormFlag ? 8 : 0 ) + 	(8-830)
	( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + dep_quant_enabled_flag

rectNorm = ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1rectNormFlag ?  181  :  1	(8-831)

bdOffset = ( 1 << bdShift ) >> 1		(8-832)

The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is set equal to 16.

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If dep_quant_enabled_flag is equal to 1, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ (qP + 1) % 6 ] ) << ( (qP + 1) / 6 )	(8-833)

–	Otherwise (dep_quant_enabled_flag is equal to 0), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ qP % 6 ] ) << ( qP / 6 )	(8-834)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = 			(8-835)
	( TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] * ls[ x ][ y ] * rectNorm +bdOffset )  >>  bdShift

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(8-836)