| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0120 |
| --- | --- |

| Title: | Proposed NAL Unit Header Design Principles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger, Byeongdoo Choi, Shan Liu Tencent America LLC 2747 Park Blvd Palo Alto, CA 94036 USA | Email: | stewe@stewe.org bdchoi@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Proposed are design principles to be adhered to when evaluating proposals to the NAL unit header design.  It is asserted that most or all of these design principles have been followed in previous ITU/MPEG video codec designs, at least to the point where the technology in question was available.  It is further asserted that following the design principles outlined herein, the design and implementation of video processing entities other than decoders (such as: MANEs) will be simplified.  Finally, it is asserted that, when following the proposed design principles, the standardization of specifications surrounding the video coding spec (such as: RTP payload format and file format specs) will also be facilitated.  If accepted, this document could form a checklist against which NAL unit header proposals can be evaluated.

# Introduction

The concept of a NAL unit including a self-contained header has been introduced early in the design of H.264/AVC, and has been followed since then with minor departures.  However, the author does recall having intervened quite a few times during the later phases of JVT, and also later in JCT-VC when proponents became somewhat ambitious in optimizing the NAL unit header (NUH).  Similarly, the author has recollection of proposals that would have moved certain functionality away from the NUH and into the slice header and similar, perhaps for minor gains in coding efficiency.  Not all of these were rejected.  Some of these design choices were, in 20:20 hindsight and with the video conferencing industry as an example industry in mind, simply bad—to the point that somewhat more sophisticated MANEs may need not only to follow the parameter set activation sequence, but also interpret significant parts of the slice header syntax before being able to decide what to do with a given NAL unit.  That is against the intention and purpose of the NAL unit concept and should be avoided if possible.

This document proposes a number of design principles based on the lessons learned.  There may be a few more that the author has missed, and the author volunteers to keep this document updated over the next several meeting cycles if so requested by JVET.

# Design Principles

Design principles are listed below in a numbered list.  The numbering does not reflect any preference or importance.

- NUH must be octet aligned

- NUH must be reasonably small (1, 2, 3 octets?)

- A variable length NUH header (as we had it for the scalable/Multiview extensions in H.264) is OK as long as there is no parsing dependency between NUH and parameter sets (perhaps excluding the highest parameter set, perhaps excluding the profile)

- If there were a parsing dependency from a parameter set, then the NUH for that parameter set must be self-explanatory (no circular parsing dependency)

- Parsing dependencies inside the NUH are OK.  For example, it’s fine to increase the length of the NUH for certain values of the nal_unit_type as we had it in H.264/SVC.

- Start code emulation for MPEG-2 System Layer: forbidden-zero-bit needs to be in as-is where-is.

- As the NUH is not subject to start code emulation, it must be designed to avoid start code emulation.  Note that this can be done in the NUH design, or by selecting sufficiently long start codes.

- Individual codepoints in the NUH should be fixed length binary coded or otherwise easily parseable.

- For reasons of error resilience, no NUH-to-NUH prediction can be allowed.

- The NUH should allow, by itself and without access to syntax in the slice/tile group headers, those operations a MANE can reasonably be expected to perform.  That may include

  - Bitstream pruning according to layers/sublayers

  - Bitstream pruning so to discard damaged pictures

  - Bitstream pruning by component type?

  - Bitstream pruning by tile?

  - …

- Decisions on whether to move functionality currently (such as: in HEVC/H.264) in the NUH to headers inside the NAL unit’s start code emulation prevention protected area (NAL unit payload) should be avoid if it were implying that start code emulation prevention needs to be applied to the payload.  In other words, those syntax elements must be allocated at the begin of the NAL unit’s payload, and must be restricted in their numbering space such that no start code emulation can occur.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the design principles described in this contribution.