| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0123r1 |
| --- | --- |

| Title: | AHG12: On Hierarchical Tile Design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmad Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Current VVC tile design divides a picture into rigid tile grid, then groups a number of consecutive tiles into a tile group. The design follows HEVC tile design but is not flexible to address needs of emerging video applications. This contribution proposes a two-level hierarchical design as summarized below:

- The design divides a picture into rectangular tile groups, and each tile group may further be partitioned into multiple tiles.

- Propose to signal tile group partitioning structure at PPS, and signal local tile grid within each tile group at tile group header to make tile group a self-contained entity for MCTS sub-bitstream access and reposition.

The design is asserted to be flexible and efficient to satisfy both application needs as well as parallelization.

# Introduction

In JVET-L0686, Pictures are divided into tile groups and tiles. A tile group is a sequence of tiles in tile raster scan of a picture. A tile is a sequence of CTUs that cover a rectangular region of a picture. For example, a picture may be divided into 9 tiles and 3 tile groups as shown in Figure 1.

Figure 1 A picture with 12 by 6 luma CTUs that is partitioned into 9 tiles and 3 tile groups

Such design keeps the rigid tile grid as specified in HEVC, but not flexible to address the needs of emerging video applications such as load balancing for different projection formats, sub-picture based rendering, extraction and reposition, etc. JVET-K0260 proposed a flexible tile design to indicate tile size individually; JVET-L0394 proposed a tile hierarchy design to address application driven and parallelization driven tile grid. This contribution proposes a two-level tile design to enable flexible tile grid.

In current VVC WD, tile grid is indicated in PPS and the tile group position is indicated in the tile group header using absolute tile ID. When subsets of multiple bitstreams are stitched together, PPS and tile group header needs to be modified to accommodate new tile grid and tile group layouts. Due to the rigid tile partitioning, the converge of multiple tile grid is limited. This contribution proposes to carry local tile grid indication in the tile group header to make each tile group a self-contained entity, which simplifies sub-bitstream extraction and reposition and allows more converged partitioning structures.

# Hierarchical tile design

Current tile partitioning method may not be sufficient to address tile layouts required by emerging video applications. Partitioning structures not supported by current tile design were described in JVET-K0260 and JVET-L0127. Figure 2 shows partitioning structures stitched from sub-pictures of multiple different resolution video sources; Figure 3 is a partitioning structure for load balancing parallelization or sub-picture access.

Figure 2 – The OMAF/VRIF partition structure

Figure 3 Load balancing partitioning structure

In most use cases, the tile grid maintains column and row structure locally within a rectangular region of picture, different tile grid may occur in different region of the picture due to sub-picture merging or region of interest processing. As a result, a two-level tile design is proposed to divide a picture into multiple rectangular tile groups first, each tile group covers integer number of complete CTBs, and then each tile group may choose different local tile partitioning grid. Figure 4 shows such partitioning process where a picture is first divided into 3 tile groups, tile group #1 and #3 only contain one tile respectively, while tile group #2 is further divided into 3 tiles.

Figure 4 Two-level tile hierarchy example

Figure 5 shows tile layout examples supporting the partitioning structures in Figure 2.

Figure 5 hierarchical tile design example

It is proposed to signal tile group partitioning at PPS as such application-driven partitioning shall be consistent for multiple pictures or entire CVS. Table 1 is the proposed tile group partitioning syntax elements. The tile partitioning structure is removed as each tile group may be partitioned differently.

Table 1 Proposed picture parameter set syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| num_tile_groups_minus1 | ue(v) |
| for (i = 0; i <= num_tile_groups_minus1; i ++) { |  |
| tile_group_start_address[ i ] | u(v) |
| tile_group_end_address[ i ] | u(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

num_tile_groups_minus1 plus 1 specifies the number of tile groups in a picture.

tile_group_start_address[ i ] and tile_group_end_address[ i ] specify the position of the first CTB, or top left CTB, and the position of the last CTB, or bottom-right CTB of i-th tile group. The address value is an address of associated CTB in CTB raster scan of a picture. The length of tile_group_start_address[ i ] and tile_group_end_address[ I ] are Ceil( Log2 ( PicSizeInCtbsY ) ) bits. The value of tile_group_start_address[ i ] and tile_group_end_address[ i ] shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive, and the value of tile_group_start_address[ i ] and tile_group_end_address[ i ] shall not be equal to the value of tile_group_start_address[ j ] and tile_group_end_address[ j ] of any other coded tile group NAL unit of the same coded picture.

Each tile group may be further divided into multiple tiles, the tile grid within each tile group is the same as HEVC tile grid structed by tile column and tile row. The local tile grid is signaled at tile group header to make each tile group a self-contained entity. Such structure would simplify the tile group sub-bitstream extraction and reposition process, the extractor does not have to signal the merged tile grid from multiple streams at PPS, especially when merged tile grid may be too complicated to be supported by traditional tile column and tile row structure. The proposed local tile grid signaled in tile group header is shown in Table 2 with changes highlighted in turquoise.

Table 2 Tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_index | u(v) |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| single_tile_in_tile_group_flag | u(1) |
| if( !single_tile_in_tile_group_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| tile_group_type | ue(v) |
| … |  |
| } |  |

tile_group_index specifies the tile group raster scan index. The length of tile_group_index is Ceil( Log2 ( NumTileGroupsInPic ) ) bits. The value of tile_group_index shall be in the range of 0 to NumTileGroupsInPic − 1, inclusive, and the value of tile_group_index shall not be equal to the value of tile_group_index of any other coded tile group NAL unit of the same coded picture. When tile_group_index is not present it is inferred to be equal to 0.

single_tile_in_tile_group_flag equal to 1 specifies that there is only one tile in associated tile group. single_tile_in_tile_group_flag equal to 0 specifies that there is more than one tile in associated tile group.

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the tile group. num_tile_columns_minus1 shall be in the range of 0 to TileGrpWidthInCtbs − 1, inclusive. When not present, the value of num_tile_columns_minus1 is inferred to be equal to 0.

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the tile group. num_tile_rows_minus1 shall be in the range of 0 to TileGrpHeightInCtbs − 1, inclusive. When not present, the value of num_tile_rows_minus1 is inferred to be equal to 0.

The i-th tile group width and height in the unit of CTBs can be derived as follows:

TileGrpWidthInCtbs[ i ] = (tile_group_end_address[ i ] % PicWidthInCtbsY) – ((tile_group_startd_address[ i ] % PicWidthInCtbsY) + 1

TileGrpHeightInCtbs[ i ] = (tile_group_end_address[ i ] / PicWidthInCtbsY) – ((tile_group_startd_address[ i ] / PicWidthInCtbsY) + 1

The total number of CTBs in i-th tile group can be derived as follows:

TileGrpSizeInCtbs[ i ] =  TileGrpWidthInCtbs[ i ] * TileGrpHeightInCtbs[ i ]

uniform_tile_spacing_flag equal to 1 specifies that tile column boundaries and likewise tile row boundaries are distributed uniformly across the tile group. uniform_tile_spacing_flag equal to 0 specifies that tile column boundaries and likewise tile row boundaries are not distributed uniformly across the tile group but signalled explicitly using the syntax elements tile_column_width_minus1[ i ] and tile_row_height_minus1[ i ]. When not present, the value of uniform_tile_spacing_flag is inferred to be equal to 1.

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs.

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs.

loop_filter_across_tiles_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile boundaries in associated tile group. loop_filter_across_tiles_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries in associated tile group. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_tiles_enabled_flag is inferred to be equal to 1.

# CTB raster and tile scanning conversion process

As tiles are partitioned within each tile group, the CTB raster and tile scanning process is specified as follows:

The list TileGrpColBd[ i ] for i ranging from 0 to num_tile_groups_minus1, inclusive, specifying the location of the i-th tile group left boundary in units of CTBs, is derived as follows:

for( i = 0; i <= num_tile_groups_minus1; i++ )
		TileGrpColBd[ i ] = tile_group_start_address[ i ] % PicWidthInCtbsY

The list TileGrpRowBd[ i ] for i ranging from 0 to num_tile_groups_minus1, inclusive, specifying the location of the i-th tile group top boundary in units of CTBs, is derived as follows:

for( i = 0; i <= num_tile_groups_minus1; i++ )
		TileGrpRowBd[ i ] = tile_group_start_address[ i ] / PicWidthInCtbsY

The list ColWidth[ i ][ j ] for j ranging from 0 to num_tile_columns_minus1[ i ], inclusive, specifying the width of the j-th tile column of i-th tile group in units of CTBs, is derived as follows:

if( uniform_spacing_flag ) for( j = 0; j <= num_tile_columns_minus1[ i ]; j++ ) ColWidth[ i ][ j ] = (( j + 1) * TileGrpWidthInCtbs[ i ]) / ( num_tile_columns_minus1[ i ] + 1 ) − ( j * TileGrpWidthInCtbs[ i ] ) / ( num_tile_columns_minus1[ i ] + 1 ) else { ColWidth[ i ][ num_tile_columns_minus1[ i ] ] = TileGrpWidthInCtbs[ i ] for( j = 0; j < num_tile_columns_minus1; j++ ) { ColWidth[ i ][ j ] = column_width_minus1[ i ] [ j ] + 1 ColWidth[ i ][ num_tile_columns_minus1[ i ] ] −= ColWidth[ i ] [ j ] } }

The list RowHeight[ i ][ j ] for j ranging from 0 to num_tile_rows_minus1[ i ], inclusive, specifying the height of the j-th tile row of i-th tile group in units of CTBs, is derived as follows:

if( uniform_spacing_flag ) for( j = 0; j <= num_tile_rows_minus1[ i ]; j++ ) RowHeight[ i ][ j ] = ( ( j + 1 ) * TileGrpHeightInCtbs[ i ] ) / ( num_tile_rows_minus1[ i ] + 1 ) − ( j * TileGrpHeightInCtbs[ i ] ) / ( num_tile_rows_minus1[ i ] + 1 ) else { RowHeight[ i ] [ num_tile_rows_minus1[ i ] ] = TileGrpHeightInCtbs[ i ] for( j = 0; j < num_tile_rows_minus1[ i ]; j++ ) { RowHeight[ i ][ j ] = row_height_minus1[ i ] [ j ] + 1 RowHeight[ i ][ num_tile_rows_minus1 ] −= RowHeight[ i ] [ j ] } }

The list ColBd[ i ][ j ] for i ranging from 0 to num_tile_columns_minus1[ i ] + 1, inclusive, specifying the location of the j-th tile column boundary of i-th tile group in units of CTBs, is derived as follows:

| for( ColBd[ i ][ 0 ] = 0, j = 0; j <= num_tile_columns_minus1[ i ]; j++ ) ColBd[ i ][ j + 1 ] = ColBd[ j ] + ColWidth[ i ] [ j ] |  |
| --- | --- |

The list RowBd[ i ][ j ] for j ranging from 0 to num_tile_rows_minus1[ i ] + 1, inclusive, specifying the location of the j-th tile row boundary of i-th tile group in units of CTBs, is derived as follows:

| for( RowBd[ 0 ] = 0, j = 0; j <= num_tile_rows_minus1[ i ]; j++ ) RowBd[ i ][ j + 1 ] = RowBd[ i ] [ j ] + RowHeight[ i ] [ j ] |  |
| --- | --- |

The list CtbAddrRsToTs[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in CTB raster scan of a picture to a CTB address in tile scan, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) { tbX = ctbAddrRs % PicWidthInCtbsY tbY = ctbAddrRs / PicWidthInCtbsY for( i = 0; i <= num_tile_groups_minus1; i++ ) if ( tbY >= TileGrpRowBd[i] && tbX >= TileGrpColBd[i] ) tileGrpIdx = i for( CtbAddrRsToTs[ ctbAddrRs ] = 0, i = 0; i < tileGrpIdx; i++ ) CtbAddrRsToTs[ ctbAddrRs ] += TileGrpWidthInCtbs[ i ] * TileGrpHeightInCtbs[ i ] tbX −= TileGrpColBd[tileGrpIdx] tbY −= TileGrpRowBd[tileGrpIdx] for( i = 0; i <= num_tile_columns_minus1[tileGrpIdx]; i++ ) if( tbX >= ColBd[tileGrpIdx][ i ] ) tileX = i for( j = 0; j <= num_tile_rows_minus1[tileGrpIdx]; j++ ) if( tbY >= RowBd[ tgIdx ][ j ] ) tileY = j for( i = 0; i < tileX; i++ ) CtbAddrRsToTs[ ctbAddrRs ]+= RowHeight[tileGrpIdx][tileY]*ColWidth[tileGrpIdx][i] for( j = 0; j < tileY; j++ ) CtbAddrRsToTs[ ctbAddrRs ] += TileGrpWidthInCtbs[tileGrpIdx] * RowHeight[tileGrpIdx][j] CtbAddrRsToTs[ ctbAddrRs ] += ( tbY – RowBd[tileGrpIdx][tileY] ) * ColWidth[ tileGrpIdx ][ tileX ] + tbX – ColBd[tileGrpIdx][tileX] }

The list CtbAddrTsToRs[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a CTB address in CTB raster scan of a picture, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) CtbAddrTsToRs[ CtbAddrRsToTs[ ctbAddrRs ] ] = ctbAddrRs

Figure 6 shows CTB raster and tile scanning example. A picture is first divided into 3 tile groups. The first and thirf tile group each contains one tile, and the second tile group is further divided into 3 tiles.

Figure 6 CTB raster and tile group scan example

The list TileGrpId[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbsY  − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile group index, is derived as follows:.

for( ctbAddrTs = 0; ctbAddrTs < PicSizeInCtbsY; ctbAddrTs ++) { for( numCtbs = 0, i = 0; i < NumTileGrpsInPic; i++ ) { if( ctbAddrTs >= numCtbs ) TileGrpIdx[ ctbAddrTs ] = i numCtbs += TileGrpSizeInCtbs[ i ] } }

The list TileId[ ctbAddrTs ] for ctbAddrTs ranging from 0 to PicSizeInCtbs  − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID for the tile group, is derived as follows:

for( ctbAddrTs = 0; ctbAddrTs < PicSizeInCtbsY; ctbAddrTs ++) {
	tileGrpIdx = TileGrpIdx[ ctbAddrTs ] 
	for ( numCtbs = 0, i = 0; i < tileGrpIdx; i ++ ) 
		numCtbs += TileGrpSizeInCtbs[ i ] 
	for( j = 0, tileIdx = 0; j  <=  num_tile_rows_minus1[  tileGrpIdx ]; j++ )
		for( i = 0; i  <=  num_tile_columns_minus1[  tileGrpIdx ]; i++, tileIdx++ ) {
			FirstCtbAddrsTs[  tileGrpIdx ][ tildIdx ] = numCtbs +  RowBd[  tileGrpIdx ][ j ] * TileGrpWidthInCtbs[ tileGrpIdx ] + ColBd[ tileGrpIdx ][ i ] )
			if ( ctbAddrTs >= FirstCtbAddrsTs[  tileGrpIdx ][ tildIdx ] ) 
				TileId[ ctbAddrTs] = tileIdx
}

The list NumCtusInTile[ tileIdx ] for tileIdx ranging from 0 to NumTilesInTileGroups − 1, inclusive, specifying the conversion from a tile index to the number of CTUs in the tile of each tile group, is derived as follows:

for( tileGrpIdx = 0; tileGrpIdx < NumTileGrpsInPic; tileGrpIdx ++ ) 
	for( j = 0, tileIdx = 0; j  <=  num_tile_rows_minus1[  tileGrpIdx ]; j++ )
		for( i = 0; i  <=  num_tile_columns_minus1[  tileGrpIdx ]; i++, tileIdx++ ) 
			NumCtusInTile[tileGrpIdx][ tileIdx ] = ColWidth[tileGrpIdx][ i ] * RowHeight[tileGrpIdx][ j ]

The list FirstCtbAddrRs[ tileGrpIdx ] for tileGrpIdx ranging from 0 to NumTileGroupsInPic − 1, inclusive, specifying the CTB address in raster scan of the first CTB in the tile group.

# Tile group data syntax

Table 4 shows the proposed tile group data syntax structure.

Table 4 Proposed tile group data structure

| tile_group_data( ) { | Descriptor |
| --- | --- |
| ctbAddrInTs = CtbAddrRsToTs[ FirstCtbAddrRs[ tile_group_index ] ] |  |
| for (i = 0, tileIdx = 0; i <= num_tile_rows_minus1; i++ ) { |  |
| for (j = 0; j <= num_tile_columns_minus1; j++, tileIdx++ ) { |  |
| for (n = 0; n < NumCtusInTile[ tile_group_index ][ tileIdx ]; n++, ctbAddrInTs++) { |  |
| ctbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < (NumTilesInTileGroup[tile_group_id] – 1) ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |
| } |  |

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).