| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0135-v1 |
| --- | --- |

| Title: | On adaptive resolution change (ARC) for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Ye-Kui Wang Jianle Chen (Huawei) Thomas Davies Arild Fuldseth (Cisco) Yu-Chen Sun Tsui-Shan Chang Jian Lou (Alibaba) | Tel: Email: | fnu.hendry@huawei.com yekui.wang@huawei.com jianle.chen@huawei.com thdavies@cisco.com arilfuld@cisco.com yuchen.s@alibaba-inc.com tsuishan.cts@alibaba-inc.com j.lou@alibaba-inc.com |
| Source: | Huawei Technologies Co., Ltd., Cisco Systems, Alibaba Group |  |  |

_____________________________

# Abstract

This contribution discusses use cases or application scenarios that would benefit from an adaptive resolution change (ARC) capability where the spatial resolution may change at a non-IRAP picture. Then a preliminary design of ARC as is presented and suggested to be a place holder just to trigger the discussion. It is proposed that the support of ARC for VVC is discussed, and if there is sufficient interest, either start a CE or include aspects of ARC support as one or more mandates of an AHG.

# Introduction

AVC and HEVC does not have the ability to change resolution without having to introduce an IDR or IRAP picture; such ability can be referred to as adaptive resolution change (ARC). There are use cases or application scenarios that would benefit from an ARC feature, including the following:

- Rate adaption in video telephony and conferencing: For adapting the coded video to the changing network conditions, when the network condition gets worse so that available bandwidth becomes lower, the encoder may adapt to it by encoding smaller resolution pictures. Currently, changing picture resolution can be done only after an IRAP picture; this has several issues. An IRAP picture at reasonable quality will be much larger than an inter-coded picture, and will be correspondingly more complex to decode: this costs time and resource. This is a problem if the resolution change is requested by the decoder for loading reasons. It can also break low-latency buffer conditions, forcing an audio re-sync, and the end-to-end delay of the stream will increase, at least temporarily. This can give a poor user experience.

- Active speaker change in multi-party video conferencing: For multi-party video conferencing, it is common that the active speaker is shown in bigger video size than the video for the rest of conference participants. When the active speaker changes, picture resolution for each participant may also need to be adjusted. The need to have ARC feature becomes more important when such change in active speaker happens frequently.

- Fast start in streaming: For streaming application, it is common that the application would buffer up to certain length of decoded picture before start displaying. Starting the bitstream with smaller resolution would allow the application to have enough pictures in the buffer to start displaying faster.

- Adaptive stream switching in streaming: The Dynamic Adaptive Streaming over HTTP (DASH) specification includes a feature named @mediaStreamStructureId. This enables switching between different representations at open-GOP random access points with non-decodable leading pictures, e.g., CRA pictures with associated RASL pictures in HEVC. When two different representations of the same video have different bitrates but the same spatial resolution while they have the same value of @mediaStreamStructureId, switching between the two representations at a CRA picture with associated RASL pictures can be performed, and the RASL pictures associated with the switching-at CRA pictures can be decoded with acceptable quality hence enabling seamless switching. With ARC, the @mediaStreamStructureId feature would also be usable for switching between DASH representations with different spatial resolutions.

Based on the above, we think that ARC would be a nice feature to be added to VVC, making the codec performs better in various versatile application scenarios.

# Support for adaptive resolution change

A preliminary design of ARC as described below, with some parts taken from JCTVC-F158, is suggested to be a place holder just to trigger the discussion.

## Description of basic tools

The basic tools constraints for supporting ARC are as follows:

- The spatial resolution may differ from the nominal resolution by a factor 0.5, applied to both dimensions. The spatial resolution may increase or decrease, yielding scaling ratios of 0.5 and 2.0.

- The aspect ratios and chroma formats of the video format are not changed.

- The cropping areas are scaled in proportion to spatial resolutions.

- The reference pictures are simply re-scaled as needed and inter prediction is applied as usual.

## Scaling operations

It is proposed to use simple, zero-phase separable down- and up-scaling filters. Note that these filters are for prediction only; a decoder may use more sophisticated scaling for output purposes.

The following 1:2 down-scaling filter is used, which has zero phase and 5 taps:

( −1, 9, 16, 9, −1 ) / 32

The down-sampling points are at even sample positions and are co-sited. The same filter is used for luma and chroma.

For 2:1 up-sampling, additional samples at odd grid positions are generated using the half-pel motion compensation interpolation filter coefficients in the latest VVC WD.

The combined up- and down-sampling will not change phase or the position of chroma sampling points.

## Resolution description in parameter set

The signalling of picture resolution in the SPS is changed as shown below.

Sequence parameter set RBSP syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| num_pic_size_in_luma_samples_minus1 | ue(v) |
| for( i = 0; i <= num_pic_size_in_luma_samples_minus1; i++ ) { |  |
| pic_width_in_luma_samples[ i ] | ue(v) |
| pic_height_in_luma_samples[ i ] | ue(v) |
| } |  |
| ... |  |
| } |  |

pic_width_in_luma_samples specifies the width of each decoded picture in units of luma samples. pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

pic_height_in_luma_samples specifies the height of each decoded picture in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

num_pic_size_in_luma_samples_minus1 plus 1 specifies the number of picture sizes (width and height) in units of luma samples that may be present in the coded video sequence.

pic_width_in_luma_samples[ i ] specifies the i-th width of decoded pictures in units of luma samples that may be present in the coded video sequence. pic_width_in_luma_samples[ i ] shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

pic_height_in_luma_samples[ i ] specifies the i-th height of decoded pictures in units of luma samples that may be present in the coded video sequence. pic_height_in_luma_samples[ i ] shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

Picture parameter set RBSP syntax and semantics

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| pic_size_idx | ue(v) |
| ... |  |
| } |  |

pic_size_idx specifies the index to the i-th picture size in the sequence parameter set. The width of pictures that refer to the picture parameter set is pic_width_in_luma_samples[ pic_size_idx ] in luma samples. Likewise, the height of pictures that refer to the picture parameter set is pic_height_in_luma_samples[ pic_size_idx ] in luma samples.

# Proposal

It is proposed that the support of ARC for VVC is discussed, and if there is sufficient interest, either start a CE or include aspects of ARC support as one or more mandates of an AHG.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd., Cisco Systems, and Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).