| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0138 |
| --- | --- |

| Title: | Intra reference sample deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Z. Zhang, K. Andersson, R. Sjöberg | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

Angular intra prediction modes can create annoying artifacts along the prediction direction for large blocks due to use of reference samples which are populated from a boundary between two large blocks. This contribution presents an Intra Reference Sample Deblocking (IRSDB) method for intra prediction. IRSDB is applied to the luma plane of large blocks that have both a block width and a block height larger than or equal to 32 luma samples. IRSDB is only applied to unfiltered reference samples for blocks that are predicted by an angular intra prediction mode. IRSDB is designed to symmetrically deblock 7 unfiltered reference samples on respective side of a block boundary to eliminate the discontinuity between reference samples which are populated from two neighboring large blocks. IRSDB is asserted to significantly reduce artifacts from angular intra prediction. The luma BDR impact for AI/RA/LDB is -0.02%/-0.02%/0.01% compared to VTM-3.0. Encoding time impact for AI/RA/LDB is 100%/100%/100% and decoding time impact for AI/RA/LDB is 100%/99%/99%, compared to VTM-3.0.

# Introduction

In Versatile Video Coding (VVC), blocking artifacts in previously decoded CUs may introduce blocking artifacts along the intra prediction direction in a current CU. The value discontinuity of reference samples can not be eliminated sufficiently by the [1 2 1] / 4 low-pass filter or 4-tap interpolation filter. Figure 1 shows an example of blocking artifacts along the intra prediction direction using VTM 3.0. The example is a 512x512 block at position (1408, 384) from FoodMarket RA coding with QP34 and ALF disabled, the POC is 11 and slice QP is 43. The artifacts in yellow circle is a 64x64 CU with intra prediction mode 34.

Figure 1, an example of a 64x64 CU with intra prediction mode 34

# Proposed method

We propose that an IRSDB filter is added and applied to the unfiltered intra reference samples for luma blocks that are predicted by angular intra prediction modes and have both a block width and a block height larger than or equal to 32 luma samples.

A set of checkpoint positions are defined that includes the following positions:

- at the sample to the top-left of the current block

- at each position (X, Y) above the current block that X is divisible by 32

- at each position (X, Y) to the left of the current block that is Y divisible by 32

Figure 2 shows example of a set of checkpoint positions for a 32x64 block.

Figure 2,  Example of a set of checkpoint positions for a 32x64 block

At each checkpoint, the block to the left or below of the checkpoint is block P, the block to the right or above of the checkpoint is block Q. For filter decision and operation, we use 8 reference samples p0,i to p7,i in block P, where p0,i is nearest to checkpoint and 8 reference samples q0,i to q7,i in block Q, where q0,i is nearest to checkpoint. Figure 3 shows the block P, block Q and reference sample pn,i and qn,i when checkpoint positions at top-left, above and left of current CU, wherein n ranges from 0 to 7 and i = multiple reference line index.

Figure 3, An example of block P, block Q for checkpoint positions at top-left, above and left of current CU

The unfiltered reference samples are examined to check whether to apply filtering or not:

- The top-left checkpoint is filtered if all these conditions are fulfilled:

  - At least the 32 reference samples for the block (block P) to the left of the current block have been decoded

  - At least 32 reference samples for the block above (block Q) the current block have been decoded

  - abs(p0,i-q0,i) < 2.5*tc, where tc value depends on the slice QP similar to HEVC deblocking.

  - abs(p0,i-p3,i-p4,i+p7,i) + abs(q0,i-q3,i-q4,i+q7,i) < (3 * beta / 32), where beta value depends on the slice QP similar to HEVC deblocking.

where p0,i, p3,i, p4,i, p7,i, q0,i, q3,i, q4,i and q7,i corresponds to positions (xPos-1-i, yPos),  (xPos-1-i, yPos+3), (xPos-1-i, yPos+4), (xPos-1-i, yPos+7), (xPos, yPos-1-i), (xPos+3, yPos-1-i), (xPos+4, yPos-1-i) and  (xPos+7,yPos-1-i).

- A checkpoint to the left of current CU at position (xPos-1-i, yPos+yO+32*k) where k ranges from 1 to (2*CUheight/32-1) is filtered if all these conditions are fulfilled:

  - The block P to the left of the current block have been decoded and block P height >= 32

  - The block Q to the left of the current block have been decoded and block Q height >=32

  - Block Q and block P belong to two different PUs or TUs

  - abs(p0,i-q0,i) < 2.5*tc

  - abs(p0,i-p3,i-p4,i+p7,i) + abs(q0,i-q3,i-q4,i+q7,i) < (3 * beta / 32)

where p0,i, p3,i, p4,i, p7,i, q0,i, q3,i, q4,i and q7,i corresponds to positions (xPos-1-i, yPos+yO+32*k), (xPos-1-i, yPos+yO+32*k+3), (xPos-1-i, yPos+yO+32*k+4), (xPos-1-i, yPos+yO+32*k+7), (xPos-1-i, yPos+yO+32*k-1), (xPos-1-i, yPos+yO+32*k-4), (xPos-1-i, yPos+yO+32*k-5) and (xPos-1-i, yPos+yO+32*k-8).

- A checkpoint to the above of current CU at position (xPos+xO+32*k,yPos-1-i) where k ranges from 1 to (2*CUwidth/32-1) is filtered if all these conditions are fulfilled:

  - The block P to the above of the current block have been decoded and block P width >= 32

  - The block Q to the above of the current block have been decoded and block Q width >= 32

  - Block Q and block P belong to two different PUs or TUs

  - abs(p0,i-q0,i) < 2.5*tc

  - abs(p0,i-p3,i-p4,i+p7,i) + abs(q0,i-q3,i-q4,i+q7,i) < (3 * beta / 32)

where p0,i, p3,i, p4,i, p7,i, q0,i, q3,i, q4,i and q7,i corresponds to positions (xPos+xO+32*k-1, yPos-1-i), (xPos+xO+32*k-4, yPos-1-i), (xPos+xO+32*k-5,yPos-1-i), (xPos+xO+32*k-7, yPos-1-i), (xPos+xO+32*k,yPos-1-i), (xPos+xO+32*k+3,yPos-1-i), (xPos+xO+32*k+4,yPos-1-i) and (xPos+xO+32*k+7,yPos-1-i).

Where the top-left sample of the current block is at (xPos,yPos), i = multi reference line index, xO is the initial offset of xPos for checkpoint position at above of current CU, yO is the initial offset of yPos for checkpoint position at left of current CU, xO is equal to 16 when CU width is equal to 32 and xPos % 32 is not equal to 0, otherwise xO is equal to 0. yO is equal to 16 when CU height is equal to 32 and yPos % 32 is not equal to 0, otherwise yO is equal to 0.

The IRSDB filter operation:

- For intra reference samples pn,i and qn,i for n = 0 to 6 on each side of the checkpoint are then replaced by linear interpolation as follows:

  - pn,i´=(fn*Middle+64-fn*P)+32)≫6), clipped to pn,i±tc

  - qn,i´=(fn*Middle+64-fn*Q+32)≫6), clipped to qn,i±tc

where f, Middle, P and Q are given below:

fn=59-n*9,  can also be described as f=59,50,41,32,23,14,5

Middle=(2*p0,i+q0,i+p1,i+q1,i+p2,i+q2,i+p3,i+q3,i+p4,i+q4,i+p5,i+q5,i+p6,i+q6,i+8)≫4

P=(p6,i+p7,i+1)≫1,      Q=(q6,i+q7,i+1)≫1

Where the positions of the unfiltered samples pn,i and qn,i,

wherein n ranges from 0 to 6, i = multiple reference line index, are given by:

- pn,i = (xPos-1-i,yPos+n), qn,i = (xPos+n, yPos-1-i) for a top-left checkpoint,

- pn,i = (xPos-1-i, yPos+yO+32*k+n), qn,i = (xPos-1-i, yPos+yO+32*k-1-n) for a left checkpoint at k,

- pn,i = (xPos+xO+32*k-1-n, yPos-1-i), qn,i = (xPos+xO+32*k+n, yPos-1-i) for an above checkpoint at k,

where the top-left sample of the current block is at (xPos,yPos),

wherein, xO is equal to 16 when CUwidth is equal to 32 and xPos % 32 is not equal to 0, otherwise xO is equal to 0.

wherein, yO is equal to 16 when CUheight is equal to 32 and yPos % 32 is not equal to 0, otherwise yO is equal to 0.

- The top-left checkpoint is filtered as follows:

reftop-leftxPos+a,yPos+b´=Middle,

clipped to reftop-left(xPos+a,yPos+b)±tc

  - Multi reference line index is 0, reftop-left is the reference sample at top-left of current block where both a and b are equal to -1.

  - Multi reference line index > 0, reftop-left are the reference samples at top-left of current block where both a and b are equal to -i-1, and i samples at below of the top-left reference sample where a is equal to -i-1 and b ranges from -1 to -i and i samples at right of the top-left reference sample where a ranges from -1 to -i and b is equal to -i-1, where i = multi reference line index.

The IRSDB reference samples is further filtered by [1 2 1] / 4 low-pass filter or 4-tap interpolation filter depending on the intra prediction mode. [ref JVET-L0628]

The directional intra-prediction mode is classified into one of the following groups:

  - vertical or horizontal modes (HOR_IDX, VER_IDX),

  - diagonal modes that represent angles which are multiple of 45 degree (2, DIA_IDX, VDIA_IDX),

  - remaining directional modes;

|  | Reference sample filter |
| --- | --- |
| Group A | IRSDB |
| Group B | IRSDB + [1 2 1] / 4 low-pass filter |
| Group C | IRSDB + 4-tap interpolation filter |

# Simulation result

Objective result:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,06% | -0,06% | 0,01% | 100% | 101% |
| Class A2 | -0,01% | 0,01% | 0,00% | 100% | 102% |
| Class B | 0,00% | -0,06% | -0,02% | 100% | 99% |
| Class C | 0,00% | -0,02% | -0,07% | 100% | 99% |
| Class E | -0,03% | -0,03% | 0,05% | 100% | 100% |
| Overall | -0,02% | -0,03% | -0,01% | 100% | 100% |
| Class D | 0,01% | -0,02% | -0,01% | 100% | 99% |
| Class F | 0,01% | 0,03% | 0,19% | 100% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,03% | -0,06% | -0,01% | 101% | 98% |
| Class A2 | -0,02% | -0,03% | 0,12% | 100% | 98% |
| Class B | -0,02% | 0,07% | -0,12% | 99% | 98% |
| Class C | -0,01% | 0,05% | 0,01% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0,02% | 0,02% | -0,02% | 100% | 99% |
| Class D | 0,01% | 0,02% | 0,02% | 100% | 98% |
| Class F | -0,03% | 0,02% | -0,07% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,01% | -0,18% | -0,46% | 101% | 98% |
| Class C | 0,03% | 0,13% | 0,21% | 100% | 100% |
| Class E | -0,02% | 0,88% | -0,32% | 99% | 98% |
| Overall | 0,01% | 0,19% | -0,20% | 100% | 99% |
| Class D | 0,05% | -0,07% | 0,19% | 100% | 98% |
| Class F | -0,12% | -0,23% | 0,08% | 100% | 102% |

Subjective result:

Figure 4 shows an example to apply IRSDB on a 64x64 CU, the prediction mode and coding tree are same as anchor. Anchor is a 512x512 block at position (1408, 384) from FoodMarket RA coding with QP34 and ALF disabled, the POC is 11 and slice QP is 43.

Figure 4, an example of applying IRSDB on a 64x64 CU

Figure 5 shows an example to encode / decode the entire video sequence with IRSDB. Anchor is a 1152x696 block at position (768, 384) from RedKayak RA coding with QP39, and ALF enabled, the POC is 20, slice QP is 46.

Figure 5, an example of applying IRSDB for encoding / decoding on entire video sequence.

# Conclusions

We propose to adopt intra reference sample deblocking for angular intra luma prediction to VVC and VTM to improve subjective performance of intra prediction of large blocks.

# References

[1]. JVET-L0628: CE3: A combination of tests 3.1.2 and 3.1.4 for intra reference sample interpolation filter

[2]. JVET-L0072: CE11: Long deblocking filters for luma (CE11.1.1) and for both luma and chroma (CE11.1.9)

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).