| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0140-v3 |
| --- | --- |

| Title: | CE6: Sub-block transform for inter blocks (CE6.4.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Han Gao, Haitao Yang, Jianle Chen | Email: | yin.zhao@huawei.com han.gao@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Draft text

The draft text corresponds to CE6.4.1-a with replacing DST7/DCT8 as DCT2 for all 64xX and Xx64 sub blocks (X is smaller than 64).

#### 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| … |  |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| sps_sbt_enable_flag | u(1) |
| if ( sps_sbt_enable_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.4.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && sps_sbt_enable_flag ) { |  |
| if( cbWidth <= maxSbtSize && cbHeight <= maxSbtSize ) { |  |
| allowSbtVerHalf = cbWidth >= 8 |  |
| allowSbtVerQuad = cbWidth >= 16 |  |
| allowSbtHorHalf = cbHeight >= 8 |  |
| allowSbtHorQuad = cbHeight >= 16 |  |
| if( allowSbtVerHalf ¦ ¦ allowSbtHorHalf ¦ ¦ allowSbtVerQuad ¦ ¦ allowSbtHorQuad ) |  |
| cu_sbt_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( cu_sbt_flag[ x0 ][ y0 ] ) { |  |
| if( ( allowSbtVerHalf ¦ ¦ allowSbtHorHalf ) && ( allowSbtVerQuad ¦ ¦ allowSbtHorQuad ) ) |  |
| cu_sbt_quad_flag[ x0 ][ y0 ] | ae(v) |
| if( ( cu_sbt_quad_flag[ x0 ][ y0 ] && allowSbtVerQuad && allowSbtHorQuad ) ¦ ¦ ( !cu_sbt_quad_flag[ x0 ][ y0 ] && allowSbtVerHalf && allowSbtHorHalf ) ) |  |
| cu_sbt_horizontal_flag[ x0 ][ y0 ] | ae(v) |
| cu_sbt_pos_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |
| } |  |

#### 7.3.4.10 Transform tree syntax

| transform_tree( x0, y0, tbWidth, tbHeight , treeType) { | Descriptor |
| --- | --- |
| if( tbWidth > MaxTbSizeY ¦ ¦ tbHeight > MaxTbSizeY ) { |  |
| trafoWidth = ( tbWidth > MaxTbSizeY ) ? (tbWidth / 2) : tbWidth |  |
| trafoHeight = ( tbHeight > MaxTbSizeY ) ? (tbHeight / 2) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight ) |  |
| if( tbWidth > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType ) |  |
| if( tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType ) |  |
| if( tbWidth > MaxTbSizeY && tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0 + trafoHeight, trafoWidth, trafoHeight, treeType ) |  |
| } else if( cu_sbt_flag[ x0 ][ y0 ] ) { |  |
| factorTb0 = cu_sbt_quad_flag[ x0 ][ y0 ] ? 1 : 2 |  |
| factorTb0 = cu_sbt_pos_flag[ x0 ][ y0 ] ? ( 4 - factorTb0 ) : factorTb0 |  |
| noResiTb0 = cu_sbt_pos_flag[ x0 ][ y0 ] ? 1 : 0 |  |
| if( !cu_sbt_horizontal_flag[ x0 ][ y0 ] ) { |  |
| trafoWidth = tbWidth * factorTb0 / 4 |  |
| transform_unit( x0, y0, trafoWidth, tbHeight, treeType , noResiTb0 ) |  |
| transform_unit( x0 + trafoWidth, y0, tbWidth − trafoWidth, tbHeight, treeType , !noResiTb0 ) |  |
| } |  |
| else { |  |
| trafoHeight = tbHeight * factorTb0 / 4 |  |
| transform_unit( x0, y0, tbWidth, trafoHeight, treeType , noResiTb0 ) |  |
| transform_unit( x0, y0 + trafoHeight, tbWidth, tbHeight − trafoHeight, treeType , !noResiTb0 ) |  |
| } |  |
| } else { |  |
| transform_unit( x0, y0, tbWidth, tbHeight, treeType , 0 ) |  |
| } |  |
| } |  |

#### 7.3.4.11 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType , noResi ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) && !noResi ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && !noResi ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |
| if( ( ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) && sps_mts_intra_enabled_flag ) ¦ ¦ ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ) && sps_mts_inter_enabled_flag ) ) && tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && !cu_sbt_flag[ x0 ][ y0 ] ) |  |
| tu_mts_flag[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_luma[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth ), log2( tbHeight ), 0 ) |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth / 2 ), log2( tbHeight / 2 ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth / 2 ), log2( tbHeight / 2 ), 2 ) |  |
| } |  |

Note that in VTM 3.0, the implementation of tu_cbf_cb, tu_cbf_cr and tu_cbf_luma is not fully aligned with the syntax table in VVC draft 3. In general, VTM 3.0 uses HEVC RQT-like signaling of cbf in transform_tree() syntax structure, while the VVC draft 3 simply signals tu_cbf_luma, tu_cbf_cb and tu_cbf_cr in transform_unit() syntax structure, without avoiding the redundancy between cu_cbf and tu_cbf_luma in the case that tu_cbf_cb and tu_cbf_cr are both 0..

Therefore, the proponent provides draft text corresponding to VTM3.0 cbf signaling, as shown in the following two tables (changes from the VVC draft 3 are highlighted in blue). On top of it, the changes from SBT on cbf syntax are added (highlighted in yellow). For clarity, the following two tables are not prepared in trace mode.

#### 7.3.4.10 Transform tree syntax

| transform_tree( x0, y0, tbWidth, tbHeight , treeType, trafoDepth, xBase, yBase ) { | Descriptor |
| --- | --- |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) && !( cu_sbt_flag[ x0 ][ y0 ] && trafoDepth = = 1 ) ) |  |
| tu_cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( ( trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) && !( cu_sbt_flag[ x0 ][ y0 ] && trafoDepth = = 1 ) ) |  |
| tu_cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( tbWidth > MaxTbSizeY ¦ ¦ tbHeight > MaxTbSizeY ) { |  |
| trafoWidth = ( tbWidth > MaxTbSizeY ) ? (tbWidth / 2) : tbWidth |  |
| trafoHeight = ( tbHeight > MaxTbSizeY ) ? (tbHeight / 2) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight , treeType, trafoDepth + 1, x0, y0 ) |  |
| if( tbWidth > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| if( tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| if( tbWidth > MaxTbSizeY && tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0 + trafoHeight, trafoWidth, trafoHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| } else if( cu_sbt_flag[ x0 ][ y0 ] && trafoDepth = = 0 ) { |  |
| factorTb0 = cu_sbt_quad_flag[ x0 ][ y0 ] ? 1 : 2 |  |
| factorTb0 = cu_sbt_pos_flag[ x0 ][ y0 ] ? ( 4 - factorTb0 ) : factorTb0 |  |
| noResiTb0 = cu_sbt_pos_flag[ x0 ][ y0 ] ? 1 : 0 |  |
| if( !cu_sbt_horizontal_flag[ x0 ][ y0 ] ) { |  |
| trafoWidth = tbWidth * factorTb0 / 4 |  |
| if ( !noResiTb0 ) |  |
| transform_tree( x0, y0, trafoWidth, tbHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| else |  |
| transform_tree( x0 + trafoWidth, y0, tbWidth − trafoWidth, tbHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| } |  |
| else { |  |
| trafoHeight = tbHeight * factorTb0 / 4 |  |
| if ( !noResiTb0 ) |  |
| transform_tree( x0, y0, tbWidth, trafoHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| else |  |
| transform_tree( x0, y0 + trafoHeight, tbWidth, tbHeight − trafoHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| } |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ ( trafoDepth != 0 && !cu_sbt_flag[ x0 ][ y0 ] ) ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| tu_cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| transform_unit( x0, y0, tbWidth, tbHeight, treeType ) |  |
| } |  |
| } |  |

#### 7.3.4.11 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |
| if( ( ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) && sps_mts_intra_enabled_flag ) ¦ ¦ ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ) && sps_mts_inter_enabled_flag ) ) && tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && !cu_sbt_flag[ x0 ][ y0 ] ) |  |
| tu_mts_flag[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_luma[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth ), log2( tbHeight ), 0 ) |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth / 2 ), log2( tbHeight / 2 ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth / 2 ), log2( tbHeight / 2 ), 2 ) |  |
| } |  |

#### 7.4.5.XX Transform tree semantics

tu_cbf_luma[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the luma transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When tu_cbf_luma[ x0 ][ y0 ][ trafoDepth ] is not present, the value of tu_cbf_luma[ x0 ][ y0 ][ trafoDepth ] is inferred as follows:

- If treeType is not equal to DUAL_TEE_CHROMA, tu_cbf_luma[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1

- Otherwise, tu_cbf_luma[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

tu_cbf_cb[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cb transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the top-left location ( x0, y0 ) of the considered transform unit. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When tu_cbf_cb[ x0 ][ y0 ][ trafoDepth ] is not present, the value of tu_cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred as follows:

- If trafoDepth is greater than 0, tu_cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, tu_cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

tu_cbf_cr[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cr transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the top-left location ( x0, y0 ) of the considered transform unit. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When tu_cbf_cr[ x0 ][ y0 ][ trafoDepth ] is not present, the value of tu_cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred as follows:

- If trafoDepth is greater than 0, cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to tu_cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, tu_cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

#### 7.4.3.1 Sequence parameter set RBSP semantics

sps_sbt_enabled_flag equal to 0 specifies that sub-block transform for inter-predicted CU is disabled. sps_sbt_enabled_flag equal to 1 specifies that sub-block transform for inter-predicted CU is enabled.

sps_sbt_max_size_64_flag equal to 0 specifies that the maximum CU width and height for allowing sub-block transform is 32. slice_sbt_max_size_64_flag equal to 1 specifies that the maximum CU width and height for allowing sub-block transform is 64.

maxSbtSize = sps_sbt_max_size_64_flag? 64 : 32

#### 7.4.5.5 Coding unit semantics

cu_sbt_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, sub-block transform is used. cu_sbt_flag[ x0 ][ y0 ] equal to 0 specifies that for the current coding unit, sub-block transform is not used.

When cu_sbt_flag[ x0 ][ y0 ] is not present, its value is inferred to be equal to 0.

NOTE – : When sub-block transform is used, a coding unit is tiled into two transform units; one transform unit has residual, the other does not have residual.

cu_sbt_quad_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, the sub-block transform includes a transform unit of 1/4 size of the current coding unit. cu_sbt_quad_flag[ x0 ][ y0 ] equal to 0 specifies that for the current coding unit the sub-block transform includes a transform unit of 1/2 size of the current coding unit.

When cu_sbt_quad_flag[ x0 ][ y0 ] is not present, its value is inferred to be equal to 0.

cu_sbt_horizontal_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is tiled into 2 transform units by a horizontal split. cu_sbt_horizontal_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is tiled into 2 transform units by a vertical split.

When cu_sbt_horizontal_flag[ x0 ][ y0 ] is not present, its value is derived as follows:

- If cu_sbt_quad_flag[ x0 ][ y0 ] is equal to 1, cu_sbt_horizontal_flag[ x0 ][ y0 ] is set to be equal to allowSbtHoriQuad.

- Otherwise (cu_sbt_quad_flag[ x0 ][ y0 ] is equal to 0), cu_sbt_horizontal_flag[ x0 ][ y0 ] is set to be equal to allowSbtHoriHalf.

cu_sbt_pos_flag[ x0 ][ y0 ] equal to 1 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the first transform unit in the current coding unit are not present in the bitstream. cu_sbt_pos_flag[ x0 ][ y0 ] equal to 0 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the second transform unit in the current coding unit are not present in the bitstream.

### 8.4.4 Transformation process for scaled transform coefficients

#### 8.4.4.1 General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

When cu_sbt_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Table 8-X depending on cu_sbt_horizontal_flag[ xTbY ][ yTbY ] and cu_sbt_pos_flag[ xTbY ][ yTbY ].

When cu_sbt_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is greater than 0, the variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are both derived to be equal to 0.

When cu_sbt_flag[ xTbY ][ yTbY ] is equal to 0, the variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Table 8-16 depending on mts_idx[ xTbY ][ yTbY ][ cIdx ].

The variables nonZeroW and nonZeroH are derived as follows:

nonZeroW = Min( nTbW, 32 )		(8-810)

nonZeroH = Min( nTbH, 32 )		(8-811)

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

- Each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nonZeroH − 1 is transformed to e[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.4.4.2 for each column x = 0..nonZeroW − 1 with the height of the transform block nTbH, the non-zero height of the scaled transform coefficients nonZeroH, the list d[ x ][ y ] with y = 0..nonZeroH − 1 and the transform type variable trType set equal to trTypeVer as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- The intermediate sample values g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )	(8-812)

- Each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.4.4.2 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the non-zero width of the resulting array g[ x ][ y ] nonZeroW, the list g[ x ][ y ] with x = 0..nonZeroW − 1 and the transform type variable trType set equal to trTypeHor as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

Table 8-X – Specification of trTypeHor and trTypeVer depending on cu_sbt_horizontal_flag[ x ][ y ] and cu_sbt_pos_flag[ x ][ y ]

|  | cu_sbt_horizontal_flag [ xTbY ][ yTbY ] = = 0 |  | cu_sbt_horizontal_flag [ xTbY ][ yTbY ] = = 1 |  |
| --- | --- | --- | --- | --- |
|  | cu_sbt_pos_flag [ xTbY ][ yTbY ] = = 0 | cu_sbt_pos_flag [ xTbY ][ yTbY ] = = 1 | cu_sbt_pos_flag [ xTbY ][ yTbY ] = = 0 | cu_sbt_pos_flag [ xTbY ][ yTbY ] = = 1 |
| trTypeHor | nTbH > 32 ? 0 : 2 | nTbH > 32 ? 0 : 1 | nTbW > 32 ? 0 : 1 | nTbW > 32 ? 0 : 1 |
| trTypeVer | nTbH > 32 ? 0 : 1 | nTbH > 32 ? 0 : 1 | nTbW > 32 ? 0 : 2 | nTbW > 32 ? 0 : 1 |

Table 8-16 – Specification of trTypeHor and trTypeVer depending on mts_idx[ x ][ y ][ cIdx ]

| mts_idx[ xTbY ][ yTbY ][ cIdx ] | trTypeHor | trTypeVer |
| --- | --- | --- |
| −1 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 2 | 1 |
| 2 | 1 | 2 |
| 3 | 2 | 2 |

### 9.5.3 Binarization process

Table 9-4 – Syntax elements and associated binarizations

| coding_unit( ) | cu_skip_flag[ ][ ] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | pred_mode_flag | FL | cMax = 1 |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_ref_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | intra_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | … |  |  |
|  | cu_sbt_flag[ ][ ] | FL | cMax = 1 |
|  | cu_sbt_quad_flag[ ][ ] | FL | cMax = 1 |
|  | cu_sbt_horizontal_flag[ ][ ] | FL | cMax = 1 |
|  | cu_sbt_pos_flag[ ][ ] | FL | cMax = 1 |

### 9.5.4.2 Derivation process for ctxTable, ctxIdx and bypassFlag

Table 9-10 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_tile_group_flag | terminate | na | na | na | na | na |
| alf_ctb_flag[ ][ ][ ] | 0..8 (clause 9.5.4.2.2) | na | na | na | na | na |
| … |  |  |  |  |  |  |
| cu_cbf | 0 | na | na | na | na | na |
| cu_sbt_flag[ ][ ] | 0 | na | na | na | na | na |
| cu_sbt_quad_flag[ ][ ] | 0 | na | na | na | na | na |
| cu_sbt_horizontal_flag[ ][ ] | ( cbWidth = = cbHeight ) ? 0 : ( (cbWidth < cbHeight ) ? 1 : 2 ) | na | na | na | na | na |
| cu_sbt_pos_flag[ ][ ] | 0 | na | na | na | na | na |

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).