| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0140-v3 |
| --- | --- |

| Title: | CE6: Sub-block transform for inter blocks (CE6.4.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Han Gao, Haitao Yang, Jianle Chen | Email: | yin.zhao@huawei.com han.gao@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports the coding performance of sub-block transform for inter blocks. Simulation results reportedly show that in test CE6.4.1a (allowing BT and ABT partitioning in sub-block transform) -0.47% and -0.83% PSNR-Y BD-rates are achieved for RA and LB configurations, with EncT 108% and 113%, respectively. In test CE6.4.1e (allowing BT, ABT and QT partitioning in sub-block transform), -0.55% and -0.93% PSNR-Y BD-rates are achieved for RA and LB configurations, with EncT 109% and 115%, respectively. In test CE6.4.1d (allowing BT and ABT partitioning in sub-block transform with multiple transform selection enabled for a sub-block), -0.52% and -0.90% PSNR-Y BD-rates are achieved for RA and LB configurations, with EncT 114% and 121%, respectively.

(In version 3, the draft text is updated)

# Proposed Method

For an inter-predicted CU with cu_cbf equal to 1, cu_sbt_flag may be signaled to indicate whether the whole residual block or a sub-part of the residual block is decoded. In the former case, inter MTS information is further parsed to determine the transform type of the CU. In the latter case, a part of the residual block is coded with inferred adaptive transform and the other part of the residual block is zeroed out.

The SBT is not applied to the combined inter-intra mode, since almost no coding gain is achieved.

## Sub-block TU tiling

When SBT is used for a inter CU, SBT type and SBT position information are further decoded from the bitstream. In CE6.4.1a, CE6.4.1b, CE6.4.1c and CE6.4.1d, there are two SBT types and two SBT positions, as indicated in Figure 1. For SBT-V (or SBT-H), the TU width (or height) may equal to half of the CU width (or height) or 1/4 of the CU width (or height), signaled by another flag, resulting in 2:2 split or 1:3/3:1 split. The 2:2 split is like a binary tree (BT) split while the 1:3/3:1 split is like an asymmetric binary tree (ABT) split. If one side of CU is 8 in luma samples, the 1:3/3:1 split along this side is not allowed. Hence, there are at most 8 SBT modes for a CU.

In CE6.4.1e, Quad-tree (QT) split is further used to tile one CU into 4 sub-blocks, and still one sub-block has residual. This SBT type is denoted as SBT-Q.

SBT-V, SBT-H and SBT-Q are allowed for CU with width and height both no larger than maxSbtSize. The maxSbtSize is signaled in SPS. For HD and 4K sequences, maxSbtSize is set as 64 by encoder; for other smaller resolution sequences, maxSbtSize is set as 32.

## Transform type of the sub-block

In CE6.4.1a, position-dependent transform is applied on luma transform blocks in SBT-V and SBT-H (chroma TB always using DCT-2). The two positions of SBT-H and SBT-V are associated with different core transforms. More specifically, the horizontal and vertical transforms for each SBT position is specified in Figure 1. For example, the horizontal and vertical transforms for SBT-V position 0 is DCT-8 and DST-7, respectively. When one side of the residual TU is greater than 32, the corresponding transform is set as DCT-2. Therefore, the sub-block transform jointly specifies the TU tiling, cbf, and horizontal and vertical transforms of a residual block, which may be considered a syntax shortcut for the cases that the major residual of a block is at one side of the block.

In CE6.4.1e, the transform types for SBT-V and SBT-H are identical to CE6.4.1a, and the transform type for SBT-Q is illustrated in Figure 2. For SBT-Q, the residual is assumed to be at one corner the CU. Compared to CE6.4.1a, around 0.08% and 0.10% additional coding gain is achieved in RA and LDB.

Figure 1 Illustration of sub-block transform modes SBT-V and SBT-H

Figure 2 Illustration of sub-block transform modes SBT-Q used in CE6.4.1-e

To test potential gain of using more generic transform type signaling for the sub-block TU, the following three tests are made.

In CE6.4.1b, the horizontal and vertical transform of the sub-block are both DCT-2. This test is to verify the coding gain from using the position dependent transform. Compared to CE6.4.1a, the gain of SBT is reduced by half, i.e., from -0.47% to -0.25% in RA and from -0.83% to -0.42% in LB.

In CE6.4.1c, the sub-block is allowed to use two transform types, and the transform type selection is signaled as a flag. The first transform type is that in CE6.4.1.a, and the second transform type is complimentary to the first transform type. For example, if the first transform type for the sub-block is DCT-8/DST-7 (horizontal transform/vertical transform), the second transform type for the sub-block is DST-7/DCT-8. The second transform type aims to be the most diverse from the first transform type. Compared to CE6.4.1a, around 0.07% and 0.11% additional gain is achieved in RA and LDB.

In CE6.4.1d, the sub-block is allowed to use DST-7 and DCT-8 for both horizontal and vertical transform, i.e., transform type of each dimension is signaled by a flag, like the inter MTS. If CU width is 64 and SBT-H mode is used, the horizontal transform of the sub-block TU is fixed as DCT-2 (since the TU width is 64), and the vertical transform of the sub-block TU is signaled. Similarly, if CU height is 64 and SBT-V mode is used, the vertical transform of the sub-block TU is fixed as DCT-2 and the horizontal transform of the sub-block is signaled. Compared to CE6.4.1a, around 0.05% and 0.07% additional gain is achieved in RA and LDB.

## Fast algorithm

To reduce encoder run-time, some fast algorithms have been developed in RDO of SBT, and commonly used in all tests.

Algorithm 1: For each SBT mode, a RD cost is estimated based on the SSD of the residual-skipped part. An SBT mode is skipped in RDO if the estimated RD cost of the SBT mode is larger than the actual RD cost of the best mode. In addition, only the best 4 SBT (or 6 SBT) modes in terms of the estimated RD cost are tried in RDO of test 6.4.1-a to 6.4.1-d (or test 6.4.1e).

Algorithm 2: a transform mode save & load is applied (which is improved from that proposed in JVET-K0358). The residual energy (i.e., SSD) and the best transform mode (one among whole block transform with DCT-2, whole block transform with inter MTS, and sub-block transform) of a PU is saved as history information. When the residual energy of a PU matches a previous case, the best transform mode associated with the residual energy is tried while the other transform modes are skipped. This fast algorithm reduces the encoding time of both SBT and inter MTS.

Algorithm 3: if the RD cost of whole residual block being transformed by DCT-2 is much worse than the current best RD cost, the SBT is skipped.

Algorithm 4: if the RD cost of whole residual block being transformed by DCT-2 is small enough, the SBT is skipped.

The fast algorithms are all almost lossless (all together around 0.1% loss in RA).

# Simulation results

The proposed methods were integrated into the VTM 3.0 reference software, and are tested in two configurations: VTM, and VTM plus inter MTS. Table 1 summarizes the simulation results of the tests. Detailed results can be found in the attached excel sheets.

Table 1 Simulation results of the two tests

|  |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| Over VTM 3.0 |  |  |  |  |  |  |
| CE6.4.1-a | RA | -0.47% | -0.16% | 0.00% | 108% | 101% |
|  | LB | -0.83% | -0.98% | -0.06% | 113% | 102% |
| CE6.4.1-b | RA | -0.25% | -0.16% | -0.07% | 107% | 101% |
|  | LB | -0.42% | -0.89% | -0.04% | 111% | 101% |
| CE6.4.1-c | RA | -0.54% | -0.20% | 0.08% | 110% | 101% |
|  | LB | -0.94% | -0.75% | 0.23% | 116% | 102% |
| CE6.4.1-d | RA | -0.52% | -0.10% | 0.14% | 114% | 101% |
|  | LB | -0.90% | -0.70% | 0.25% | 121% | 102% |
| CE6.4.1-e | RA | -0.55% | -0.26% | 0.00% | 109% | 101% |
|  | LB | -0.93% | -0.91% | 0.07% | 115% | 102% |
| Over VTM 3.0 + Inter MTS |  |  |  |  |  |  |
| CE6.4.1-a | RA | -0.28% | -0.21% | -0.06% | 98% | 101% |
|  | LB | -0.43% | -0.72% | -0.26% | 96% | 100% |
| CE6.4.1-b | RA | -0.12% | -0.15% | -0.06% | 97% | 101% |
|  | LB | -0.20% | -0.67% | -0.21% | 95% | 101% |
| CE6.4.1-c | RA | -0.33% | -0.25% | 0.03% | 100% | 100% |
|  | LB | -0.53% | -0.88% | -0.28% | 98% | 101% |
| CE6.4.1-d | RA | -0.29% | -0.14% | 0.03% | 103% | 100% |
|  | LB | -0.47% | -0.64% | -0.06% | 102% | 100% |
| CE6.4.1-e | RA | -0.34% | -0.23% | -0.06% | 99% | 101% |
|  | LB | -0.55% | -0.73% | -0.21% | 97% | 101% |

Compared to VTM anchor, the sub-block transform (in CE6.4.1-a) achieves -0.47% and -0.83% gain in RA and LDB. SBT with more flexible transform types (e.g., CE6.4.1-c) gives additional 0.07% gain; SBT with Quad-tree TU tiling (CE6.4.1-e) gives 0.08% gain in RA.

# Combined Tests of JVET-M0140 and JVET-M0046

The proposed method 6.4.1a and 6.4.1e are combined tested with proposal JVET-M0046 CE6-related: A study of primary transforms. The simulation results of 6.4.1.a plus JVET-M0046 test 4/5 and 6.4.1.e plus JVET-M0046 test 4/5 is attached.

Table 2 6.4.1a + JVET-M0046 test 4 (inter MTS off)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.19% | 0.01% | 104% | 100% |
| Class A2 | -0.54% | -0.47% | -0.17% | 109% | 101% |
| Class B | -0.50% | -0.09% | 0.33% | 109% | 102% |
| Class C | -0.47% | -0.28% | 0.11% | 110% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.45% | -0.23% | 0.11% | 108% | 102% |
| Class D | -0.65% | -0.67% | -0.32% | 112% | 103% |
| Class F | -0.35% | -0.18% | -0.11% | 106% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.87% | -1.41% | -0.07% | 115% | 103% |
| Class C | -0.88% | -0.74% | -0.15% | 115% | 103% |
| Class E | -0.41% | 0.63% | -0.17% | 108% | 102% |
| Overall | -0.76% | -0.68% | -0.12% | 113% | 103% |
| Class D | -1.08% | -1.05% | 0.16% | 120% | 105% |
| Class F | -1.33% | -0.88% | -0.53% | 110% | 102% |

Table 3 6.4.1a + JVET-M0046 test 5 (inter MTS on)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 emt3 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.26% | -0.09% | 94% | 99% |
| Class A2 | -0.26% | -0.52% | -0.34% | 98% | 100% |
| Class B | -0.23% | -0.18% | 0.22% | 97% | 100% |
| Class C | -0.27% | -0.19% | -0.03% | 95% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.27% | -0.02% | 96% | 100% |
| Class D | -0.36% | -0.55% | -0.41% | 98% | 101% |
| Class F | -0.27% | -0.19% | -0.09% | 95% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 emt3 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.45% | -0.79% | -0.19% | 94% | 99% |
| Class E | -0.15% | 0.34% | -0.79% | 93% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.60% | -1.33% | -0.24% | 99% | 102% |
| Class F | -1.19% | -0.78% | -1.25% | 93% | 100% |

Table 4 6.4.1e + JVET-M0046 test 4 (inter MTS off)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.18% | 0.03% | 105% | 100% |
| Class A2 | -0.61% | -0.45% | -0.18% | 111% | 101% |
| Class B | -0.56% | 0.00% | 0.25% | 111% | 102% |
| Class C | -0.55% | -0.30% | 0.17% | 111% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.51% | -0.21% | 0.10% | 110% | 102% |
| Class D | -0.74% | -0.58% | -0.37% | 114% | 104% |
| Class F | -0.42% | -0.17% | -0.11% | 107% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -1.02% | -0.63% | -0.25% | 119% | 104% |
| Class E | -0.52% | 0.36% | -0.85% | 109% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -1.19% | -1.17% | 0.66% | 125% | 105% |
| Class F | -1.40% | -0.66% | -0.55% | 111% | 102% |

Table 5 6.4.1e + JVET-M0046 test 5 (inter MTS on)

|  |  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 emt3 |  |  |  |
|  | Y | U | V |  | EncT | DecT |
| Class A1 | -0.13% | -0.27% | -0.03% |  | 95% | 99% |
| Class A2 | -0.33% | -0.48% | -0.31% |  | 99% | 100% |
| Class B | -0.32% | -0.08% | 0.19% |  | 98% | 100% |
| Class C | -0.37% | -0.16% | 0.02% |  | 97% | 102% |
| Class E |  |  |  |  |  |  |
| Overall | -0.30% | -0.22% | 0.00% |  | 97% | 100% |
| Class D | -0.49% | -0.41% | -0.24% |  | 100% | 103% |
| Class F | -0.30% | -0.22% | -0.14% |  | 96% | 101% |
|  |  |  | Low delay B Main10 |  |  |  |
|  |  |  | Over VTM 3.0 emt3 |  |  |  |
|  | Y | U | V |  | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |  | #VALUE! | #VALUE! |
| Class C | -0.58% | -0.91% | -0.09% |  | 96% | 100% |
| Class E | -0.14% | 0.15% | -0.72% |  | 94% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! |  | #VALUE! | #VALUE! |
| Class D | -0.71% | -0.95% | 0.04% |  | 102% | 103% |
| Class F | -1.24% | -0.72% | -0.72% |  | 95% | 101% |

# Tests of removing 64xX and Xx64 shape DST7/DCT8

In this tests, the DST7/DCT8 transform cores are replaced by DCT2 for sub block shape of 64xX and Xx64 (X is smaller than 64) in luma samples. In other word, if any dimension of the sub block size is greater than 32, the DCT2 transform core is used for both horizontal and vertical transform of this block. The simulation results (Table 6 and Table 7) shows 0.02% RA loss compare to 6.4.1a for both inter MTS off and on case.

The tests results on top of 6.4.1a with inter MTS on and off are illustrated in Table 6 and 7.

Table 6 6.4.1a + 64xX and Xx64 DCT2 transform (inter MTS off)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.16% | 0.06% | 105% | 100% |
| Class A2 | -0.53% | -0.44% | -0.17% | 110% | 101% |
| Class B | -0.52% | -0.17% | 0.03% | 110% | 102% |
| Class C | -0.51% | -0.21% | 0.01% | 110% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.45% | -0.23% | -0.01% | 109% | 102% |
| Class D | -0.70% | -0.34% | 0.10% | 113% | 103% |
| Class F | -0.34% | -0.19% | -0.12% | 106% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.94% | -0.78% | -0.05% | 116% | 102% |
| Class E | -0.52% | -0.48% | -0.06% | 107% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -1.11% | -1.45% | 0.39% | 121% | 103% |
| Class F | -1.31% | -1.04% | -0.44% | 110% | 100% |

Table 7 6.4.1a + 64xX and Xx64 DCT2 transform (inter MTS on)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 emt3 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.14% | 0.02% | 97% | 100% |
| Class A2 | -0.26% | -0.42% | -0.27% | 100% | 101% |
| Class B | -0.31% | -0.16% | 0.08% | 100% | 101% |
| Class C | -0.33% | -0.05% | -0.14% | 98% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -0.18% | -0.06% | 99% | 101% |
| Class D | -0.47% | -0.20% | 0.01% | 101% | 103% |
| Class F | -0.27% | -0.26% | -0.21% | 98% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 emt3 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.53% | -0.84% | -0.16% | 97% | 100% |
| Class E | -0.25% | -0.63% | -0.68% | 94% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.70% | -1.35% | -0.46% | 102% | 102% |
| Class F | -1.16% | -0.74% | -0.44% | 96% | 101% |

It is noted that if only replace the DST7/DCT8 as DCT7 for a block size of 32x64 or 64 x32, The simulation results (Table 8 and Table 9) shows 0.01% RA loss compare to 6.4.1a for both inter MTS off and on case.

Table 8 6.4.1a + 64x32 and 32x64 DCT2 transform (inter MTS off)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.07% | 0.08% | 105% | 101% |
| Class A2 | -0.56% | -0.37% | -0.13% | 110% | 102% |
| Class B | -0.52% | -0.15% | 0.03% | 110% | 102% |
| Class C | -0.51% | -0.21% | 0.01% | 110% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.46% | -0.20% | 0.00% | 109% | 102% |
| Class D | -0.70% | -0.34% | 0.10% | 113% | 103% |
| Class F | -0.36% | -0.20% | -0.11% | 106% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.94% | -0.78% | -0.05% | 115% | 101% |
| Class E | -0.52% | -0.48% | -0.06% | 107% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -1.11% | -1.45% | 0.39% | 120% | 103% |
| Class F | -1.30% | -1.09% | -0.38% | 110% | 99% |

Table 9 6.4.1a + 64x32 and 32x64 DCT2 transform (inter MTS on)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 emt3 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.19% | 0.01% | 98% | 101% |
| Class A2 | -0.29% | -0.39% | -0.25% | 100% | 101% |
| Class B | -0.30% | -0.18% | 0.02% | 100% | 101% |
| Class C | -0.33% | -0.05% | -0.14% | 98% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.19% | -0.08% | 99% | 101% |
| Class D | -0.47% | -0.20% | 0.01% | 101% | 103% |
| Class F | -0.27% | -0.26% | -0.22% | 98% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 emt3 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.53% | -0.84% | -0.16% | 97% | 100% |
| Class E | -0.25% | -0.63% | -0.68% | 94% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.70% | -1.35% | -0.46% | 102% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusions

This contribution reports the performance of a sub-block transform method. -0.47% and -0.83% BD-rates are achieved from the basic method in test CE6.4.1-a under CTC RA and LB configuration. More generic transform type selection for SBT increases marginal coding gain, similar to that from further allowing Quad-tree like TU tiling.

It is recommended to adopt the sub-block transform method into VTM.

# Acknowledgement

We would like to thank Tencent, MediaTek, Sony , Samsung and LGE for cross-checking this proposal.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft text

The draft text corresponds to CE6.4.1-a with replacing DST7/DCT8 as DCT2 for all 64xX and Xx64 sub blocks (X is smaller than 64).

#### 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| … |  |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| sps_sbt_enable_flag | u(1) |
| if ( sps_sbt_enable_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.4.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && sps_sbt_enable_flag ) { |  |
| if( cbWidth <= maxSbtSize && cbHeight <= maxSbtSize ) { |  |
| allowSbtVerHalf = cbWidth >= 8 |  |
| allowSbtVerQuad = cbWidth >= 16 |  |
| allowSbtHorHalf = cbHeight >= 8 |  |
| allowSbtHorQuad = cbHeight >= 16 |  |
| if( allowSbtVerHalf ¦ ¦ allowSbtHorHalf ¦ ¦ allowSbtVerQuad ¦ ¦ allowSbtHorQuad ) |  |
| cu_sbt_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( cu_sbt_flag[ x0 ][ y0 ] ) { |  |
| if( ( allowSbtVerHalf ¦ ¦ allowSbtHorHalf ) && ( allowSbtVerQuad ¦ ¦ allowSbtHorQuad ) ) |  |
| cu_sbt_quad_flag[ x0 ][ y0 ] | ae(v) |
| if( ( cu_sbt_quad_flag[ x0 ][ y0 ] && allowSbtVerQuad && allowSbtHorQuad ) ¦ ¦ ( !cu_sbt_quad_flag[ x0 ][ y0 ] && allowSbtVerHalf && allowSbtHorHalf ) ) |  |
| cu_sbt_horizontal_flag[ x0 ][ y0 ] | ae(v) |
| cu_sbt_pos_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |
| } |  |

#### 7.3.4.10 Transform tree syntax

| transform_tree( x0, y0, tbWidth, tbHeight , treeType) { | Descriptor |
| --- | --- |
| if( tbWidth > MaxTbSizeY ¦ ¦ tbHeight > MaxTbSizeY ) { |  |
| trafoWidth = ( tbWidth > MaxTbSizeY ) ? (tbWidth / 2) : tbWidth |  |
| trafoHeight = ( tbHeight > MaxTbSizeY ) ? (tbHeight / 2) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight ) |  |
| if( tbWidth > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType ) |  |
| if( tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType ) |  |
| if( tbWidth > MaxTbSizeY && tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0 + trafoHeight, trafoWidth, trafoHeight, treeType ) |  |
| } else if( cu_sbt_flag[ x0 ][ y0 ] ) { |  |
| factorTb0 = cu_sbt_quad_flag[ x0 ][ y0 ] ? 1 : 2 |  |
| factorTb0 = cu_sbt_pos_flag[ x0 ][ y0 ] ? ( 4 - factorTb0 ) : factorTb0 |  |
| noResiTb0 = cu_sbt_pos_flag[ x0 ][ y0 ] ? 1 : 0 |  |
| if( !cu_sbt_horizontal_flag[ x0 ][ y0 ] ) { |  |
| trafoWidth = tbWidth * factorTb0 / 4 |  |
| transform_unit( x0, y0, trafoWidth, tbHeight, treeType , noResiTb0 ) |  |
| transform_unit( x0 + trafoWidth, y0, tbWidth − trafoWidth, tbHeight, treeType , !noResiTb0 ) |  |
| } |  |
| else { |  |
| trafoHeight = tbHeight * factorTb0 / 4 |  |
| transform_unit( x0, y0, tbWidth, trafoHeight, treeType , noResiTb0 ) |  |
| transform_unit( x0, y0 + trafoHeight, tbWidth, tbHeight − trafoHeight, treeType , !noResiTb0 ) |  |
| } |  |
| } else { |  |
| transform_unit( x0, y0, tbWidth, tbHeight, treeType , 0 ) |  |
| } |  |
| } |  |

#### 7.3.4.11 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType , noResi ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) && !noResi ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && !noResi ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |
| if( ( ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) && sps_mts_intra_enabled_flag ) ¦ ¦ ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ) && sps_mts_inter_enabled_flag ) ) && tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && !cu_sbt_flag[ x0 ][ y0 ] ) |  |
| tu_mts_flag[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_luma[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth ), log2( tbHeight ), 0 ) |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth / 2 ), log2( tbHeight / 2 ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth / 2 ), log2( tbHeight / 2 ), 2 ) |  |
| } |  |

Note that in VTM 3.0, the implementation of tu_cbf_cb, tu_cbf_cr and tu_cbf_luma is not fully aligned with the syntax table in VVC draft 3. In general, VTM 3.0 uses HEVC RQT-like signaling of cbf in transform_tree() syntax structure, while the VVC draft 3 simply signals tu_cbf_luma, tu_cbf_cb and tu_cbf_cr in transform_unit() syntax structure, without avoiding the redundancy between cu_cbf and tu_cbf_luma in the case that tu_cbf_cb and tu_cbf_cr are both 0..

Therefore, the proponent provides draft text corresponding to VTM3.0 cbf signaling, as shown in the following two tables (changes from the VVC draft 3 are highlighted in blue). On top of it, the changes from SBT on cbf syntax are added (highlighted in yellow). For clarity, the following two tables are not prepared in trace mode.

#### 7.3.4.10 Transform tree syntax

| transform_tree( x0, y0, tbWidth, tbHeight , treeType, trafoDepth, xBase, yBase ) { | Descriptor |
| --- | --- |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) && !( cu_sbt_flag[ x0 ][ y0 ] && trafoDepth = = 1 ) ) |  |
| tu_cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( ( trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) && !( cu_sbt_flag[ x0 ][ y0 ] && trafoDepth = = 1 ) ) |  |
| tu_cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( tbWidth > MaxTbSizeY ¦ ¦ tbHeight > MaxTbSizeY ) { |  |
| trafoWidth = ( tbWidth > MaxTbSizeY ) ? (tbWidth / 2) : tbWidth |  |
| trafoHeight = ( tbHeight > MaxTbSizeY ) ? (tbHeight / 2) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight , treeType, trafoDepth + 1, x0, y0 ) |  |
| if( tbWidth > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| if( tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| if( tbWidth > MaxTbSizeY && tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0 + trafoHeight, trafoWidth, trafoHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| } else if( cu_sbt_flag[ x0 ][ y0 ] && trafoDepth = = 0 ) { |  |
| factorTb0 = cu_sbt_quad_flag[ x0 ][ y0 ] ? 1 : 2 |  |
| factorTb0 = cu_sbt_pos_flag[ x0 ][ y0 ] ? ( 4 - factorTb0 ) : factorTb0 |  |
| noResiTb0 = cu_sbt_pos_flag[ x0 ][ y0 ] ? 1 : 0 |  |
| if( !cu_sbt_horizontal_flag[ x0 ][ y0 ] ) { |  |
| trafoWidth = tbWidth * factorTb0 / 4 |  |
| if ( !noResiTb0 ) |  |
| transform_tree( x0, y0, trafoWidth, tbHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| else |  |
| transform_tree( x0 + trafoWidth, y0, tbWidth − trafoWidth, tbHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| } |  |
| else { |  |
| trafoHeight = tbHeight * factorTb0 / 4 |  |
| if ( !noResiTb0 ) |  |
| transform_tree( x0, y0, tbWidth, trafoHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| else |  |
| transform_tree( x0, y0 + trafoHeight, tbWidth, tbHeight − trafoHeight, treeType, trafoDepth + 1, x0, y0 ) |  |
| } |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ ( trafoDepth != 0 && !cu_sbt_flag[ x0 ][ y0 ] ) ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| tu_cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| transform_unit( x0, y0, tbWidth, tbHeight, treeType ) |  |
| } |  |
| } |  |

#### 7.3.4.11 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |
| if( ( ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) && sps_mts_intra_enabled_flag ) ¦ ¦ ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ) && sps_mts_inter_enabled_flag ) ) && tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && !cu_sbt_flag[ x0 ][ y0 ] ) |  |
| tu_mts_flag[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_luma[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth ), log2( tbHeight ), 0 ) |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth / 2 ), log2( tbHeight / 2 ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth / 2 ), log2( tbHeight / 2 ), 2 ) |  |
| } |  |

#### 7.4.5.XX Transform tree semantics

tu_cbf_luma[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the luma transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When tu_cbf_luma[ x0 ][ y0 ][ trafoDepth ] is not present, the value of tu_cbf_luma[ x0 ][ y0 ][ trafoDepth ] is inferred as follows:

- If treeType is not equal to DUAL_TEE_CHROMA, tu_cbf_luma[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1

- Otherwise, tu_cbf_luma[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

tu_cbf_cb[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cb transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the top-left location ( x0, y0 ) of the considered transform unit. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When tu_cbf_cb[ x0 ][ y0 ][ trafoDepth ] is not present, the value of tu_cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred as follows:

- If trafoDepth is greater than 0, tu_cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, tu_cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

tu_cbf_cr[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cr transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the top-left location ( x0, y0 ) of the considered transform unit. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When tu_cbf_cr[ x0 ][ y0 ][ trafoDepth ] is not present, the value of tu_cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred as follows:

- If trafoDepth is greater than 0, cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to tu_cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, tu_cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

#### 7.4.3.1 Sequence parameter set RBSP semantics

sps_sbt_enabled_flag equal to 0 specifies that sub-block transform for inter-predicted CU is disabled. sps_sbt_enabled_flag equal to 1 specifies that sub-block transform for inter-predicted CU is enabled.

sps_sbt_max_size_64_flag equal to 0 specifies that the maximum CU width and height for allowing sub-block transform is 32. slice_sbt_max_size_64_flag equal to 1 specifies that the maximum CU width and height for allowing sub-block transform is 64.

maxSbtSize = sps_sbt_max_size_64_flag? 64 : 32

#### 7.4.5.5 Coding unit semantics

cu_sbt_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, sub-block transform is used. cu_sbt_flag[ x0 ][ y0 ] equal to 0 specifies that for the current coding unit, sub-block transform is not used.

When cu_sbt_flag[ x0 ][ y0 ] is not present, its value is inferred to be equal to 0.

NOTE – : When sub-block transform is used, a coding unit is tiled into two transform units; one transform unit has residual, the other does not have residual.

cu_sbt_quad_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, the sub-block transform includes a transform unit of 1/4 size of the current coding unit. cu_sbt_quad_flag[ x0 ][ y0 ] equal to 0 specifies that for the current coding unit the sub-block transform includes a transform unit of 1/2 size of the current coding unit.

When cu_sbt_quad_flag[ x0 ][ y0 ] is not present, its value is inferred to be equal to 0.

cu_sbt_horizontal_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is tiled into 2 transform units by a horizontal split. cu_sbt_horizontal_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is tiled into 2 transform units by a vertical split.

When cu_sbt_horizontal_flag[ x0 ][ y0 ] is not present, its value is derived as follows:

- If cu_sbt_quad_flag[ x0 ][ y0 ] is equal to 1, cu_sbt_horizontal_flag[ x0 ][ y0 ] is set to be equal to allowSbtHoriQuad.

- Otherwise (cu_sbt_quad_flag[ x0 ][ y0 ] is equal to 0), cu_sbt_horizontal_flag[ x0 ][ y0 ] is set to be equal to allowSbtHoriHalf.

cu_sbt_pos_flag[ x0 ][ y0 ] equal to 1 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the first transform unit in the current coding unit are not present in the bitstream. cu_sbt_pos_flag[ x0 ][ y0 ] equal to 0 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the second transform unit in the current coding unit are not present in the bitstream.

### 8.4.4 Transformation process for scaled transform coefficients

#### 8.4.4.1 General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

When cu_sbt_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Table 8-X depending on cu_sbt_horizontal_flag[ xTbY ][ yTbY ] and cu_sbt_pos_flag[ xTbY ][ yTbY ].

When cu_sbt_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is greater than 0, the variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are both derived to be equal to 0.

When cu_sbt_flag[ xTbY ][ yTbY ] is equal to 0, the variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Table 8-16 depending on mts_idx[ xTbY ][ yTbY ][ cIdx ].

The variables nonZeroW and nonZeroH are derived as follows:

nonZeroW = Min( nTbW, 32 )		(8-810)

nonZeroH = Min( nTbH, 32 )		(8-811)

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

- Each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nonZeroH − 1 is transformed to e[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.4.4.2 for each column x = 0..nonZeroW − 1 with the height of the transform block nTbH, the non-zero height of the scaled transform coefficients nonZeroH, the list d[ x ][ y ] with y = 0..nonZeroH − 1 and the transform type variable trType set equal to trTypeVer as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- The intermediate sample values g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )	(8-812)

- Each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.4.4.2 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the non-zero width of the resulting array g[ x ][ y ] nonZeroW, the list g[ x ][ y ] with x = 0..nonZeroW − 1 and the transform type variable trType set equal to trTypeHor as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

Table 8-X – Specification of trTypeHor and trTypeVer depending on cu_sbt_horizontal_flag[ x ][ y ] and cu_sbt_pos_flag[ x ][ y ]

|  | cu_sbt_horizontal_flag [ xTbY ][ yTbY ] = = 0 |  | cu_sbt_horizontal_flag [ xTbY ][ yTbY ] = = 1 |  |
| --- | --- | --- | --- | --- |
|  | cu_sbt_pos_flag [ xTbY ][ yTbY ] = = 0 | cu_sbt_pos_flag [ xTbY ][ yTbY ] = = 1 | cu_sbt_pos_flag [ xTbY ][ yTbY ] = = 0 | cu_sbt_pos_flag [ xTbY ][ yTbY ] = = 1 |
| trTypeHor | nTbH > 32 ? 0 : 2 | nTbH > 32 ? 0 : 1 | nTbW > 32 ? 0 : 1 | nTbW > 32 ? 0 : 1 |
| trTypeVer | nTbH > 32 ? 0 : 1 | nTbH > 32 ? 0 : 1 | nTbW > 32 ? 0 : 2 | nTbW > 32 ? 0 : 1 |

Table 8-16 – Specification of trTypeHor and trTypeVer depending on mts_idx[ x ][ y ][ cIdx ]

| mts_idx[ xTbY ][ yTbY ][ cIdx ] | trTypeHor | trTypeVer |
| --- | --- | --- |
| −1 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 2 | 1 |
| 2 | 1 | 2 |
| 3 | 2 | 2 |

### 9.5.3 Binarization process

Table 9-4 – Syntax elements and associated binarizations

| coding_unit( ) | cu_skip_flag[ ][ ] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | pred_mode_flag | FL | cMax = 1 |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_ref_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | intra_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | … |  |  |
|  | cu_sbt_flag[ ][ ] | FL | cMax = 1 |
|  | cu_sbt_quad_flag[ ][ ] | FL | cMax = 1 |
|  | cu_sbt_horizontal_flag[ ][ ] | FL | cMax = 1 |
|  | cu_sbt_pos_flag[ ][ ] | FL | cMax = 1 |

### 9.5.4.2 Derivation process for ctxTable, ctxIdx and bypassFlag

Table 9-10 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_tile_group_flag | terminate | na | na | na | na | na |
| alf_ctb_flag[ ][ ][ ] | 0..8 (clause 9.5.4.2.2) | na | na | na | na | na |
| … |  |  |  |  |  |  |
| cu_cbf | 0 | na | na | na | na | na |
| cu_sbt_flag[ ][ ] | 0 | na | na | na | na | na |
| cu_sbt_quad_flag[ ][ ] | 0 | na | na | na | na | na |
| cu_sbt_horizontal_flag[ ][ ] | ( cbWidth = = cbHeight ) ? 0 : ( (cbWidth < cbHeight ) ? 1 : 2 ) | na | na | na | na | na |
| cu_sbt_pos_flag[ ][ ] | 0 | na | na | na | na | na |