| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0148-v2 |
| --- | --- |

| Title: | Non-CE9: Simplifications to DMVR search pattern and interpolation for refinement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sriram Sethuraman | Email: | sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

In this proposal, two simplifications to the DMVR studied in CE9.2 are proposed. The first simplification is related to changing the refinement range from a square shaped pattern to a rhombus shaped pattern with the same +/-2 refinement range in order to facilitate simultaneous evaluation of cost at all 13 positions in order to get the best timing in hardware. Along with this simplification, for the purposes of average decoder time reduction, a 2-stage search pattern is proposed. Also, to counter the BDRATE drop due to the reduced number of refinement positions, the parametric 2-D error surface concept of JVET-K0041 is extended to consider a 1-D parametric error surface. These changes together result in a 0.08% BDRATE drop relative to CE9.2.1g.

The second simplification is related to the bilinear interpolation performed for DMVR. Currently, bilinear interpolation is performed at 1/16th of a pixel precision for the fractional part of the merge MV in the horizontal and vertical directions. This requires 10-bit x 4-bit multiplications for horizontal interpolation, and 14-bit x 4-bit multiplications for the vertical interpolation. Integer grid samples based refinement proposed in JVET-K0275 obviates the need for such interpolation and has an average BDRATE drop of ~0.15% when compared to using bilinear interpolation. In this proposal, a multiplication-free interpolation option is proposed that uses quarter-pixel accuracy when performing 1-D interpolation and half-pixel accuracy when performing 2-D interpolation and retains 0.11% BDRATE gain over integer grid samples based refinement.

The combination results of these two are presented with three different cost functions of which two of them facilitate a row-level pipeline between interpolation for refinement and the cost evaluations at the refinement positions.

# Introduction

The current DMVR methods studied in CE9.2 employ a square +/-2 refinement range of positions. For achieving the best possible timing for DMVR in hardware implementations, it is preferable for the cost calculations to happen concurrently for all possible refinement positions. However, performing concurrent evaluation of cost at 25 positions has a significant impact on the gate count of the hardware. In this proposal, a method is suggested to reduce the refinement range to a rhombus-shaped refinement range. Also, in order to ease the software average decoding complexity, the 13-point evaluation is broken into two stages of cost evaluation so that the second stage can be skipped when the first stage refinement is satisfactory. Further, the parametric 2-D error surface based sub-pixel delta-MV estimation from integer-distance cost values has been extended to consider a 1-D parametric error surface as well.

The second improvement to DMVR that is proposed is related to simplifications to the interpolation performed for refinement by having a coarser level of precision in sub-pixel accuracy at which bilinear interpolation is performed.

Section 2 provides the detailed description of these two proposed simplifications to DMVR.

The results with these two proposal in isolation and in combination and for 3 different cost functions are presented in Section 3 and compared against CE9.2.1(g) which offers the best performance among the CE9.2.1 sub-tests. In combination with SAD or PR-MR-SAD cost function, the proposed simplifications help DMVR achieve the best possible hardware timing and also serve to reduce the gate count and average decoding time.

# Proposed Simplifications

## 13-point rhombus refinement range

The refinement range used in the current DMVR method is a square +/-2 range that has 5x5=25 possible integer delta motion vectors. For achieving the best timing, hardware designs desire simultaneous evaluation of costs at all refinement positions. However, evaluating 25 positions can have a significant impact on gate count. In this proposal, a rhombus-shaped refinement range covering 13 integer distance refinement positions around the merge MV centers in L0 and L1 reference is proposed.  The additional advantage of the rhombus shaped refinement range when used in conjunction with evaluating the cost function only on alternate rows is that while the 25-positions require 15 positions on even lines and 10 positions on odd lines of the refinement range data, the 13-positions require 7 positions on even lines and 6 positions on odd lines. This allows only 7 row-level cost evaluation units to handle all the cost evaluation in a row-level pipeline between interpolation for refinement and cost calculations. Hence, with the rhombus shaped search range and alternate row based cost evaluation, the gate count for concurrent evaluation of costs at all refinement positions can be kept minimal while achieving the best timing. Such simplification can allow sub-PU DMVR and final motion compensation to possibly fit within the same pipeline slot that can keep the overall internal memory requirements due to DMVR minimal.

### 1-D parametric error surface based sub-pixel distance estimation

Reducing the integer distance refinement range with the rhombus-shaped refinement range results in the parametric 2-D error surface proposed first in JVET-K0041 to be possible only when the delta-MV is zero. At all the other delta-MVs, all the 4 integer-distance costs at positions above, below and to the left and right of the position with the least cost are not guaranteed to be available. In addition to the reduction in integer distance search positions, this also can adversely impact the compression gains possible with the earlier refinement range and 2-D parametric error surface based sub-pixel distance estimation. To offset some of these compression losses, a 1-D parametric error surface is proposed. With 1-D parametric error surface, the cost values at only 2 positions around the best cost position are required. Including these 2 positions, the 3 cost values must be on a straight line which can be a horizontal line, vertical line, or a diagonal or anti-diagonal line. When the line is horizontal or vertical, the sub-pixel displacement computed from the 1-D error surface provides a sub-pixel distance only in the horizontal or vertical directions respectively. When the line is diagonal, the sub-pixel displacement computed is applied to both the horizontal and vertical directions. With the 1-D error surface, the 13-point rhombus shaped refinement range offers a minimal drop in BDRATE when compared to the earlier search pattern that covered 25 positions.

### 2-stage search pattern for average decoding time reduction

While hardware designs may desire to evaluate all the 13-position costs unconditionally for the best timing, the earlier search pattern was performing only 9 cost evaluations in software by conditionally evaluating the costs in 4-stages. In order to provide a similar advantage, the normative search is modified to be done in 2 stages. In the first stage, the center and its 4 1-pixel neighbors, namely, its left, top, right, and bottom neighbor positions are evaluated. If the best cost is found at the center of these 5 positions, then the second stage is skipped. On the other hand, if the best cost position is one among the 4 neighbor positions, a second set of 5 positions are evaluated with the best position as the center. These 5 positions are chosen as follows. The 1-pixel horizontal-only or vertical-only neighbor positions s of the best cost position that have not been evaluated at so far form 3 of these positions. The 2 additional positions form the 2 vertices out of the remaining 3 vertices of the rhombus that are not part of the 3 additional positions and chosen to be the 2 closest to the 2 diagonal positions added as part of the 3 additional positions so that diagonal 1-D error surface can be evaluated for those positions.

## Lower complexity multiplication-free bilinear interpolation for refinement

The current bilinear interpolation employed for performing the refinement is done at 1/16th of a pixel accuracy. Hence, the horizontal interpolation requires 2 10-bit x 4-bit multiplications per horizontal interpolation output sample for an input sample bit-depth of 10-bits. The intermediate bit-depth is maintained at 14-bits after horizontal interpolation and the vertical interpolation stage requires 2 14-bit x 4-bit multiplications per output sample which is finally rounded and shifted right by 4-bits to keep the intermediate bit-depth of 14-bits. Hence, the bilinear interpolation for refinement requires 4 multiplications per output sample for each of the (sub-PU-w + 4) x (sub-PU-h + 4) samples required for performing the refinement and for the worst-case of an 8x8 CU, the number of multiplications become a substantial percentage of the final DCTIF-based 2-D separable interpolation.

In order to reduce the complexity of the interpolation for refinement, a multiplication-free method is preferred. Though the integer grid of samples based DMVR as proposed in JVET-K0275 offers the lowest complexity by not requiring any interpolation for refinement, it suffers a 0.15% BDRATE drop when compared to bilinear interpolation. In this proposal, a reduced complexity bilinear interpolation scheme is proposed by considering a reduced sub-pixel precision for the interpolation.

In the proposed method, if horizontal sub-pixel phase and vertical sub-pixel phase both fall between 5/16 and 11/16 (both inclusive), then half-pixel position in both horizontal and vertical directions is used. If horizontal sub-pixel phase is less than or equal to 4/16 or greater than or equal to 12/16, then the vertical sub-pixel phase is rounded to the nearest quarter pixel grid position (i.e. 0, 4/16, 8/16, or 12/16). Else, if the vertical sub-pixel phase is less than or equal to 4/16 or greater than or equal to 12/16, then the horizontal sub-pixel phase is rounded to the nearest quarter pixel grid position (i.e. 0, 4/16, 8/16, or 12/16).

This is illustrated for two examples in the following two figures Fig.1A and Fig.1B.

Fig. 1A: An example of rounding the 1/16th pel accurate MV to a 2-D half-pixel accurate position

Fig. 1B: An example of rounding the 1/16th pel accurate MV to a 1-D quarter-pixel accurate position

motion

vector

Integer sample coordinate

Half sample coordinate

Rounded motion vector

Horizontal Quarter-pel sample coordinate

Vertical Quarter-pel sample coordinate

motion vector

Integer sample coordinate

Half sample coordinate

Rounded motion vector

Horizontal Quarter-pel sample coordinate

Vertical Quarter-pel sample coordinate

The half-pixel precision in both the directions just requires summing the four corner integer pixel positions surrounding the half-pixel position and then perform a rounding and shift by 2-bits to obtain the interpolated sample. Thus, nor multiplications are required. Though the quarter pixel accurate 1-D interpolations seem to  require a multiplication by 3 when the quarter pixel  accurate position does not coincide with a half-pixel accurate position, this can be written as a sum of a value left shifted by 1 and summed with the same value. When written in this manner, the complexity of this interpolation is the same as that of the interpolation with half-pixel accurate in both directions. With the proposed method, the complexity of the interpolation is only the additions/shift operations required earlier with all the multiplications eliminated. It is seen that this type of interpolation for refinement is able to recover most of the compression gains offered by bilinear interpolation at 1/16th of a pixel precision over the integer sample grid based refinement. The intermediate bit-depth used in this case is kept at 10-bits and this also helps reduce the memory requirements in hardware when compared to the 14-bit precision used earlier.

# Results

The following test results are reported related to the proposals against VTM3.0 under CTC, wherein the proposed changes are made the CE9.2.1(g) configuration that employed 16x16 sub-PU, bilinear interpolation for refinement, MR-SAD as the cost function with cost computed only on alternate rows of the blocks, and allowed use of refined MVs for purposes other than MC of current CU,

- Test-1: Replacing the square-shaped refinement range of CE9.2.1(g) with a 13-point rhombus shaped refinement range, 2-stage (5 + 5) cost evaluations, and addition of 1-D error surface, while still computing the cost only on alternate rows of the blocks

- Test-2: Replacing the 1/16th of a pixel accurate bilinear interpolation for refinement in CE9.2.1(g) with proposed multiplication-free bilinear interpolation for refinement

- Test-3: Combination of the changes in Tests 1 and 2 done in CE9.2.1(g)

For comparison, the results of CE9.2.1(g) are shown below.

Table 1: Results for CE9.2.1(g) over which the proposed changes are made

Table 2: Results of Test-1 against VTM3.0

By reducing the refinement range from a square shaped refinement range covering 25 refinement positions to a 13-position rhombus shaped refinement range and by imposing the sequential 2-stage (5 cost evaluations per stage) option and with the addition of parametric 1-D error surface, there is an overall BDRATE drop of 0.08% over the results of CE9.2.1(g). It should also be noted that an additional restriction is introduced in Test, 1, 2 and 3 wherein a coding unit of width=4 shall only perform refinement in a +/-1 square shaped refinement range in order to cap the worst-case complexity arising from +/-2 refinement range for width=4 CUs.

Table 3: Results of Test-2 against VTM3.0

The proposed 2-D half-pel or 1-D qpel accurate bilinear interpolation drops only 0.04% in average BDRATE over performing 1/16th of a pixel accurate bilinear interpolation for refinement.

Table 4: Results of Test-3 involving the combination of Tests 1 and 2 against VTM3.0

The combination of both the proposed complexity reduction methods results in only a 0.11% BDRATE drop.

Since MR-SAD cost function still does not allow row-level pipelining to be done between interpolation for refinement and the symmetric bilateral matching cost function calculations, the combination results are also shown where MR-SAD is replaced by PR-MR-SAD and SAD respectively as these two cost functions allow row-level pipelining and working with just 2 sets of 5 line buffers of interpolated samples.

- Test-4: Replacing MR-SAD by PR-MR-SAD in Test-3

- Test-5: Replacing MR-SAD by SAD in Test-3

Table 5: Results of Test-4

It can be seen that PR-MR-SAD loses an additional 0.05% BDRATE when compared to block level MR-SAD.

Table 6: Results of Test-5

It can be seen that SAD as the cost function drops 0.1% BDRATE with respect to MR-SAD as the cost function and 0.05% BDRATE with respect to PR-MR-SAD as the cost function.

# Conclusion

Two simplifications to DMVR are proposed in this contribution. The first simplification is related to the refinement range to allow concurrent evaluation of costs at all refinement positions to enable the best hardware timing for DMVR. This change has 0.08% impact on the average luma BDRATE gain when compared to using the square-shaped +/-2 refinement range of CE9.2.1(g). The second simplification is related to making the bilinear interpolation performed for DMVR multiplication-free. This is achieved by reducing the sub-pixel accuracy at which interpolation is performed. This change has a 0.04% impact on the average luma BDRATE gain when compared to 1/16th of a pixel accurate bilinear MC. The combination of these two changes results in a 0.11% BDRATE drop relative to CE9.2.1(g). In order to facilitate a row-level pipeline for DMVR between the interpolation for refinement and the cost calculations, the results with two cost functions that do not require block-level mean to be computed are also presented. It is requested that the configuration of Test-4 be considered for adoption in the next version of VTM.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).