| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0149-v2 |
| --- | --- |

| Title: | Non-CE3: On simplification of PDPC basic equation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Jianle Chen | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co. |  |  |

_____________________________

# Abstract

This document proposes a simplification of PDPC basic equation. In Test 1, a top-left reference sample is excluded from PDPC basic equation that reportedly shows the following overall results: 0.01% (Y), -0.01% (U), 0.02% (V) and 0.01% (Y), 0.05% (U), 0.06% (V) for AI and RA configurations, respectively. In Test 2, a clipping operation is removed from PDPC basic equation due to minor modification of PDPC for horizontal and vertical modes that reportedly shows the following overall results: 0.01% (Y), -0.02% (U), -0.01% (V) and 0.00% (Y), 0.09% (U), 0.07% (V) for AI and RA configurations, respectively. The running time is changed negligibly at both encoder and decoder sides for AI and RA configurations as compared with VTM-3.0.

# Introduction

PDPC (Position Dependent Prediction Combination) is an efficient tool used to improve compression performance of conventional intra prediction modes. However, PDPC can increase worst-case complexity. In this contribution, we propose several ways of how to reduce its adverse impact on worst-case complexity by simplifying PDPC basic equation that currently is as follows:

, (1)

where  is a predicted samples at position (x,y);  is a clipping operation to keep predicted samples within a range of valid values;  are left, top and top-left reference samples, respectively;  are weights for left, top and top-left reference samples, respectively.

# Proposal

In this contribution, we propose 2 modifications of PDPC basic equation. The 1st modification is to exclude the term related to a top-left reference sample for DC intra prediction mode so that PDPC basic equation can be rewritten as follows:

(2)

Horizontal and vertical modes are not touched by modification 1.

The 2nd modification is to remove a clipping operation from (2). After removing a top-left reference sample for DC intra prediction mode, the only reason to keep a clipping operation in (2) is that PDPC for horizontal and vertical modes can cause valid range violation. So, we propose to clip the difference between top or left reference samples, on the one hand, and a top-left reference sample, on the other hand, for horizontal or vertical modes in advance using the following expressions:

for vertical mode:

,

for horizontal mode:

where T() is a thresholding operation.

This thresholding operation is defined as follows:

- when , the lower limit is obtained as , the value of T(a) is max(a, );

- otherwise, the upper limit is obtained as , the value of T(a) is max(a, )

The 2nd modification results in the following PDPC basic equation (by formal substitution of  or  by the corresponding result of thresholding operation):

.                 (3)

A set of the proposed tests is summarized in Table 1.

Table 1. Summary of the proposed tests

| Test # | Description |
| --- | --- |
| 1 | Excluding the term related to a top-left reference sample from PDPC basic equation |
| 2 | Removing a clipping operation from PDPC basic equation |

# Experiment Results

The results reportedly obtained for tests 1-2 using the common test conditions [1] are presented in Table 2-Table 3 for AI and RA configurations. The tables also include results for the TGM Class.

Table 2. Experimental results for Test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.06% | 99% | 100% |
| Class A2 | 0.01% | -0.01% | 0.03% | 99% | 99% |
| Class B | 0.02% | 0.01% | 0.03% | 100% | 99% |
| Class C | 0.01% | -0.02% | -0.14% | 99% | 99% |
| Class E | 0.02% | -0.03% | 0.14% | 100% | 99% |
| Overall | 0.01% | -0.01% | 0.02% | 99% | 99% |
| Class D | 0.00% | 0.15% | -0.02% | 99% | 99% |
| Class F | 0.01% | -0.01% | 0.00% | 99% | 99% |
| TGM | 0.00% | -0.01% | -0.02% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | -0.01% | 99% | 100% |
| Class A2 | 0.01% | 0.01% | 0.17% | 99% | 100% |
| Class B | 0.00% | 0.14% | -0.03% | 99% | 99% |
| Class C | 0.02% | -0.05% | 0.15% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.06% | 99% | 100% |
| Class D | -0.01% | -0.48% | -0.21% | 100% | 99% |
| Class F | -0.03% | 0.13% | 0.07% | 99% | 99% |
| TGM | -0.03% | -0.07% | -0.02% | 100% | 100% |

Table 3. Experimental results for Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | 0.08% | 99% | 99% |
| Class A2 | 0.01% | 0.05% | 0.01% | 99% | 100% |
| Class B | 0.01% | 0.01% | 0.01% | 99% | 100% |
| Class C | 0.00% | -0.01% | -0.12% | 99% | 99% |
| Class E | 0.01% | -0.10% | 0.00% | 99% | 100% |
| Overall | 0.01% | -0.02% | -0.01% | 99% | 100% |
| Class D | -0.01% | 0.06% | -0.01% | 100% | 100% |
| Class F | -0.07% | -0.02% | -0.02% | 99% | 100% |
| TGM | -0.76% | -0.66% | -0.65% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.06% | 99% | 99% |
| Class A2 | -0.01% | 0.10% | 0.15% | 99% | 99% |
| Class B | -0.01% | 0.18% | 0.01% | 99% | 100% |
| Class C | 0.03% | 0.00% | 0.11% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.09% | 0.07% | 99% | 99% |
| Class D | -0.01% | -0.59% | -0.17% | 99% | 100% |
| Class F | -0.05% | -0.19% | -0.09% | 99% | 100% |
| TGM | -0.34% | -0.32% | -0.32% | 100% | 100% |

# Conclusion

We propose to adopt the proposed simplifications of PDPC basic equation into the VTM software and the VVC spec draft.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th JVET Meeting, Macao, China, October 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).