| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0160-v1 |
| --- | --- |

| Title: | AHG17: Flexible tile grouping for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add support in VVC for flexible and rectangular tile grouping and the flag for filtering processing across tile group boundaries in addition to the flag for filtering processing across tile boundaries. The suggested syntax addition and modification and semantics are provided.

# Introduction

JVET-L0686-v2 [2], adopted in the JVET L-meeting, specifies the tile group in substitution of the classic slice in prior standards, e.g. HEVC. The picture parameter set, the tile group header and data syntax are as follows.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### Tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| if ( tile_group_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

#### General tile group data syntax

| tile_group_data( ) { | Descriptor |
| --- | --- |
| tileIdx = tile_group_address |  |
| for( i = 0; i <= num_tiles_in_tile_group_minus1; i++, tileIdx++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ tileIdx ] |  |
| for( j = 0; j < NumCtusInTile[ tileIdx ]; j++, ctbAddrInTs++ ) { |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < num_tiles_in_tile_group_minus1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |

Pictures are divided into tile groups and tiles. A tile group is a sequence of tiles in tile raster scan of a picture. A tile is a sequence of CTUs that cover a rectangular region of a picture. A picture may be divided into 12 tiles and 3 tile groups as shown in Figure 6-4.

Figure 6-4 – A picture with 18 by 12 luma CTUs that is partitioned into 12 tiles and 3 tile groups (informative)

# Proposed method

For use cases of viewport dependent streaming and processing, rectangular viewports or rectangular regions of interest are often used in the applications. Currently, tile grouping [2] forms a tile group in tile scan (raster scan in tile unit). To support flexible position and rectangular shape of tile grouping for VVC based on JVET-L0686-v2, the proposed syntax addition and modification are highlighted as follows. And the flag for filtering processing across tile group boundaries is added in addition to the flag for filtering processing across tile boundaries.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if(loop_filter_across_tiles_enabled_flag) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### Tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| rectangular_tile_group_enabled_flag | u(1) |
| if( rectangular_tile_group_enabled_flag ) { |  |
| num_tile_columns_in_tile_group_minus1 | ue(v) |
| num_tile_rows_in_tile_group_minus1 | ue(v) |
| } else { |  |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| } |  |
| tile_group_type | ue(v) |
| if ( tile_group_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

#### General tile group data syntax

| tile_group_data( ) { | Descriptor |
| --- | --- |
| if( rectangular_tile_group_enabled_flag ) { |  |
| tileIdx = tile_group_address |  |
| for( m = 0; m <= num_tile_rows_in_tile_group_minus1; m++, tileIdx++ ) { |  |
| for( i = 0; i <= num_tile_columns_in_tile_group_minus1; i++, tileIdx++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ tileIdx ] |  |
| for( j = 0; j < NumCtusInTile[ tileIdx ]; j++, ctbAddrInTs++ ) { |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < num_tiles_in_tile_group_minus1 ) |  |
| byte_alignment( ) |  |
| } |  |
| tileIdx+= num_tile_columns_minus1 |  |
| } |  |
| } else { |  |
| tileIdx = tile_group_address |  |
| for( i = 0; i <= num_tiles_in_tile_group_minus1; i++, tileIdx++ ) { |  |
| ctbAddrInTs = FirstCtbAddrTs[ tileIdx ] |  |
| for( j = 0; j < NumCtusInTile[ tileIdx ]; j++, ctbAddrInTs++ ) { |  |
| CtbAddrInRs = CtbAddrTsToRs[ ctbAddrInTs ] |  |
| coding_tree_unit( ) |  |
| } |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| if( i < num_tiles_in_tile_group_minus1 ) |  |
| byte_alignment( ) |  |
| } |  |
| } |  |
| } |  |

Semantics

loop_filter_across_tiles_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile boundaries except tile group boundaries in pictures referring to the PPS. loop_filter_across_tiles_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_tiles_enabled_flag is inferred to be equal to 1.

loop_filter_across_tile_groups_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile group boundaries in pictures referring to the PPS. loop_filter_across_tile_group_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile group boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_tile_groups_enabled_flag is inferred to be equal to 1. When loop_filter_across_tiles_enabled_flag equal to 0, loop_filter_across_tile_groups_enabled_flag shall be 0.

rectangular_tile_group_enabled_flag equal to 1 specifies that tile group area is in rectangular shape.

num_tile_columns_in_tile_group_minus1 plus 1 specifies the number of tile columns partitioning the rectangular tile group. num_tile_columns_in_tile_group_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of num_tile_columns_minus1 is inferred to be equal to 0.

num_tile_rows_in_tile_group_minus1 plus 1 specifies the number of tile rows partitioning the rectangular tile group. num_tile_rows_in_tile_group_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of num_tile_rows_minus1 is inferred to be equal to 0.

As an example, a picture may be divided into 12 tiles and 3 rectangular tile groups (in three colors: blue, yellow, and red) as shown in Figure below.

# References

- “Versatile Video Coding (Draft 3)”, JVET-L1001-v4, October 2018, Macau, China.

- “Spec text for the agreed starting point on slicing and tiling”, JVET-L0686-v2, October 2018, Macau, China.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).