| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0170-v1 |
| --- | --- |

| Title: | CE4.3.1: Shared merging candidate list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chia Chen, Yu-Cheng Lin, Man-Shu Chiang, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chunchia.chen, yucheng.lin,, man-shu.chiang, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the test results of Core Experiment 4.3.1 (CE4.3.1). It proposes to share the same merging candidate list for all leaf coding units (CUs) of one ancestor node in the CU split tree for enabling parallel processing of small skip/merge-coded CUs. The ancestor node is named merge sharing node. The shared merging candidate list is generated at the merge sharing node pretending the merge sharing node is a leaf CU. When the proposed method is applied to VTM3.0, luma BD-rates are 0.00% and 0.02% for RA and LB under an area threshold equal to 32, respectively; the luma BD-rates are 0.04% and 0.08% for RA and LB under the threshold equal to 64, respectively. Other test results of different thresholds are also provided. It is claimed that the shared merging candidate list can effectively reduce difficulty for real-time 8K hardware decoding.

# Introduction

In a high-end (e.g., 8K 120fps) video decoder, a parallel-design-friendly algorithm is preferred. However, merging candidate list construction of one CU cannot be computed in parallel at the same time with that of the previous CU. Table 1 shows the clock cycle budgets for small CUs to support real-time decoding. In hardware designs, the clock cycle budgets are very tight to support real-time 8K 60fps or 120fps decoding for merge modes. There are five spatial merge candidates and one temporal merge candidate. Before one new candidate is inserted into the list, pruning (i.e., comparing the new candidate with the candidates in the list to avoid redundancy) is performed to decide whether to insert. Moreover, merging candidate insertion of history candidates needs sequential pruning operation between the history entries and the original merging candidates. The sequential pruning of history candidates enlarges the cycle requirement. Moreover, several average-based candidates needs to be generated after the history candidate insertion, where the averaging operations for generating average candidates makes the cycle requirement even more demanding. Real-time hardware decoding for VVC merge mode becomes much more challenging compared to HEVC merge mode.

In addition to the translational merge mode, the subblock-based merge mode including affine merge and subblock-based temporal motion vector prediction (SbTMVP) merge has the cycle budget problem also. SbTMVP is a new merge mode that is much more complex than the traditional merge modes. The affine merge needs many sequential operations, such as availability checking for neighbouring affine CUs or corner motion vectors (MVs), reference index comparing for constructed affine candidate, neighbouring MVs fetching for inherited/constructed affine candidate, and control point MV transformation. Because of the combination of SbTMVP and affine merge into one subblock merging candidate list, the decision (when decoding) of which affine merge candidate to be constructed must wait for SbTMVP availability checking, which is also sequential operation and cycle-consuming. Moreover, since in worst case 8x8 CU is allowed for subblock-based merge mode, the real-time hardware decoding is even more challenging.

Table 1. Clock cycle budgets for one CU

| Assuming Clock @ 1GHz | 8K 60fps decoder | 8K 120fps decoder |
| --- | --- | --- |
| 4x4 CU | 8 | 4 |
| 4x8 or 8x4 CU | 16 | 8 |
| 8x8 CU | 32 | 16 |
| Assuming Clock @ 500MHz | 8K 60fps decoder | 8K 120fps decoder |
| 4x4 CU | 4 | 2 |
| 4x8 or 8x4 CU | 8 | 4 |
| 8x8 CU | 16 | 8 |

# Proposed method

In this CE, it is proposed to share the same merging candidate list for all leaf CUs of one ancestor node in the CU split tree for enabling parallel processing of small skip/merge-coded CUs. The ancestor node is named merge sharing node. The shared merging candidate list is generated at the merge sharing node pretending the merge sharing node is a leaf CU.

Figure 1 shows four examples of merge sharing nodes. The shared merging candidate list is generated for the dotted virtual CU (i.e., merge sharing node).

Figure 1. Examples of merge sharing nodes

In the proposed method, there are 2 types of size threshold definitions, which are denoted as Type-1 and Type-2 definitions. For Type-1 definition, the merge sharing node will be decided for each CU inside a CTU during parsing stage of decoding; moreover, the merge sharing node is the largest ancestor node among all the ancestor nodes of the leaf CUs satisfying the following two criteria.

- The merge sharing node size is equal to or smaller than the size threshold

- No samples of the merge sharing node are outside the picture boundary.

Therefore, in cases that the merge sharing mode is fully inside the current picture, the parent node of the merge sharing node is larger than the predefined size threshold.

For Type-2 definition, the merge sharing node will be decided for each CU inside a CTU during parsing stage of decoding; moreover, the merge sharing node is an ancestor node of leaf CU which must satisfy the following 2 criteria:

- The merge sharing node size is equal to or larger than the size threshold

- In the merge sharing node, one of the child CU size is smaller than the size threshold

Moreover, no samples of the merge sharing node are outside the picture boundary has to be guaranteed. During parsing stage, if an ancestor node satisfies the criteria (1) and (2) but has some samples outside the picture boundary, this ancestor node will not be the merge sharing node and it proceeds to find the merge sharing node for its child CUs.

Figure 2 shows an example for the difference of Type-1 and Type-2 definition. In this example, the parent node is ternary-split into 3 child CUs. The size of parent node is 128. For Type-1 definition, the 3 child-CUs will be merge sharing nodes separately. But for Type-2 definition, the parent node is the merge sharing node.

Figure 2. One example of difference of Type-1 and Type-2 definitions

The proposed shared merging candidate list algorithm supports translational merge (including merge mode and triangle merge mode, history-based candidate is also supported) and subblock-based merge mode. For all kinds of merge mode, the behavior of shared merging candidate list algorithm looks basically the same, and it just generates candidates at the merge sharing node pretending the merge sharing node is a leaf CU. It has 2 major benefits. The first benefit is to enable parallel processing for merge mode, and the second benefit is to share all computations of all leaf CUs into the merge sharing node. Therefore, it significantly reduces the hardware cost of all merge modes for hardware codec. By the proposed shared merging candidate list algorithm, the encoder and decoder can easily support parallel encoding for merge mode and it relieves the cycle budget problem of merge mode.

Besides, it is proposed to add new syntax elements to sequence parameter set (SPS) and picture parameter set (PPS). For SPS, syntax elements are designed as shown in Table 2. The sps_shared_mrg_enabled_flag is the high-level switch for shared merge mode in SPS. The sps_log2_shared_mrg_thd_minus6 is the log-2 value of threshold and then minus 6. (e.g. if threshold = 1024, sps_log2_shared_mrg_thd_minus6 = 10-6 = 4). That is, the minimum threshold is 64. An overriding flag sps_shared_mrg_override_enabled_flag is proposed to enable overriding behavior at picture level.

Table 2. SPS syntax design of shared merge mode

sps_shared_mrg_enabled_flag

if( sps_shared_mrg_enabled_flag ) {

sps_log2_shared_mrg_thd_minus6

sps_shared_mrg_override_enabled_flag

}

For PPS, syntax elements are designed as shown in Table 3. The overriding behavior is proposed. If PPS overriding flag pps_shared_mrg_override_enabled_flag is signalled and set to 1, the PPS control syntax elements pps_shared_mrg_enabled_flag and pps_log2_shared_mrg_thd_minus6 will override the enable flag and threshold for the current picture values over the SPS setting values. If PPS overriding flag is not used or set to 0, the setting of the current picture will follow the SPS setting.

Table 3. PPS syntax design of shared merge mode

if( sps_shared_mrg_override_enabled_flag && sps_shared_mrg_enabled_flag ) {

pps_shared_mrg_override_enabled_flag

if( pps_shared_mrg_override_enabled_flag ) {

pps_shared_mrg_enabled_flag

pps_log2_shared_mrg_thd_minus6

}

}

# Simulation results

The proposed method is integrated on VTM3.0. All tests were conducted under the common test condition [1]. There are five CE sub-items as shown in Table 4.

Table 4. Sub-items of CE4.3.1

| Test # | Description |
| --- | --- |
| 4.3.1.a | Share merging candidate list for translational merge and subblock-based merge with threshold = 32 |
| 4.3.1.b | Share merging candidate list for translational merge and subblock-based merge with threshold = 64 |
| 4.3.1.c | Share merging candidate list for translational merge and subblock-based merge with threshold = 128 |
| 4.3.1.d | Share merging candidate list for translational merge and subblock-based merge with threshold = 256 |
| 4.3.1.e | Share merging candidate list for translational merge and subblock-based merge with threshold = 512 |

The results of test 4.3.1.a, 4.3.1.b, 4.3.1.c, 4.3.1.d, and 4.3.1.e are shown in Table 5, Table 6, Table 7, Table 8, and Table 9, respectively. Moreover, the tests are based on Type-1 threshold definition.

Moreover, for this CE, we have other supplementary results. The first group of supplementary results are using shared merging candidate list for subblock-based merge mode only based on Type-1 threshold definition. There are 3 supplementary tests:
Supplementary test A: Shared subblock-based merge mode with threshold = 512
Supplementary test B: Shared subblock-based merge mode with threshold = 1024
Supplementary test C: Shared subblock-based merge mode with threshold = 2048
The results of test A, test B, and test C are shown in Table 10, Table 11, and Table 12, respectively.

The second group of supplementary results are tests all based on Type-2 threshold definition. There are 8 supplementary tests:
Supplementary test D.1: Shared merging candidate list for translational merge and subblock-based merge with threshold = 32

Supplementary test D.2: Share merging candidate list for translational merge and subblock-based merge with threshold = 64

Supplementary test D.3: Share merging candidate list for translational merge and subblock-based merge with threshold = 128

Supplementary test D.4: Share merging candidate list for translational merge and subblock-based merge with threshold = 256

Supplementary test D.5: Share merging candidate list for translational merge and subblock-based merge with threshold = 512

Supplementary test D.6: Shared subblock-based merge mode with threshold = 512

Supplementary test D.7: Shared subblock-based merge mode with threshold = 1024

Supplementary test D.8: Shared subblock-based merge mode with threshold = 2048

The results of test D.1, test D.2, test D.3, test D.4, test D.5, test D.6, test D.7, and test D.8 are shown in Table 13, Table 14, Table 15, Table 16, Table 17, Table 18, Table 19, and Table 20, respectively.

Table 5. Test results of CE 4.3.1.a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | 0.04% | 101% | 103% |
| Class A2 | 0.01% | 0.03% | 0.04% | 101% | 101% |
| Class B | 0.01% | -0.07% | -0.08% | 101% | 102% |
| Class C | 0.00% | 0.05% | 0.04% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class D | 0.02% | 0.02% | 0.22% | 102% | 102% |
| Class F (optional) | -0.02% | -0.06% | -0.03% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.13% | 0.10% | 101% | 103% |
| Class C | 0.04% | 0.06% | -0.01% | 102% | 103% |
| Class E | 0.01% | -0.08% | -0.31% | 101% | 102% |
| Overall | 0.02% | -0.05% | -0.04% | 101% | 102% |
| Class D | 0.03% | -0.19% | 0.15% | 101% | 104% |
| Class F (optional) | 0.06% | -0.06% | 0.29% | 101% | 101% |

Table 6. Test results of CE 4.3.1.b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.08% | -0.01% | 100% | 104% |
| Class A2 | 0.02% | 0.07% | 0.13% | 100% | 104% |
| Class B | 0.03% | 0.04% | -0.03% | 100% | 104% |
| Class C | 0.11% | 0.05% | 0.15% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.02% | 0.05% | 101% | 104% |
| Class D | 0.16% | 0.02% | 0.27% | 101% | 102% |
| Class F (optional) | 0.08% | 0.09% | 0.10% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.22% | 0.04% | 101% | 102% |
| Class C | 0.12% | 0.27% | 0.28% | 102% | 103% |
| Class E | 0.08% | -0.17% | 0.18% | 101% | 101% |
| Overall | 0.08% | -0.04% | 0.15% | 101% | 102% |
| Class D | 0.19% | 0.49% | 0.57% | 102% | 104% |
| Class F (optional) | 0.13% | 0.21% | 0.11% | 101% | 101% |

Table 7. Test results of CE 4.3.1.c

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.08% | -0.03% | 101% | 102% |
| Class A2 | 0.05% | 0.18% | 0.23% | 101% | 100% |
| Class B | 0.07% | 0.20% | 0.04% | 101% | 102% |
| Class C | 0.31% | 0.35% | 0.37% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.18% | 0.16% | 101% | 102% |
| Class D | 0.40% | 0.42% | 0.49% | 102% | 102% |
| Class F (optional) | 0.17% | 0.18% | 0.26% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.07% | 0.25% | 102% | 102% |
| Class C | 0.32% | 0.55% | 0.69% | 103% | 103% |
| Class E | 0.01% | 0.26% | -0.20% | 101% | 103% |
| Overall | 0.14% | 0.22% | 0.28% | 102% | 102% |
| Class D | 0.48% | 0.30% | 0.38% | 102% | 104% |
| Class F (optional) | 0.51% | 0.82% | 0.14% | 102% | 103% |

Table 8. Test results of CE 4.3.1.d

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.03% | 0.25% | 101% | 105% |
| Class A2 | 0.27% | 0.35% | 0.56% | 102% | 103% |
| Class B | 0.30% | 0.45% | 0.45% | 102% | 103% |
| Class C | 0.99% | 1.09% | 1.31% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.44% | 0.52% | 0.66% | 102% | 103% |
| Class D | 1.27% | 1.29% | 1.31% | 103% | 101% |
| Class F (optional) | 0.54% | 0.61% | 0.65% | 102% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.40% | 0.33% | 0.69% | 103% | 102% |
| Class C | 1.13% | 1.22% | 1.34% | 105% | 102% |
| Class E | 0.33% | 0.53% | -0.17% | 102% | 102% |
| Overall | 0.63% | 0.68% | 0.69% | 103% | 102% |
| Class D | 1.59% | 1.51% | 1.67% | 104% | 102% |
| Class F (optional) | 1.15% | 1.14% | 1.27% | 103% | 101% |

Table 9. Test results of CE 4.3.1.e

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.21% | 0.34% | 101% | 105% |
| Class A2 | 0.49% | 0.80% | 0.84% | 102% | 101% |
| Class B | 0.55% | 0.73% | 0.67% | 102% | 102% |
| Class C | 1.37% | 1.53% | 1.73% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.68% | 0.85% | 0.92% | 102% | 102% |
| Class D | 1.70% | 1.60% | 1.96% | 103% | 102% |
| Class F (optional) | 0.78% | 0.83% | 0.92% | 102% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.63% | 0.31% | 0.83% | 103% | 101% |
| Class C | 1.59% | 1.80% | 1.89% | 104% | 102% |
| Class E | 0.55% | 0.51% | 0.44% | 102% | 102% |
| Overall | 0.93% | 0.86% | 1.09% | 103% | 101% |
| Class D | 1.91% | 1.98% | 2.03% | 104% | 103% |
| Class F (optional) | 1.42% | 1.44% | 1.51% | 104% | 104% |

Table 10. Test results of supplementary test A

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | -0.08% | 102% | 101% |
| Class A2 | 0.01% | -0.06% | 0.01% | 101% | 100% |
| Class B | 0.00% | 0.02% | -0.05% | 102% | 101% |
| Class C | 0.06% | 0.00% | 0.03% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.02% | -0.02% | 102% | 101% |
| Class D | 0.06% | -0.03% | -0.09% | 102% | 107% |
| Class F (optional) | 0.03% | -0.01% | 0.03% | 102% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.30% | 0.11% | 102% | 100% |
| Class C | 0.12% | 0.27% | 0.20% | 103% | 101% |
| Class E | 0.16% | 0.08% | 0.16% | 103% | 102% |
| Overall | 0.08% | -0.02% | 0.15% | 103% | 101% |
| Class D | 0.20% | 0.28% | 0.30% | 103% | 102% |
| Class F (optional) | -0.03% | -0.34% | -0.10% | 102% | 100% |

Table 11. Test results of supplementary test B

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.04% | 0.00% | 102% | 103% |
| Class A2 | 0.05% | 0.04% | 0.15% | 102% | 102% |
| Class B | 0.07% | 0.10% | 0.06% | 101% | 105% |
| Class C | 0.16% | 0.06% | 0.08% | 102% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.06% | 0.07% | 102% | 104% |
| Class D | 0.19% | -0.09% | 0.08% | 101% | 104% |
| Class F (optional) | 0.05% | 0.03% | 0.02% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.17% | 0.20% | 102% | 100% |
| Class C | 0.17% | 0.21% | 0.13% | 102% | 102% |
| Class E | 0.13% | 0.43% | 0.31% | 102% | 100% |
| Overall | 0.12% | 0.11% | 0.21% | 102% | 101% |
| Class D | 0.20% | -0.03% | -0.22% | 101% | 104% |
| Class F (optional) | -0.01% | 0.26% | -0.03% | 102% | 100% |

Table 12. Test results of supplementary test C

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.02% | 0.07% | 102% | 100% |
| Class A2 | 0.12% | 0.00% | 0.16% | 101% | 98% |
| Class B | 0.11% | 0.08% | 0.06% | 101% | 103% |
| Class C | 0.19% | 0.10% | 0.14% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.05% | 0.10% | 101% | 101% |
| Class D | 0.19% | 0.04% | 0.10% | 102% | 105% |
| Class F (optional) | 0.07% | 0.06% | 0.10% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | -0.34% | 0.02% | 102% | 102% |
| Class C | 0.17% | 0.29% | 0.24% | 101% | 101% |
| Class E | 0.21% | -0.11% | 0.11% | 104% | 101% |
| Overall | 0.16% | -0.07% | 0.12% | 102% | 101% |
| Class D | 0.36% | 0.29% | 0.08% | 102% | 103% |
| Class F (optional) | 0.06% | 0.03% | 0.27% | 102% | 105% |

Table 13. Test results of supplementary test D.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.08% | 100% | 98% |
| Class A2 | -0.02% | 0.14% | 0.08% | 100% | 98% |
| Class B | -0.01% | 0.07% | -0.06% | 100% | 101% |
| Class C | 0.03% | -0.01% | 0.05% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | 0.03% | 100% | 100% |
| Class D | 0.05% | 0.02% | -0.01% | 100% | 101% |
| Class F (optional) | 0.02% | -0.05% | 0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.09% | 0.13% | 100% | 100% |
| Class C | 0.05% | 0.16% | 0.27% | 101% | 101% |
| Class E | 0.03% | 0.34% | 0.17% | 100% | 100% |
| Overall | 0.04% | 0.10% | 0.18% | 100% | 101% |
| Class D | 0.05% | -0.24% | 0.07% | 101% | 102% |
| Class F (optional) | 0.07% | -0.10% | 0.20% | 101% | 101% |

Table 14. Test results of supplementary test D.2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | -0.01% | 101% | 98% |
| Class A2 | 0.02% | 0.05% | 0.13% | 101% | 98% |
| Class B | 0.05% | 0.13% | -0.04% | 101% | 101% |
| Class C | 0.18% | 0.24% | 0.25% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.11% | 0.08% | 101% | 100% |
| Class D | 0.25% | 0.31% | 0.25% | 101% | 102% |
| Class F (optional) | 0.11% | 0.08% | 0.07% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.13% | 0.08% | 101% | 101% |
| Class C | 0.21% | 0.55% | 0.32% | 101% | 101% |
| Class E | 0.11% | 0.29% | -0.28% | 101% | 101% |
| Overall | 0.13% | 0.20% | 0.07% | 101% | 101% |
| Class D | 0.29% | 0.48% | 0.74% | 102% | 102% |
| Class F (optional) | 0.33% | 0.85% | 0.53% | 101% | 100% |

Table 15. Test results of supplementary test D.3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.07% | 0.10% | 101% | 100% |
| Class A2 | 0.11% | 0.21% | 0.36% | 102% | 98% |
| Class B | 0.14% | 0.30% | 0.27% | 101% | 101% |
| Class C | 0.58% | 0.70% | 0.83% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.23% | 0.34% | 0.40% | 101% | 100% |
| Class D | 0.77% | 0.71% | 0.82% | 103% | 102% |
| Class F (optional) | 0.36% | 0.41% | 0.46% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.17% | -0.03% | 0.32% | 102% | 102% |
| Class C | 0.68% | 0.89% | 0.87% | 104% | 101% |
| Class E | 0.12% | 0.64% | 0.31% | 103% | 104% |
| Overall | 0.33% | 0.44% | 0.50% | 103% | 102% |
| Class D | 0.93% | 1.45% | 0.97% | 107% | 103% |
| Class F (optional) | 0.79% | 1.27% | 1.68% | 102% | 100% |

Table 16. Test results of supplementary test D.4

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.30% | 0.24% | 101% | 98% |
| Class A2 | 0.34% | 0.51% | 0.69% | 102% | 97% |
| Class B | 0.41% | 0.61% | 0.53% | 102% | 100% |
| Class C | 1.16% | 1.25% | 1.51% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.54% | 0.70% | 0.77% | 102% | 99% |
| Class D | 1.47% | 1.47% | 1.64% | 103% | 101% |
| Class F (optional) | 0.64% | 0.74% | 0.78% | 102% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.51% | 0.22% | 0.76% | 103% | 100% |
| Class C | 1.41% | 1.32% | 1.71% | 104% | 103% |
| Class E | 0.34% | 0.57% | 0.32% | 103% | 104% |
| Overall | 0.77% | 0.67% | 0.96% | 103% | 102% |
| Class D | 1.80% | 1.89% | 1.58% | 104% | 102% |
| Class F (optional) | 1.17% | 1.10% | 1.16% | 103% | 100% |

Table 17. Test results of supplementary test D.5

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.34% | 0.48% | 0.55% | 101% | 100% |
| Class A2 | 0.86% | 1.20% | 1.35% | 103% | 98% |
| Class B | 0.92% | 1.13% | 1.06% | 102% | 100% |
| Class C | 2.23% | 2.38% | 2.57% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | 1.14% | 1.35% | 1.42% | 103% | 100% |
| Class D | 2.59% | 2.64% | 2.88% | 106% | 99% |
| Class F (optional) | 1.18% | 1.32% | 1.43% | 103% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.04% | 0.71% | 0.97% | 104% | 101% |
| Class C | 2.55% | 2.58% | 3.00% | 107% | 100% |
| Class E | 0.93% | 1.01% | 0.72% | 102% | 101% |
| Overall | 1.52% | 1.41% | 1.59% | 105% | 101% |
| Class D | 3.09% | 3.08% | 3.57% | 108% | 101% |
| Class F (optional) | 2.25% | 2.31% | 2.48% | 105% | 100% |

Table 18. Test results of supplementary test D.6

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.08% | 0.02% | 101% | 99% |
| Class A2 | 0.02% | 0.00% | 0.09% | 101% | 99% |
| Class B | 0.03% | 0.05% | 0.01% | 101% | 101% |
| Class C | 0.10% | 0.11% | 0.11% | 104% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.03% | 0.05% | 102% | 100% |
| Class D | 0.12% | 0.04% | 0.04% | 101% | 100% |
| Class F (optional) | 0.05% | 0.04% | 0.00% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.30% | -0.03% | 102% | 100% |
| Class C | 0.13% | 0.25% | 0.13% | 103% | 100% |
| Class E | 0.06% | 0.03% | -0.09% | 101% | 101% |
| Overall | 0.07% | -0.03% | 0.01% | 102% | 100% |
| Class D | 0.20% | 0.44% | 0.50% | 103% | 101% |
| Class F (optional) | 0.06% | 0.15% | -0.16% | 102% | 100% |

Table 19. Test results of supplementary test D.7

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.05% | 0.02% | 102% | 97% |
| Class A2 | 0.08% | 0.14% | 0.12% | 101% | 98% |
| Class B | 0.06% | 0.03% | -0.01% | 101% | 99% |
| Class C | 0.15% | 0.05% | 0.15% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.04% | 0.06% | 101% | 99% |
| Class D | 0.17% | 0.10% | 0.18% | 101% | 100% |
| Class F (optional) | 0.07% | 0.05% | 0.05% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.09% | 0.01% | 102% | 101% |
| Class C | 0.16% | 0.19% | 0.25% | 101% | 101% |
| Class E | 0.18% | 0.56% | 0.31% | 103% | 101% |
| Overall | 0.13% | 0.17% | 0.17% | 102% | 101% |
| Class D | 0.31% | 0.05% | -0.29% | 101% | 102% |
| Class F (optional) | 0.05% | 0.01% | 0.10% | 102% | 100% |

Table 20. Test results of supplementary test D.8

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.11% | 0.08% | 101% | 99% |
| Class A2 | 0.19% | 0.17% | 0.31% | 101% | 98% |
| Class B | 0.15% | 0.19% | 0.15% | 101% | 100% |
| Class C | 0.23% | 0.24% | 0.26% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | 0.18% | 0.19% | 101% | 100% |
| Class D | 0.26% | 0.16% | 0.20% | 101% | 101% |
| Class F (optional) | 0.16% | 0.10% | 0.16% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | -0.15% | 0.00% | 102% | 99% |
| Class C | 0.25% | 0.26% | 0.29% | 101% | 101% |
| Class E | 0.35% | 0.47% | 0.19% | 102% | 101% |
| Overall | 0.23% | 0.14% | 0.15% | 102% | 100% |
| Class D | 0.40% | 0.06% | 0.35% | 101% | 100% |
| Class F (optional) | 0.13% | 0.06% | -0.17% | 101% | 101% |

# Conclusions

In this contribution, it is proposed to share the same merging candidate list for all leaf CUs of one ancestor node in the CU split tree. It is shown that the proposed concept can facilitate parallel processing of merge mode and effectively reduce hardware cost for merge mode with minor coding efficiency penalty.

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).