| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0174-v2 |
| --- | --- |

| Title: | CE8-related: Removal of subblock-based chroma MC in CPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Tzu-Der Chuang, Yu-Ling Hsiao, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, peter.chuang, yuling.hsiao, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modification on current picture referencing (CPR). In the current CPR, when dual tree coding is enabled, a chroma coding unit (CU) selecting CPR inter mode has to check all collocated luma subblock motion vectors (MVs), scale the collocated luma subblock MVs for chroma, and perform subblock-based chroma motion compensation (MC) if all the collocated luma subblock MVs are available and valid. To simplify the chroma CPR inter CUs in dual tree coding, it is proposed to use only the center collocated luma subblock MV to replace subblock-based chroma MC with CU-based chroma MC. Two default MVs can be checked when the center collocated luma subblock MV is unavailable or invalid. Simulation results are as follows, where the anchor enabled CPR.

For common test condition (CTC) sequences without class F
VTM3.0-AI: {Y/U/V-BD-rate = 0.00% / -0.04% / -0.06%, EncT=100%, DecT=99%}
VTM3.0-RA: {Y/U/V-BD-rate = 0.01%/ -0.07% / -0.04%, EncT=100%, DecT=99%}

For class F sequences
VTM3.0-AI: {Y/U/V-BD-rate = -0.04% / -0.25% / -0.14%, EncT=101%, DecT=100%}
VTM3.0-RA: {Y/U/V-BD-rate = -0.04% / -0.21% / -0.07%, EncT=100%, DecT=101%}

For TGM sequences
VTM3.0-AI: {Y/U/V-BD-rate = -0.27% / -0.47% / -0.55%, EncT=101%, DecT=101%}
VTM3.0-RA: {Y/U/V-BD-rate = -0.06% / -0.25% / -0.25%, EncT=100%, DecT=99%}

# Introduction

CPR is adopted in JVET-L meeting and is disabled in the current CTC. A constraint limiting reference samples within the current coding tree unit (CTU) is used. CPR can be supported by dual tree coding.

In the current design of CPR, for luma component of a CU, CPR inter mode and CPR merge mode are tested at the encoder. For chroma components of a CU in dual tree coding, if all collocated luma subblock MVs are available and valid, chroma CPR inter mode is allowed; otherwise, chroma CPR inter mode of the CU will be skipped at the encoder. As described above, subblock-based luma MV checks are required for chroma CPR inter CUs in dual tree coding. Besides, MC for chroma CPR inter CU needs to apply scaling to collocated luma MVs to generate chroma MVs and chroma prediction samples. The subblock-based chroma MC is complex.

# Proposed method

It is proposed to use the center collocated luma subblock MV for the entire chroma CPR inter CU. If the center collocated luma subblock MV is unavailable or invalid, a default MV may be used. The first default MV is [-CuWidth, 0]. If the first default MV needs to reference any sample outside the current CTU, it is invalid, and a second default MV [0, -CuHeight] is used. If the second default MV is also invalid, CPR inter mode is not allowed for the chroma CU. In the proposed method, the subblock-based MC in chroma CPR inter CU is replaced with CU-based MC, so the MC complexity is significantly reduced.

# Experimental results

The proposed methods have been integrated into VTM3.0. All the tests were conducted according to the AI and RA CTCs [1] except that the anchor enables CPR. Additional tests for class F and TGM sequences are also conducted. Table 1 shows the test results.

Table 1.  Simulation results of the proposed method.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.00% | 100% | 98% |
| Class A2 | -0.01% | -0.01% | -0.02% | 101% | 98% |
| Class B | -0.01% | -0.05% | -0.03% | 100% | 101% |
| Class C | 0.00% | -0.12% | -0.13% | 100% | 97% |
| Class E | -0.01% | 0.04% | -0.16% | 100% | 100% |
| Overall | 0.00% | -0.04% | -0.06% | 100% | 99% |
| Class D | 0.00% | 0.08% | -0.16% | 101% | 97% |
| Class F (optional) | -0.04% | -0.25% | -0.14% | 101% | 100% |
| TGM | -0.27% | -0.47% | -0.55% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.08% | 0.00% | 100% | 98% |
| Class A2 | -0.02% | -0.01% | -0.02% | 100% | 100% |
| Class B | 0.01% | -0.13% | 0.01% | 100% | 99% |
| Class C | 0.02% | -0.16% | -0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.07% | -0.04% | 100% | 99% |
| Class D | -0.01% | -0.16% | -0.21% | 101% | 102% |
| Class F (optional) | -0.04% | -0.21% | -0.07% | 100% | 101% |
| TGM | -0.06% | -0.25% | -0.25% | 100% | 99% |

# Conclusions

This contribution proposes to simplify the chroma CPR inter CU in dual tree coding by only using the center collocated luma subblock MV for the entire chroma CU instead of performing the subblock-based chroma MC. Two default MVs can be checked when the center collocated luma subblock MV is unavailable or invalid. It is reported that the simulation results are 0.00%, -0.04%, and -0.27% luma BD-rates for ClassA/B/C, ClassF, and TGM sequences in AI, respectively.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).